3GPP TSG-RAN WG4 Meeting # 113                                                             R4-2417855

Orlando, US, 18th – 22nd November, 2024

Title: 	                 Discussion on A-IoT BS requirements

Source: 	CATT

Agenda item:	7.21.3.1

Document for:	Discussion

- Introduction

This contribution provides our further analysis for A-IoT BS Tx and Rx requirements according to the WF [1].

- A-IoT BS RF requirements analysis

2.1 Tx requirements

We have no conclusion about whether the A-IoT BS supports RE power boosting, so RAN4 hold on discussion on whether or not to specify RE power control dynamic range until RAN1 concludes the RE power boosting for A-IoT.

The signal resources of R2D can be several RBs, so total power dynamic range requirement is necessary.

Proposal 1: RAN4 hold on discussion on whether or not to specify RE power control dynamic range until RAN1 concludes the RE power boosting for A-IoT.

Proposal 2: Total power dynamic range requirement is needed.

In the current BS Tx requirements, transmit OFF power requirement only applies to TDD operation, however, if the frequency bands of R2D and D2R/CW are same and CW node is outside the topology, the requirement is necessary, otherwise, the requirement is not needed.

Proposal 3: Transmitter OFF power requirement is needed when the frequency spectrums of R2D and D2R/CW are same.

For an A-IoT BS communicating with device 1 and device 2a,  the A-IoT BS doesn’t transmit R2D signal all the time and the transmission needs to be turned on and off, and R2D transmission and CW transmission /D2R signal reception operations are TDM, this requirement is needed.

Proposal 4: For A-IoT BS with TDM operation, transmitter transient period requirement is needed.

Traditionally the transmitter intermodulation requirement was created to guarantee emission levels in a co-location scenario. The requirement is based on robustness against injection of a reverse interference signal from a co-located BS. If the A-IoT BS R2D signal use NR BS DL spectrum and the two BSs are also co-located, the transmitter intermodulation requirement is necessary.

Proposal 5: If the A-IoT BS R2D signal use NR BS DL spectrum and the two BSs are also co-located, the transmitter intermodulation requirement is necessary.

2.2 Rx requirements

ICS is a measure of the receiver ability to receive a wanted signal at its assigned resource block locations in the presence of another in-channel wanted signal received at a much larger power spectral density, but device 1 and device 2a are TDM operation, and device 2b are FDM operation, and whether uplink support FDM mode haven't been decided, so the ICS requirement need further study in WI phase.

Proposal 6: ICS requirement need further discussion in WI phase.

- Summary

This contribution provides our further analysis for A-IoT BS requirements. We have the following proposals.

Proposal 1: RAN4 hold on discussion on whether or not to specify RE power control dynamic range until RAN1 concludes the RE power boosting for A-IoT.

Proposal 2: Total power dynamic range requirement is needed.

Proposal 3: Transmitter OFF power requirement is needed when the frequency spectrums of R2D and D2R/CW are same.

Proposal 4: For A-IoT BS with TDM operation, transmitter transient period requirement is needed.

Proposal 5: If the A-IoT BS R2D signal use NR BS DL spectrum and the two BSs are also co-located, the transmitter intermodulation requirement is necessary.

Proposal 6: ICS requirement need further discussion in WI phase.

Reference

[1] R4-2417088, “WF on impacts of A-IoT on RF requirements”, Huawei, RAN4#112bis