3GPP TSG RAN WG4 Meeting #113		R4-2417866

Orlando, USA, Nov 18th – 22nd , 2024

Title: 	Further discussion on Testability for UE RF requirements for LP-WUS

Agenda Item:	7.23.2.4

Source: 	CATT

Document for:	Discussion

# 1 Introduction

In RAN4#112bis, test aspects of UE RF requirements for LP-WUS have not been discussed sufficiently, with the following open issues [1]:

Issue 4-1-1: General framework on LP-WUS testing Way forward: FFS Issue 4-1-2: Separate RF test case for idle and connection mode Way forward: FFS Issue 4-1-3: How to get feedback from LP-WUR during the test Way forward: FFS Issue 4-1-4: detailed Test procedure Way forward: FFS Issue 4-1-5: UE test mode for LP-WUR Agreements: - FFS A UE test mode for LP-WUR is needed. Issue 4-1-6: whether a LS to RAN5 on test issue Way forward: FFS

In this contribution, we present our views on these open issues.

# 2 Discussion

It is understood that LP-WUS must be supported in all three RRC states (i.e., RRC_IDLE, RRC_INACTIVE, and RRC_CONNECTED). The primary goal of introducing LP-WUS/WUR is to allow the main radio (MR) to remain dormant while only the LP-WUR is active, thereby conserving power. Clearly, regardless of the UE’s RRC state, the process for waking the MR when it is dormant and only LP-WUR is active remains consistent. The LP-WUR receives and accurately decodes the wake-up signal intended for the UE. Thus, from a testing perspective, LP-WUR performance is uniform across all three RRC states. That’s the foundation for introducing the same performance requirements for all three RRC states.

Observation 1: The same LP-WUS performance requirements are defined for all three RRC states.

With such consideration, LP-WUS can be tested under only one RRC state for testing effort reduction.

Observation 2: Only one RRC state can be selected for verifying LP-WUS performance.

To reduce testing time, there is a proposal suggesting that the MR does not need to actually wake up during the test. However, implementing this approach may necessitate a dedicated test mode with altered UE behaviours, and potentially even auxiliary tools for accurately measuring metrics like MDR and FAR.

Observation 3: If MR does not actually wake up during tests, a dedicated test mode with different UE behaviours and some auxiliary tool may be required.

Hence, the general framework for LP-WUS testing can be summarized as:

- The same LP-WUS performance requirements for all three RRC states are assumed;

- Only one RRC state needs to be verified;

- MR wakes up during tests, thus no dedicated test mode with different UE behaviours is required.

Proposal 1: RAN4 to consider the following as part of the general framework for LP-WUS testing:

- The same LP-WUS performance requirements for all three RRC states are assumed;

- Only one RRC state needs to be verified;

- MR wakes up during tests, thus no dedicated test mode with different UE behaviours is required.

If the UE is tested in the RRC_CONNECTED state, a dedicated test mode for the UE is unnecessary. Additionally, metrics such as MDR and FAR statistics can be obtained via the MR, making the process more convenient and eliminating the need for auxiliary tools. Therefore, we propose to test LP-WUS performance solely in the RRC_CONNECTED state.

Proposal 2: RAN4 not to consider separate test cases for IDLE and CONNECTED modes, and LP-WUS is tested only in RRC_CONNECTED state.

One potential benefit of only testing in RRC_CONNECTED mode is that NR MR can be utilized to send feedback of LP-WUR during the test, and feedback can be buffered and packed before it is sent to TE.

Proposal 3: RAN4 to consider utilize MR to send feedback of LP-WUR during the test.

Actually, an OTT app may be considered for this purpose, thus no dedicated UE test mode is needed for LP-WUR.

Proposal 4: RAN4 not to consider a dedicated UE test mode for LP-WUR.

At this stage, RAN1 is still focusing on LP-WUS signal designs, and RAN4 needs the complete signal design to finalize defining UE RF requirements for LP-WUR, thus an LS to RAN5 on test aspects are not ready, and RAN4 can wait and collect sufficient inputs and progress before sending an LS to RAN5.

Proposal 5: RAN4 to wait and collect sufficient inputs and progress before sending an LS to RAN5.

# 3 Conclusion

In this contribution, we have the following observations and proposals on testability for UE RF requirements for LP-WUS:

Observation 1: The same LP-WUS performance requirements are defined for all three RRC states.

Observation 2: Only one RRC state can be selected for verifying LP-WUS performance.

Observation 3: If MR does not actually wake up during tests, a dedicated test mode with different UE behaviours and some auxiliary tool may be required.

Proposal 1: RAN4 to consider the following as part of the general framework for LP-WUS testing:

- The same LP-WUS performance requirements for all three RRC states are assumed;

- Only one RRC state needs to be verified;

- MR wakes up during tests, thus no dedicated test mode with different UE behaviours is required.

Proposal 2: RAN4 not to consider separate test cases for IDLE and CONNECTED modes, and LP-WUS is tested only in RRC_CONNECTED state.

Proposal 3: RAN4 to consider utilize MR to send feedback of LP-WUR during the test.

Proposal 4: RAN4 not to consider a dedicated UE test mode for LP-WUR.

Proposal 5: RAN4 to wait and collect sufficient inputs and progress before sending an LS to RAN5.

# Reference

[1] R4-2417112, “WF on UE RF requirements for LP-WUS”, vivo