3GPP TSG RAN WG4 Meeting #113		R4-2417876

Orlando, USA, Nov 18th – 22nd , 2024

Title: 	Further discussion on radio and antenna parameters for ITU-R response for 14.8G-15.35GHz

Agenda Item:	7.2.4.2

Source: 	CATT

Document for:	Discussion

# 1 Introduction

In RAN4#112bis, discussions continued on radio and antenna parameters for ITU-R response for the frequency range 14.8G – 15.35GHz, with two WFs approved for BS and UE parameters respectively [1][2].

In this contribution, we propose values for ACLR and ACS for BS and UE respectively based on our updated simulation results [3], as well as other radio parameters targeted in the third reply LS.

# 2 Discussion

## 2.1 BS parameters

ACLR and ACS for BS are the main left BS parameters:

Open issues Issu 4-13: ACLR Potential agreement: Postpone to next meeting according to the simulation results. Issue 4-16: ACS agreement: Postpone to next meeting according to the simulation results.

As shown in our companion paper [3], the ACIR values are recommended as 24dB and 20dB for downlink and uplink respectively. The tables below show a good candidate for ACLR/ACS for BS and UE respectively according to the simulated ACIR values.

Table – 1, BS ACLR and UE ACS for downlink

| Downlink | ACIR | 24.0 |
| --- | --- | --- |
| BS | ACLR | 31.0 |
| UE | ACS | 25.0 |

Table – 2, UE ACLR and BS ACS for uplink

| Uplink | ACIR | 20.0 |
| --- | --- | --- |
| UE | ACLR | 21.0 |
| BS | ACS | 27.0 |

Proposal 1: Set BS ACLR as 31dB for the frequency range 14.8G – 15.35GHz.

Proposal 2: Set BS ACS as 27dB for the frequency range 14.8G – 15.35GHz.

## 2.2 UE parameters

Regarding ACLR and ACS for UE, the tables above show a good candidate.

Proposal 3: Set UE ACLR as 21dB for the frequency range 14.8G – 15.35GHz.

Proposal 4: Set UE ACS as 25dB for the frequency range 14.8G – 15.35GHz.

In addition, there are also other open issues related to UE parameters [2]:

Power dynamic range - Proposals - Option 1: 56dB for 100MHz assuming 23dBm - Option 2: 56dB for 200MHz assuming 23dBm Emissions mask - Option 1: UE SEM given in Section 7.1.2, TR 38.921. - Option 2: Other options. Spurious emission - FFS if RAN4 to adopt the UE general spurious emissions defined in TS 38.101-1 clause 6.5.3.1 Blocking and spurious response - FFS if RAN4 to consider the blocking characteristic specified in clause 7.6 of TS 38.101-1 for frequency larger than 3300 MHz could be applied.

For power dynamic range, both options are actually 56dB, with different typical channel bandwidths, so it is sufficient to use 56dB in the reply.

Proposal 5: Use 56dB as power dynamic range in the third reply LS.

For emissions mask, considering the third LS will be sent out in this meeting, the previous works captured in TR 38.921 would be a feasible choice.

Proposal 6: For emissions marks, UE SEM given in Section 7.1.2 in TR 38.921 can be reused.

Similar considerations can also apply to spurious emission and blocking and spurious response.

Proposal 7: For spurious emission, RAN4 to adopt the UE general spurious emissions defined in TS 38.101-1 clause 6.5.3.1.

Proposal 8: For blocking and spurious response, RAN4 to consider the blocking characteristic specified in clause 7.6 of TS 38.101-1 for frequency larger than 3300 MHz could be applied.

# 3 Conclusion

In this contribution, we have the following proposals for radio and antenna parameters for ITU-R response for the frequency range 14.8G – 15.35GHz:

Proposal 1: Set BS ACLR as 31dB for the frequency range 14.8G – 15.35GHz.

Proposal 2: Set BS ACS as 27dB for the frequency range 14.8G – 15.35GHz.

Proposal 3: Set UE ACLR as 21dB for the frequency range 14.8G – 15.35GHz.

Proposal 4: Set UE ACS as 25dB for the frequency range 14.8G – 15.35GHz.

Proposal 5: Use 56dB as power dynamic range in the third reply LS.

Proposal 6: For emissions marks, UE SEM given in Section 7.1.2 in TR 38.921 can be reused.

Proposal 7: For spurious emission, RAN4 to adopt the UE general spurious emissions defined in TS 38.101-1 clause 6.5.3.1.

Proposal 8: For blocking and spurious response, RAN4 to consider the blocking characteristic specified in clause 7.6 of TS 38.101-1 for frequency larger than 3300 MHz could be applied.

# Reference

[1] R4-2417107, “WF on 15GHz BS parameters”, ZTE

[2] R4-2417108, “WF on 15GHz UE parameters”, Qualcomm

[3] R4-241xxxx, “Co-existence simulations for IMT parameters for 14.8G-15.35GHz”, CATT