3GPP TSG RAN WG4 Meeting #113		R4-2417879

Orlando, USA, Nov 18th – 22nd , 2024

Title: 	On solution to enable HPUE maximum transmit power in downlink CA with single UL transmission

Agenda Item:	9.2

Source: 	CATT

Document for:	Discussion

# 1 Introduction

For the RAN task concerning the enablement of HPUE maximum transmit power in DL CA with a single UL configuration [1], papers submitted prior to the meeting were not addressed in RAN4#112bis, where sporadic offline discussions happened. As a final attempt at making a way forward was made [2], but similarly, no consensus was reached during RAN4#112bis with the WF noted.

In our understanding, we could primarily focus on resolving several key issues:

(1) How to handle cases where single UL transmission is in a higher power class while the corresponding MSD is missing for the band combination;

(2) How to report the higher power capability for DL CA with single UL configuration.

(3) How network allows for such feature transmitting higher power while the corresponding MSD requirements are not defined.

In this contribution, we further discuss these three issues and present our views.

# 2 Discussion

Issue on the missing MSD

To enable HPUE maximum transmit power in DL-only CA with single UL configuration for a band combination whose MSD for the high power class is not specified yet but the high power class of the said UL band is supported, either it is required to only verify MSD for the default power class, or wait until MSD for the high power class is defined (Option B in the WF[2]).

In our view, nothing comes without a cost. In this case, the trade-off could be a degradation in downlink performance, which the network should be aware of and prepared for, e.g., compensated by more re-transmissions in time domain. Essentially, downlink performance degradation is exchanged for higher uplink power transmission. Under such consideration, the feature could be enabled.

Proposal 1: A UE is allowed for HPUE maximum transmit power in DL-only CA with single UL configuration for a band combination whose MSD for the high power class is not specified yet but the high power class of the said UL band is supported, with awareness by network that the downlink performance may suffer from a degradation and is able to be compensated in other means.

Issue on the capability reporting

If a UE support such a feature, should a new UE capability signaling be introduced or some existing UE capability, e.g., ue-PowerClass in BandNR or ue-PowerClassPerBandPerBC-r17 for the band combination.

If an existing UE capability is reused for this purpose, it could lead to NBC issues. This is because legacy UEs that do not support the feature might share the same existing UE capability, making it impossible for the network to differentiate between the two types of UEs. Therefore, a new UE capability would be necessary if RAN4 agrees to support this feature.

Proposal 2: RAN4 to introduce a new UE capability if agreeing to support this feature.

Issue on network controlling

If a UE indicates to the network that it supports the feature, the network should have the authority to decide whether the UE can activate it. The network may evaluate the potential benefits and drawbacks, considering the current network load, before making this decision. Therefore, a mechanism for the network to enable this feature for a UE that reports its support should be introduced.

Proposal 3: RAN4 to introduce a mechanism for network to enable this feature for a UE that reports its support of the feature.

# 3 Conclusion

In this contribution, we have the following proposals on solution to enable HPUE maximum transmit power in downlink CA with single UL transmisstion:

Proposal 1: A UE is allowed for HPUE maximum transmit power in DL-only CA with single UL configuration for a band combination whose MSD for the high power class is not specified yet but the high power class of the said UL band is supported, with awareness by network that the downlink performance may suffer from a degradation and is able to be compensated in other means.

Proposal 2: RAN4 to introduce a new UE capability if agreeing to support this feature.

Proposal 3: RAN4 to introduce a mechanism for network to enable this feature for a UE that reports its support of the feature.

# Reference

[1] RP-241687, “Moderator's summary for offline discussion on CA Power Class Support”, Moderator(AT&T)

[2] R4-2417203, “Way Forward for RAN task on HPUE with 1UL and DL CA”, Qualcomm.