3GPP TSG-RAN WG4 Meeting # 113	                                                R4-2417894

Orlando, US, Nov. 18 – Nov. 22, 2024

Agenda item:	7.17.4.2

Source:	MediaTek Inc.

Title:	Discussion on testability issues of AI/ML RAN4 BM use case

Document for:	Discussion

# Introduction

In this contribution, we provide our views on testability issues of AI/ML RAN4 BM use case.

# Discussion

## 2.1 Input and output of AI/ML BM models

As analyzed in our contribution R4-2417893, we have the following three observations.

Observation 1: Both DL Tx beam prediction and beam prediction are studied in R18 AI/ML SI, but only DL Tx beam prediction is in the scope of R19 AI/ML WI.

Observation 2: According to the submitted input from different companies in R18 SI, for DL Tx beam prediction, the model inputs are L1-RSRP of Tx beams in Set B with Tx beam ID information as input or implicitly indicated. The outputs are Probability of each beam in set A to be the Top-1 beam or predicted L1-RSRP of beams in Set A.

Observation 3: AoA information is not used at least in all the submitted simulations in R18 SI (R18 AI/ML BM evaluation in RAN1). Rx beam information is not used in DL Tx beam prediction.

      - 2.2 Test setup for AI/ML BM

### 2.2.1 Testing goal

In R18 SI, the following principle was identified: The design of test should ensure performance is guaranteed and avoid that a UE can pass the test but perform poorly in the field (in TR38.843).

In General part, RAN4 discussed the testing goals of this WI and reached the following agreement in RAN4#112.

Issue 1-3: Testing goals Agreement: - The testing goal is to verify whether the minimum performance of AI/ML functionality/feature can be achieved - FFS on whether and how many different test configurations/parameters are used for tests. - LCM would be tested

As analyzed in our contribution R4-2417893, we had done some simulations with different Set B patterns. In the simulation, there are 32 Tx beams with 2 elevation angles and 16 azimuth angles in Set A and 8 Tx beams in Set B. We tried different set B patterns (e.g., in pattern 0, Set B = the first 8 beams in Set A) and got the Top 5/1 prediction accuracy requirements as in Fig. 4. It can be observed that the top 5/1 prediction accuracy of different patterns are varied. The gap is more than 10% between the best and worst.

Fig. 1

Observation 4: In terms of Top K/1 prediction accuracy, the performance of AI/ML BM case 1 models for different scenarios may have much difference.

It is impossible to try all the scenarios in the simulation, we are not sure whether it is possible to define a scenario-agnostic performance requirements based on the above observation. RAN4 once agreed to use synthetic channels in R19 AI/ML test. Now we wonder whether the simulation results under synthetic channels can reflect the performance of AI/ML models in the real field. If not, we think it may be necessary to design a test system that is possible to use field data in the test. The key issue to use field data in the test is how to get the test dataset. We understand that it is not easy to figure this out.

Considering the potential fact that the performance under synthetic channels may not reflect the performance of AI/ML models in the real field and the difficulty to use field data in the test, we think at least for AI/ML BM, RAN4 needs to discuss what the testing goal is, to verify the model can work well in real field or just to test UE can train a model that works well under a specific scenario.

Proposal 1: For AI/ML BM, further discuss whether the performance under synthetic channels can reflect the performance of AI/ML models in the real field.

Proposal 2: For AI/ML BM, discuss what the testing goal is, to verify the model can work well in real field or just to test UE can train a model that works well under a specific scenario.

Proposal 3: If the testing goal is to verify the model can work well in real field, and the performance under a specific channel/scenario cannot reflect the performance in the real field, it is necessary to design a test system that is possible to use field data in the test.

### 2.2.2 Test set up requirements

According to the agreement last meeting, companies are encouraged to analyse the test set up requirements of using CDL channel model or TDL channel.

Issue 2-9: Test setup requirements Agreement: - For CDL or simplified CDL channel model, RAN4 to study the relationship between AoD range [and granularity], number of beams and number of probes. - Set A and B should be generated in the chamber for ground-truth extraction. - For TDL channel model, RAN4 to study the relationship between [dynamic power range], number of beams and number of probes. - Set A and B should be generated in the chamber for ground-truth extraction. - For single AoA, FSS how to derive Set A

In our understanding, the key issue should be the test up requirements for single AoA and multiple AoA test systems but not only for CDL or TDL channel. The channel that matters is that from the BS emulator to UE as shown in Fig.1. No matter single AoA or multiple AoA test system, the channel from the BS emulator to UE can be CDL or TDL channel, but the implementation efforts will be quite different.

Fig. 2

In our understanding, single AoA test system can be applicable to all kinds of synthetic channels as well as using real field data as analysed later in 2.2.2.2. Using single AoA test system, it can be like virtue conductive test. From the point of implementation, we don’t see much different on using CDL channel or TDL channel. The only limitation would be the dynamic power range which will be discussed later.

With multiple AoA test systems, according to the information provided by TE vendors [2-5], the impact on current test system are quite different using CDL channel or TDL channel.

Observation 5: Using single AoA test system in AI/ML BM test, it can be like virtue conductive test. From the point of TE implementation, there is no much different on using CDL channel or TDL channel.

Observation 6: Using multiple AoA test system in AI/ML BM test, emulating CDL channel and TDL channel are on different difficulty levels according to TE vendors’ past analysis.

Since it is also possible to use single AoA test system to emulate CDL channel and TDL channel, some discussion shall be also focus on the comparison of single AoA and multiple AoA test systems.

Proposal 4: Discussion shall be focused on comparison of Single AoA or multiple AoA test systems, but not only CDL or TDL channels.

#### 2.2.2.1 Multiple AoAs test system

In the past few meetings, there were some proposals to further enhance multiple AoAs test system for AI/ML BM test. In the next, we would like to provide our views on that.

As the exact Rx beamforming gain is unknown at TE and the range of Rx antenna gain margin in Rel-15 FR2 test is from -10 to 20dB, RAN4#111 agreed that the ground truth is the approximate as the reported RSRP measurement under the certain SNR on the predicted Tx beam. A possible way to get the “ground truth” is separating the test into two steps. In the first step, TE generates the channel and transmits Tx beams in Set B only. UE reports prediction results. In the next step, TE tries to generate the same channel as in the first step and transmits all the Set A Tx beams. The order of the above steps can also be reversed.

To use multiple AoAs test system, to get the ground truth, i.e., approximate as the reported RSRP measurement under the certain SNR on the predicted Tx beam, TE is supposed to transmit all the Tx beams in set A. And the number of Tx beams that test system should be able to emulate is equal to number of set A Tx beams, that is [64-128].

Proposal 5: If to use multiple AoAs test system for AI/ML BM test, test system should be able to emulate as many Tx beams as in set A, that is at least 64.

In the test, as the measurement results may be 10dB lower or 20dB higher the transmitted RSRP at TE, it is difficult to check whether UE reports the real measurement results of Set A or reports fake values according to the previous predicted results to pass the test. Another issue is how to ensure the channels in these two steps are the same. In RAN4#111, when reaching the agreement of using ideal measured RSRP as the ground truth, the 3rd and 4th FFSs were added for further discussion. In our understanding, even setting the random seeds the same, when to perform measurement with a specific beam is uncertain. It is difficult to make sure the channels are the same at the prediction phase and the phase of measuring all Set A.

RAN4#111 Issue 2-3: Reported measurements and ground truth Agreement: - The ground truth for the predicted RSRP is the ideal measurement of RSRP on the predicted Tx beam - In RAN4, the ground truth is the approximate as the reported RSRP measurement under the certain SNR on the predicted Tx beam - FFS on SNR level to ensure that SNR is high enough for sufficient accuracy of reported RSRP - FFS on impact of multiple-AoA test setup - FFS on the channel condition - FFS on whether the ground truth will be changing or not - Other solutions are not precluded

Observation 7: It is difficult to ensure reference RSRP is measured under similar channel conditions as in prediction phase.

Proposal 6: How to avoid UE cheating in the test, such as reports fake values according to the previous predicted results, shall be further discussed to use the reported RSRP measurement as ground truth.

In our understanding, it is difficult to model the real UE Rx beamforming gain in the test. The Rx beamforming pattern used to generate training dataset would not be the same as the test UE. This mismatch will lead to some uncertainty in the test results and likely also extra requirement margin. In the past meetings, there were some proposals to support UE rotation in the test. We understand the intention, i.e., without UE rotation during the test, Rx beamforming gain for each AoA is basically fixed. But according to the information provided by TE vendors [2-5], it may be difficult and costly to support frequent and flexible UE rotation, i.e., rotation 2o, 5o.

Proposal 7: If to use multiple AoAs test system for AI/ML BM test, UE rotation should be supported. Otherwise, Rx beamforming gain for each AoA will be basically fixed during the test.

Last meeting, TE vendors analysed how to emulate multiple Tx beams and a simplified CDL channel was proposed to reduce the complexity. We wonder whether it is possible for test UE to generate the same simplified CDL channel in the training phase as used in the test. If not, there will be some performance loss due to different channels used during training phase and inference phase. RAN4 shall discuss how to handle this.

Proposal 8: RAN4 should further discuss how to handle potential performance loss if simplified CDL channel is used in AI/ML BM test.

In addition, AI/ML BM models are highly data sensitive. As captured in TR38.843, there is significant generalization performance degradation with unseen various gNB setting (i.e., different gNB antenna array dimensions, and/or DL Tx beam codebook) or unseen various Set B of beam(pairs). AI/ML BM models are quite scenario/configuration specific. The model that works well under defined CDL channel may perform badly in real field. We wonder whether it is enough to use CDL channel in the test. It is very likely that test UE will train a specific model just to pass the test and this model will always have mismatch to the real field. If the test goal is to verify the model can work well in real field, CDL channel is not enough.

If the test is designed by using data collected from real field, the need of multiple AoAs test system is still questionable. As AoA information in real field is simply not available, TE has no idea how to generate those Tx beams (with which AoA). Blindly setting each Tx beam at a random AoA in the test chamber leads to the uncertainty to the RSRP measured by UE in Set B. I.e., each Tx beam will be measured via an unknown Rx beamforming gain. In that case, even the measured RSRP in Set B requires a big accuracy margin, and the margin of the predicted RSRP in Set A perhaps needs to be even larger. Such test cases seem not so effective.

With above discussion, we don’t see the need to emulate AoAs and AoDs of each Tx beam.

Proposal 9: It is not necessary to emulate AoAs and AoDs of each Tx beam as the information is not used in all the submitted simulations in R18 SI.

#### 2.2.2.2 Single AoA test system

As shown in Observation 2, for DL Tx beam prediction, the model inputs are L1-RSRP of Tx beams in Set B with Tx beam ID information as input or implicitly indicated. So it is only the final L1-RSRP value that matters. All the spatial characteristics will be reflected on RSRP. Instead of emulating CDL channels and [64-128] Tx beams through multiple AoAs and AoDs, the spatial-selective and time-varying propagation conditions can be finally reflected on received RSRP at the test UE through adjusting Tx power at TE on each beam at different time. This is also the method which is widely used in legacy RAN4 RRM tests.

Proposal 10: In AI/ML BM test, emulate the spatial-selective and time-varying in propagation conditions through adjusting Tx power at TE on each beam at different time.

Single AoA test system can be applicable to all kinds of synthetic channels as well as using real field data. In the next, we would like to explain that using real field data is also possible with single AoA test system.

As shown in Fig.3, in the data collected from real field, the measured RSRP (at baseband, including Rx beamforming gain) of Tx#1 and Tx#2 are P1 and P2, respectively. Using single AoA test system, with Rx beam lock, the Rx beamforming gain during the test can be fixed, assuming it is G. If the transmit RSRP at TE for Tx#1 and Tx#2 is P1-G and P2-G, then the received RSRP at UE baseband will be P1 for Tx#1 and P2 for Tx#2 which are exactly the same as the measured RSRP in real field. The Rx beamforming gain G can be derived by letting TE transmits a fixed high power P0, G is equal to P0’-P0 where P0’ is the RSRP measured at UE. This can be done multiple times to derive an average G to reduce the impact of noise.

Fig. 3

If to use field data, TE transmission power for each Tx beam in set B can be determined as shown in Fig.4.

Fig. 4

If to use synthetic channels defined in 38.901, BS/UE antenna configuration & radiation pattern defined in 38.901 can be used to determine TE transmission power for each Tx beam in set B (as shown in Fig.4). RAN4 can further discuss which antenna configuration to use during training and inference. Using aligned or different antenna configurations during training and inference phase are both possible. Single AoA test system can be easily extended to different scenarios and settings.

Fig. 5

Observation 8: Single AoA test system can be applicable to all kinds of synthetic channels as well as using real field data. It can be easily extended to different scenarios and settings.

With fixed Rx beamforming gain which is known to the TE, TE can also know the expected RSRP of Tx beams at UE baseband, i.e., transmitted RSRP at TE + fixed Rx beamforming gain.

Observation 9: With single AoA, TE can know the expected RSRP of Tx beams at UE baseband.

With single AoA, existing IFF and enhance IFF test systems can be re-used for AI/ML BM test.

Based on the potential issues to use multiple AoAs test system and the benefits to use single AoA test system, we propose to use single AoA test system for R19 AI/ML BM test.

Proposal 11: Use existing IFF or enhance IFF test systems with single AoA for R19 AI/ML BM test.

#### 2.2.2.3 Dynamic range of Single AoA and Multiple AoA test systems

In early releases, dynamic range of IFF and DFF test systems had been analysed. Since several years have passed, there may be some new progress. But we are not experts, so here we just copy the previous results in TR38.810 here for information.

Table 1

| Test methods | Wanted signal (dBm/SCS) at reference point: upper bound | SNR at baseband (dB): upper bound | SNR at baseband (dB): upper bound – 30dB |
| --- | --- | --- | --- |
| IFF, peak fine beam | -83.5 | 25.1 | -4.9 |
| IFF, peak rough beam | -83.5 | 18.1 | -11.9 |
| IFF, non-peak fine beam | -83 | 13 | -17 |
| IFF, non-peak rough beam | -83 | 6 | -24 |
| DFF, peak fine beam | -83.8 | 24.8 | -5.2 |
| DFF, peak rough beam | -83.8 | 17.8 | -12.2 |
| DFF, non-peak fine beam | -83.3 | 12.7 | -17.3 |
| DFF, non-peak rough beam | -83.3 | 5.7 | -24.3 |

Considering the upper bound of SNR with rough beam are much lower than that with fine beam as shown in Table 1, fine beam should be used in AI/ML BM test to get a larger dynamic power range. Next we will only focus testing using fine beam.

Observation 10: The upper bound of SNR with rough beam are much lower than that with fine beam.

Proposal 12: Fine beam should be used in AI/ML test.

It can be observed that the power level at reference point (before UE Rx beam forming) for different cases are basically the same as it is determined by maximum Tx power at TE.

The upper bound of SNR at UE baseband is determined by several factors. One of the factors is Tx power at TE. If TE can do better now, the upper bound of SNR can be higher.

Regarding the lower bound, in our understanding, it is not quite a limitation. As observed through the simulation results given in our contribution R4-2417893, the week beams should be excluded otherwise the predict accuracy of AI/ML BM model will be very poor after considering the impact of measurement error.  In our simulation, we excluded the beams which is 30dB weaker than the best beam, and the Top 5/1 prediction accuracy is above 90%. Therefore 30dB dynamic range may be enough for the test based on our initial simulations. We can get the corresponding SNR at baseband in the last column. It can be observed that SNR in the last column is above -6dB only when using peak fine beam. It is important to have a large SNR dynamic range, otherwise the measurement error will be large for many beams. If to use multiple AoA test system, it is not possible to use peak fine beam only. With single AoA, it is possible to use peak fine beam. And we propose to use peak fine beam in AI/ML BM test. It can be observed that the upper bound of SNR with non-peak fine beam is 12.1dB lower than that of peak fine beam, which means SNR dynamic range of multiple AoA test system would be at least 12dB smaller than single AoA test system.

Observation 11: Among all the test methods, only using peak fine beam can make sure the lowest SNR above -6dB with 30dB dynamic power range.

Observation 12: Multiple AoA test systems can not use peak fine beam only.

Proposal 13: Using peak fine beam in AI/ML BM test with single AoA test system.

Observation 13: SNR dynamic range of multiple AoA test system would be at least 12dB smaller than single AoA test system.

# Conclusion

In this contribution, we provide our views for AI/ML BM use case. Observations and proposals are summarized as follows.

Observation 1: Both DL Tx beam prediction and beam prediction are studied in R18 AI/ML SI, but only DL Tx beam prediction is in the scope of R19 AI/ML WI.

Observation 2: According to the submitted input from different companies in R18 SI, for DL Tx beam prediction, the model inputs are L1-RSRP of Tx beams in Set B with Tx beam ID information as input or implicitly indicated. The outputs are Probability of each beam in set A to be the Top-1 beam or predicted L1-RSRP of beams in Set A.

Observation 3: AoA information is not used at least in all the submitted simulations in R18 SI (R18 AI/ML BM evaluation in RAN1). Rx beam information is not used in DL Tx beam prediction.

Observation 4: In terms of Top K/1 prediction accuracy, the performance of AI/ML BM case 1 models for different scenarios may have much difference.

Proposal 1: For AI/ML BM, further discuss whether the performance under synthetic channels can reflect the performance of AI/ML models in the real field.

Proposal 2: For AI/ML BM, discuss what the testing goal is, to verify the model can work well in real field or just to test UE can train a model that works well under a specific scenario.

Proposal 3: If the testing goal is to verify the model can work well in real field, and the performance under a specific channel/scenario cannot reflect the performance in the real field, it is necessary to design a test system that is possible to use field data in the test.

Observation 5: Using single AoA test system in AI/ML BM test, it can be like virtue conductive test. From the point of TE implementation, there is no much different on using CDL channel or TDL channel.

Observation 6: Using multiple AoA test system in AI/ML BM test, emulating CDL channel and TDL channel are on different difficulty levels according to TE vendors’ past analysis.

Since it is also possible to use single AoA test system to emulate CDL channel and TDL channel, some discussion shall be also focus on the comparison of single AoA and multiple AoA test systems.

Proposal 4: Discussion shall be focused on comparison of Single AoA or multiple AoA test systems, but not only CDL or TDL channels.

Proposal 5: If to use multiple AoAs test system for AI/ML BM test, test system should be able to emulate as many Tx beams as in set A, that is at least 64.

Observation 7: It is difficult to ensure reference RSRP is measured under similar channel conditions as in prediction phase.

Proposal 6: How to avoid UE cheating in the test, such as reports fake values according to the previous predicted results, shall be further discussed to use the reported RSRP measurement as ground truth.

Proposal 7: If to use multiple AoAs test system for AI/ML BM test, UE rotation should be supported. Otherwise, Rx beamforming gain for each AoA will be basically fixed during the test.

Proposal 8: RAN4 should further discuss how to handle potential performance loss if simplified CDL channel is used in AI/ML BM test.

Proposal 9: It is not necessary to emulate AoAs and AoDs of each Tx beam as the information is not used in all the submitted simulations in R18 SI.

Proposal 10: In AI/ML BM test, emulate the spatial-selective and time-varying in propagation conditions through adjusting Tx power at TE on each beam at different time.

Observation 8: Single AoA test system can be applicable to all kinds of synthetic channels as well as using real field data. It can be easily extended to different scenarios and settings.

Observation 9: With single AoA, TE can know the expected RSRP of Tx beams at UE baseband.

Proposal 11: Use existing IFF or enhance IFF test systems with single AoA for R19 AI/ML BM test.

Observation 10: The upper bound of SNR with rough beam are much lower than that with fine beam.

Proposal 12: Fine beam should be used in AI/ML test.

Observation 11: Among all the test methods, only using peak fine beam can make sure the lowest SNR above -6dB with 30dB dynamic power range.

Observation 12: Multiple AoA test systems cannot use peak fine beam only.

Proposal 13: Use peak fine beam in AI/ML BM test with single AoA test system.

Observation 13: SNR dynamic range of multiple AoA test system would be at least 12dB smaller than single AoA test system.

# Reference

- 3GPP TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

- R4-2411177, Testability and interoperability issues for beam management, Korea Testing Laboratory, 3GPP TSG-RAN4 # 112, Aug. 2024.

- R4-2411254, OTA Test System/Testability Considerations for FR2 AI/ML Beam Management Use Cases, Keysight Technologies, 3GPP TSG-RAN4 # 112, Aug. 2024.

- R4-2411279, Study on CDL channel model for AI/ML beam management in FR2, Anritsu Corporation, 3GPP TSG-RAN4 # 112, Aug. 2024.

- R4-2411587, Discussion on test setup for AI/ML based beam management, Rohde & Schwarz, 3GPP TSG-RAN4 # 112, Aug. 2024.