3GPP TSG-RAN WG4 Meeting # 113	                                       R4-2417897

Orlando, US, Nov. 18 – Nov. 22, 2024

Agenda Item: 5.23.1

Source: MediaTek Inc.

Title: Discussion on R18 mobility core requirements

Document for: Discussion

# Introduction

Although R18 mobility has been closed, there are some further optimization proposals which are still open. In addition, after further checking, we found some missed points in the past discussions. We will discuss the related issues in this contribution.

# Discussion

## 2.1 Early TCI state activation

Regarding core part requirements of early TCI state activation, there is still an “FFS” as shown below. In addition, after reviewing the requirements, we found a contradiction in the requirements. In the next, we would like to further discuss the FFS and resolve the contradiction we found.

### 8.25.3 SSB based TCI state activation delay In FR1, when the UE receives LTM candidate cell TCI state activation command at slot n, the UE shall have completed the LTM TCI state list update in slot n + THARQ +3Nslotsubframe,µ + [ TOk*( Tfirst-SSB + TSSB-proc) / NR slot length]. In FR2, when all the target TCI states in the active TCI state list are known, when the UE receives TCI state activation command at slot n, UE shall have completed the LTM TCI state list update in slot n + THARQ +3Nslotsubframe,µ + [ TOk*( Tfirst-SSB + TSSB-proc) / NR slot length]. Where: - THARQ is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3]. - TOk = 1 if the target TCI state is not in the serving cell active TCI state list or LTM candidate cell active TCI state list, 0 otherwise. - Tfirst-SSB is the time to first SSB occasion after slot n + THARQ +3Nslotsubframe,µ when the target SSB is within active BWP, upon receiving PDSCH carrying MAC-CE activation command in slot n; - Tfirst-SSB is the time to first SSB occasion overlapping with MGL after slot n + THARQ +3Nslotsubframe,µ + 2ms if the SSB needs to be measured with MG, upon receiving PDSCH carrying MAC-CE activation command in slot n; - FFS: this is only applicable to UE supporting inter-frequency L1 measurement with MG. - TSSB-proc = 2 ms.

In our understanding, two cases are considered when defining the requirements, T/F tracking without gap and T/F tracking with gap. As highlighted above, it seems there are two different logics in defining the requirements. One is to check whether target SSB is within active BWP or not (grey highlighted part above). Another is to check whether MG is needed to perform measurement (yellow highlighted part above). They are not complementary to each other. For an example, SSB within active BWP may still need to be measured within gap if all SSB occasions are fully overlapped with gap. Another example is that for UE not supporting inter-frequency without gap, it is not clear which one to follow when the target SSB is inter-frequency but within active BWP. Therefore, Clarifications are needed.

In our view, RAN4 already figured out a framework since Rel-15 on which to be measured within gap or outside gap. We should re-use the conclusions as much as possible, i.e., check whether the measurement is done within or outside MG. Here we only define the requirements on the first T/F tracking. After TCI state activation, UE is supposed to do periodic or several shots of T/F tracking within or outside MG following measurement. We prefer to have aligned requirements and UE behaviour.

Observation 1: In early TCI state activation requirements, it is not precise to capture the requirement based on whether SSB is within the BWP or whether the SSB is to be measured with MG.

Proposal 1: In early TCI state activation requirements, differentiate the cases based on whether the SSB is to be measured within or outside gap.

Regarding the “FFS”, we think the proponent’s concern is what if inter-frequency tracking is not supported. It seems reasonable to discuss the applicable condition, but the current proposed condition is not acceptable to us. In RAN4#112 meeting, given the RAN2 agreement that RAN2 has defined separate capabilities for intra and inter frequency LTM cell switch, RAN2 sent an LS to check with RAN1 if the UE support of RAN1 features 45-3a/4a, 45-5/5a/6 should also be indicated separately for intra and inter-frequency LTM cell switch. And RAN1 replied as below:

R1-2409045 Agreement: - Send a reply LS to RAN2 to inform the following RAN1’s understanding. - FG45-3a/4a, 45-5/5a/6 do not need to be indicated separately for intra and inter-frequency LTM cell switch. - If UE supports ltm-InterFreq-r18, the support of above FGs implies the support for inter-frequency scenarios in addition to intra-frequency scenarios. If UE only supports ltm-MCG-IntraFreq-r18 or ltm-SCG-IntraFreq-r18, the support of above FGs implies the support for intra-frequency scenarios. - Same understanding is applicable to FG45-3/4 as well

For information: 45-3a; MAC-CE activated joint LTM TCI states. 45-4; Beam indication with separate DL/UL LTM TCI states. 45-4a; MAC-CE activated DL/UL LTM TCI states. 45-5; RACH-based early TA acquisition. 45-5a; RACH-based early TA acquisition with simultaneous transmission. 45-6; UE-based TA measurement. 45-7; TA indication in cell switch command.

According to RAN1 reply LS, early TCI state activation (45-3a/45-4a) on inter-frequency candidate cells is only supported when UE supports ltm-InterFreq-r18. If to add some condition, we can live with adding an applicable condition such as “The requirements for inter-frequency candidate cells are applicable to UE capable of ltm-InterFreq-r18” in the introduction, i.e., cl. 8.25.1. But it is common understanding that the requirements are only applicable to UEs supporting the corresponding capability. We don’t think it is necessary to add such an applicable rule.

Proposal 2: Remove the sentence “FFS: this is only applicable to UE supporting inter-frequency L1 measurement with MG.” in early TCI state activation requirements.

For PDCCH order RACH, there are similar issues, the above two Proposals are applicable to PDCCH order RACH delay requirements too.

Proposal 3: In PDCCH order delay requirements:

- differentiate the cases based on whether the SSB is to be measured within or outside gap.

- Remove the sentence “FFS: this is only applicable to UE supporting inter-frequency L1 measurement with MG.”

## 2.2 Cell switch delay requirements

### 2.2.1. Conditions of Tfirst-RS =0 in cell switch delay

In RAN4#112, it was agreed to extend the condition for Tfirst-RS = 0 “the time gap between TCI state activation and the cell switch command is not more than 160 ms” to “480ms” in FR1 for UE not supporting/configured with L1 measurement. It was also clarified that “This is not to extent 160ms to 480ms directly. Longer L3 measurement delay may be expected in the 480ms duration”, which means UE may perform several shots of T/F tracking during the 480ms. Following the timing requirements, UE is supposed to track the target cell at least every 160ms.

Issue 1-4-1-2: Conditions of Tfirst-RS =0 in cell switch delay Ad hoc agreement <Agreement>: - For cell switch with L1 measurement: Extend the condition for Tfirst-RS = 0 “the time gap between receiving the LTM candidate cell TCI state activation MAC-CE and the cell switch command is not more than [160 ms]” from the agreed 160 ms, to TCI state activation delay stated in section 8.25.3 + 160 ms. - In FR1, for UE not supporting/configured with L1 measurement, one SSB occasion is needed from RAN4 requirement point of view for T/F fine tracking, if - the time gap between completion of TCI activation and cell switch command is larger than 480ms - Note: This is not to extent 160ms to 480ms directly. Longer L3 measurement delay may be expected in the 480ms duration. - Discuss the wording in CR. - FFS the following assumption - In FR1, for UE not supporting/configured with L1 measurement, one SSB occasion is needed from RAN4 requirement point of view for T/F fine tracking, if - the time gap between early RACH transmission and cell switch command is larger than [160ms] and the L3 measurement interval is larger than 160 ms.

After further checking the agreement, we found the agreement is not applicable to most of inter-frequency scenarios, where UE is supposed to measure and track the target cells within MG. The typical MGRP is 80ms. If NW activates the TCI states on more than two different frequency layers, it is not possible to track each cell every 160ms. We understand that the above agreement is workable for some inter-frequency cases, such as a shorter MGRP configured and cells on less frequency layers to track. But we don’t think it is good to go that far by adding complicated conditions. Therefore we propose to limit the above agreement to intra-frequency.

Proposal 4: The following agreement is only applicable to intra-frequency:

- Extend the condition for Tfirst-RS = 0 “the time gap between TCI state activation and the cell switch command is not more than 160 ms” to “480ms” in FR1 for UE not supporting/configured with L1 measurement.

### 2.2.2. known TCI state definition for cell switch

As shown below which is copied from the latest spec, there are still some open parts.

The target joint DL/UL TCI state or separate DL and UL TCI states in the LTM cell switch command are known if the following conditions are met: [- The target TCI state in the cell switch command is activated not more than TBD ms before the reception of the cell switch command and SNR of the SSB associated to TCI state ≥ -3dB; or] [- The target TCI state in cell switch command is activated before receiving the cell switch command and the SSB associated to target TCI state is available at least once every TBD ms after the TCI state activation command is received and SNR of the SSB associated to TCI state ≥ -3dB; or] - During the period from the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target DL/UL TCI state to the completion of LTM cell switch, where the RS resource for L1-RSRP measurement is the RS in target DL/UL TCI state or QCLed to the target DL/UL TCI state - LTM cell switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement - The UE has sent at least 1 L1-RSRP report for the target DL/UL TCI state before the LTM cell switch command - The target DL/UL TCI state remains detectable during the LTM cell switching period - The SSB associated with the target DL/UL TCI state remain detectable during the cell switching period - SNR of the TCI state ≥ -3dB Otherwise, the target joint DL/UL TCI state or separate DL and UL TCI state is unknown.

In our understanding, the initial intention of the yellow highlighted part is to support cell switch without L1 measurement report under some conditions. In the past meetings, RAN4 had agreed to support cell switch without L1 measurement report in FR1 with conditions. That is, the defined cell switch delay requirements are applicable to unknown TCI state if reported beam level L3 results in 5s in FR1. In previous meetings, RAN4 also reached the consensus that the RAN4 Rel-18 related LTM requirements are not applicable if L1 measurement of LTM candidate cell is not supported or configured in FR2. With this understanding, the whole issue was already concluded, and therefore the yellow highlighted sentences are no longer needed.

There was some proposal that L1 report may be not necessary for cell switch delay especially when TCI state is activated. One of the motivations comes that periodic L1 report leads to much overhead. As well known, R19 mobility is designing event triggered L1 report. In R18, aperiodic report can also be used to reduce report overhead. In our understanding, the intention of configuring L1 measurement is to switch to target cell’s best beam. If cell switch decision is made based on L3 report, we don’t see the necessary and the benefit to configure L1 measurement in FR2. We are open to discuss how to optimize the known TCI state definition in R19 considering L1 report can be triggered when leaving condition is met. In R18, we prefer not to do further optimization on known TCI state definition.

Proposal 5: Open to discuss how to optimize the known TCI state definition in R19 considering L1 report can be triggered when leaving condition is met. Not to further optimize known TCI state definition in R18.

Proposal 6: Not to extend cell switch delay requirements to FR2 without L1 report and remove the following in known TCI state definition for cell switch:

[-	The target TCI state in the cell switch command is activated not more than TBD ms before the reception of the cell switch command and SNR of the SSB associated to TCI state ≥ -3dB; or]

[-	The target TCI state in cell switch command is activated before receiving the cell switch command and the SSB associated to target TCI state is available at least once every TBD ms after the TCI state activation command is received and SNR of the SSB associated to TCI state ≥ -3dB; or]

### 2.2.3. Conditions for early ASN.1 decoding and validity check

Issue 1-4-3-1: Which cell(s) TLTM-RRC-processing = 0 apply to when candidate cells configured are more than UE capability? Online agreement <Agreement>: - If the total number of cells to be fast RRC decoded does not exceed UE capability maxNumberStoredConfigCells-r18 and the number of LTMCandidateConfigs fast decoded does not exceed maxNumberConfigs-r18, TLTM_RRC-processing =0 applies to the LTM candidates with early TCI activation or early PDCCH order RACH, - Further discuss: - FFS: Otherwise, TLTM_RRC-processing =0 applies to the LTM candidates with the most recently activated TCI states (if any) [and/or] PDCCH-order PRACH transmission (if any) within UE capability maxNumberStoredConfigCells-r18 and maxNumberConfigs-r18. - FFS: This is at least applicable to the case that NW does not trigger TCI state activation or PDCCH-order RACH on different candidate cells at the same occasion. - FFS: Whether to support and further optimize the case that NW triggers TCI state activation or PDCCH-order RACH on different candidate cells at the same occasion.

In previous meetings, RAN4 agreed that both early TCI state activation command and PDCCH-order can be used to trigger early RRC decoding and validity check. In RAN4#112, RAN4 further agreed that “If the total number of cells to be fast RRC decoded does not exceed UE capability maxNumberStoredConfigCells-r18 and the number of LTMCandidateConfigs fast decoded does not exceed maxNumberConfigs-r18, TLTM_RRC-processing =0 applies to the LTM candidates with early TCI activation or early PDCCH order RACH”. But we are still discussing whether to further optimize the case that the total number of candidates triggered early TCI activation or early PDCCH order RACH exceed UE capability. We are open to do further optimization if a simple and clear rule can be defined. Based on the discussion in RAN4#112, we don’t think it is easy to converge. If so, we prefer not to do further optimization.

Proposal 7: When the configured candidate cells are more than number of candidates that UE supports early RRC decoding and validity check, we are open to further optimize this scenario only if a simple and clear rule can be agreed. Otherwise, we prefer not to optimize further.

## 2.3 UE feature

In the UE feature list we sent to RAN2, the prerequisite FG of 39-6 “Fast processing of LTM candidate cell RRC configuration” is 45-3a/45-4a (45-3a: MAC-CE activated joint LTM TCI states, 45-4a: MAC-CE activated DL/UL LTM TCI states). In our view, it is not necessary. Whether a UE is capable of fast RRC processing should be independent of 45-3a/45-4a. UE can get benefit if the number of pre-configured candidates and the total number of serving cell(s) and candidate cell(s) are within UE capability. Or PDCCH-order RACH can be used to trigger fast RRC processing. We prefer to fix this.

| Features | Index | Feature group | Components | Prerequisite feature groups |
| --- | --- | --- | --- | --- |
| 39. NR_Mob_enh2 | 39-6 | Fast processing of LTM candidate cell RRC configuration | 1. Indicates the maximum number of serving cell(s) and candidate cell(s), including serving SpCell(s), serving SCell(s) in MCG and SCG, SpCell in LTMCandidateConfig(s) and Scell(s) in LTMCandidateConfig(s) for MCG and SCG, that UE can store the configurations. 2. Indicates the maximum number of LTMCandidateConfigs that UE can support fast processing . | 45-3a or 45-4a in RAN1 feature list |

Proposal 8: Send LS to RAN2 to inform them that 45-3a/45-4a (MAC-CE activated joint LTM TCI states) are not the prerequisite FG of 39-6 “Fast processing of LTM candidate cell RRC configuration”.

## 2.4 RAN2 LS on the fast RRC processing for LTM

Last meeting, RAN2 sent us an LS on fast RRC processing for LTM (R2-2409377/ R4-2417509). As far as we know, the background is as below:

RAN4 introduced the capability LTM fast RRC processing “ltm-FastProcessingConfig-r18”. There are two components included:

•	maxNumberConfigs-r18, represents the maximum number of LTM candidate configuration for which the UE can perform early ASN.1 decoding and validity check

•	maxNumberStoredConfigCells, indicates the maximum number of serving cell(s) and candidate cell(s), including serving SpCell(s), serving SCell(s) in MCG and SCG, SpCell in LTM candidate configurations and Scell(s) in LTM candidate configurations for MCG and SCG, that UE can store the configurations.

UE will decide whether to perform early RRC decoding and store the configurations of a candidate based on its UE capability and the conditions defined in 38.133. NW will also use this information to determine the cell switch delay needed by UE.

For the first component, maxNumberConfigs-r18, it is very easy for UE to judge whether the number of LTM candidate configurations exceeds its capability and whether the number of LTM candidate configurations NW triggered early TCI state activation and PDCCH order RACH.

But for the second component, maxNumberStoredConfigCells, due to the number of SpCell/SCell(s) in LTM candidate configurations is not visible until the UE decodes LTM candidate configuration(s), UE needs to perform early RRC decoding at first in order to know whether the fast RRC processing conditions are fulfilled or not. This creates a chicken-egg problem.

So RAN2 discussed whether to add a new field in LTM-Candidate IE to indicate the number of serving cells configured by this LTM candidate configuration. However, it seems RAN2 companies don’t have common understanding on this capability and need more RAN4 inputs.

In the LS, they have the following three questions. In the next, we would like to share our views on their questions.

RAN2 would like to know the answers for the following three questions: - As far as the network configures a number of total serving cell(s) + SpCell/SCell(s) in LTM candidate configurations which is up to maxNumberStoredConfigCells and where up to maxNumberConfigs LTM candidate configurations are included within the configured total cells, is it the correct understanding that UE will be capable of performing early ASN.1 decoding on all the configured LTM candidate configurations? - Does the UE need to know the number of SpCells and SCells within the configured LTM candidate configurations before doing the early ASN.1 decoding of an LTM candidate configuration? - Is it possible that the number of serving cell(s) + SpCell/SCell(s) in LTM candidate configurations exceeds the UE reported maxNumberStoredConfigCells?

Question 1: As far as the network configures a number of total serving cell(s) + SpCell/SCell(s) in LTM candidate configurations which is up to maxNumberStoredConfigCells and where up to maxNumberConfigs LTM candidate configurations are included within the configured total cells, is it the correct understanding that UE will be capable of performing early ASN.1 decoding on all the configured LTM candidate configurations?

The answer is yes according to the following condition for “TLTM-RRC-processing = 0”

Proposal 9: For Q1, confirm RAN2’s understanding that if the network configures a number of total serving cell(s) + SpCell/SCell(s) in LTM candidate configurations which is up to maxNumberStoredConfigCells and where up to maxNumberConfigs LTM candidate configurations are included within the configured total cells, UE will be capable of performing early ASN.1 decoding on all the configured LTM candidate configurations.

Question 2: Does the UE need to know the number of SpCells and SCells within the configured LTM candidate configurations before doing the early ASN.1 decoding of an LTM candidate configuration?

The question is related to the chicken-egg problem mentioned above. In our view, without the information, the number of SpCells and SCells within the to-be-decoded LTM candidate configuration(s), UE can perform early decoding. After decoding, if the number of serving cell(s) + SpCell/SCell(s) in decoded LTM candidate configuration(s) exceeds maxNumberStoredConfigCells, UE may fail to store them. At cell switch, UE still needs extra time to perform RRC decoding and validity check again. Doing like this, it is just a waste of UE efforts and power.

With the prior information, the number of SpCells and SCells within the to-be-decoded LTM candidate configuration(s), UE can determine whether to perform early decoding avoiding the waste of UE efforts and power. In our understanding, there is almost no difficulty to indicate the number of serving cells configured by a LTM candidate configuration at NW side.

To make the feature better, we propose to add a new field in LTM-Candidate IE to indicate the number of serving cells configured by this LTM candidate configuration.

Proposal 10: For Q2, reply RAN2 that:

- No. But if the number of serving cell(s) + SpCell/SCell(s) in decoded LTM candidate configuration(s) exceeds maxNumberStoredConfigCells, UE will not store the decoded configurations and still needs extra time to perform RRC decoding and validity check again during cell switch. This leads to a waste of UE efforts and power, and not preferred in RAN4.

- RAN4 suggests RAN2 adding a new field in LTM-Candidate IE to indicate the number of serving cells configured by this LTM candidate configuration to avoid potential waste of UE efforts and power.

Question 3: Is it possible that the number of serving cell(s) + SpCell/SCell(s) in LTM candidate configurations exceeds the UE reported maxNumberStoredConfigCells?

The capability “ltm-FastProcessingConfig-r18” is optional. Pre-configuration is an essential step to enable LTM. Without early decoding, cell switch is still workable, just with longer delay. In addition, RAN4 also agreed that both early TCI state activation command and PDCCH-order can be used to trigger early RRC decoding on LTM candidate(s) if the number of total serving cell(s) + SpCell/SCell(s) in LTM candidate configurations exceeds maxNumberStoredConfigCells and/or the number of LTM candidate configurations exceeds maxNumberConfigs. So it is allowed that the number of serving cell(s) + SpCell/SCell(s) in LTM candidate configurations exceeds the UE reported maxNumberStoredConfigCells.

It seems RAN2 assumes UE early decoding is only triggered when UE receives the pre-configuration. To make them understand the capability introduced in RAN4 better, we think it is helpful to inform them the information “RAN4 also agreed that both early TCI state activation command and PDCCH-order can be used to trigger early RRC decoding on LTM candidate(s) if the number of total serving cell(s) + SpCell/SCell(s) in LTM candidate configurations exceeds maxNumberStoredConfigCells and/or the number of LTM candidate configurations exceeds maxNumberConfigs”.

Proposal 11: For Q3, reply RAN2 that: Yes. RAN4 also agreed that both early TCI state activation command and PDCCH-order can be used to trigger early RRC decoding.

# Summary

In this paper, we provide some views on the open issues of R18 L1/L2 triggered mobility. We have the following proposals:

Observation 1: In early TCI state activation requirements, it is not precise to capture the requirement based on whether SSB is within the BWP or whether the SSB is to be measured with MG.

Proposal 1: In early TCI state activation requirements, differentiate the cases based on whether the SSB is to be measured within or outside gap.

Proposal 2: Remove the sentence “FFS: this is only applicable to UE supporting inter-frequency L1 measurement with MG.” in early TCI state activation requirements.

Proposal 3: In PDCCH order delay requirements:

- differentiate the cases based on whether the SSB is to be measured within or outside gap.

- Remove the sentence “FFS: this is only applicable to UE supporting inter-frequency L1 measurement with MG.”

Proposal 4: The following agreement is only applicable to intra-frequency:

- Extend the condition for Tfirst-RS = 0 “the time gap between TCI state activation and the cell switch command is not more than 160 ms” to “480ms” in FR1 for UE not supporting/configured with L1 measurement.

Proposal 5: Open to discuss how to optimize the known TCI state definition in R19 considering L1 report can be triggered when leaving condition is met. Not to further optimize known TCI state definition in R18.

Proposal 6: Not to extend cell switch delay requirements to FR2 without L1 report and remove the following in known TCI state definition for cell switch:

[-	The target TCI state in the cell switch command is activated not more than TBD ms before the reception of the cell switch command and SNR of the SSB associated to TCI state ≥ -3dB; or]

[-	The target TCI state in cell switch command is activated before receiving the cell switch command and the SSB associated to target TCI state is available at least once every TBD ms after the TCI state activation command is received and SNR of the SSB associated to TCI state ≥ -3dB; or]

Proposal 7: When the configured candidate cells are more than number of candidates that UE supports early RRC decoding and validity check, we are open to further optimize this scenario only if a simple and clear rule can be agreed. Otherwise, we prefer not to optimize further.

Proposal 8: Send LS to RAN2 to inform them that 45-3a/45-4a (MAC-CE activated joint LTM TCI states) are not the prerequisite FG of 39-6 “Fast processing of LTM candidate cell RRC configuration”.

Proposal 9: For Q1, confirm RAN2’s understanding that if the network configures a number of total serving cell(s) + SpCell/SCell(s) in LTM candidate configurations which is up to maxNumberStoredConfigCells and where up to maxNumberConfigs LTM candidate configurations are included within the configured total cells, UE will be capable of performing early ASN.1 decoding on all the configured LTM candidate configurations.

Proposal 10: For Q2, reply RAN2 that:

- No. But if the number of serving cell(s) + SpCell/SCell(s) in decoded LTM candidate configuration(s) exceeds maxNumberStoredConfigCells, UE will not store the decoded configurations and still needs extra time to perform RRC decoding and validity check again during cell switch. This leads to a waste of UE efforts and power, and not preferred in RAN4.

- RAN4 suggests RAN2 adding a new field in LTM-Candidate IE to indicate the number of serving cells configured by this LTM candidate configuration to avoid potential waste of UE efforts and power.

Proposal 11: For Q3, reply RAN2 that: Yes. RAN4 also agreed that both early TCI state activation command and PDCCH-order can be used to trigger early RRC decoding.

# 4 Reference

R4-2410302, “WF on NR mobility enhancements (part 1)”, MediaTek Inc., RAN4#111 meeting, May 2024