3GPP TSG-RAN WG4 Meeting #113	R4-2417916
Orlando, US, 18th – 22nd November, 2024

Title:	R19 Simulation results of 6Layer performance

Source:	OPPO

Agenda item:	7.1.1.3.2

Document for:	Approval

- Introduction

6Layer was discussed in last meeting and captured in WF [1]. This paper studies the 6Layer performance in handheld UE with real antenna parameters.

- Discussion

2.1 Simulation assumptions

The simulations are based on the assumptions agreed in last meeting.

| Table: Feasibility Study Link-Level Simulation Assumptions Parameters |  | Values |
| --- | --- | --- |
| Duplex mode |  | TDD |
| Bandwidth |  | 40MHZ |
| SCS |  | 30 |
| Antenna configuration |  | {8Tx, 6Rx} as baseline Interested companies can submit results with {32Tx, 6Rx} and specify the precoding type |
| Propagation channel |  | TDLA30-10 |
| Rank |  | Fixed rank {4,6}, interested companies can bring {5} |
| MCS |  | Adaptive MCS (target BLER 10%) MCS Table 2 Interested companies can bring MCS Table 1 |
| PDSCH configuration | Mapping type | Type A |
|  | Starting symbol | 2 |
|  | Length | 12 |
|  | PRB bundling size | 2 |
|  | VRB-to-PRB mapping type | Non-interleaved |
|  | Precoding | Random PMI (Rel-15 Type 1 codebook) for 8Tx SVD based precoding for 32Tx; companies are encouraged to provide details on the SVD update periodicity |
| PDSCH DMRS configuration | DMRS Type | Type 1 [Rel-15] and eType-1 [Rel-18] |
|  | DMRS Configurations | Option 1: Rel-18 1+1(6L), Rel-15 1+1(4L) Option 2: Rel-15 1+1(4L), Rel-15 2+0(6L) Option 3: Rel-15 1+1(4L), Rel-15 2+2(6L) Option 4: Rel-15 2+2(4L), Rel-15 2+2(6L) |
|  | Precoding | Random PMI (Rel-15 Type 1 codebook) for 8Tx SVD based precoding for 32Tx, companies are encouraged to provide details on the SVD update periodicity |
| UE Correlation matrix |  | Option 1: Antenna efficiency is considered in the following simulations. Option 2: 1.000.100.101.000.300.10 0.300.150.100.141.000.12 0.130.120.300.200.200.120.150.140.130.300.120.20 0.121.000.200.110.120.30 0.110.301.000.100.101.00 Option 3: 1.000.220.221.000.300.580.600.150.150.100.050.150.300.600.580.151.000.150.151.000.150.150.150.100.150.050.100.150.150.150.150.101.000.300.301.00 Option 4: 1.000.100.101.000.250.350.500.100.100.050.050.100.250.500.350.101.000.100.101.000.100.150.100.050.100.050.050.100.100.100.150.051.000.150.151.00 Option 5: 3GPP MIMO correlation matrix with beta=0.00718 (Option 5 is parametric and to be justified since Options 1-4 are based on measurements) Note: For the feasibility analysis, antenna efficiency can be considered. Companies are encouraged to provide the antenna efficiency numbers. |
| BS correlation matrix |  | Option 1: 3GPP MIMO correlation matrix with Alpha = 0.1 Option 2: 3GPP MIMO correlation matrix with Alpha = 0.0 Other options are not precluded. Matrices requested from interested BS vendors for RAN4#113 |

2.1 Simulation results

2.1.1 CB 8Tx TDL-A

Figure 3 UE correlation option 1 with TDL-A 8Tx and CB precoding

Observation 1:   Antenna efficiency has some impact on the spectrum efficiency, but it doesn’t change the trend that rank 6 has better performance than rank 4 when the SNR is high.

Observation 2:   When antenna efficiency is not considered (left figure), rank 6 has better performance than rank 4 when SNR is higher than 27dB with CB 8Tx and TDL-A.

Observation 3:   When antenna efficiency is considered (right figure), rank 6 has better performance than rank 4 when SNR is higher than 30dB with CB 8Tx and TDL-A.

2.1.2 NCB 32Tx CDL-C

Figure 2 UE correlation option 1 with CDL-C 32Tx and NCB precoding

Observation 4:   Rank 6 has better or similar performance than rank 4 in all SNR ranges with NCB 32Tx in CDL-C no matter antenna efficiencies are considered or not.

Observation 5:   6Layers is feasible and can perform better than 4Layer in some conditions.

- Conclusion

This paper analyzed the 6Layer throughput performances with real handheld UE antenna correlations in both TDL-A and CDL-C channels, and got observations as below.

Observation 1:   Antenna efficiency has some impact on the spectrum efficiency, but it doesn’t change the trend that rank 6 has better performance than rank 4 when the SNR is high.

Observation 2:   When antenna efficiency is not considered (left figure), rank 6 has better performance than rank 4 when SNR is higher than 27dB with CB 8Tx and TDL-A.

Observation 3:   When antenna efficiency is considered (right figure), rank 6 has better performance than rank 4 when SNR is higher than 30dB with CB 8Tx and TDL-A.

Observation 4:   Rank 6 has better or similar performance than rank 4 in all SNR ranges with NCB 32Tx in CDL-C no matter antenna efficiencies are considered or not.

Observation 5:   6Layers is feasible and can perform better than 4Layer in some conditions.

- Reference

[1] R4-2417209 WF on 6Rx UE requirements, AT&T, RAN4#112bis 2024-10