3GPP TSG-RAN WG4 Meeting#113							         		R4-2417920

Orlando, Florida, USA, 18 – 22 November 2024

Title: 	Emission requirement to enable single CC MPR when only one CC is allocated

Source: 	Skyworks Solutions, Inc.

Agenda Item:	7.1.1.2.2 MPR applicability for FR1 intra-band UL CA [NR_ENDC_RF_Ph4-Core]

Document for:	Approval

# 1	Background

In RAN#112bis, in [1], we discussed which emissions requirement can apply for ULCA to enable the use of single CC MPR when there is active RBs in only one CC and avoiding the retuning of the LO. The different options from different companies related to contiguous and non-contiguous ULCA were captured in way forward [2]. In this contribution, we amend our proposal for contiguous ULCA and non-contiguous ULCA for a consistent approach to all intra-band ULCA cases and implementations.

# Discussion

## 2.1 Way Forward input on intra-band contiguous ULCA and proposal

#### The agreements and way forward from [2] relevant to the contiguous ULCA emissions requirements are copied below in italic.

#### Issue 2-1-2: Single CC CBW or aggregated CBW for applying requirements of ACLR/SEM/SE

- Proposals

  - Proposal 1: activated CC CBW or aggregated CBW adopted for integral region and boundary of spurious emissions/ACLR/SEM depending on dualPA-Architecture IE indication.

  - Proposal 2: Emission requirements are based on aggregated channel bandwidth also in case that only one CC is activated.

- WF

  - FFS in next meeting

#### Issue 2-1-3: Single CC or CA requirements of ACLR/SEM/SE applied for single activated cell

- Proposals

  - Proposal 1: CA requirements of ACLR/SEM/SE should be applied for intra-band contiguous CA with single activated cell.

  - Proposal 2:

    - For UE supporting contiguous ULCA with the dualPA IE:

      - The applicable SEM, ACLR and spurious emissions are the single CC emissions defined in Table 6.5.2.2-1, Table 6.5.2.4.1-1 and Table 6.5.3.1-2 respectively.

    - For UE supporting contiguous ULCA without signalling the dualPA IE or signalling PC1.5 or TxD or UL MIMO support:

      - The applicable SEM, ACLR and spurious emissions are the configured contiguous ULCA

  - Proposal 1: After the initial configuration of contiguous ULCA, upon SCELL deactivation with only 1CC active, the applied SEM mask should be the aggregated CA BW SEM as stated in sub-clause 6.5A.2.2.1 in TS38.101-1. Further discuss SEM mask for single CC fall back only if dualPA-architecture is supported

  - Proposal 3: Apply single carrier spurious emission/ACLR/SEM requirements for contiguous UL CA with only 1 CC transmitted.

- WF

  - FFS in next meeting

#### Issue 2-1-4: Whether UL interruption is allowed for intra-band contiguous CA with single activated cell

- Proposals

  - Proposal 1: No interruptions are allowed for enabling single carrier MPR for the case when only one CC is activated in intra-band contiguous UL CA.

- Agreement in AH

  - Proposal 1

Observations:

- It is agreed that UL interruptions due to LO retuning are not allowed to enable the use of single carrier MPR

- Accordingly, at least for the UEs supporting contiguous ULCA without the dualPA IE, it is not possible to meet single CC emissions with the single CC MPR as shown in [1].

- If contiguous ULCA emission requirements for ACLR, SEM and SE based on aggregated BW is needed for one implementation, it may be applied generally.

Proposal for intra-band contiguous ULCA: For PC3/PC2 intra-band contiguous carrier aggregation with single CC with activated cell and active RBs allocated in only one CC, the following TS 38.101-1 MPR requirements are applied.

- MPR defined in Table 6.2.2-1 applies for UE power class 3 CA bandwidth classes B and C;

- MPR defined in Table 6.2D.2-1 applies for power class 2 CA bandwidth classes B and C when TxD and/or UL-MIMO capability are indicated.

- MPR defined in Table 6.2.2-2 applies for power class 2 CA bandwidth classes B and C when TxD and/or UL-MIMO capability are absent.

And associated with applicable TS 38.101-1 ULCA emission requirements based on the aggregated BW, independent of whether or not the UE signals the dualPA IE:

- SEM requirements in Table 6.5A.2.2.1-1 applies.

- ACLR requirements in Table 6.5A.2.4.1.1-1 and Table 6.5A.2.4.1.1-2 applies for PC3 and PC2 respectively.

- Spurious emission requirements in Table 6.5A.3.1-1 apply.

- Spurious emissions for UE co-existence in Table 6.5A.3.2.1-1 apply.

- Where applicable the additional contiguous ULCA emission requirements in section 6.5A.2.3 apply.

## 2.2 Way Forward input on intra-band non-contiguous ULCA and proposal

#### The agreements and way forward from [2] relevant to the non-contiguous ULCA emissions requirements are copied below in italic.

#### Issue 2-2-1: Applicable MPR for FR1 intra-band non-contiguous UL CA

- Proposals

Option 1: No spec impact or minimum spec impact with some clarification but no changes of requirements

  - Proposal 1: for PC3 and PC2 intra-band non-contiguous CA as the standard already accounts for the use of the single CC MPR tables when only 1 CC is scheduled no further changes to the standard are required.

  - Proposal 2: adding 1CC activation as a supplementary scenario in the spec but no changes of the requirements

  - Proposal 3: single CC MPR applied with single CC emission requirements:

Option 2: Single CC MPR/SEM applied on indication of dualPA IE

  - Proposal 4: To apply non-CA (single CC) MPR based on indication of dualPA IE

  - Proposal 5: After the initial configuration of non-contiguous ULCA, upon SCELL deactivation with only 1CC active, the applied SEM mask should be the composite SEM of the 2CCs as stated in sub-clause 6.5A.2.2.2 in TS38.101-1. Further discuss to support the single CC SEM only if dualPA-architecture is supported.

Option 3: Possible spec impact on LO retuning

  - Proposal 6: Discuss impact on requirements if LO needs to be re-configured to support single CC MPR. This discussion would be required for intra-band contiguous and non-contiguous carrier aggregation

- WF

  - FFS on whether for PC2 and PC3, no MPR requirements to be changed.

  - FFS PC1.5 when NC CA requirements are available

  - FFS clarification for the applicable emission requirements whether they are single CC based or composite based

Agreement in main session

  - Drop the discussion for LO retuning since it is out of WI scope

    - FFS for FR2

Observations:

- It is agreed that LO retuning is out of scope for non-contiguous ULCA to enable the use of single CC MPR when only one CC has active RBs

- Accordingly, at least for the UEs supporting contiguous ULCA without the dualPA IE, it is not possible to meet single CC emissions with the single CC MPR as shown in [1].

- We do not see a reason to change requirements if the non-contiguous ULCA emission requirements are used based on the composite SEM, ACLR and SE.

- If this approach can be used, it may be applied independent from whether or not the UE signalling DualPA, and is consistent with the approach proposed for contiguous ULCA.

Proposal for intra-band non-contiguous ULCA: For PC3/PC2 intra-band non-contiguous carrier aggregation with single CC with activated cell and active RBs allocated in only one CC, the following TS 38.101-1 MPR requirements are applied.

- MPR defined in Table 6.2.2-1 applies for UE power class 3 CA bandwidth classes B and C;

- MPR defined in Table 6.2D.2-1 applies for power class 2 CA bandwidth classes B and C when TxD and/or UL-MIMO capability are indicated.

- MPR defined in Table 6.2.2-2 applies for power class 2 CA bandwidth classes B and C when TxD and/or UL-MIMO capability are absent.

And associated with applicable TS 38.101-1 ULCA requirements based on the composite emission, independent of whether or not the UE signals the dualPA IE:

- Composite SEM requirements in section 6.5A.2.2.2 apply.

- ACLR requirements in section 6.5A.2.4.1.2 and Table 6.5A.2.4.1.2-1 and Table 6.5A.2.4.1.2-2 applies for PC3 and PC2 respectively.

- Spurious emission requirements in section 6.5A.3.1 bellow Table 6.5A.3.1-1 apply.

- Spurious emissions for UE co-existence in Table 6.5A.3.2.2-1 apply.

- Where applicable the additional contiguous ULCA emission requirements in section 6.5A.2.3.2 apply.

# Conclusion

In this contribution, based on the analysis in [1] and further considering the consistency of the requirement between contiguous and non-contiguous ULCA, and whether or not implementation signals DualPA, we make the following proposals.

Proposal for intra-band contiguous ULCA: For PC3/PC2 intra-band contiguous carrier aggregation with single CC with activated cell and active RBs allocated in only one CC, the following TS 38.101-1 MPR requirements are applied.

- MPR defined in Table 6.2.2-1 applies for UE power class 3 CA bandwidth classes B and C;

- MPR defined in Table 6.2D.2-1 applies for power class 2 CA bandwidth classes B and C when TxD and/or UL-MIMO capability are indicated.

- MPR defined in Table 6.2.2-2 applies for power class 2 CA bandwidth classes B and C when TxD and/or UL-MIMO capability are absent.

And associated with applicable TS 38.101-1 ULCA emission requirements based on the aggregated BW, independent of whether or not the UE signals the dualPA IE:

- SEM requirements in Table 6.5A.2.2.1-1 applies.

- ACLR requirements in Table 6.5A.2.4.1.1-1 and Table 6.5A.2.4.1.1-2 applies for PC3 and PC2 respectively.

- Spurious emission requirements in Table 6.5A.3.1-1 apply.

- Spurious emissions for UE co-existence in Table 6.5A.3.2.1-1 apply.

- Where applicable the additional contiguous ULCA emission requirements in section 6.5A.2.3 apply.

Proposal for intra-band non-contiguous ULCA: For PC3/PC2 intra-band non-contiguous carrier aggregation with single CC with activated cell and active RBs allocated in only one CC, the following TS 38.101-1 MPR requirements are applied.

- MPR defined in Table 6.2.2-1 applies for UE power class 3 CA bandwidth classes B and C;

- MPR defined in Table 6.2D.2-1 applies for power class 2 CA bandwidth classes B and C when TxD and/or UL-MIMO capability are indicated.

- MPR defined in Table 6.2.2-2 applies for power class 2 CA bandwidth classes B and C when TxD and/or UL-MIMO capability are absent.

And associated with applicable TS 38.101-1 ULCA requirements based on the composite emission, independent of whether or not the UE signals the dualPA IE:

- Composite SEM requirements in section 6.5A.2.2.2 apply.

- ACLR requirements in section 6.5A.2.4.1.2 and Table 6.5A.2.4.1.2-1 and Table 6.5A.2.4.1.2-2 applies for PC3 and PC2 respectively.

- Spurious emission requirements in section 6.5A.3.1 bellow Table 6.5A.3.1-1 apply.

- Spurious emissions for UE co-existence in Table 6.5A.3.2.2-1 apply.

- Where applicable the additional contiguous ULCA emission requirements in section 6.5A.2.3.2 apply.

# References

[1] R4-2415273 Using single CC MPR and ULCA related emissions when only one CC has RBs allocated, Skyworks Solutions Inc., RAN4#112bis

[2] R4-2417204 WF on power enhancement for UE Huawei, RAN4#112bis