# 3GPP TSG-RAN WG4 Meeting #113	R4-2417942

# Orlando, US, 18th – 22nd November, 2024

Source: 	Xiaomi

Title: 	Discussion on simulation assumptions and results for LP-WUS/WUR

Agenda Item:	7.23.4.1

Document for:	Discussion

# 1Introduction

In RAN4#112bis meeting, the updated link level simulation assumptions for LP-SS evaluation were agree in [1].

In this contribution, our initial link level simulation results for OOK based LP-SS are provided.

- Discussion

- Simulation assumptions

The simulation assumptions are given in Table 1:

Table 1: General parameters

| Parameters | Value |
| --- | --- |
| Carrier frequency | 2.6 GHz |
| Prior knowledge of Cell 1 / Cell 2 by the UE | Interfering cell (Cell 2) is not known to UE |
| LP-SS block BW | 132 subcarriers for SCS=30kHz |
| LP-SS | OOK-4 with M = 2,4 |
| LP-SS overhead | 2 OFDM symbols |
| LP-SS sequences | M sequence |
| BS transmit antennas for LP-SS blocks | 1 tx |
| UE receive antennas | 1 rx |
| Subcarrier spacing | 30 kHz |
| Number of LP-SS blocks per SS burst set, K | 1 |
| SNR | -3dB |
| Propagation conditions | TDL-C 300ns |

Table 2: Cell-specific parameters

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| RF Channel number | - | Channel 1 | Channel 1 |
| LP-SS (OOK based LP-WUR receiver) |  | OOK-4 with M = 2,4 | when Cell 1 uses OOK-4, OOK-4 is used for Cell 2. |
| LP-SS pattern |  | M sequence | M sequence |
| CP Length | - | Normal | Normal |
| Frequency offset relative to UE frequency reference | Hz | OOK: 10 ppm | N/A |
| Timing error |  | Residual timing error + timing drift (frequency offset* 320ms (reference signal periodicity) Residual timing error: T= 2us for OOK-4 with M=2 T= 1us for OOK-4 with M=4 |  |
| 1)Relative Delay of 1st Path (synchronous) | µs | 0 | CP/2 |
| 2) Relative Delay of 1st Path (asynchronous): Fixed delay | ms | 0 | 3 ms |
| SNR | dB | Set 1: SNR = -2.7 Set 2: SNR = -2.4 | Set 1: SNR = -11.7 Set 2: SNR = -8.4 |
| Ês/Iot | dB | -3 | N/A |
| Propagation conditions | - | FR1: AWGN TDL-C 300ns |  |
| UE speed |  | FR1: 3 km/h |  |

- Simulation results

Table 3: simulation results for OOK-4, M=2, async

|  |  |  | 5% CDF (dB) | 95% CDF (dB) |
| --- | --- | --- | --- | --- |
| Set1: [-2.7, -11.7] | AWGN | 1 sample | -2.05 | 1.25 |
|  |  | 2 sample | -1.43 | 0.83 |
|  |  | 3 sample | -1.16 | 0.69 |
|  |  | 4 sample | -1.03 | 0.59 |
|  |  | 5 sample | -0.91 | 0.52 |
|  | TDL-C | 1 sample | -3.72 | 3.42 |
|  |  | 2 sample | -3.35 | 2.49 |
|  |  | 3 sample | -3.11 | 2.28 |
|  |  | 4 sample | -2.42 | 1.86 |
|  |  | 5 sample | -2.25 | 1.82 |
| Set2: [-2.4, -8.4] | AWGN | 1 sample | -1.94 | 1.17 |
|  |  | 2 sample | -1.31 | 0.84 |
|  |  | 3 sample | -1.07 | 0.68 |
|  |  | 4 sample | -0.94 | 0.59 |
|  |  | 5 sample | -0.83 | 0.51 |
|  | TDL-C | 1 sample | -3.58 | 3.41 |
|  |  | 2 sample | -2.70 | 2.48 |
|  |  | 3 sample | -2.51 | 2.11 |
|  |  | 4 sample | -2.19 | 1.85 |
|  |  | 5 sample | -2.31 | 1.61 |

Table 4: simulation results for OOK-4, M=4, async

|  |  |  | 5% CDF (dB) | 95% CDF (dB) |
| --- | --- | --- | --- | --- |
| Set1: [-2.7, -11.7] | AWGN | 1 sample | -1.71 | 1.63 |
|  |  | 2 sample | -1.05 | 1.27 |
|  |  | 3 sample | -0.78 | 1.08 |
|  |  | 4 sample | -0.59 | 0.97 |
|  |  | 5 sample | -0.52 | 0.88 |
|  | TDL-C | 1 sample | -3.40 | 3.58 |
|  |  | 2 sample | -2.48 | 2.60 |
|  |  | 3 sample | -2.41 | 2.36 |
|  |  | 4 sample | -2.22 | 2.00 |
|  |  | 5 sample | -1.85 | 1.93 |
| Set2: [-2.4, -8.4] | AWGN | 1 sample | -1.29 | 1.78 |
|  |  | 2 sample | -0.68 | 1.46 |
|  |  | 3 sample | -0.43 | 1.29 |
|  |  | 4 sample | -0.28 | 1.21 |
|  |  | 5 sample | -0.19 | 1.13 |
|  | TDL-C | 1 sample | -3.09 | 3.37 |
|  |  | 2 sample | -2.26 | 2.70 |
|  |  | 3 sample | -1.84 | 2.46 |
|  |  | 4 sample | -1.71 | 2.07 |
|  |  | 5 sample | -1.38 | 1.92 |

In last meeting, the following agreement was achieved.

Issue 2-1-4: Accuracy baseline for simulation Agreement: Candidate absolute baseband accuracy to derive the number of samples for simulation alignment purpose only: - For FR1: - For OOK based LP-WUR, {3.5, 2.5} dB for LP-RSRP and {3.5, 2.5} dB for LP-RSRQ - 1st priority: 3.5dB, 2nd priority: 2.5dB - For OFDM based LP-WUR, {3.5, 2.5} dB for SS-RSRP and {3.5, 2.5} dB for SS-RSRQ - 1st priority: 3.5dB, 2nd priority: 2.5dB - Note 1: the numbers can be updated if needed based on the simulation results and/or the progress in RF session - Note 2: the numbers for accuracy requirement is a separate discussion, and different numbers for OOK based and OFDM based LP-WUS for accuracy requirement are not precluded.

Based on evaluation results shown in previous section, we could make following observations:

- For LP-SS based RSRP measurement, accuracy results show slightly difference between OOK-4 with M=2 and OOK-4 with M=4;

- To achieve 3.5 dB absolute baseband accuracy, measurement with 3 LP-SS samples is needed at SNR = -3dB in TDL-C channel.

- To achieve 2.5 dB absolute baseband accuracy, measurement with 5 LP-SS samples is needed at SNR = -3dB in TDL-C channel.

Observation 1: For LP-SS based RSRP measurement, accuracy results show slightly difference between OOK-1, OOK-4 with M=2 and OOK-4 with M=4.

Observation 2: To achieve 3.5 dB absolute baseband accuracy, measurement with 3 LP-SS samples is needed at SNR = -3dB in TDL-C channel.

Observation 3: To achieve 2.5 dB absolute baseband accuracy, measurement with 5 LP-SS samples is needed at SNR = -3dB in TDL-C channel.

- Conclusion

Observation 1: For LP-SS based RSRP measurement, accuracy results show slightly difference between OOK-1, OOK-4 with M=2 and OOK-4 with M=4.

Observation 2: To achieve 3.5 dB absolute baseband accuracy, measurement with 3 LP-SS samples is needed at SNR = -3dB in TDL-C channel.

Observation 3: To achieve 2.5 dB absolute baseband accuracy, measurement with 5 LP-SS samples is needed at SNR = -3dB in TDL-C channel.

- Reference

- R4-2410296, WF for RRM core requirements for LP-WUS/WUR, Vivo