3GPP TSG-RAN WG4 Meeting # 113	R4-2417952

# Orlando, US, 18th – 22nd November, 2024

Source: 	Xiaomi

Title: 	Discussion on event triggered L1 measurement reporting for mobility

Agenda Item:	7.24.2.1

Document for:	Discussion

# 1 Introduction

In this contribution, we will mainly discuss the following issues for Rel-19 Mobility.

- Event-triggered L1 reporting

- Discussion

From RAN2#127bis, the agreement is as below:

- Event triggered periodic MR can be supported, based on NW configuration.

It shows that Event triggered periodic MR can be supported, based on NW configuration.

Proposal 1: Both event triggered L1 reporting and event triggered periodic L1 reporting requirement will be defined by RAN4.

Issue 1-5: filtering assumption in event evaluation - Candidate solutions: - Option 1: Wait for more RAN1 progress to discuss how to handle L1 filtering in RAN4 for event triggered L1 report (MTK, Apple, HW, Nokia, E///, Qualcomm, CATT). - Option 1a: For whether to consider the RRM impact due to L1 filtering for event triggered L1 report, RAN4 needs to wait for more RAN1 input. (CATT) - If L1 filtering is introduced in RAN1, RAN4 needs to define additional SSB based L1 measurement delay requirements with filtering for event triggered L1 report. - Otherwise, there is no impact on RAN4. - Option 2: For filtering, we think it is better to wait the conclusion of RAN1. If it is up to UE implementation, it is unnecessary to define the related requirements. (CTC) - Event triggered reporting delay is up to whether the filtering is needed, when filtering is used an additional delay can be expected. - In case without filtering, we think current LTM L1-RSRP measurement delay can be as baseline. - Option 3: no need to define additional SSB based L1 measurement delay requirements with filtering, since the agreement about filtering in RAN2 is about filtering with filter coefficient as defined in Clause 5.5.3.2, TS38.331 or about Time-to-Trigger (TTT), neither of them have impact on L1 measurement delay requirements. (CMCC) - Recommended WF - Continue discussion.

In RAN1 #118bis, it’s agreed that no L1 specified filtering for time and spatial domain is introduced.

Agreement The agreement “Rel-18 LTM CSI reporting framework is the baseline for CSI-RS based L1-measurement report by gNB scheduled measurement reporting” made in RAN#118 is further clarified for L1-RSRP as follows: - UCI format defined in Table 6.3.1.1.2-8C of TS38.212 can be used by replacing SSBRI with CRI. - Whether the L1-RSRP(s) of serving cell is always included is configurable (in line with Rel-18) - The quantization method defined in clause 5.2.1.4.3 of TS38.214 and bit width defined in Table 6.3.1.1.2-6 of TS38.212 can be used - No L1 specified filtering for time and spatial domain is introduced - No enhancement on how to report L cells x M beams - Periodic reporting on PUCCH is supported - FFS: semi-persistent reporting on PUCCH/PUSCH, and aperiodic reporting on PUSCH

Proposal 2: No L1 based filtering needs to be considered in event evaluation.

From RAN2 agreement, TTT operates based on Timer, which is used similar as L3 event triggered MR. For event triggered L3 reporting, minimum configurable TTT is zero, then the shortest reporting delay is just one measurement period. For L1 reporting, if minimum configured TTT can still be 0, no additional delay needs to be considered. If not, minimum configurable TTT may needs to be added into total evaluation delay. Here, the impact of TTT is not about L1 filtering, TTT make sure that L1 reporting can only be triggered if all L1 results in TTT satisfy the event triggering condition.

Proposal 3: RAN4 to wait for more progress in RAN2 about whether to consider the impact of TTT on event evaluation time.

Issue 1-7: beam level or cell level measurement result for new LTM event evaluation - Candidate solutions: - Option 1: For LTM event triggered L1 reporting, beam level measurement result will be used for evaluation. (Xiaomi, CMCC, Apple, Samsung) - Recommended WF - Continue discussion.

RAN2 #127bis agreements are as below:

1. The entire event evaluation procedure is handled by MAC based on the latest L1 measured results reported by L1. 2. Basic information included in MR MAC CE: - Beam information: FFS if SSBRI/CRI of N beams or (LTM configuration id + SSB/CSI-RS id) - Beam quantity: L1-RSRP or SINR (up to RAN1) of N beams - Triggered event information (e.g., ReportConfigID) MR MAC CE can include up to N beams (FFS whether the beam should satisfy the event or not). N is configurable by NW. 3. Additional information included in MR MAC CE: - The information and quantity of current beam, based on NW configuration.

From RAN2 agreement, beam information will be included in L1 report and event evaluation will be based on L1 results in the report, therefore, bean level measurement will be used for LTM event evaluation.

Proposal 4: For LTM event triggered L1 reporting, beam level measurement result will be used for evaluation.

Measurement delay

In order to evaluate whether L1 reporting will be triggered, UE needs to compare L1 results with certain criteria. There are several agreed events in RAN2:

Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events. - Event LTM2: Beam of serving cell becomes worse than absolute threshold; - Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell; - Event LTM4: Beam of candidate cell becomes better than absolute threshold; - Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

For event LTM3 and LTM5, UE needs to measure for both serving cell and candidate cell to obtain L1 results for comparing. For Event LTM4, UE may need to measure for multiple neighbor cells to find one qualified cell.

For L1 measurement, the RS may be based on SSB or CSI-RS. It’s possible that the periodicity of RS is different from each other and then the measurement period is different. We plot a figure for event LTM3 where the RS periodicity for serving cell and neighbour cell is different. RAN4 needs to discuss how to define measurement time when the measurement periods for different cells are different.

Fig.1 Measurement time for different cells

Observation 1: It’s possible that the periodicity of RS is different from each other and then the measurement period is different.

If multiple beams with different measurement period need to be measured, the maximum measurement period will be considered as measurement delay.

Proposal 5: If multiple beams with different measurement period need to be measured, the maximum measurement period will be considered as measurement delay.

Impact of UL resource waiting time

In legacy L3 event triggered reporting, according to 38.331, if L3 measurement result satisfies the event triggering condition, UE will initiate the message reporting procedure and submit the MeasurementReport message to lower layers for transmission. It means that the L3 measurement result which triggers reporting will be included in the L3 report.

For L1 event triggered reporting, different from L3, L1 measurement is much faster, it’s possible that UE can obtain several new L1 results between the time UE trigger L1 reporting and the time when UL resource arrives.

Similar as L3 event triggered reporting, it needs to specify which measurement result will be included in L1 reporting. Following the same logic of L3, L1 result which triggers L1 reporting will be included in L1 reporting. It means that if L1 reporting procedure starts, UE can continue to measure for L1, however, the following L1 result will not be evaluated for event anymore.

Proposal 6: For L1 event triggered reporting, it needs to specify that L1 result which triggers L1 reporting will be included in L1 reporting.

- Conclusion

In this contribution, we provide the following proposals：

Proposal 1: Both event triggered L1 reporting and event triggered periodic L1 reporting requirement will be defined by RAN4.

Proposal 2: No L1 based filtering needs to be considered in event evaluation.

Proposal 3: RAN4 to wait for more progress in RAN2 about whether to consider the impact of TTT on event evaluation time.

Proposal 4: For LTM event triggered L1 reporting, beam level measurement result will be used for evaluation.

Observation 1: It’s possible that the periodicity of RS is different from each other and then the measurement period is different.

Proposal 5: If multiple beams with different measurement period need to be measured, the maximum measurement period will be considered as measurement delay.

Proposal 6: For L1 event triggered reporting, it needs to specify that L1 result which triggers L1 reporting will be included in L1 reporting.

- Reference