3GPP TSG-RAN WG4 Meeting #113	R4-2417974

Orlando, United States, Nov 18th – Nov 22nd, 2024

Source:	Nokia

Title:	Simulations for UE demodulation performance requirements for 8Rx with MMSE-IRC

Agenda item:	7.16.2

Document for:	Discussion

Introduction

In RAN4#112bis agreements were made which enables companies to provide initial PDSCH simulation results for a limited number of configurations for initial alignment. In the following we will provide Nokia’s simulation results.

Simulation results

PDSCH requirements with inter cell interference

Agreed setup

The following configuration parameters are extracted from the agreements made in RAN4#112bis (see [1]). For remaining parameters, we have re-used the configuration from in Rel.17 2Rx/4Rx:

| Parameter | Value | Comments |
| --- | --- | --- |
| Receivers | MMSE-MRC, MMSE-IRC (baseline), MMSE-IRC(simplified) |  |
| Interference profile | Reuse from Rel-17 2/4Rx |  |
| Network type and SCS/CBW | - FDD: 15kHz/10MHz - TDD: 30kHz/40MHz, 7D1S2U(S=6D+4G+4U) |  |
| Propagation conditions | - TDLC300-100 for 2 interfering cells with rank 1 (HomNet) -TDLA30-10 for all other configurations. |  |
| Rank + MCS | Rank 1: MCS17, 2T8R ULA Low Rank 2: MCS17, 2T8R ULA Low Rank 4: MCS13, 4T8R ULA Low |  |
| Interference cell | - 16QAM - 2Tx - Frequency shift to target cell: 300Hz for interference cell 1, -100Hz for interference cell 2 - Transmission rank: Random rank with 70% and 30% probability for rank 1 and rank 2 for all interference cells |  |

Simulation results

Table 1 shows Nokia’s Baseline receiver PDSCH simulation results with inter-cell interference.

Table 1: PDSCH simulation results with inter-cell interference (SNR [dB] @ 70% Max TP)

| Priority | TC Number | Duplex Mode and SCS | CHBW | Scenario | MIMO | Rank (target UE) | MCS (target UE) | INR1 | INR2 | Channel Model | Antenna correlation | Nokia |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Baseline Receiver |  |  |  |  |  |  |  |  |  |  |  | MMSE-IRC | MMSE | Gain over MMSE |
| Priority for eval phase | 1 | FDD 15KHz SCS | 10 MHz | HomNet | 2T8R | 1 | 17 | 7.77 | 2.29 | TDLC300-100 | ULA Low | 4.9 | 11.1 | 6.2 |
|  | 2 |  |  |  | 2T8R | 2 | 17 |  |  | TDLA30-10 |  | 7.8 | 12.3 | 4.5 |
|  | 3 |  |  |  | 4T8R | 4 | 13 |  |  |  |  | 11.2 | 17.0 | 5.8 |
|  | 4 |  |  | HetNet | 2T8R | 1 | 17 | 7.58 | N/A | TDLA30-10 | ULA Low | 2.5 | 6.8 | 4.3 |
|  | 5 |  |  |  | 2T8R | 2 | 17 |  |  |  |  | 6.7 | 10.9 | 4.2 |
|  | 6 |  |  |  | 4T8R | 4 | 13 |  |  |  |  | 10.2 | 15.6 | 5.4 |
| 2nd prio/optional | 7 |  |  | HomNet | 2T8R | 1 | 19 | 7.77 | 2.29 | TDLC300-100 | ULA Low | 6.4 | 12.6 | 6.2 |
|  | 8 |  |  |  |  | 2 | 13 |  |  | TDLA30-10 |  |  |  | 0 |
|  | 9 |  |  |  |  |  | 19 |  |  |  |  |  |  | 0 |
|  | 10 |  |  | HetNet | 2T8R | 1 | 19 | 7.58 | N/A | TDLA30-10 | ULA Low | 4.0 | 8.3 | 4.3 |
|  | 11 |  |  |  |  | 2 | 13 |  |  |  |  |  |  | 0 |
|  | 12 |  |  |  |  |  | 19 |  |  |  |  |  |  | 0 |

PDSCH requirements with intra cell inter user interference

Agreed setup

The following configuration parameters are extracted from the agreements made in RAN4#112bis (see [1]). For remaining parameters, we have re-used the configuration from in Rel.17 2Rx/4Rx:

| Parameter | Value | Comments |
| --- | --- | --- |
| Receivers | MMSE-MRC, MMSE-IRC (baseline), MMSE-IRC(simplified) |  |
| NoF CoUEs | 1 |  |
| Network type and SCS/CBW | - FDD: 15kHz/10MHz (initial analysis) - TDD: 30kHz/40MHz, 7D1S2U(S=6D+4G+4U) (Not needed for RAN4#113) |  |
| Propagation conditions | TDLA30-10 for all configurations. |  |
| Rank + MCS | Rank 1+1: MCS13, 2T8R ULA Low, random + orthogonal precoder Rank 2+2: MCS13, 4T8R ULA Low, orthogonal precoder Rank 4+4: MCS13, 8T8R ULA Low, orthogonal precoder |  |
| Precoder for target UE | Target UE: Random precoder |  |
| DMRS ports | - Rank 1+1 and 2+2, reuse same configuration as Rel-17 - Rank 4+4, use different CDM group for 4+4 case. Serving UE with ports {1000, 1001,1004, 1005} and co-scheduled UE with ports {1002, 1003, 1006, 1007} as a starting point. |  |
| Codebook | Type I Single Panel only |  |
| Other parameters | - Use symbols #0 and #1 of each slot for PDCCH - PDSCH mapping type A, Start symbol 2, Duration 12 - PDSCH allocation in frequency domain: Full PRB |  |

Simulation results

Table 2 show Nokia’s Baseline receiver PDSCH simulation results with intra-cell inter-user interference

Table 2: PDSCH simulation results with intra-cell inter-user interference (SNR [dB] @ 70% Max TP)

| Priority | TC Number | Duplex Mode and SCS | CHBW | MIMO | Rank for target + Co-UE | MCS (target UE) | Precoder selection | Channel Model | Antenna correlation | Modulation order for the co-scheduled UE | Nokia |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Baseline Receiver |  |  |  |  |  |  |  |  |  |  | MMSE-IRC | MMSE | Gain over MMSE |
| Priority for eval phase | 1 | FDD 15KHz SCS | 10 MHz | 2T8R | 1+1 | MCS 13 | Random | TDLA30-10 | ULA Low | 16QAM | 7.5 | 11.3 |  |
|  | 2 |  |  |  |  |  | Orthogonal |  |  |  | 5.2 | 5.7 | 0.5 |
|  | 3 |  |  | 4T8R | 2+2 | MCS 13 | Orthogonal |  |  |  | 8.3 | Inf |  |
|  | 4 |  |  | 8T8R | 4+4 | MCS 13 | Orthogonal |  |  |  | TBA | TBA |  |
| 2nd prio/optional | 5 |  |  | 2T8R | 1+1 | MCS 17 | Random | TDLA30-10 | ULA Low | 16QAM | 10.5 | TBA |  |
|  | 6 |  |  |  |  |  | Orthogonal |  |  |  | 8.1 | 10.2 |  |
|  | 7 |  |  | 4T8R | 2+2 | MCS 19 | Orthogonal |  |  |  |  |  |  |

Conclusion

In this paper we have provided Nokia’s simulation results for PDSCH with:

- Inter-cell interference.

- Intra-cell inter-user interference.

- References

- R4-2416555 - WF for [112bis][320] NR_demod_Ph5_Part2_UE