3GPP TSG-RAN WG4 Meeting #113	R4-2418003

Orlando, USA, 18-22 Nov 2024

Title: 	BS Signaling and UE behavior during initial connection

Agenda Item:	7.5.2.2

Source: 	Nokia

Document for:	Approval

# 1	Introduction

In previous RAN4 meeting WF on remaining open items was approved [1]. In this contribution we present out view for new BS Signaling and UE behavior during initial connection.

# 2	Discussio.

## 2.1 New BS Signaling

First open issue is the new BS signalling [1]

## < Issue 2-3: BS Signaling to switch between Type 4a/4b and Type 1 4L/CC(collocated) >

Way Forward:

- Continue further discussion on BS signalling in the next meeting.

## < Issue 2-4: BS Signaling to switch between Type 4a/4b and Type 1 6L/8L/CC(collocated) >

Way Forward:

- Continue further discussion on BS signalling in the next meeting.

In previous meeting our contribution [2] proposed to define new R19 network indication IEs to command UE to switch from type 4 to type 1 4L/CC and from type 4 to type 1 6L/8L/CC. If the BS signaling only indicates collocated type, there is still ambiguity on the number of layers UE is operating in type 1. In addition, UE may not be able to operate 6L/8L/CC in type 1 in some cases, then it would have to fall back to 4L/CC e.g. in bad channel condition. It is hence expected that network can indicate the maximum number of layers/CC besides collocation i.e. Type 1 indication. We still have the same view.

Proposal 1: Define new R19 network indication IEs to command UE to switch from type 4 to type 1 4L/CC or from type 4 to type 1 6L/8L/CC.

## 2.2 UE behavior during initial connection

Moderator topic summary[3] listed several options on this. We have re-considered our earlier proposal for this and concluded that KDDI, Samsung, Qualcomm proposals which build on number of MIMO layers can be the best way forward. Below we have captured KDDI proposal from [3]

  - Option 3: (KDDI)

Confirm the following understanding on maxMIMO-layers and Rel-18/19 BS signaling:

- The case of maxMIMO-layers larger than 4 is Type 1.

- The case of maxMIMO-layers equal to 4 and

  - Rel-19 New BS signaling is present with value 0 is Type 4.

  - Rel-19 New BS signaling is present with value 1 is Type 1.

  - Rel-19 New BS signaling is absent is that UE follows Rel-18 specification.

- The case of maxMIMO-layers equal to 2 and

  - Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is present is Type 1.

  - Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is absent is Type 2.

Observation: Above proposal from KDDI (Samsung, Qualcomm had similar) is a good basis. If our proposal 1 is agreed, then some modification is necessary.

# 3	Conclusion

In this paper we have proposed

Proposal 1: Define new R19 network indication IEs to command UE to switch from type 4 to type 1 4L/CC or from type 4 to type 1 6L/8L/CC.

# 4	References

[1] R4-2417115, WF on rest issues for intra-band non-collocated EN-DC/NR-CA, RAN4#112bis

[2] R4-2415373, On Rel-19 non-collocated deployment UE Capability/UE behavior and network signaling, RAN4#112bis

[3] R4-2415225, Topic summary for [112bis][121] NonCol_intraB_ENDC_NR_CA, RAN4#112bis