3GPP TSG-RAN WG4 Meeting #113	R4-2418011

Orlando, US, Nov 18th – Nov 22nd, 2024

Source:	Nokia

Title:	Simulation Results for on MIMO layer evaluation for 6Rx UE

Agenda item:	7.1.1.3.2

Document for:	Information

Introduction

During RAN#104 the work item on UE RF enhancements in Rel-19 was revised [2].

The performance part of this work item has not commenced yet, however discussions during RAN4#110-bis and RAN4#111 on the number of MIMO layers to support 6Rx has led to an Agenda Item to be introduced during RAN4#112 to specifically discuss MIMO layers for this Work Item with regards to 6Rx.

Specifically looking at the following aspects of the WID:

6Rx for handheld and FWA UE [RAN4, RAN1] - Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario - Example bands: n41, n77/n78, n79, n104 - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers - Specify the Rx requirements including reference sensitivity requirements for support 6Rx - Note: the specified requirements can be applicable to both handheld UE and FWA devices - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, depending on UE capability - RAN1 to study and specify, if necessary, SRS antenna switching to support 3T6R depending on UE capability. - Note: RAN1 discussion starts from RAN1#118bis (Oct. 2024) - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution.

In this document we present simulation results according to some proposals captured in the Way Forward from RAN4#112-bis [1].

Discussion

Simulation Parameters from RAN4#112-bis

Annex A from the WF [1] stipulates the following parameters which can be simulated for feasibility analysis, this is copied below to provide context for simulations.

| Parameters |  | Values |
| --- | --- | --- |
| Duplex mode |  | TDD |
| Bandwidth |  | 40MHZ |
| SCS |  | 30 |
| Antenna configuration |  | {8Tx, 6Rx} as baseline Interested companies can submit results with {32Tx, 6Rx} and specify the precoding type |
| Propagation channel |  | TDLA30-10 |
| Rank |  | Fixed rank {4,6}, interested companies can bring {5} |
| MCS |  | Adaptive MCS (target BLER 10%) MCS Table 2 Interested companies can bring MCS Table 1 |
| PDSCH configuration | Mapping type | Type A |
|  | Starting symbol | 2 |
|  | Length | 12 |
|  | PRB bundling size | 2 |
|  | VRB-to-PRB mapping type | Non-interleaved |
|  | Precoding | Random PMI (Rel-15 Type 1 codebook) for 8Tx SVD based precoding for 32Tx; companies are encouraged to provide details on the SVD update periodicity |
| PDSCH DMRS configuration | DMRS Type | Type 1 [Rel-15] and eType-1 [Rel-18] |
|  | DMRS Configurations | Option 1: Rel-18 1+1(6L), Rel-15 1+1(4L) Option 2: Rel-15 1+1(4L), Rel-15 2+0(6L) Option 3: Rel-15 1+1(4L), Rel-15 2+2(6L) Option 4: Rel-15 2+2(4L), Rel-15 2+2(6L) [To be further discussed online] |
|  | Precoding | Random PMI (Rel-15 Type 1 codebook) for 8Tx SVD based precoding for 32Tx, companies are encouraged to provide details on the SVD update periodicity |
| UE Correlation matrix |  | Option 1: Option 2: 1.000.100.101.000.300.10 0.300.150.100.141.000.12 0.130.120.300.200.200.120.150.140.130.300.120.20 0.121.000.200.110.120.30 0.110.301.000.100.101.00 Option 3: 1.000.220.221.000.300.580.600.150.150.100.050.150.300.600.580.151.000.150.151.000.150.150.150.100.150.050.100.150.150.150.150.101.000.300.301.00 Option 4: 1.000.100.101.000.250.350.500.100.100.050.050.100.250.500.350.101.000.100.101.000.100.150.100.050.100.050.050.100.100.100.150.051.000.150.151.00 Option 5: 3GPP MIMO correlation matrix with beta=0.00718 (Option 5 is parametric and to be justified since Options 1-4 are based on measurements) Note: For the feasibility analysis, antenna efficiency can be considered. Companies are encouraged to provide the antenna efficiency numbers. |
| BS correlation matrix |  | Option 1: 3GPP MIMO correlation matrix with Alpha = 0.1 Option 2: 3GPP MIMO correlation matrix with Alpha = 0.0 Other options are not precluded. Matrices requested from interested BS vendors for RAN4#113 |

Performance in CDL-C

The results that are shown in Figure 1 below are based on the following configuration:

| Parameters |  | Values |
| --- | --- | --- |
| Duplex mode |  | TDD |
| Bandwidth |  | 40MHZ |
| SCS |  | 30 |
| Antenna configuration |  | 8Tx, 6Rx as baseline |
| Propagation channel |  | CDL-C 365-100 |
| Rank |  | Fixed rank {4,6} |
| MCS |  | Adaptive MCS (target BLER 10%) MCS Table 2 |
| PDSCH configuration | Mapping type | Type A |
|  | Starting symbol | 2 |
|  | Length | 12 |
|  | PRB bundling size | 2 |
|  | VRB-to-PRB mapping type | Non-interleaved |
|  | Precoding | Random PMI (Rel-15 Type 1 codebook) for 8Tx SVD based precoding for 32Tx; companies are encouraged to provide details on the SVD update periodicity |
| PDSCH DMRS configuration | DMRS Type | Type 1 [Rel-15] and eType-1 [Rel-18] |
|  | DMRS Configurations | Rel-15 1+1(4L), Rel-15 2+0(6L) |
|  | Precoding | Random PMI (Rel-15 Type 1 codebook) for 8Tx |
| UE Correlation matrix |  | : 1.000.100.101.000.300.10 0.300.150.100.141.000.12 0.130.120.300.200.200.120.150.140.130.300.120.20 0.121.000.200.110.120.30 0.110.301.000.100.101.00 As presented, noting that the Huawei, Oppo and Qualcomm Matrix all performed similarly. |
| BS correlation matrix |  | Correlation matrix with Alpha = 0.0 |

Figure 1 : Throughput curves for 8T6R with 4 and 6 Layer in CDL-C 365-100.

Performance in TDL-A

The results that are shown in Figure 2 are based on the following configuration:

| Parameters |  | Values |
| --- | --- | --- |
| Duplex mode |  | TDD |
| Bandwidth |  | 40MHZ |
| SCS |  | 30 |
| Antenna configuration |  | 8Tx, 6Rx as baseline |
| Propagation channel |  | CDL-C 365-100 |
| Rank |  | Fixed rank {4,6} |
| MCS |  | Adaptive MCS (target BLER 10%) MCS Table 2 |
| PDSCH configuration | Mapping type | Type A |
|  | Starting symbol | 2 |
|  | Length | 12 |
|  | PRB bundling size | 2 |
|  | VRB-to-PRB mapping type | Non-interleaved |
|  | Precoding | Random PMI (Rel-15 Type 1 codebook) for 8Tx SVD based precoding for 32Tx; companies are encouraged to provide details on the SVD update periodicity |
| PDSCH DMRS configuration | DMRS Type | Type 1 [Rel-15] and eType-1 [Rel-18] |
|  | DMRS Configurations | Rel-15 1+1(4L), Rel-15 2+0(6L) |
|  | Precoding | Random PMI (Rel-15 Type 1 codebook) for 8Tx |
| UE Correlation matrix |  | : 1.000.100.101.000.300.10 0.300.150.100.141.000.12 0.130.120.300.200.200.120.150.140.130.300.120.20 0.121.000.200.110.120.30 0.110.301.000.100.101.00 As presented, noting that the Huawei, Oppo and Qualcomm Matrix all performed similarly. |
| BS correlation matrix |  | Correlation matrix with Alpha = 0.0 |

Figure 2 : Throughput curves for 8T6R with 4 and 6 Layer in TDLA 30-10.

Conclusion

The paper presents simulations to support the discussions in R4-2417559, no observations nor proposals are made.

- References

- R4-2417095, “WF on requirements for 6Rx”, AT&T, RAN4#112-bis, Hefei, October  2024

- RP-241656, “New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, Huawei, RAN#104 Shanghai, China, June 2024