3GPP TSG RAN WG4 #113		                                                       R4-2418025

Orlando, USA 18th – 22nd November 2024

Agenda item:	7.23.4

Source: 	Vodafone, vivo

Title: 	On LR and MR operating frequencies and the answers to RAN1 LS

Document for:	Discussion

- Introduction

In the last RAN1#118 meeting, RAN1 sent an LS to RAN2/RAN4 in R4-2414909/R1-2407559:

# 1 Overall description In RAN1, the common understanding is that UE may not support LP-WUS reception on all the bands supported by the UE. RAN1 respectfully asks RAN2 and RAN4 to check if there is any issue and specification support needed for IDLE/INACTIVE UEs. # 2 Actions To RAN2 and RAN4: ACTION: RAN1 respectfully asks RAN2 and RAN4 to check if there is any issue and specification support needed for IDLE/INACTIVE UEs.

Based on this LS, the following agreement was captured in the latest RAN4 WF (R4-2416861):

Issue 1-1-8: LP-WUR operating carrier frequency Agreement: - RAN4 will further discuss whether any technical issues regarding the LS from RAN1. - From Rel-19 RAN4 RRM requirement of MR offloading and relaxation perspective, RAN4 assumed LR and MR are operating on the same carrier frequency as baseline. FFS for the case of MR and LR working on different carrier frequencies if it is supported in RAN1/2. Issue for further discussion based on contribution driven: - For WUS monitoring assumption, further discuss whether or not the same carrier frequency is assumed for LR and MR.]

- Discussion

As highlighted in the RAN4 RRM WF R4-2416861, the current discussion in RRM has considered the case where the MR and LR are operating on the same carrier frequency as baseline for the definition of requirements for MR measurements offloading to LR and measurements relaxation. From the RAN4 RF perspective the WF R4-2417112  highlighted that there is no specific work related to the RAN1 LS.

As should be evident to all companies, in the case of the LR and MR operating on the same carrier frequency there should not be any further specification impact other than what is already envisioned by RAN4, since it is the common understanding that it is the baseline scenario.

Given that there are concerns from operators on precluding the support for the operation of MR and LR on different bands (R1-2408592/R2-2409157), from RAN4 RRM perspective there are two issues highlighted in the WF if this scenario is supported:

- Requirements for the offloading and relaxation of RRM measurements

- WUS monitoring assumptions

Starting with latter issue 2), from RAN4 perspective there should not be any impact as the WUS monitoring assumptions are in scope of RAN1/RAN2. As RAN1/RAN2 are the groups responsible to define the WUS monitoring occasions in relation to paging in the case of MR and LR operating with the same or different carrier frequency, there should not be any impact on the measurement requirements defined in RAN4.

Concerning the former issue in 1), there ought to be specification impacts on both offloading and relaxation of RRM measurements, given that the measurements performed by LR in a different frequency than the MR may not be accurate. From this understanding, we believe that RAN4 should discuss potential approaches to this issue.  One approach is to investigate the feasibility of defining entry/exit conditions concerning the operating frequencies of MR and LR and the deployment scenarios. A possibility to solve this issue was proposed already in RAN1 contribution (R1-2406296) with the definition of a delta factor to counteract the different frequencies of the two receivers:

LR and MR operating on different bands Another issue is offloading RRM measurement to LR when LP-SS and NR channels are located in different carriers/bands. Since LP-SS and SSB are located far away in frequency, the channel experienced by LP-SS and SSB may not be very similar, for example, when the measurement results by LR is still acceptable, the channel quality for NR may degraded to the level that paging can not be successfully received. But since the MR is deactivated, UE is not aware of it. In this case, when UE receives LP WUS and wake up MR, UE has to do cell reselection procedures first and then try to monitor PO. So the latency can be quite long, and UE may even miss the paging. One solution is that gNB configures a delta factor to reflect the difference exists between RRM measurements of LR and MR when they are in different bands. Proposal 22: Support offloading RRM measurement to LR when LP-SS and NR channels are located in different carriers/bands.

Of course there can be any other multitude solutions, but the important aspect to highlight is to not reduce the usage of the LR dramatically by restricting it to operate in the same carrier frequency as MR.

- How to proceed

RAN1 asked RAN4 if we see any problems if UEs are not supporting LP-WUS reception on all bands and we believe, we should answer that RAN4 sees a problem that the usage of the LR is limited when its operation is only at the same carrier frequency as MR.

RAN4 has also concerns on the impact of requirements for both offloading and relaxation of RRM measurements, given that the measurements performed by LR in a different frequency than the MR may not be accurate. Some solutions were proposed to mitigate this issue. From RAN4 RF perspective it was agreed that there is no specific work related to RAN1 LS.

Taking into account that the LS, we might send, will not reach RAN1 before the next plenary, we would suggest to address this LS to RAN plenary, RAN1 and RAN2.

This would help if companies would like to update the existing WI.

- DRAFT LS to RAN Plenary and RAN1

RAN4 thanks RAN1 for their LS in R4-2414909/R1-2407559 and would like to provide the following answer:

Question: “In RAN1, the common understanding is that UE may not support LP-WUS reception on all the bands supported by the UE.

RAN1 respectfully asks RAN2 and RAN4 to check if there is any issue and specification support needed for IDLE/INACTIVE UEs.”

Answer: During discussions, RAN4 sees the need to not restrict the usage of the LR dramatically by limiting its operation to the same carrier frequency as MR.

From RAN4 RRM perspective, LR measurement requirements depends only on LR band. For LR measurement requirements, there is no extra specification work on RAN4 if LR and MR work at the different band. From RAN4 RF perspective it was agreed that there is no specific work related to RAN1 LS.

RAN4 has also identified impacts on requirements for both offloading and relaxation of RRM measurements, given that the measurements performed by LR in a different frequency than the MR might not be accurate. Some solutions to mitigate this problem were proposed by a number of companies.

- References

- R4-2414909, LS on LP-WUS operation in IDLE/INACTIVE mode, RAN1, 3GPP RAN4#112-bis meeting, 14th – 18th October 2024.

- R1-2407559, LS on LP-WUS operation in IDLE/INACTIVE mode, Moderator (Apple), 3GPP RAN1#118 meeting, 19 – 23 August 2024.

- R4-2416861, WF on RRM requirements for NR_LPWUS, Moderator (vivo), 3GPP RAN4#112-bis meeting, 14th – 18th October 2024.

- R4-2417112, WF on UE RF requirements for LP-WUS, Moderator (vivo), 3GPP RAN4#112-bis meeting, 14th – 18th October 2024.

- R1-2408592, On LR and MR operating frequencies, Vodafone, 3GPP RAN1#118-bis meeting, 14th – 18th October 2024.

- R2-2409157, On LR and MR operating frequencies (related to LS in R1-2407559), Vodafone, 3GPP RAN2#127-bis meeting, 14th – 18th October 2024.

- R1-2406296, Discussion on LP-WUS operation in Idle/Inactive modes, Xiaomi, 3GPP RAN#118 meeting, 19 – 23 August 2024.

3GPP TSG-RAN WG4 Meeting #113		R4-XXX

Orlando, USA 18th – 22nd November 2024

Title:	DRAFT Reply LS on LP-WUS operation in IDLE/INACTIVE mode

Response to:	LS R1-2407559 from RAN1

Release:	Rel-19

Work Item:	NR_LPWUS-Core

Source:	Vodafone [to be RAN4]

To:	RAN Plenary, RAN1

Cc:	RAN2

Contact person:	Diogo Martins,	diogo.martins@vodafone.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

RAN4 thanks RAN1 for their LS on R4-2414909/R1-2407559 and would like to provide the following answer:

Question:

“In RAN1, the common understanding is that UE may not support LP-WUS reception on all the bands supported by the UE.

RAN1 respectfully asks RAN2 and RAN4 to check if there is any issue and specification support needed for IDLE/INACTIVE UEs.”

Answer:

During discussions, RAN4 sees the need to not restrict the usage of the LR dramatically by limiting its operation to the same carrier frequency as MR.

From RAN4 RRM perspective, LR measurement requirements depends only on LR band. For LR measurement requirements, there is no extra specification work on RAN4 if LR and MR work at the different band. From RAN4 RF perspective it was agreed that there is no specific work related to RAN1 LS.

RAN4 has also identified impacts on requirements for both offloading and relaxation of RRM measurements, given that the measurements performed by LR in a different frequency than the MR might not be accurate. Some solutions to mitigate this problem were proposed by a number of companies.

# 2	Actions

To RAN Plenary and RAN 1:

ACTION: 	RAN4 kindly asks RAN Plenary and RAN 1 to take the information above into account once deciding on the support of the use case mentioned above.

# 3	Dates of next TSG RAN WG 4 meetings

TSG-RAN WG4 Meeting #114 		17th – 21st February 2025		Athens, Greece

TSG-RAN WG4 Meeting #114-bis 	7th – 11th April 2025			TBC, China