3GPP RAN4 Meeting #113		R4-2418027

Orlando, USA, 18th – 22nd November, 2024

Agenda item:	7.12.2.1

Source:	Ericsson

Title:	Discussion on XR test method

WI/SI:	TRP_TRS_MIMO_OTA_Ph3-Core

Release:	Rel-19

Document for:	Approval

# 1	Introduction

During RAN4 #112bis meeting, the following agreements were made in WF [1].

This contribution provides our views related to the remaining issues regarding XR devices performance metrics and measurement time reduction solutions.

# 2	Discussion

## 2.1 Performance metric for XR devices

The performance of an antenna (radiation pattern, radiation efficiency, insertion loss, etc.) depends on its size, the used material, and the interactions with the near objects. Given the different form factors of the two XR device types (head-mounted devices and glasses), the different materials and the different positions on the head phantom, their antennas will have different performances. Hence, the two device types should have different performance requirements. Furthermore, since we are studying OTA requirements, it is worth using some antenna related parameters (e.g. maximum directivity, beamwidth, front-to-back ratio, etc.) as performance metrics.

Observation 1: The antennas of the two XR device types will have different performances. Hence, the two device types should have different performance requirements.

Proposal 1: The two XR device types (head-mounted devices and glasses) should have different performance requirements. The difference in performance requirements (delta) can be decided depending on the measurement campaign results.

Proposal 2: Use some antenna related parameters (e.g. maximum directivity, beamwidth, front-to-back ratio, etc.) as performance metrics.

## 2.1 Testing time reduction solutions for XR OTA

### 2.1.1 Coarse measurement grids for XR device testing

#### 2.1.1.1 TRP measurement

According to TS 38.161, “the TRP of the DUT is measured by sampling the radiated transmit power of the DUT with three-dimensional scan at various locations surrounding the device. The measurement is performed with a constant sampling step of 15 degrees in both theta (θ) and phi (φ) axes for TRP measurement”. However, TS 38.161 is mainly focused on handheld UEs with a different shape factor and antenna environment compared to XR devices.  Furthermore, as seen from TR 38.870, even for small size UEs that have quasi-omnidirectional antennas, TRP measurement error increases when increasing the grid step size as shown below:

XR devices have larger available area for antenna design. Thus, they can have more directive antennas that are more sensitive to the sampling grid step. Hence, XR devices’ TRP measurement should not use coarse measurement grids.

Observation 2: Considering the results given in TR 38.870 and the potential higher directivity of XR devices’ antennas compared to hand-held UEs’ antennas, the XR devices’ TRP should be measured without using a coarse grid measurement.

Proposal 3: For XR devices TRP testing, use the same procedure as in TS38.161 without using a coarse measurement grid, i.e. test with a grid size of 15 degrees.

#### 2.1.1.2 TRS measurement

According to TS 38.161, “the TRS of the DUT is measured by sampling effective isotropic sensitivity (EIS) of the DUT with three-dimensional scan at various locations surrounding the device. The measurement is performed with a constant sampling step of 30 degrees in both theta (θ) and phi (φ) axes for TRS measurement”. Given, the same arguments as in the case of TRP measurement, XR devices’ TRS measurement should not use coarse measurement grids.

Proposal 4: For XR devices TRS testing, use the same procedure as in TS38.161 without using a coarse measurement grid, i.e. test with a grid size of 30 degrees.

# 3	Conclusions

This contribution provides our views on the remaining issues regarding XR devices performance metrics and measurement time reduction solutions. The contribution makes the following proposals.

Proposal 1: The two XR device types (head-mounted devices and glasses) should have different performance requirements. The difference in performance requirements (delta) can be decided depending on the measurement campaign results.

Proposal 2: Use some antenna related parameters (e.g. maximum directivity, beamwidth, front-to-back ratio, etc.) as performance metrics.

Proposal 3: For XR devices TRP testing, use the same procedure as in TS38.161 without using a coarse measurement grid, i.e. test with a grid size of 15 degrees.

Proposal 4: For XR devices TRS testing, use the same procedure as in TS38.161 without using a coarse measurement grid, i.e. test with a grid size of 30 degrees.

# 4	References

R4-2416590, “Way Forward for [112bis][323] TRP_TRS_Ph3” vivo, 3GPP RAN4 #112bis.