3GPP TSG-RAN WG4 Meeting #113	R4-2418028

Orlando, USA, 18th – 22th November, 2024

Source: 	Qualcomm Incorporated

Title: 	Study of SCM Model Parameters for demodulation performance requirements

Agenda item:	7.14.2

Document for:	Discussion

- Introduction

In the previous RAN4 Meeting #112bis, we contributed to the SCM Study Item sharing considerations on the Study Item on the use spatial channel model for demodulation performance requirements.

According to the outcome of the discussion during the meeting and based on the approved Way Forward [3], we provide in this contribution further observations as part of the ongoing study.

# SCM Methodology Alignment

## Test Cases for study

With respect to the test cases to be simulated to study the Spatial Channel Modelling, RAN4 reached some tentative agreements in the previous meeting, which helped set up a baseline direction for the scenarios to focus on.

Considering that, we believe it would be productive for RAN4 to prioritize collection of SU-MIMO PDSCH test cases results, for the purposes of channel modelling alignment and study. After this step, RAN4 should further consider what is most appropriate to test and collect results for in the case of SU PMI and MU-MIMO.

Proposal 1: RAN4 to prioritize SU-MIMO PDSCH results collection and alignment with the agreed initial configuration for the purpose of SCM study and alignment. Baseline agreements for the configurations with Single User PMI and Multi-User MIMO can be postponed and reviewed once the PDSCH results are available and reviewed.

## Definition of the BS Antenna Framework for CDL Modeling

In the previous RAN4 meeting a lot of productive work was invested in defining a comprehensive framework that would allow for a clear and transparent understanding of the BS antenna panel configuration when used in test setup.

We believe that the result, pasted here below verbatim from R4-2416591 provides a very good reference, and as such it should be captured in the TR:

the BS antenna panel is composed by antenna elements arranged in columns and rows, single or dual polarized, and can be split in subarrays according to the following description: (M,N,P,Ms,Ns)

M is the number of antenna elements with the same polarization in each column (vertical size)

N is the number of antenna elements with the same polarization in each row (horizontal size)

P is the number of polarizations

Ms is the number of Subarray Elements with the same polarization in each column (vertical size)

Ns is the number of Subarray Elements with the same polarization in each row (horizontal size)

However, we would like to present one definition that is missing and might leave some room for misunderstanding: the usage of “subarray” is not specified in the text, and we propose for RAN4 to agree on the following definition:

Proposal 2: RAN4 to agree on the following definition for an Antenna Subarray: One subarray is a unique set of co-polarized antenna elements mapped exclusively to one CSI-RS Port through a virtualizer (vector of complex values).

If that is agreed, then RAN4 should capture the whole description in the TR:

Proposal 3: RAN4 to capture the description below in the TR:
The BS antenna panel is composed by antenna elements arranged in columns and rows, single or dual polarized, and can be split in subarrays. One subarray is a unique set of co-polarized antenna elements mapped exclusively to one CSI-RS Port through a virtualizer (vector of complex values).

The BS antenna panel configuration is defined as (M,N,P,Ms,Ns), according to the description below:

- M is the number of antenna elements with the same polarization in each column (vertical size)

- N is the number of antenna elements with the same polarization in each row (horizontal size)

- P is the number of polarizations

- Ms is the number of Subarray Elements with the same polarization in each column (vertical size)

- Ns is the number of Subarray Elements with the same polarization in each row (horizontal size)

## CDL Modelling: Number of Channel Taps

It is worth noting that in existing RAN4 Channel modelling based on 38.901, the established RAN4 methodology is based on a simplification procedure to reduce the number of channel taps to 12. This procedure is listed in 38.101-4, Annex B.2.1.

Observation 1: Existing methodology for Channel Modelling based on 38.901 in RAN4 is based on a simplified procedure to reduce the number of channel taps to a maximum of 12 taps for 5ns resolution or 16 taps for 2ns resolution.

For the purpose of CDL Channeling Modeling for RAN4 purposes, we believe that RAN4 should discuss how to address the extension of the existing procedure of simplification for Spatial Channel Models to reduce the number of taps.

Proposal 4: RAN4 to discuss whether the existing simplification procedure in 38.901 Annex B.2.1 is applicable to SCM, and if not how to extend the existing simplification methodology to reduce the number of channel taps for CDL channels based on 38.901.

## BS and UE Antenna Detailed parameters

In the WF there are some details regarding the choice of BS and UE parameters still open, and in this section we address our view on some of them, that will impact the future alignment of the results.

### BS and UE Antenna Elements

In particular, for the single BS Antenna Element pattern we think that there should ok to use the radiation power pattern defined in 38.901 Table 7.3-1.

Proposal 5: Support the radiation power pattern defined in 38.901 Table 7.3-1 to be used for Antenna Element Pattern at the BS;

On the other hand, we propose to clarify that the assumptions at the UE side exclude the application of any virtualization, and each antenna element is considered to be connected with one RX Chain:

Proposal 6: RAN4 to agree that the UE applies no virtualizer at the RX and that each Antenna Elements (omnidirectional as per previous agreements) is connected exclusively with one RX Chain;

### BS and UE GCS Location and Conversion Angles to LCS

Based on offline discussions after the last RAN4 meeting, we believe that the parameters that were agreed in the R4#112-bis WF need to be updated.

To that end, we support the following sets of parameters:

Proposal 7: For the gNB, use the following parameters for alignment with UMa CDL-C model:

- GCS Location Coordinates for the gNB:

  - Height = 25 m

  - Azimuth = 0 (placement on the x axis)

  - X Coordinate = 0 m

- GCS to LCS Conversion angles  (α,β,γ = bearing, downtilt, panel slant) for the gNB:

  - α,β,γ=0, 10, 0;

  - Polarization Slant: ζ=(+45) deg

Proposal 8: For the UE, use the following parameters for alignment with UMa CDL-C model:

- GCS Location Coordinates for the UE:

  - Height = 1.5 m;

  - Azimuth = 0; (placement on the x axis);

  - X Coordinate = 100 m; (Distance from gNB = 100 m);

- GCS to LCS Conversion angles  (α,β,γ = bearing, downtilt, panel slant) for the UE:

  - α,β,γ=180, 0, 0;

  - Polarization Slant: ζ=(0) deg

## Additional observations not captured in TR 38.827

### SNR Definition/Signal Power Normalization

In the previous meeting there were discussions regarding the choice of received power normalization and SNR definition.

While it was agreed offline that SNR definition needs not to be substantially changed from the established RAN4 methodology, we believe that the following should be captured in the WF and future TR for clarifications:

Proposal 9: For the purpose of received signal power normalization and SNR definition, RAN4 to consider capturing in the TR the following understanding: Assuming constant total received signal power summed across all RX RF Chain, the sum of Signal Power across all TX Beams is normalized using the spatially filtered long-term average PDP.

# TR Structure and how to capture additional CDL parameters for TR 38.827

## Proposal for Detailed TR Structure

In R4#112-bis RAN4 agreed the TR skeleton structure was agreed. However, it was not discussed how the agreements reached for channel modelling assumptions and parameters should be captured in the TR. To that end, we propose to discuss the following options:

Proposal 10: RAN4 should capture the detailed SCM approach used to generate simulation results for alignment in the TR according to one of the following options:

- Option 1: Copy/paste of the relevant portions of 38.827 and/or 38.901, adding extensions for RAN4 Demod Usage;

- Option 2: (Self Contained?) Rewrite of the “randomness reduction” changes from 38.827, specific to RAN4 Demod Usage;

- Option 3: (Self Contained) rewrite of the relevant portions of TR 38.901 including 38.827 Changes, specific to RAN4 Demod;

- Conclusions

Proposal 1: RAN4 to prioritize SU-MIMO PDSCH results collection and alignment with the agreed initial configuration for the purpose of SCM study and alignment. Baseline agreements for the configurations with Single User PMI and Multi-User MIMO can be postponed and reviewed once the PDSCH results are available and reviewed.

Proposal 2: RAN4 to agree on the following definition for an Antenna Subarray: One subarray is a unique set of co-polarized antenna elements mapped exclusively to one CSI-RS Port through a virtualizer (vector of complex values).

Proposal 3: RAN4 to capture the description below in the TR:
The BS antenna panel is composed by antenna elements arranged in columns and rows, single or dual polarized, and can be split in subarrays. One subarray is a unique set of co-polarized antenna elements mapped exclusively to one CSI-RS Port through a virtualizer (vector of complex values).

The BS antenna panel configuration is defined as (M,N,P,Ms,Ns), according to the description below:

- M is the number of antenna elements with the same polarization in each column (vertical size)

- N is the number of antenna elements with the same polarization in each row (horizontal size)

- P is the number of polarizations

- Ms is the number of Subarray Elements with the same polarization in each column (vertical size)

- Ns is the number of Subarray Elements with the same polarization in each row (horizontal size)

Observation 1: Existing methodology for Channel Modelling based on 38.901 in RAN4 is based on a

simplified procedure to reduce the number of channel taps to a maximum of 12 taps for 5ns resolution or 16 taps for 2ns resolution.

Proposal 4: RAN4 to discuss whether the existing simplification procedure in 38.901 Annex B.2.1 is applicable to SCM, and if not how to extend the existing simplification methodology to reduce the number of channel taps for CDL channels based on 38.901.

Proposal 5: Support the radiation power pattern defined in 38.901 Table 7.3-1 to be used for Antenna Element Pattern at the BS;

Proposal 6: RAN4 to agree that the UE applies no virtualizer at the RX and that each Antenna Elements (omnidirectional as per previous agreements) is connected exclusively with one RX Chain;

Proposal 7: For the gNB, use the following parameters for alignment with UMa CDL-C model:

- GCS Location Coordinates for the gNB:

  - Height = 25 m

  - Azimuth = 0 (placement on the x axis)

  - X Coordinate = 0 m

- GCS to LCS Conversion angles  (α,β,γ = bearing, downtilt, panel slant) for the gNB:

  - α,β,γ=0, 10, 0;

  - Polarization Slant: ζ=(+45) deg

Proposal 8: For the UE, use the following parameters for alignment with UMa CDL-C model:

- GCS Location Coordinates for the UE:

  - Height = 1.5 m;

  - Azimuth = 0; (placement on the x axis);

  - X Coordinate = 100 m; (Distance from gNB = 100 m);

- GCS to LCS Conversion angles  (α,β,γ = bearing, downtilt, panel slant) for the UE:

  - α,β,γ=180, 0, 0;

  - Polarization Slant: ζ=(0) deg

Proposal 9: For the purpose of received signal power normalization and SNR definition, RAN4 to consider capturing in the TR the following understanding: Assuming constant total received signal power summed across all RX RF Chain, the sum of Signal Power across all TX Beams is normalized using the spatially filtered long-term average PDP.

Proposal 10: RAN4 should capture the detailed SCM approach used to generate simulation results for alignment in the TR according to one of the following options:

- Option 1: Copy/paste of the relevant portions of 38.827 and/or 38.901, adding extensions for RAN4 Demod Usage;

- Option 2: (Self Contained?) Rewrite of the “randomness reduction” changes from 38.827, specific to RAN4 Demod Usage;

- Option 3: (Self Contained) rewrite of the relevant portions of TR 38.901 including 38.827 Changes, specific to RAN4 Demod;

- References

[1] RP-241610, “Study on spatial channel model for demodulation performance requirements”, 3GPP TSG RAN Meeting #104, Shanghai, China, 17th – 20th June, 2024;

[2] 3GPP TR 38.827 V16.8.0 (2022-09), “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study on radiated metrics and test methodology for the verification of multi-antenna reception performance of NR User Equipment (UE); (Release 16)”

[3] R4-2416591, “Way Forward Minutes for [112bis][318] NR_SCM”, 3GPP TSG-RAN WG4 Meeting #112-bis, Hefei, Anhui, China October 14 – 18, 2024