3GPP TSG-RAN4 Meeting #113	R4-2418052

# Orlando, US, 18th – 22nd November, 2024

Title:	Cross Link Interference (CLI) handling impact on MPR for SBFD operation

Source:	Qualcomm Incorporated

Agenda item:	7.20.1

WI code:	NR_duplex_evo-Core

Release:	Rel-19

Lead WG: 	RAN1

Document for:	Discussion

# 1.	Introduction

In the RAN plenary #104 meeting, RAN-P approved some schemes for CLI handling under the WI of evolution of NR duplex operation [1]. One of the schemes for handing inter-gNB CLI is based on UL resource muting of the PUSCH for both CP-OFDM and DFT-s-OFDM waveform. RAN4 is listed a WG to check the impact on transmit signal quality/MPR requirement.

- Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4] - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4] - Indication/determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-s-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain - PUSCH resource mapping, i.e., rate-matching around the muted REs - UCI resource determination in symbols with muted REs - Notes: Following are assumed for UL resource muting - No new DCI field / MAC CE is introduced for indication/determination of UL resource muting - No impact on data and control multiplexing as defined in section 6.2.7, TS 38.212 - UL resource muting does not apply for Msg A PUSCH and Msg 3 PUSCH - UL resource muting applies only for UEs in RRC_CONNECTED mode - UE assumes that the UL resource muting pattern does not overlap with UL DMRS or PT-RS in the same symbol - Power boosting is assumed for REs in the symbol with UL resource muting. PUSCH transmit power does not change across symbols - No changes on TBS determination for PUSCH - Check impact on transmit signal quality/MPR requirement, if any, with RAN4 - This feature is subject to UE capability

# 2. 	Discussion on PUSCH with RE muting

In this section, we describe how UL resource muting (RM) is applied to PUSCH waveform. The muting pattern in frequency domain is based on comb-2 structure, where every other RE is reserved and not used for PUSCH resource mapping. The UL-RM is applied only to two OFDM symbol within the PUSCH.

Figure 0-1: UL resource muting for PUSCH

In CP-OFDM, the muting is applied in frequency domain (pre IFFT) in the muted PUSCH OFDM symbols. Since, half of the REs are muted, the amplitude of the remaining REs are multiplied by sqrt (2), to keep the same power. This will lead to higher PSD in the muted symbols that may affect emissions which could in turn impact MPR

Observation 1: Resource muting changes the waveform characteristics and could have an impact on MPR.

For DFT-s-OFDM waveform, the muting can be implemented by using one of the followings methods.

- Method #1: The DFT size on the muted symbol is half compared to the non-muted OFDM symbols. Then, the complex valued symbols after DFT are oversampled by factor of 2 (i.e. mapped to every other RE) at the input of the IFFT.

- Method #2: The DFT size is the same, however, the input samples are repeated to generate a comb-2 RE muting post DFT.

From the recommendation above of resource muting, it can be observed that in time domain muting happens only for two symbols and not for the entire slot. This raises the question of how should the emissions be calculated now for SBFD operation i.e still at a slot level or a symbol level?

Observation 2: With symbol level resource muting, emissions calculation (like ACLR, EVM, IBE, etc.)  needs to be revisited.

# 3. 	Analysis

Based on the potential description of a waveform in section 2, other parameters described below could be used to generate a waveform.

- Setup:

  - Waveform type

  - Location and # of RBs : Based on the agreed sub-band configuration (a follow up discussion is required)

  - Tx power level

For the generated waveforms, linearity metrics namely ACLR, IBE, EVM, and others should be analyzed.

# Conclusion

We discussed an approach to construct a waveform to combat CLI in SBFD operation, its impact on UE performance should be studied and evaluated.

Observation 1: Resource muting changes the waveform characteristics and could have an impact on MPR.

Observation 2: With symbol level resource muting, emissions calculation (like ACLR, EVM, IBE, etc.)  needs to be revisited.

# Reference

[1]	3GPP TSG RAN Meeting #104, RP-241614 “New WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”