3GPP TSG-RAN4 Meeting #113	R4-2418053

# Orlando, US, 18th – 22nd November, 2024

Source:	LG Electronics

Title:	Tx requirements for SL intra-band contiguous CA

Agenda item:	7.3.3

Document for:	Discussion

Introduction

In this paper, we provide our views on UE RF requirements for intra-band contiguous SL CA for the Band n47 based on the approved WF[1].

Discussion

In WF[1], MPR requirement for PC2 SL intra-band C CA was agreed to be defined for 1x26dBm+1LO and 2x23dBm+1LO. And, MPR for PSSCH was agreed.

For MPR for PSFCH and S-SSB, based on the submitted [2] in the last meeting, we propose MPR requirement as follows.

< PC2 PSFCH MPR >

Table 2-1: PSFCH MPR for SL contiguous CA

|  | MPR for ratio (R) in bandwidth class B(dB) |  |  |
| --- | --- | --- | --- |
|  | R ≤ 0. 3 | 0.3 < R ≤ 0. 5 | 0.5 < R ≤ 1.0 |
| 1x26dBm+1LO | ≤ 5.0 | ≤ 10.0 | ≤ 14.0 |
| 2x23dBm+1LO | ≤ 6.0 | ≤ 11.0 | ≤ 15.0 |

Proposal 1: Consider PC2 PSFCH MPR in Table 2-1 for SL contiguous CA with 1x26dBm PA + 1LO, and 2x23dBm PA + 1LO.

< PC2 S-SSB MPR >

Table 2-2: S-SSB MPR for SL contiguous CA

|  | MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- | --- |
|  | Inner | Outer1 | Outer2 |
| 1x26dBm + 1LO | ≤ 4.0 | ≤ 9.5 | ≤ 14.0 |
| 2x23dBm + 1LO | ≤ 4.5 | ≤ 10.0 | ≤ 14.5 |

Proposal 2: Consider PC2 S-SSB MPR in Table 2-2 for SL contiguous CA with 1x26dBm PA + 1LO, and 2x23dBm PA + 1LO.

Conclusion

In this contribution, we provided PC2 PSFCH, and S-SSB MPR simulation results for SL intra-band contiguous CA. Based on those, the followings are proposed.

- MPR for SLCA PSFCH

Proposal 1: Consider PC2 PSFCH MPR in Table 2-1 for SL contiguous CA with 1x26dBm PA + 1LO, and 2x23dBm PA + 1LO.

Table 2-1: PSFCH MPR for SL contiguous CA

|  | MPR for ratio (R) in bandwidth class B(dB) |  |  |
| --- | --- | --- | --- |
|  | R ≤ 0. 3 | 0.3 < R ≤ 0. 5 | 0.5 < R ≤ 1.0 |
| 1x26dBm+1LO | ≤ 5.0 | ≤ 10.0 | ≤ 14.0 |
| 2x23dBm+1LO | ≤ 6.0 | ≤ 11.0 | ≤ 15.0 |

- MPR for SLCA S-SSB

Proposal 2: Consider PC2 S-SSB MPR in Table 2-2 for SL contiguous CA with 1x26dBm PA + 1LO, and 2x23dBm PA + 1LO.

Table 2-2: S-SSB MPR for SL contiguous CA

|  | MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- | --- |
|  | Inner | Outer1 | Outer2 |
| 1x26dBm + 1LO | ≤ 4.0 | ≤ 9.5 | ≤ 14.0 |
| 2x23dBm + 1LO | ≤ 4.5 | ≤ 10.0 | ≤ 14.5 |

- References

[1] R4-2417090, “WF on NR_SL_ intraB_CA_ITS_part2”, LG Electronics

[2] R4-2415894, “Tx requirements for SL intra-band contiguous CA”, LG Electronics