3GPP TSG-RAN WG4 Meeting #113 	R4-2418066

Orlando, Florida, USA, 18th – 22nd November, 2024

Source:	T-Mobile USA

Title:	TP for TR38.750 for DL CA_n66-n85 with PC2 2BUL

Agenda item:	6.8.4

Document for:	Approval

- Introduction

This contribution is a text proposal to introduce PC2 2BDL/2BUL for CA_n66-n85.

The power class 2 MSD is calculated by calculating the interference power for PC3 IMD MSD, increasing the interference by 3 dB * IMD order, and then calculating the resulting MSD for PC2. The equations are included.

- Reference

[1] RP-242003, Revised WID: Rel-19 High power UE (power class 1.5 or 2) for NR intra-band Carrier Aggregation (CA) or NR inter-band CA/Dual connectivity (DC) band combinations with/without NR Supplementary uplink (SUL)

- Text Proposal

<Beginning of Text Proposal>

## 5.x	CA_n66-n85

### 5.x.1	UE maximum output power

Table 5.x.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n66A-n85A | n668 CA_n66A-n85A8 | n66 | See n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n85 | See n85 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 8: Minimum requirements for Power Class 2 are applicable for this uplink combination with 1Tx antenna connector in each band or single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination |  |  |  |  |

Table 5.x.1-2: UE Power Class for uplink inter-band CA (two bands)

| Uplink CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n66A-n85A |  |  | 26y | +2/-3 | 23 | +2/-3 |  |  |
| NOTE y: The UE supports PC3 within each NR FDD band. |  |  |  |  |  |  |  |  |

### 5.x.2	Reference sensitivity requirements

For PC3, DL CA_n66A-n85A with UL CA_n66A-n85A has IMD4 MSD. This section will propose MSD for UL CA_n66A-n85A based on the PC3 MSD. The PC3 MSD for CA_n66-n85 is:

Table 5.x.2.1-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC3 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n66-n85 | n66 | 1770 | 5 | 25 | 2138 | 5 | FDD | IMD4 |
|  | n85 | 701 | 5 | 25 | 731 | N/A | FDD | N/A |

MSD for PC2 UL CA is calculated from IMD4 MSD for PC2 as follows:

If the input signal increases by 3 dB, the IMD4 increases by 3*4=12 dB.

MSD due to interference power is given by

where N is the noise spectral density and BW is the bandwidth of the carrier. If the initial MSD is known,

then we have

If is increased by dB, then is given by

The proposed value for PC2 UL CA MSD can be found in Table 7.3A.5-2a:

Table 5.x.2.1-2: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n66-n85 | n66 | 1770 | 5 | 25 | 2138 | 15.5 | FDD | IMD4 |
|  | n85 | 701 | 5 | 25 | 731 | N/A | FDD | N/A |

### 5.x.3	∆TIB and ∆RIB values (Void)

<End of Text Proposal>