3GPP TSG-RAN WG4 Meeting #113	R4- 2418067

Orlando, US, November 18 – 22, 2024

Source:	Nokia, Nokia Shanghai Bell

Title:	RRM requirements for on-demand SSB SCell operation

Agenda item:	7.22.2.1

Document for:	Discussion

- Introduction

At RAN4#113 meeting, the R19 WI on network energy saving was first discussed and the agreements were captured in the WF [1]. In this contribution, we continue the discussion on the RRM impact from on-demand SSB SCell operation.

- Discussion

At RAN4#112bis meeting, the followings were agreed on the general aspects of OD-SSB SCell operation.

- Sub-topic 1-1: General OD-SSB requirements Issue 1-1-2: OD-SSB impact on neighbor cell measurement - Agreement: - For the RRM measurement on neighboring cell, the legacy measurement framework is applied. - Note: The discussion of SMTC configuration is a separate issue. - This agreement can be revisited if it is not aligned with further agreement in other WGs. Issue 1-1-3: OD-SSB type - Agreement: - RAN4 to first take NCD-SSB as a start point for OD-SSB requirements.

OD-SSB impact on neighbor cell measurement

In last meeting, it was concluded to reuse the legacy measurement framework irrespective of OD-SSB cell, but there were different understandings if existing measurement requirement can still apply. In our view, we can see two cases:

- OD-SSB cell is a neighbor cell on a serving CC with legacy SCell

In this case, UE is required to perform measurements on the serving CC i.e. the servingCellMO including all the cells. As UE does not know if the OD-SSB is triggered in neighbor cell, it just measure the MO based on SMTC and may or may not be able to detect the OD-SSB neighbor cell. But if OD-SSB is activated in the neighbor cell, the existing measurement requirement shall still apply to the OD-SSB neighbor cell.

- OD-SSB cell is configured as an SCell

In this case, it was agreed a UE is not required to measure OD-SSB before OD-SSB is activated. Hence the existing measurement requirement for deactivated SCell in TS 38.133 clause 9.2 is applicable to OD-SSB SCell only if OD-SSB is activated. But this shall not impact the measurement on the legacy neighbor cells in the servingCellMO.. The UE is still required to perform the intra-frequency measurement on the serving MO as in legacy and existing measurement requirement for neighbor cells on the MO still apply.

- If there is an OD-SSB neighbor cell on a serving CC with legacy SCell, the existing measurement requirement based on SMTC shall apply to the OD-SSB neighbor cell only if OD-SSB is activated on the cell.

- If there is OD-SSB SCell on the serving CC, the existing measurement requirement for neighbor cells still apply to the legacy neighbor cells on the CC.

OD-SSB type

While RAN4 agreed to take NCD-SSB as a starting point, there seem to be different understanding what NCD-SSB is. We understood NCD-SSB means it is not associated with SIB1 and generally there is no limitation on the frequency location. That is, it may or may not be located on synchronization raster as shown in Fig.1. Meanwhile RAN1 agreed OD-SSB is not located in sync raster but there was different understanding if this is alternative or in addition to the NCD-SSB condition. To simplify the RAN4 discussion, we propose to start the discussion assuming OD-SSB is NCD-SSB and not on synchronization raster.

Fig. 1 OD-SSB SCell operation in and off sync raster (taking case 1 as example)

As in Case 1, there is only OD-SSB i.e. no always-on CD-SSB available in the cell for measurement, the UE is required to measure the OD-SSB once OD-SSB transmission is triggered. This requirement is independent of the OD-SSB location on synchronization raster or not. We don’t see any impact on RRM requirements in Case 1 because of OD-SSB being located on sync raster or not.

- RAN4 to start the discussion assuming OD-SSB is NCD-SSB and not on synchronization raster.

- For Case 1, there is no impact on RRM requirements because of OD-SSB being located on the sync raster or not.

  - OD-SSB based deactivated SCell measurement (Case 1)

Deactivated SCell measurement

From last meeting, a compromised solution was agreed on deactivated SCell measurement considering the energy saving on both network and UE sides. But the details are still FFS.

Issue 1-2-1: OD-SSB based deactivated SCell measurement(Case 1) Agreements: Once UE receives the OD-SSB activation indication, UE performs the OD-SSB based fast L3 measurement for deactivated SCell within a time window. - After the time window, UE can perform deactivated SCell measurement with a slow mode. - Option 2-1: fallback legacy requirement - Option 2-2: stop measurement - Option 2-3: No requirement - Time window means the time duration starting from receiving OD-SSB activation indication - FFS the time window configuration or pre-defined - FFS the fast L3 measurement requirement - FFS the relation between time window and OD-SSB deactivation command in the time domain When UE receives OD-SSB deactivation, UE is not required to measure the deactivated SCell.

In this WI, OD-SSB is introduced for network energy saving by minimizing the transmissions of SSBs. When UE receives the OD-SSB indication, it is expected to utilize the OD-SSBs for measurement as much as possible hence the measurement based on OD-SSB periodicity is preferred within the time window. This helps UE with a faster cell detection and measurement on the SCell, and network could deactivate the OD-SSB transmission at the earliest time for power saving. Hence, within the time window after receiving OD-SSB activation indication, the UE is expected to perform the OD-SSB based fast L3 measurement based on the OD-SSB periodicity hence the deactivated measurement requirement shall be defined based on OD-SSB periodicity.

- Within the time window after receiving OD-SSB activation indication, the deactivated measurement requirement is defined based on OD-SSB periodicity.

After the time window, we believe the UE is required to keep the measurement on OD-SSB unless it is deactivated in the SCell. Otherwise, the network triggered OD-SSBs would be wasted and cause inefficiency from energy saving point of view. But in order to save UE power, the UE can perform the measurement in a slow mode e.g. following MeasCycleSCell. In this case, the legacy measurement requirements on deactivated SCell can still apply.

- After the time window, the UE is required to keep the measurement on OD-SSB unless it is deactivated in the SCell.

- After the time window, the legacy measurement requirements on deactivated SCell apply based on MeasCycleSCell.

Regarding to the length of the time window, we think it can be defined as the time to identify the SCell based on OD-SSB periodicity e.g. Tidentify_intra_without_index or Tidentify_intra_with_index as specified in TS38.133 clause 9.2.5. Since the UE is able to acquire a valid measurement result within this time period, it can switch to a relaxed measurement mode afterward to save power.

- The time window is defined as the time to identify the SCell based on OD-SSB periodicity e.g. Tidentify_intra_without_index or Tidentify_intra_with_index as specified in TS38.133 clause 9.2.5.

According to TS38.133 clause 9.2.1, a UE is always required to perform intra-frequency measurements on a serving cell including deactivated SCells. However, for OD-SSB based deactivated SCell in Case 1, a UE is not required to perform SSB based measurement before OD-SSB is activated and after OD-SSB is deactivated. That is, the deactivated SCell measurement requirement would apply only if and once OD-SSB transmission is triggered. Hence for OD-SSB SCell in deactivated state, the intra-frequency measurement requirement applies only if OD-SSB transmission is activated.

- For OD-SSB operation in Case 1, the intra-frequency measurement requirement for deactivated SCell applies only when OD-SSB is triggered.

  - OD-SSB based deactivated SCell measurement (Case 2)

Relation between Always-on SSB and OD-SSB

In Case 2, it is assumed there is some regular or always-on SSBs in the SCell and OD-SSB may be triggered in addition. In last meeting, an LS was sent to RAN1 asking about the feasible scenarios in terms of the relation between always-on SSB and OD-SSB. These scenarios correspond to the respective cases in Fig.2.

Issue 1-3-1: Always-on SSB and OD-SSB time/frequency domain relation Agreement: - In OD-SSB Case 2, RAN4 to first discuss the deactivated SCell requirement for Scenario 1. RAN4 to further discuss whether to define the requirement for the scenario 2 and 3 if scenario 2 and 3 are supported in RAN1 based on RAN1’s agreement. - Scenario 1: OD-SSB and always-on SSB are with same SSB carrier frequency, same offset but different periodicities. - Scenario 2: OD-SSB and always-on SSB are with same SSB carrier frequency, different offsets and periodicities. - Scenario 3: OD-SSB and always-on SSB are within different SSB carrier frequencies. - Send the above information to RAN1, further discuss the text including the questions to RAN1 in the LS. - RAN4 will inform RAN1 if any further agreement.

Fig. 2 OD-SSB SCell operation in Case 2 and Scenario #2/#2A

In our view, it would be more energy efficient to assume the same time offset and different periodicity between OD-SSB and regular SSBs. While the SSB configuration is up to network, RAN4 could start the discussion from scenario 1 (i.e. Case 2-1) and scenario 3 (i.e. Case 2-3) as in Fig.2 while waiting for the RAN1 response to the LS.

- RAN4 to start the Case 2 discussion assuming OD-SSB and regular SSB have the same time offset and different periodicity on the same or different frequencies i.e. scenario 1 and scenario 3 in the LS.

Deactivated SCell measurement

When OD-SSB is triggered on a deactivated SCell, it needs to be discussed how a UE would perform the measurements and how to define the measurement requirement while both OD-SSB and regular SSBs are present. In our view, this depends on the relation between OD-SSB and regular SSB configurations which are illustrated in Fig.2.

- If OD-SSBs are on the same frequency and with same time offset with regular SSBs e.g. Case 2-1 in Fig.2, the regular SSBs may fully overlap with the OD-SSB in some of the time occasions. It is unclear whether regular SSB or OD-SSB is to be transmitted on the overlapping time occasions that is more up to RAN1 discussion. But from UE side, if there is no difference between the two types of SSBs, UE may assume OD-SSB overrides the regular SSBs hence no need to measure the regular SSBs. RAN4 needs to discuss how to define the measurement requirement when OD-SSB is activated.

- Otherwise, if the regular SSB is not on the same frequency with OD-SSB e.g. Case 2-3 in Fig.2, from existing configuration point of view they would be residing in different measurement object (MO)s as no MO can have multiple frequencies of SSB associated. This seems to be easiest approach in both RAN2 and RAN4 as if OD-SSB and regular SSB are on different frequencies it is not clear if they would actually be 1:1 due to differences in frequency they are transmitted.. So we think the measurements and reporting on OD-SSB and always-on SSB are up to network configuration and follow legacy behavior. From RAN4 point of view, the legacy measurement requirements apply to the MO with always-on SSB, and RAN4 can discuss how to define the measurement requirement for the OD-SSB when it is activated.

In any case, the SCell measurement in Case 2 depends on the scenarios and RAN1/RAN2 discussion on the SSB configuration, measurement configuration etc. We can wait for RAN1/2 progress before defining the measurement requirement.

- If OD-SSBs are on the same frequency and with same time offset with regular SSBs, UE assumes OD-SSB overrides the regular SSBs. RAN4 needs to discuss how to define the measurement requirement when OD-SSB is activated.

- If OD-SSB and regular SSB are not on the same frequency, follow legacy measurement behavior assuming they are configured as separate measurement objects. RAN4 can discuss how to define the measurement requirement on the MO with OD-SSB when it is activated.

  - OD-SSB based SCell activation delay

Known/unknown condition

On SCell activation delay, there was consensus to discuss the known/unknown condition irrespective of the OD-SSB operation cases and scenarios.

Issue 1-4-1: General Agreement: - RAN4 focuses on OD-SSB based SCell activation known and unknown requirement for both Case 1 and Case 2 and not specifically differentiate Scenario 2/2A scenarios. Issue 1-4-3: known cell condition design – principle Agreement: - The legacy known condition principle can be reused, which is defined based on if the UE has sent a valid measurement report during a ‘period’ before the reception of the SCell activation command. - When OD-SSB transmission is before/together with the SCell activation command (Scenario #2/Scenario #2A), UE follows the principles of legacy known/unknown SCell activation requirement once the known/unknown condition is met. - FFS how known/unknown condition is defined i.e. based on always-on SSB and/or OD-SSB - Other principles are not precluded.

For Case 1, the difference from legacy SCell activation is that the known/unknown condition is based on the measurement reporting on OD-SSB in the deactivated SCell. OD-SSB may or may not be triggered in different scenarios, but this will only impact the known/unknown condition of the SCell and the SCell activation delay requirement shall be defined for known/unknown conditions respectively based on OD-SSB measurement reporting.

In Case 2, the UE may be able to send the measurement reporting based on either regular SSB or OD-SSB. The known/unknown condition needs to consider the availability of any of the measurement reporting for the deactivated SCell. Hence the SCell shall be considered as known if the UE has sent a measurement reporting for either regular SSB or OD-SSB on the deactivated SCell. This can also cover the definition of known condition in Case 1.

- The SCell known/unknown condition shall be defined based on if the UE has sent a measurement reporting based on either OD-SSB or always-on SSB within a certain time period before receiving SCell activation command (in addition to the radio conditions).

As for the time period, this depends on the UE measurement behaviour after the time window discussed in Issue 1-2-1.  If the UE falls back to the legacy measurement on deactivated SCell following MeascycleSCell, the validity of the measurement reporting can still refer to the legacy time period max(5*measCycleSCell,  5*DRX cycles). This can minimize the impact on known condition.

- In the known condition, the time period is the same as in legacy SCell activation i.e. max(5*measCycleSCell,  5*DRX cycles).

There is also a question if the UE is using SMTC or SSB periodicity to activate the SCell. Since OD-SSB is configured to help network energy saving, a faster SCell activation based on OD-SSB periodicity would be preferred so that network may deactivate the OD-SSB transmission afterwards. Therefore, the SCell activation delay requirement can be defined based on OD-SSB periodicity instead of SMTC.

- For OD-SSB SCell, the SCell activation delay requirement is defined based on OD-SSB periodicity instead of SMTC.

SCell activation delay

In Case 1, OD-SSB shall be triggered no later than receiving SCell activation command in order to activate the SCell. The UE is expected to monitor OD-SSB immediately after receiving the SCell activation command. This is the same as in legacy SCell activation except OD-SSB is used instead of legacy SSBs. Therefore, exiting SCell activation delay requirement shall apply except OD-SSB is used instead of legacy SSBs.

However, in Case 2, OD-SSB may not be triggered at SCell activation. When receiving the SCell activation command, UE will perform the SCell activation procedure based on if OD-SSB is triggered at the time instance T2. In our view, OD-SSB shall be used for SCell activation if available in order to reduce the SCell activation delay e.g. scenario 2A. Otherwise, if OD-SSB is not triggered at the reception of SCell activation command, the regular SSB shall be used for SCell activation.

- For Case 1, existing SCell activation delay requirement shall apply except OD-SSB is used instead of legacy SSBs.

- For Case 2, RAN4 to define the SCell activation delay requirement based on OD-SSB if OD-SSB is triggered no later than the time instance when receiving the SCell activation command. Otherwise, the regular SSB shall be used for SCell activation.

  - Others

In RAN1#118 meeting, it was further concluded to at least support L1 measurement based on OD-SSBs. In our understanding, UE is required to perform only L3 measurements on a deactivated SCell i.e. L1 measurement would happen only after SCell is activated. This implies the OD-SSB will be triggered after SCell activation procedure is completed hence leads to scenario #3B. However, the scenarios other than scenario #2 and #2A were not concluded explicitly in RAN1.

We would like to wait for further RAN1 clarification on the applicable scenarios for L1 measurement before discussing the L1 measurement requirements based on OD-SSB in RAN4.

Agreements (RAN1#118) [Signalling] Agreement​ Update the previous RAN1 agreement as follows.​ - At least support L1 measurement based on on-demand SSB ​ - For L1 measurement based on on-demand SSB, periodic, semi-persistent, [and aperiodic] L1 measurement reports based on existing CSI framework are supported.​ - FFS on potential enhancements of CSI report configuration and/or triggering/activation mechanisms for L1 measurement based on on-demand SSB​ - The support of LTM is a separate discussion point​

Observation 1: A UE is required to perform only L3 measurements on a deactivated SCell i.e. L1 measurement would happen only after SCell is activated i.e. scenario #3B.

- Wait for further RAN1 clarification on the applicable scenarios for L1 measurement before discussing the L1 measurement requirements based on OD-SSB in RAN4.

- Conclusion

In this contribution, we discussed the RRM requirements for OD-SSB SCell operation based on RAN1/2 progress and presented our views on the potential RAN4 impact. The following observations and proposals are made:

- If there is an OD-SSB neighbor cell on a serving CC with legacy SCell, the existing measurement requirement based on SMTC shall apply to the OD-SSB neighbor cell only if OD-SSB is activated on the cell.

- If there is OD-SSB SCell on the serving CC, the existing measurement requirement for neighbor cells still apply to the legacy neighbor cells on the CC.

- RAN4 to start the discussion assuming OD-SSB is NCD-SSB and not on synchronization raster.

- For Case 1, there is no impact on RRM requirements because of OD-SSB being located on the sync raster or not.

- Within the time window after receiving OD-SSB activation indication, the deactivated measurement requirement is defined based on OD-SSB periodicity.

- After the time window, the UE is required to keep the measurement on OD-SSB unless it is deactivated in the SCell.

- After the time window, the legacy measurement requirements on deactivated SCell apply based on MeasCycleSCell.

- The time window is defined as the time to identify the SCell based on OD-SSB periodicity e.g. Tidentify_intra_without_index or Tidentify_intra_with_index as specified in TS38.133 clause 9.2.5.

- For OD-SSB operation in Case 1, the intra-frequency measurement requirement for deactivated SCell applies only when OD-SSB is triggered.

- RAN4 to start the Case 2 discussion assuming OD-SSB and regular SSB have the same time offset and different periodicity on the same or different frequencies i.e. scenario 1 and scenario 3 in the LS.

- If OD-SSBs are on the same frequency and with same time offset with regular SSBs, UE assumes OD-SSB overrides the regular SSBs. RAN4 needs to discuss how to define the measurement requirement when OD-SSB is activated.

- If OD-SSB and regular SSB are not on the same frequency, follow legacy measurement behavior assuming they are configured as separate measurement objects. RAN4 can discuss how to define the measurement requirement on the MO with OD-SSB when it is activated.

- The SCell known/unknown condition shall be defined based on if the UE has sent a measurement reporting based on either OD-SSB or always-on SSB within a certain time period before receiving SCell activation command (in addition to the radio conditions).

- In the known condition, the time period is the same as in legacy SCell activation i.e. max(5*measCycleSCell,  5*DRX cycles).

- For OD-SSB SCell, the SCell activation delay requirement is defined based on OD-SSB periodicity instead of SMTC.

- For Case 1, existing SCell activation delay requirement shall apply except OD-SSB is used instead of legacy SSBs.

- For Case 2, RAN4 to define the SCell activation delay requirement based on OD-SSB if OD-SSB is triggered no later than the time instance when receiving the SCell activation command. Otherwise, the regular SSB shall be used for SCell activation.

Observation 1: A UE is required to perform only L3 measurements on a deactivated SCell i.e. L1 measurement would happen only after SCell is activated i.e. scenario #3B.

- Wait for further RAN1 clarification on the applicable scenarios for L1 measurement before discussing the L1 measurement requirements based on OD-SSB in RAN4.

- References

[1]	R4-2416916, “WF on Netw_Energy_NR_enh”, Ericsson