3GPP TSG-RAN WG4 Meeting #113	R4-2418088

Orlando, USA, 18th - 22nd November 2024

Source:	Qualcomm Incorporated

Title:	3 Tx NR specifications

Agenda item:	7.19.2

Document for:	Approval

# 1.	Introduction

In RAN4#112-bis there were some discussion on 3 Tx issues. In this paper we present further views on some of the unresolved issues mentioned in the WF.

# 2. 	Discussion

The 3Tx WI was started in RAN1#116 and to date the following 1-layer, 2-layer and 3-layer TPMIs have been defined.

In RAN4#112 it was agreed to support 1-layer and 3-layer transmit precoder matrices from the above list. Also, in RAN4#112-bis companies agreed to support full power mode 0 for PC2 and PC3.

Observation 1: Based on previous agreements in RAN4 PC3 and PC2 support 1-layer and 3-layer transmit precoder matrices approved by RAN1.

It is not possible for a UE configured for multiple ports to declare PC1.5 in mode 0 because such a device would have to support the same power class for all ranks, and based on current technology the highest power achievable from a single PA is PC2.

Observation 2: Not possible to configure a UE for PC1.5 in mode 0 using a single PA based on current technology.

Another way of achieving PC1.5 operation is by using TxD where multi-PA transmission is allowed. However, for 3Tx TxD is precluded so PC1.5 power is not achievable.

Observation 3: Not possible for a UE to support PC1.5 if Tx diversity is not allowed and 1 PA cannot deliver PC1.5 power.

In RAN4#112-bis it was agreed that the following PA architectures would be used for deriving MPRs for PC3 and PC2 and it was also agreed that separate MPR requirements would be created for UE handheld and FWA/CPE devices:

For 3 port configuration 3x23dBm, PC3 will have to be supported by the single layer precoder matrices defined by RAN1 given below:

This means that at least 1 PA will have to be PC3 capable. So, for the 3X23dBm scenario the single PA PC3 MPR in table 6.2.2-1[1] would apply. For a 3-layer PC3 3Tx 3X23dBm architecture each PA would operate at 18.2dBm. This would mean that some MPRs will be smaller for certain operating conditions where non-linearities determine the MPR specification compared to the single Tx MPR values captured in the standard. However, it has been stated in section 6.2D.2 of the standard [1] :

According to section 6.2D.2 [1] for the 2 or 4 TX PC3 architecture the same MPR values are used as for the 1 PA case. Therefore, we suggest that for the 3 Tx architecture the same procedure be followed.

Proposal 1: For 3Tx 3X23dBm PC3 architecture use the PC3 single PA MPR values specified in Table 6.2.2-1 of TS 38.101-1 for UE handheld and CPE/FWA devices

Achieving PC2 with 3X23dBm PAs is not possible as no single PA can achieve PC2 power with the single layer precoder matrices shown above and TxD has also been precluded for 3Tx.

Observation 4:  Not possible to achieve PC2 with 3X23dBm PAs as no single PA can achieve PC2 power and TxD is precluded.

Achieving PC2 with 3X26dBm PAs is possible as all PAs can achieve PC2 power. For a 3-layer PC2 3Tx 3X26dBm architecture each PA would operate at 21.2dBm. This would mean that some MPRs will be smaller for certain operating conditions where non-linearities determine the MPR specification compared to the single Tx MPR values. Following the same methodology as used for PC3 detailed above it is proposed that the same MPR values as used for the single layer PC2 case captured in table 6.2.2-2 [1] be used for the 3X26dBm architecture targeting PC2 power.

Proposal 2: For 3 Tx 3X26dBm PC2 architecture use the PC2 single PA MPR values specified in Table 6.2.2-2 of TS 38.101-1 for UE handheld and CPE/FWA devices

As a UE supporting 3X23dBm architecture is not capable of achieving PC2 power using a single PA diversity will allow multiple PAs to be used to achieve the required power. It is proposed to allow a UE having a 3X23dBm PA architecture to indicate support for 2 Tx operation with diversity. This will enable a 3X23dBm PA architecture to support PC2. The MPR in this case would be given by table 6.2D.2-1 for DCI_0_0 and UL MIMO rank2

Proposal 3: For 3X23dBm for PC2 allow the UE to indicate support for 2Tx operation with diversity. MPR for this case is given in table 6.2D.2-1 [1]

# Conclusion

In this paper we further present our views on power class support for 3 Tx, MPR table creation and support for 2-layers in 3 Tx and make the following observations and proposals:

Observation 1: Based on previous agreements in RAN4 PC3 and PC2 support 1-layer and 3-layer transmit precoder matrices approved by RAN1.

Observation 2: Not possible to configure a UE for PC1.5 with mode 0 using a single PA based on current technology.

Observation 3: Not possible for a UE to support PC1.5 if Tx diversity is not allowed and 1 PA cannot deliver PC1.5 power.

Proposal 1: For 3Tx 3X23dBm PC3 architecture use the PC3 single PA MPR values specified in Table 6.2.2-1 of TS 38.101-1 for UE handheld and CPE/FWA devices

Observation 4:  Not possible to achieve PC2 with 3X23dBm PAs as no single PA can achieve PC2 power and TxD is precluded.

Proposal 2: For 3 Tx 3X26dBm PC2 architecture use the PC2 single PA MPR values specified in Table 6.2.2-2 of TS 38.101-1 for UE handheld and CPE/FWA devices

Proposal 3: For 3X23dBm for PC2 allow the UE to indicate support for 2Tx operation with diversity. MPR for this case is given in table 6.2D.2-1 [1]

# References

[1] TS38.101-1, User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone, V18.7.0

[2] RP-234007, New WID: NR MIMO Phase 5, RAN#102, December 11th – December 15th , 2023