3GPP TSG-RAN WG4 Meeting #113	R4-2418090

Orlando, USA, 18th - 22nd November, 2024

Source:	Qualcomm Incorporated

Title:	MPR for Intra-band contiguous UL CA for PC1.5

Agenda item:	7.1.1.1.2

Document for:	Approval

# 1.	Introduction

For this WI companies were requested to measure/simulate MPR/AMPR numbers for several intra-band contiguous and non-contiguous scenarios. In this paper we present details on the MPR measurements that were done on contiguous CC with contiguous RBs.

# 2. 	Discussion

In RAN4#111 a WF was created that stated the following [1]:

- Evaluate MPR/A-MPR numbers for contiguous CA first and then work on non-contiguous CA MPR/A-MPR afterwards.

- The intra-band PC1.5 ULCA R19 work focusses on UE implementing with two 26dBm PAs. Architectures requiring a 29dBm PA are not specified but can be implemented by fulfilling the requirements based on two 26dBm PAs.

- For intra-band contiguous ULCA w/ and w/o UL MIMO, the Architecture#2 is prioritized for MPR/A-MPR evaluation.

  - If time is allowed, architecture #1 can be evaluated.

| Architecture | Description | Indicated capability | Whether can support UL MIMO | Applicable cases |
| --- | --- | --- | --- | --- |
| #1 | 2x26 dBm PA + 2 LO with 100MHz BW | dualPA-Architecture | No | Contiguous CA |
| #2 | 2x26 dBm PA + 1 LO with 200MHz BW | TxD | Yes | Contiguous CA |

In this paper we describe our measurements for architecture#2 given in the above table for contiguous CA with contiguous RBs. The MPR for band n41C was measured using two 26 dBm PAs setup with an antenna-to-antenna isolation of 10 dB. The contiguous CCs with contiguous RBs scenario have RBs allocated in one CC exclusively, or in both CCs with the RB allocation being contiguous across the CC1 and CC2 RB boundary. Numerous RB allocations belonging to this scenario were measured for MPR performance. For the cases where RBs were present in both CCs their PSDs were set to be equal. Based on these measurements the following MPRs are suggested for intra-band contiguous CC with contiguous RBs for bandwidth class C operation.

| Modulation |  | MPR for bandwidth class C(dB) |  |
| --- | --- | --- | --- |
|  |  | inner | outer |
| DFT-s-OFDM | Pi/2 BPSK | 4.0 | 8.5 |
|  | QPSK | 4.0 | 8.5 |
|  | 16QAM | 4.0 | 8.5 |
|  | 64QAM | 6.5 | 8.5 |
|  | 256QAM | 8.5 | 9.0 |
| CP-OFDM | QPSK | 4.7 | 9.0 |
|  | 16QAM | 4.7 | 9.0 |
|  | 64QAM | 6.0 | 9.0 |
|  | 256QAM | 8.0 | 9.0 |

Table 1 – MPR for contiguous CC and contiguous RBs

It is proposed that the above MPRs be used for contiguous CC with contiguous RBs.

Proposal 1: For bandwidth class C for contiguous CC with contiguous RBs use the following MPR table

| Modulation |  | MPR for bandwidth class C(dB) |  |
| --- | --- | --- | --- |
|  |  | inner | outer |
| DFT-s-OFDM | Pi/2 BPSK | 4.0 | 8.5 |
|  | QPSK | 4.0 | 8.5 |
|  | 16QAM | 4.0 | 8.5 |
|  | 64QAM | 6.5 | 8.5 |
|  | 256QAM | 8.5 | 9.0 |
| CP-OFDM | QPSK | 4.7 | 9.0 |
|  | 16QAM | 4.7 | 9.0 |
|  | 64QAM | 6.0 | 9.0 |
|  | 256QAM | 8.0 | 9.0 |

# Conclusion

In this paper we present our measurements for contiguous CC with contiguous RB MPRs for various modulations and waveform types.

Proposal 1: For bandwidth class C for contiguous CC with contiguous RBs use the following MPR table

| Modulation |  | MPR for bandwidth class C(dB) |  |
| --- | --- | --- | --- |
|  |  | inner | outer |
| DFT-s-OFDM | Pi/2 BPSK | 4.0 | 8.5 |
|  | QPSK | 4.0 | 8.5 |
|  | 16QAM | 4.0 | 8.5 |
|  | 64QAM | 6.5 | 8.5 |
|  | 256QAM | 8.5 | 9.0 |
| CP-OFDM | QPSK | 4.7 | 9.0 |
|  | 16QAM | 4.7 | 9.0 |
|  | 64QAM | 6.0 | 9.0 |
|  | 256QAM | 8.0 | 9.0 |

# References

[1] R4-2410565, WF on higher power UE, May 20th – May 24th, 2024