3GPP TSG-RAN WG4 Meeting #113                                                               R4-2418098

Orlando, US, 18th – 22nd November, 2024

Source:	Samsung

Title:	On the potentially new requirements for SBFD operation for FR1 and FR2-1

Agenda item:			7.20.2.1

Document for:	Approval

# 1. Introduction

Based on Rel-18 study item on evolution of NR duplex operation, the support of subband non-overlapping full duplex has been studied, which is targeted to provide enhanced UL coverage, reduced latency, improved system capacity, and improved configuration flexibility for NR TDD operation [2].

In RAN#102, the work item on evolution of NR duplex operation (SBFD) has been approved, with WID further revised in the follow-up RAN plenary [1]. According to the objectives in WID, RAN1 is tasked to specify the mechanisms to support SBFD, including semi-static indication of time/frequency location, random access in SBFD symbols, and other transmission, reception and measurement behavior and procedures for SBFD aware UE. Furthermore, the enhancement for CLI handing, including gNB-to-gNB and UE-to-UE CLI handling, will also be specified in RAN1.

Accordingly, from RAN4 perspective, it is tasked to “Specify BS RF requirements for SBFD operation at gNB [RAN4]”, and this work shall be based on the good outcome from the study, i.e., Section 10.1, “Impact on BS RF requirements” in TR 38.858. Furthermore, the way forward encourages companies to continue contributing to ongoing discussions on (1) the modification of existing requirements for SBFD-capable BS and (2) potentially new requirement for SBFD operation, and address issues that have been raised during meetings [4]. In the RAN4#111, #112 and #112bis, further agreements are achieved on the potentially new requirements for SBFD operation [6] [8] [10]. In this contribution, accordingly, we would like to provide our analysis and viewpoints on the potentially new BS RF requirements for SBFD operation for FR1 and FR2-1.

# 2. Transient period requirement for SBFD

From Rel-18 study, the requirement of transmitter transient period will be introduced for the transmitter mode switching between SBFD and non-SBFD or SBFD reconfigurations if needed. RAN4 has confirmed that the transition period is needed between the non-SBFD slot and SBFD slot and vice versa. Furthermore, RAN4 has concluded that the transient period shall be introduced to BS in SBFD symbols.

For transmitter transient period between SBFD and non-SBFD or SBFD reconfigurations if needed, the requirement shall be introduced to BS in SBFD symbols/slots, by defining the transient period as the time period which the transmitter is changing from the SBFD operation to non-SBFD operation or vice versa, or during SBFD reconfigurations. Regarding the transition period requirement, RAN4 mainly focus on the transition period related with SBFD. Based on the RAN4 study, between the non-SBFD slot and SBFD slot and vice versa, a transition period is needed. If the SBFD configuration between adjacent SBFD slots is the same, then no transition period is needed.

The following agreements related to the transient period between SBFD and non-SBFD are achieved in RAN4#112 [8]:

#### <Agreement achieved in RAN4#112 [8]> #### Issue 2-1-1: Requirement for transient period between SBFD and non-SBFD - Agreement: - RAN4 shall use existing transient period requirement as baseline for the following four cases identified for SBFD: Frequency regions for Transition; BS transmitter behavior during transition. Case-A (SBFD to DL); SBFD UL subband and guardband(s); BS transmitter OFF-to-ON. Case-B (SBFD to UL); SBFD DL subband; BS transmitter ON-to-OFF. Case-C (DL to SBFD); SBFD UL subband and guardband(s); BS transmitter ON-to-OFF. Case-D (UL to SBFD); SBFD DL subband; BS transmitter OFF-to-ON. #### Issue 2-1-2: Transient period length between SBFD and non-SBFD - Agreement: - Transient period length between SBFD and non-SBFD: - The existing TDD BS transmitter transient period reused, i.e., 10us for FR1 and 3us for FR2-1. #### Issue 2-1-3: Location of transient period between SBFD and non-SBFD - Agreement: - Location of transient period between SBFD and non-SBFD in RAN4 RF requirement: - FFS the location of transient period between SBFD and non-SBFD shall be located within the SBFD slot/symbol if it needs to be decided. #### Issue 2-2-1: Requirement for transient period between different SBFD configurations - Agreement: - No need to specify transmitter transient period requirement between different SBFD configurations.

And the following way forward is achieved in RAN4#112bis [10].

#### <Agreement achieved in RAN4#112bis [10]> #### Issue 2-1-1: Transient period requirement applicability on frequency/antenna Way Forward: - FFS the text proposal for the clause of transmitter transient period for SBFD, e.g., the example provided in R4-2416498. #### Issue 2-1-2: Location of transient period between SBFD and non-SBFD Way Forward: - FFS location of transient period between SBFD and non-SBFD: - Option 1: No need to discuss and decide whether the transmitter transient period shall be located within the SBFD symbol or not. - Option 2: Place the transient period outside of both SBFD and non-SBFD slots. - Option 3: The location of the transient period between SBFD and non-SBFD shall be within the SBFD symbol).

It should be noted that RAN1 agreements as below, for both one or two TDD-UL-DL pattern(s) configured, SBFD symbols are configured in consecutive manner within each TDD-UL-DL pattern period. And the SBFD can be flexibly configured from/to any symbol of a certain slot.

<Agreements from RAN1#116> Agreement A slot can consist of SBFD symbols and non-SBFD symbols. Agreement A slot can consist of SBFD symbols and non-SBFD symbols. For semi-static indication of SBFD subband time location, - When only one TDD-UL-DL pattern is configured, SBFD symbols are configured in consecutive manner within a TDD-UL-DL pattern period. When two TDD-UL-DL patterns are configured and if SBFD symbols are configured for only one of the patterns, SBFD symbols are configured in consecutive manner within the TDD-UL-DL pattern period. When two TDD-UL-DL patterns are configured and if SBFD symbols are configured for both patterns, SBFD symbols are configured in consecutive manner within each TDD-UL-DL pattern period. - SBFD symbols are configured in DL and/or flexible symbols configured in TDD-UL-DL-ConfigCommon - The configured SBFD symbols can start from any symbol within a slot and can end in any symbol within a slot. - referenceSubcarrierSpacing in TDD-UL-DL-ConfigCommon is used as reference SCS. - FFS details <Agreements from RAN1#117> Agreement For RRC connected mode UEs, SBFD subband time period: - When only one TDD-UL-DL pattern is configured, the period is the same as TDD-UL-DL pattern period configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon. When two TDD-UL-DL patterns are configured, the period is the same as the sum of the two TDD-UL-DL pattern periods configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon. <Agreements from RAN1#118> Agreement For configuration of SBFD symbols within a TDD-UL-DL pattern period, the following parameters are supported - A starting slot index - A starting symbol index within the starting slot - An ending slot index - An ending symbol index within the ending slot

### 2.1 Requirement for transient period between SBFD and non-SBFD

Next issue to be discussed for transient period between SBFD and non-SBFD is how to capture the agreed transient period requirement into core requirement. Based on the agreement “RAN4 shall use existing transient period requirement as baseline for the following four cases identified for SBFD”, here we would like to identify the necessary changes to clause 6.4.2 and clause 9.5.3.

#### 2.1.1 Location of transient period between SBFD and non-SBFD

In our previous contribution [7] [9], different cases (Case-A, B, C and D) are identified and denoted for four cases of transitions to/from SBFD symbols, which is captured in the agreement in RAN4#112 [8].

To better illustrate the Case-A, B, C and D, the following SBFD time-domain configuration is given, in which two TDD-UL-DL patterns are configured, e.g, DDDSU and DDSUU.

Figure 1: Illustration of SBFD time-domain configuration

Corresponding to the above Figure 1, the detailed configurations of SBFD symbols within the above TDD-UL-DL patterns (DDDSU and DDSUU) are provided by the following parameters:

| TDD-UL-DL pattern 1 (DDDSU) | Starting slot index = 1 |
| --- | --- |
|  | Starting symbol index within the starting slot = 0 |
|  | Ending slot index = 2 |
|  | Ending symbol index within the ending slot = 13 |
| TDD-UL-DL pattern 2 (DDSUU) | Starting slot index = 0 |
|  | Starting symbol index within the starting slot = 0 |
|  | Ending slot index = 2 |
|  | Ending symbol index within the ending slot = 13 |

Right now, we would like to provide the analysis for individual case:

- Case-A (SBFD symbol to DL symbol): For the DL subband(s), we expect there is no transient period issue since the BS transmission is kept from SBFD symbol to DL symbol. For UL subband, we expect TA_offset is still required for SBFD UL subband, which shall be longer than the required transient period (10us for FR1 and 3us for FR2), so it is straightforward that the TA_offset can accommodate the required transient period.

- Case-B (SBFD symbol to UL symbol): For DL subband(s), we expect UE shall follow legacy TDD behaviour, i.e., guard symbols located in D/F-slot is required to accommodate (1) 10us (or 3us) transient period; (2) TA_offset. For UL subband, TA_offset shall be applied to the timing in SBFD symbol and the follow-up UL symbol, and there is no BS transient period issue at all.

- Case-C (DL symbol to SBFD symbol): For the DL subband(s), we expect there is no transient period issue since the BS transmission is kept from DL symbol to SBFD symbol. For UL subband, we expect UE shall follow legacy TDD behaviour, i.e., guard symbols should be reserved in D/F-slot to accommodate (1) 10us (or 3us) transient period; (2) TA_offset.

- Case-D (UL symbol to SBFD symbol): For DL subband, just follow legacy TDD behaviour, i.e., the transient period can be accommodated in TA_offset. For UL subband, TA_offset shall be applied to the timing in SBFD symbol and the preceeding UL symbol, and there is no BS transient period issue at all.

Give the above analysis, the following observation is provided:

Observation 1: For the identified four cases of transitions to/from SBFD symbols, the following timing behaviors are expected:

|  | Expected timing behavior for DL/UL subbands respectively |  | Location of transient period (if any) |
| --- | --- | --- | --- |
| Case-A (SBFD symbol to DL symbol) | DL subband(s) | No transient period issue since the BS transmission is kept from SBFD symbol to DL symbol. | N/A |
|  | UL subband | TA_offset is still required for SBFD UL subband, which shall be longer than the required transient period (10us for FR1 and 3us for FR2), so it is straightforward that the TA_offset can accommodate the required transient period. | Located in TA_offset in SBFD symbol |
| Case-B (SBFD symbol to UL symbol) | DL subband(s) | UE shall follow legacy TDD behaviour, i.e., guard symbols located in D/F-slot is required to accommodate (1) 10us (or 3us) transient period; (2) TA_offset. | Located in guard symbols reserved in SBFD symbols |
|  | UL subband | TA_offset shall be applied to the timing in SBFD symbol and the follow-up UL symbol, and there is no BS transient period issue at all | N/A |
| Case-C (DL symbol to SBFD symbol) | DL subband(s) | No transient period issue since the BS transmission is kept from DL symbol to SBFD symbol. | N/A |
|  | UL subband | UE shall follow legacy TDD behaviour, i.e., guard symbols should be reserved in D/F-slot to accommodate (1) 10us (or 3us) transient period; (2) TA_offset. | Located in guard symbols reserved in non-SBFD DL symbols |
| Case-D (UL symbol to SBFD symbol) | DL subband(s) | UE shall follow legacy TDD behaviour, i.e., the transient period can be accommodated in TA_offset | Located in TA_offset in non-SBFD UL symbol |
|  | UL subband | TA_offset shall be applied to the timing in SBFD symbol and the preceeding UL symbol, and there is no BS transient period issue at all. | N/A |

Based on the above analysis, we see the location of transient period is in TA_offset or guard symbols reserved in D/F-slot.

Observation 2: The location of transient periods should be located in (1) SBFD symbol for Case-A; (2) SBFD symbol(s) reserved as guard symbol(s) for Case-B; (3) Non-SBFD DL symbol(s) reserved as guard symbol(s) for Case-C; (4) Non-SBFD UL symbol for Case-D.

On the other hand, we can see that transient period is defined for different cases while there is no need in RAN4 specification to specify the detailed location of transmitter transient period. Moreover, from RAN4 conformance requirement perspective, the test on the “OFF” power on the OFF period is performed (clause 6.4.2.4 in TS 38.141-1):

3)	Measure the mean power spectral density over 70/N μs filtered with a square filter of bandwidth equal to the RF bandwidth of the antenna connector or TAB connector centred on the central frequency of the RF bandwidth. 70/N μs average window centre is set from 35/N μs after end of one transmitter ON period + 10 μs to 35/N μs before start of next transmitter ON period – 10 μs. N = SCS/15, where SCS is Sub Carrier Spacing in kHz.

Similar to the transient period for TDD operation, the similar approach can be used, which also requires no necessity of specifying the detailed location of transmitter transient period

Proposal 1: No need to discuss and decide whether the transmitter transient period shall be located within the SBFD symbol or not.

Particular for the cases in which the transient period is located in guard symbols reserved in Non-SBFD DL symbol(s), i.e., Case-C, the explicit guard should be reserved to allow UE transition from receiving to transmitting, in which the transient period, two-way propagation delay shall be accommodated for UL subband. From RAN1 perspective, another issue is to decide whether the reserved guard symbol should span over the UL subband or the whole subband, and the relevant configuration for that. However, our understanding is this is within the RAN1 scope while has no impact on the RAN4 discussion for transient period requirement.

Observation 3: For the cases in which the transient period is located in guard symbols reserved in Non-SBFD DL symbol(s), i.e., Case-C, the explicit guard should be reserved to allow UE transition from receiving to transmitting, in which the transient period, two-way propagation delay shall be accommodated for UL subband.

#### 2.1.2 Transient period requirement applicability on frequency/antenna

As we analysis in our previous contribution, different from the existing transient period for TDD operation (from transmitter OFF period to transmitter ON period or vice versa), the transient period for SBFD operation is more complex in terms of:

- Transmitter OFF-to-ON and ON-to OFF happen in part(s) of frequency resources: e.g., Case-A (SBFD to DL), the transmitter OFF-to-ON happens only on SBFD UL subband and guardband(s), while the transmitter may continue on the SBFD DL subband.

- Transmitter OFF-to-ON and ON-to OFF happen in part(s) of antenna elements: e.g., Case-A (SBFD to DL), the transmitter OFF-to-ON happens only on the antenna elements in SBFD TX panel.

Observation 4: The transient period for SBFD operation is different from TDD operation in terms of:

- Transmitter OFF-to-ON and ON-to OFF happen in part(s) of frequency resources, and

- Transmitter OFF-to-ON and ON-to OFF happen in part(s) of antenna elements.

In Rel-18 RAN1/4 study, the separate TX/RX antenna array model as below, and three SBFD antenna configuration options (provided in annex A.5 in TR 38.858) have been studied:

Figure A.5-1: Separate-Tx/Rx antenna array model For evaluation and comparison between SBFD and legacy TDD, three options can be used. - SBFD antenna configuration option-1: The total number of antenna elements of the antenna array for SBFD is the same as the total number of antenna elements of the antenna array for legacy TDD. The total number of TxRUs of the antenna array for SBFD is the same as the total number of TxRUs of the antenna array for legacy TDD. - SBFD antenna configuration option-2: The total number of antenna elements of the antenna array for SBFD is two times of the total number of antenna elements of the antenna array for legacy TDD. The total number of TxRUs of the antenna array for SBFD is the same as the total number of TxRUs of the antenna array for legacy TDD. - SBFD antenna configuration option-3: The total number of antenna elements of the antenna array for SBFD is the same as the total number of antenna elements of the antenna array for legacy TDD. The total number of TxRUs of the antenna array for SBFD is half of the total number of TxRUs of the antenna array for legacy TDD.

For SBFD antenna configuration option-1, as illustrated in the below figure, the SBFD-relevant BS transmitter ON-to-OFF could happens in TX and RX antenna panels respectively for Case B (SBFD to UL) and Case C (DL to SBFD), and the SBFD-relevant BS transmitter OFF-to-ON could happens in RX and TX antenna panels respectively for Case A (SBFD to DL) and Case D (UL to SBFD):

Case-D (UL to SBFD)

Case-B (SBFD to UL)

Case-C (DL to SBFD)

Case-A (SBFD to DL)

Figure 1: SBFD-relevant transmitter ON-to-OFF and OFF-to-ON transient period for SBFD antenna configuration Option-1

Case-B (SBFD to UL)

Case-D (UL to SBFD)

For SBFD antenna configuration option-2 and 3, even with different methods of usage of TXRUs and antenna elements (provided in annex A.5 in TR 38.858), the situation is simpler since the BS TX only happens in TX antenna panel. Take the below Option 2 (Method 2-1) as example, SBFD-relevant BS transmitter ON-to-OFF and OFF-to-ON only happens in the TX antenna panel, as illustrated below:

Figure 2: SBFD-relevant transmitter ON-to-OFF and OFF-to-ON transient period for SBFD antenna configuration Option-2 (Method 2-1)

By summarizing the above observations, the transient period for SBFD operation could be further summarized as:

Observation 5: The transient period for SBFD operation could be further summarized as below, by elaborating the four different cases for different SBFD antenna configuration:

|  | Frequency regions for transition | SBFD antenna configuration Option-1 (Same panel area as TDD) | SBFD antenna configuration Option-2&3 (Twice panel area as TDD) | BS transmitter behavior during transition |
| --- | --- | --- | --- | --- |
| Case-A (SBFD symbol to DL symbol) | SBFD UL subband and guardband(s) | SBFD RX antenna panel | N/A | BS transmitter OFF-to-ON |
| Case-B (SBFD symbol to UL symbol) | SBFD DL subband | SBFD TX antenna panel | SBFD TX antenna panel | BS transmitter ON-to-OFF |
| Case-C (DL symbol to SBFD symbol) | SBFD UL subband and guardband(s) | SBFD RX antenna panel | N/A | BS transmitter ON-to-OFF |
| Case-D (UL symbol to SBFD symbol) | SBFD DL subband | SBFD TX antenna panel | SBFD TX antenna panel | BS transmitter OFF-to-ON |

Firstly, based on WF [10], it is agreed to further study the text proposal for the clause of transmitter transient period for SBFD, e.g., the example provided in our previous contribution [9, R4-2416498]. We have updated the general description for the clause of transmitter transient period for SBFD, by removing the controversial contents related to TX/RX antenna panels.

Proposal 2: Based on the example given in R4-2416498, the text proposal is updated for the clause of transmitter transient period for SBFD:

=================== Start of Text Proposal ===================

#### 6.4.2B	Transmitter transient period for SBFD

##### 6.4.2.1B	General

Transmitter transient period for SBFD requirements apply only to SBFD operation of the BS.

The transmitter transient period for SBFD is the time period during which the transmitter is changing from the transmitter OFF period to the transmitter ON period or vice versa. The transmitter transient period is illustrated in figure 6.4.2.1-1.

The transmitter transient period for SBFD shall be measured in the SBFD TX and RX antenna penal and transmission bandwidth configuration for SBFD DL subband, UL subband and guardband, depending on different SBFD cases antenna configuration:

Table 6.4.2.1B-1: Transmitter transient period for SBFD

|  | Frequency regions for transition | SBFD antenna configuration Option-1 (Same panel area as TDD) | SBFD antenna configuration Option-2&3 (Twice panel area as TDD) | BS transmitter behavior during transition |
| --- | --- | --- | --- | --- |
| Case-A (SBFD symbol to DL symbol) | SBFD UL subband and guardband(s) | SBFD RX antenna panel | N/A | BS transmitter OFF-to-ON |
| Case-B (SBFD symbol to UL symbol) | SBFD DL subband | SBFD TX antenna panel | SBFD TX antenna panel | BS transmitter ON-to-OFF |
| Case-C (DL symbol to SBFD symbol) | SBFD UL subband and guardband(s) | SBFD RX antenna panel | N/A | BS transmitter ON-to-OFF |
| Case-D (UL symbol to SBFD symbol) | SBFD DL subband | SBFD TX antenna panel | SBFD TX antenna panel | BS transmitter OFF-to-ON |

For BS type 1-C this requirement shall be applied at the antenna connector supporting transmission in the operating band.

For BS type 1-H this requirement shall be applied at each TAB connector supporting transmission in the operating band.

=================== End of Text Proposal ===================

# 3. Discussion on in-channel adjacent subband requirements

### 3.1 In-channel adjacent subband leakage ratio

For co-channel inter-gNB interference perspective (including inter-site and co-site inter-sector cases), we also see the difficulty to define a new requirement to control the interference into a “proper” level. It should be noted that the enhancements for CLI handling likely to be specified in this release are under RAN1 discussion, including beam nulling, beam pairing and non-transparent UL resource muting. Then all these schemes to be introduced in RAN1 (see below RAN1 agreement from RAN1#116bis) will greatly mitigate the gNB-to-gNB CLI, and make it be hard to have a criterion for a “proper” interference level.

Agreement If beam nulling is supported for gNB-to-gNB CLI handling, the following are recommended to be specified - Information exchange of measurement resource configuration, i.e., periodic NZP CSI-RS Agreement If beam pairing is supported for gNB-to-gNB CLI handling, the following are recommended to be specified - Information exchange of measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS - Information exchange of recommended/not-recommended DL beam information and associated resource configuration Agreement If non-transparent UL resource muting is supported for gNB-to-gNB CLI measurement for gNB-to-gNB CLI handling, the following are recommended to be specified - Definition and indication of UL resource muting pattern - Collision with DMRS/PTRS - PUSCH resource mapping, i.e., rate-matching around the muted REs - UCI resource determination - Power allocation in symbols with muted REs considering potential impact to phase continuity - TB size determination - Exchange of information across gNBs on measurement resources Note: The existing reference signal time-frequency resource pattern, e.g., CSI-RS, are used to determine the UL resource muting pattern. Note: Consider pattern without adverse impact on PAPR Note: The potential impact on transmit signal quality/MPR requirement may need to checked with RAN4. Note: The above does not apply for PUSCH transmission during random access procedures.

Observation 6: All the gNB-to-gNB CLI handling schemes likely to be introduced in RAN1 (including beam nulling, beam pairing and non-transparent UL resource muting) will greatly mitigate the gNB-to-gNB CLI, and make it be hard to have a criterion for a “proper” interference level or in-channel adjacent subband leakage ratio.

Based on the above observation, it is hard to obtain the required level of in-channel adjacent subband leakage ratio, because the system-level evaluation for the required in-channel adjacent subband leakage ratio shall take the CLI handling into account, which could propose different level of required leakage ratio for certain level of system performance degradation.

Observation 7: For system-level evaluation (given a certain level of system performance degradation, e.g., 5%), the required in-channel adjacent subband leakage ratio levels can be different if adopting different inter-gNB CLI handling schemes.

Based on RAN4 study outcome from Rel-18, it is concluded that the necessity of in-channel adjacent subband leakage ratio needs further study. For self-interference perspective, for the in-channel adjacent subband leakage ratio, based on companies’ proposals, the purpose is to make sure the SBFD operation without issues. However, we see the difficulty to specify a reasonable requirement accordingly because the RSIC budget over various component capabilities can be an implementation-specific issue, which is highly depends on vendors’ choice. For instance, with or without TX DPD could have significant impact on in-channel adjacent subblock leakage ratio, while RAN4 can’t specify the requirement based on implementation with DPD since some vendors may use other methods to deliver the similar overall RSIC capability to make sure SBFD operate well.

Observation 8: It is difficult for RAN4 to agree on a reference scheme for self-interference suppression implemented to derive the potential new requirement in-channel adjacent subband leakage ratio.

Proposal 3: There is no necessity to introduce new requirement for in-channel adjacent subband leakage ratio.

### 3.2 In-channel adjacent subband blocking and adjacent subband selectivity

Similar story for the potential new metric, in-channel adjacent subblock blocking/selectivity: with or without RF SIC and other RF solutions to mitigate RX chain saturation and co-channel interference, the required in-channel adjacent subblock blocking requirement can be significantly different, while it is hard for RAN4 to agree on a single RF architecture and the required level of blocking and selectivity to derive the requirement.

Based on last meeting’s discussion, some companies see the necessity of in-channel adjacent subband blocking requirement because they observed that DL sub-band interference might cause some non-linearity similar as in-band blocking signal. It should be noted that the RAN1 introduced gNB-gNB CLI handling schemes have the potential to reduce the co-channel interference which will greatly mitigate this problem.

Observation 9: Considering the RAN1-introduced gNB-gNB CLI-handling mechanism, the co-channel interference from neighboring gNB DL subband transmission could be greatly mitigated.

Furthermore, even without any CLI handling mechanism, based on our evaluation for all scenarios in FR1 and FR2-1, co-channel interference from neighboring BS is not significant as adjacent channel interference, as shown in the following figures providing the interference level CDF curves.

| Scenario 1: FR1 UMa-UMa |  |
| --- | --- |
| Scenario 3: FR1 Indoor-Indoor |  |
| Scenario 5: FR1 UMi-UMi |  |
| Scenario 6: FR2 UMa-UMa |  |
| Scenario 9: FR2 Indoor-Indoor |  |

Observation 10: The blocking from co-channel gNB DL subband interference is not as important as adjacent channel gNB DL interference.

Considering the above analysis, the blocking level from co-channel gNB DL subband interference is not the major factor, compared with the adjacent channel gNB DL interference, we see no necessity to introduce new requirement for in-channel adjacent subband blocking and adjacent subband selectivity, considering that the in-band blocking requirement will be specified anyway.

Proposal 4: There is no necessity to introduce new requirement for in-channel adjacent subband blocking and adjacent subband selectivity, considering that the in-band blocking requirement will be specified anyway.

# 4. Conclusion

In this contribution, we provided BS RF requirement for SBFD-capable BS, accordingly with the following observations and proposals obtained:

Transient period requirement for SBFD

Observation 1: For the identified four cases of transitions to/from SBFD symbols, the following timing behaviors are expected:

|  | Expected timing behavior for DL/UL subbands respectively |  | Location of transient period (if any) |
| --- | --- | --- | --- |
| Case-A (SBFD symbol to DL symbol) | DL subband(s) | No transient period issue since the BS transmission is kept from SBFD symbol to DL symbol. | N/A |
|  | UL subband | TA_offset is still required for SBFD UL subband, which shall be longer than the required transient period (10us for FR1 and 3us for FR2), so it is straightforward that the TA_offset can accommodate the required transient period. | Located in TA_offset in SBFD symbol |
| Case-B (SBFD symbol to UL symbol) | DL subband(s) | UE shall follow legacy TDD behaviour, i.e., guard symbols located in D/F-slot is required to accommodate (1) 10us (or 3us) transient period; (2) TA_offset. | Located in guard symbols reserved in SBFD symbols |
|  | UL subband | TA_offset shall be applied to the timing in SBFD symbol and the follow-up UL symbol, and there is no BS transient period issue at all | N/A |
| Case-C (DL symbol to SBFD symbol) | DL subband(s) | No transient period issue since the BS transmission is kept from DL symbol to SBFD symbol. | N/A |
|  | UL subband | UE shall follow legacy TDD behaviour, i.e., guard symbols should be reserved in D/F-slot to accommodate (1) 10us (or 3us) transient period; (2) TA_offset. | Located in guard symbols reserved in non-SBFD DL symbols |
| Case-D (UL symbol to SBFD symbol) | DL subband(s) | UE shall follow legacy TDD behaviour, i.e., the transient period can be accommodated in TA_offset | Located in TA_offset in non-SBFD UL symbol |
|  | UL subband | TA_offset shall be applied to the timing in SBFD symbol and the preceeding UL symbol, and there is no BS transient period issue at all. | N/A |

Observation 2: The location of transient periods should be located in (1) SBFD symbol for Case-A; (2) SBFD symbol(s) reserved as guard symbol(s) for Case-B; (3) Non-SBFD DL symbol(s) reserved as guard symbol(s) for Case-C; (4) Non-SBFD UL symbol for Case-D.

Proposal 1: No need to discuss and decide whether the transmitter transient period shall be located within the SBFD symbol or not.

Observation 3: For the cases in which the transient period is located in guard symbols reserved in Non-SBFD DL symbol(s), i.e., Case-C, the explicit guard should be reserved to allow UE transition from receiving to transmitting, in which the transient period, two-way propagation delay shall be accommodated for UL subband.

Observation 4: The transient period for SBFD operation is different from TDD operation in terms of:

- Transmitter OFF-to-ON and ON-to OFF happen in part(s) of frequency resources, and

- Transmitter OFF-to-ON and ON-to OFF happen in part(s) of antenna elements.

Observation 5: The transient period for SBFD operation could be further summarized as below, by elaborating the four different cases for different SBFD antenna configuration:

|  | Frequency regions for transition | SBFD antenna configuration Option-1 (Same panel area as TDD) | SBFD antenna configuration Option-2&3 (Twice panel area as TDD) | BS transmitter behavior during transition |
| --- | --- | --- | --- | --- |
| Case-A (SBFD symbol to DL symbol) | SBFD UL subband and guardband(s) | SBFD RX antenna panel | N/A | BS transmitter OFF-to-ON |
| Case-B (SBFD symbol to UL symbol) | SBFD DL subband | SBFD TX antenna panel | SBFD TX antenna panel | BS transmitter ON-to-OFF |
| Case-C (DL symbol to SBFD symbol) | SBFD UL subband and guardband(s) | SBFD RX antenna panel | N/A | BS transmitter ON-to-OFF |
| Case-D (UL symbol to SBFD symbol) | SBFD DL subband | SBFD TX antenna panel | SBFD TX antenna panel | BS transmitter OFF-to-ON |

Proposal 2: Based on the example given in R4-2416498, the text proposal is updated for the clause of transmitter transient period for SBFD:

=================== Start of Text Proposal ===================

#### 6.4.2B	Transmitter transient period for SBFD

##### 6.4.2.1B	General

Transmitter transient period for SBFD requirements apply only to SBFD operation of the BS.

The transmitter transient period for SBFD is the time period during which the transmitter is changing from the transmitter OFF period to the transmitter ON period or vice versa. The transmitter transient period is illustrated in figure 6.4.2.1-1.

The transmitter transient period for SBFD shall be measured in the SBFD TX and RX antenna penal and transmission bandwidth configuration for SBFD DL subband, UL subband and guardband, depending on different SBFD cases antenna configuration:

Table 6.4.2.1B-1: Transmitter transient period for SBFD

|  | Frequency regions for transition | SBFD antenna configuration Option-1 (Same panel area as TDD) | SBFD antenna configuration Option-2&3 (Twice panel area as TDD) | BS transmitter behavior during transition |
| --- | --- | --- | --- | --- |
| Case-A (SBFD symbol to DL symbol) | SBFD UL subband and guardband(s) | SBFD RX antenna panel | N/A | BS transmitter OFF-to-ON |
| Case-B (SBFD symbol to UL symbol) | SBFD DL subband | SBFD TX antenna panel | SBFD TX antenna panel | BS transmitter ON-to-OFF |
| Case-C (DL symbol to SBFD symbol) | SBFD UL subband and guardband(s) | SBFD RX antenna panel | N/A | BS transmitter ON-to-OFF |
| Case-D (UL symbol to SBFD symbol) | SBFD DL subband | SBFD TX antenna panel | SBFD TX antenna panel | BS transmitter OFF-to-ON |

For BS type 1-C this requirement shall be applied at the antenna connector supporting transmission in the operating band.

For BS type 1-H this requirement shall be applied at each TAB connector supporting transmission in the operating band.

=================== End of Text Proposal ===================

In-channel adjacent subband requirements

Observation 6: All the gNB-to-gNB CLI handling schemes likely to be introduced in RAN1 (including beam nulling, beam pairing and non-transparent UL resource muting) will greatly mitigate the gNB-to-gNB CLI, and make it be hard to have a criterion for a “proper” interference level or in-channel adjacent subband leakage ratio.

Observation 7: For system-level evaluation (given a certain level of system performance degradation, e.g., 5%), the required in-channel adjacent subband leakage ratio levels can be different if adopting different inter-gNB CLI handling schemes.

Observation 8: It is difficult for RAN4 to agree on a reference scheme for self-interference suppression implemented to derive the potential new requirement in-channel adjacent subband leakage ratio.

Proposal 3: There is no necessity to introduce new requirement for in-channel adjacent subband leakage ratio.

Observation 9: Considering the RAN1-introduced gNB-gNB CLI-handling mechanism, the co-channel interference from neighboring gNB DL subband transmission could be greatly mitigated.

Observation 10: The blocking from co-channel gNB DL subband interference is not as important as adjacent channel gNB DL interference.

Proposal 4: There is no necessity to introduce new requirement for in-channel adjacent subband blocking and adjacent subband selectivity, considering that the in-band blocking requirement will be specified anyway.

# 5. Reference

[1] RP-240789 (revision from RP-234035), “WID revision: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, RAN#103.

[2] 3GPP TR 38.858 v18.0.0, “Study on Evolution of NR Duplex Operation (Release 18)”.

[3] R4-2405655, “Discussion on BS RF requirements for SBFD-capable BS”, Samsung, RAN4#110bis.

[4] R4-2406107, “Way Forward for [110bis][313] NR_duplex_evo”, Samsung, RAN4#110bis

[5] R4-2409765, “On the potentially new requirements for SBFD operation for FR1 and FR2-1”, Samsung, RAN4#111.

[6] R4-2409958, “Way Forward for [111][313] NR_duplex_evo”, Samsung, RAN4#111.

[7] R4-2411640, “On the potentially new requirements for SBFD operation for FR1 and FR2-1”, Samsung, RAN4#112.

[8] R4-2413514, “Way Forward for [112][307] NR_duplex_evo_General”, Samsung, RAN4#112.

[9] R4-2416498, “On the potentially new requirements for SBFD operation for FR1 and FR2-1”, Samsung, RAN4#112bis.

[10] R4-2416584, “Way forward for [112bis][306] NR_duplex_evo_General”, Samsung, RAN4#112bis.