3GPP TSG-RAN WG4 Meeting #113                                                              R4-2418101

Orlando, US, 18th – 22nd November, 2024

Source:	Samsung

Title:	Discussion on Upper-6GHz BS EEIRP mask conformance test

Agenda item:			7.11.4.1

Document for:	Approval

# 1. Introduction

In ITU-R world radiocommunication conference (WRC) in Nov. 20 - Dec. 15 2023,  the frequency band of 6 425-7 125 MHz was identified in every ITU region as in the RR Footnotes 5.6A12, 5.6B12 and 5.6C12 with associated technical condition of limits on the expected equivalent isotropically radiated power (EIRP) spectral density of IMT base-stations, for the purpose of protecting earth-to-space fixed satellite services (FSS) as in Resolution 220 (WRC-23). Based on the newly released ITU-R regulation, the expected EIRP limits have been established which require the harmonized specification for compliance tests to be developed as the task in 3GPP.

Specifically, the aim of this task is to include the new regulation in BS (and other nodes like IAB, Repeater and NCR) RF core specification (TS 38.104) and a corresponding test requirement in the conformance test specification (TS 38.141-2). Furthermore, having the requirement included in the 3GPP standard would also guarantee a harmonized terminology and conformance test method. Accordingly, the performance part objective related to expected EIRP mask for upper 6GHz is provided in WID [1] as:

Expected EIRP mask for upper 6GHz - Specify manufacturer declarations, conformance test descriptions, test tolerances and test requirements. - Note: target completion date of this objective is December 2024.

And much progress on performance requirement has been achieved in RAN4#112 [8] and #112bis [10]. In this contribution, we would like to provide our viewpoints on the remaining issues for OTA spatial emission conformance testing, based on the WRC-23 outcome.

# 2. Discussion

## 2.1 Mechanical tilt angle in conformance test

Based on the agreement from previous meetings when specifying the core requirement, “the requirement shall apply to all supported mechanical tilts.” For the mechanical tilt angle used in the conformance test, the following agreement and WF are provided [8][10]:

Agreement from RAN4#112: To declare the supported mechanical tilt for CAR/beam steering direction sets for BS conformance testing WF from RAN4#112bis: Further discuss the conformance testing for different mechanical tilts and whether the conformance testing for the worst case is sufficient (e.g. minimum mechanical tilt with maximum elevation steering direction)

Considering the much-increased test burden if all supported mechanical tilts shall be tested for BS conformance testing, we prefer to select one typical or worst-case mechanical tilt supported. For EEIRP requirement, the worst performance is more likely to be achieved at the lowest mechanical down-tilt angle, therefore we would like to propose that this angle is chosen in the conformance test.

Proposal 1: EEIRP mask conformance testing is performed under the declared minimum mechanical down-tilt angle (and accordingly the declared beam steering range associated to this mechanical down-tilt angle).

## 2.2 Declared CAR (Coverage Angular Range) definition

In RAN4#112bis, the definition of CAR declarations is given in the approved WF:

Agreement for beam steering range declaration: - Beam steering range declaration is agreeable. FFS on the declaration of minimum/maxium theta and phi. - Option 1: to follow the existing definition. E.g. beam peak direction set; - Option 2: to have specific declaration as in. Parameter; Description; Example values. min; Minimum angle along  axis; 0 degrees. max; Maximum angle along  axis; 6 degrees. min; Minimum angle along axis; -60 degrees. max; Maximum angle along  axis; 60 degrees. - EIRP declaration is per beam based. For the conformance testing for different CARs: - Agreement for conformance testing for different CARs: - FFS discussion on the test effort reduction on CARs with different mechanical tilt.

Based on the offline discussion after RAN4#112bis, the following “compromised solution” is adopted, in which the description of min, max, min, max is provided in the below table as an example:

| Parameter | Description |
| --- | --- |
| min | The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the negative θ direction, while the Φ value being the closest possible to the reference beam centre direction. |
| max | The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the positive θ direction, while the Φ value being the closest possible to the reference beam centre direction. |
| min | The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the negative Φ direction, while the θ value being the closest possible to the reference beam centre direction. |
| max | The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the positive Φ direction, while the θ value being the closest possible to the reference beam centre direction. |

As described in the proponent company’s contribution [R4-2409118], the concept of CAR is defined as “a set of directions describing the intended angular operation range for which the expected EIRP requirement is met”. However, the above exemplary declaration of the CAR is similar as D.10 (OTA peak directions set maximum steering direction(s)) as follows from TS38.141-2:

| Declaration identifier | Declaration | Description | Applicability (Note 1) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | BS type 1-H (Note 2) | BS type 1-O | BS type 2-O |
| D.10 | OTA peak directions set maximum steering direction(s) | The beam direction pair(s) corresponding to the following points: 1) The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the positive Φ direction, while the θ value being the closest possible to the reference beam centre direction. 2) The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the negative Φ direction, while the θ value being the closest possible to the reference beam centre direction. 3) The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the positive θ direction, while the Φ value being the closest possible to the reference beam centre direction. 4) The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the negative θ direction, while the Φ value being the closest possible to the reference beam centre direction. The maximum steering direction(s) may coincide with the reference beam centre direction. Declared for every beam (D.3). | x | x | x |

Based on the same description as the corresponding beam directions pair(s) given in D.10 (OTA peak directions set maximum steering direction(s)), the declared parameters min, max, min, max shall come from the corresponding beam directions pair(s). However, from our understanding, the declared min, max, min, max is different form the beam directions pair(s) given in D.10 (OTA peak directions set maximum steering direction(s)) at least in terms of (1) declared min, max, min, max is the declarations associated with a certain mechanical tilt, rather than the D.10 declared for every beam; (2) declared min, max, min, max is the parameters for one beam direction rather than direction pairs.

Observation 1: The declared parameters min, max, min, max shall come from the corresponding beam directions pair(s) given in D.10 (OTA peak directions set maximum steering direction(s)).

Observation 2: The declared min, max, min, max is different form the beam directions pair(s) given in D.10 (OTA peak directions set maximum steering direction(s)) at least in terms of

(1) declared min, max, min, max is the declarations associated with a certain mechanical tilt, rather than the D.10 declared for every beam;

(2) declared min, max, min, max is the parameters for one beam direction rather than direction pairs.

Proposal 2: During EEIRP mask conformance testing, CAR is provided as below:

| Declaration identifier | Declaration | Description | Applicability (Note 1) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | BS type 1-H (Note 2) | BS type 1-O | BS type 2-O |
| D.x | OTA coverage angular range (CAR) | The max and min in Φ direction, and max and min in θ direction: 1) The beam peak direction max corresponding to the maximum steering from the reference beam centre direction in the positive Φ direction, while the θ value being the closest possible to the reference beam centre direction. 2) The beam peak direction min corresponding to the maximum steering from the reference beam centre direction in the negative Φ direction, while the θ value being the closest possible to the reference beam centre direction. 3) The beam peak direction max corresponding to the maximum steering from the reference beam centre direction in the positive θ direction, while the Φ value being the closest possible to the reference beam centre direction. 4) The beam peak direction min corresponding to the maximum steering from the reference beam centre direction in the negative θ direction, while the Φ value being the closest possible to the reference beam centre direction. | x | x | x |

## 2.3 Confidence interval

In the Annex to Resolution 220 (WRC-23) [2], details for the calculation of the expected equivalent isotropically radiated power for assessing the compliance of an IMT base station equipment operating within the frequency band 6 425-7 125 MHz with the limit on expected EIRP are provided:

ANNEX TO RESOLUTION 220 (WRC-23) ... The e.i.r.p. of an IMT base station in the horizontal (azimuth) direction −π ≤ φ ≤ π and vertical (elevation) direction 0 ≤ θ ≤ π/2 above the horizon can be written as P(θ, φ; α, β). The parameters α and β are the horizontal and vertical beamforming directions, i.e. the angles towards which the base station electronically steers a beam. These are illustrated in Figure 1 below. The expected e.i.r.p. of an IMT base station within a vertical angle range θL ≤ θ < θH can be calculated by averaging the e.i.r.p. P(θ, φ; α, β) of the base station as follows: 1) Averaging over beamforming directions for a given vertical angle θ0 and horizontal angle φ0: for an AAS base station within a given horizontal and vertical steering range, a sufficient sampling of N beamforming directions (αn, βn) n = 1 ... N is necessary to allow an accurate averaging of the expected e.i.r.p. The beamforming directions (αn, βn) have a uniform statistical angular distribution within the steering range of the IMT base station. In other words: where wn refers to the weight for the nth beamforming direction, i.e. the fraction of the steering range represented by the nth beamforming direction. For example, wn = 1/N in the case that N uniform equispaced beams are assumed in the azimuth and elevation, respectively, and where each beam covers an equal range of angles. The set of base station configurations over which the base station complies with the limits on expected e.i.r.p. (for example, power of steering range as one of the parameters) shall be declared and the BS shall be used within one of these configurations. The set of e.i.r.p. values used to calculate the expected e.i.r.p. for each vertical angle range shall be a mathematical summation of both polarization states of the IMT base station antenna with no polarization discrimination. For a non-AAS base station, P1(θ0, φ0) = P(θ0, φ0; α1, β1) where α1 = 0 and β1 is the electrical tilt. It is noted that the compliance with the limits on expected e.i.r.p. should be limited to a defined range of electrical tilts. 2) Averaging over horizontal and vertical angles: the expected e.i.r.p. is then calculated by averaging the results of step 1 over horizontal angles φ from −π to +π with respect to the base station horizontal boresight, and vertical angles θ within vertical angle measurement window θL ≤ θ < θH with respect to the horizon. In other words: The averaging processes in steps 1 and 2 shall allow for accurate averaging of the expected e.i.r.p (e.g. to the confidence interval of 95%).

In Resolution 220 (WRC-23), the confidence interval is corresponding to the averaging processes in both steps 1 and 2, to guarantee the accurate averaging, while in existing analysis companies consider the confidence interval only for selecting the accurate test beams in step 1.

Observation 3: In Resolution 220 (WRC-23), the confidence interval is corresponding to the averaging processes in both steps 1 and 2, to guarantee the accurate averaging.

Secondly, the confidence interval of 95% is just an exemplary value, while it is not regulated by the Resolution 220.

Observation 4: In Resolution 220 (WRC-23), the confidence interval of 95% for accurate averaging is just an exemplary value, rather than a regulated value.

For averaging step 1, i.e., averaging over beamforming directions for a given vertical angle θ0 and horizontal angle φ0, the key discussion point is how to interpret the averaged expected for the sampled test beams (e.g., 9 or 25 test beams). If only the 2nd averaging is performed, for individual test beam (e.g., test beam i), the expected EIRP over the specified vertical angle range θL ≤ θ < θH  shall be calculated as

EEIRPθLθH, test beam i =θH-θLMNsinθH-sinθLn=1Nm=1MEIRPθn,φm, test beam icosθn

And the final expected EIRP should be calculated by averaging over all test beams (taking K test beams as example) as

EEIRPθLθH =i=1Kwi∙EEIRPθLθH, test beam i

By assuming the uniform weights for K test beams, the above final expected EIRP is the averaged EEIRP over K test beams:

EEIRPθLθH =avg(EEIRPθLθH, test beam i )=i=1KEEIRPθLθH, test beam i /K

Companies propose that the samples’ standard deviation by using the K sampled EEIRPθLθH, test beam i

Standard deviation s = i=1KEEIRPθLθH, test beam i -avg(EEIRPθLθH, test beam i )2/(K-1)

And the samples’ standard error can be derived as

Standard error SE = s/K

By assuming 9 test beams for the sampling, the degree of freedom is 8, and the right-tail t-distribution can be given in the below figure, which gives t-value of 1.86 for right-tail 95% confidence. It should be noted that the right-tail t-distribution shall be used, instead of two tails, since the 95% confidence interval is given to guarantee the actual EEIRP is smaller than the spatial emission mask:

Observation 5: Only by assuming K “randomly selected” test beams which provides K sampled EEIRPθLθH, test beam i  and accordingly the averaged value  avg(EEIRPθLθH, test beam i ), the actual EEIRP is smaller than the spatial emission mask (marked as TθLθH ) with 95% confidence only if:

avgEEIRPθLθH, test beam i +1.86∙sK< TθLθH

However, we see the above assumption of “K randomly selected test beams” can’t be held for the existing test beam options, such as 9 beams in diamond constellation and 17 beams in ellipsoid constellation.

Observation 6: The assumption of “K randomly selected test beams” can’t be held for the existing test beam options, such as 9 beams in diamond constellation and 17 beams in ellipsoid constellation.

Based on the above analysis, the methodology of assuming “K randomly selected test beams”, and use t-value and standard error to guarantee 95% confidence is questionable for the currently proposed test beam constellation, because the standard error is greatly overestimated.

Observation 7: By adopting the existing test beam options, such as 9 beams in diamond constellation and 17 beams in ellipsoid constellation, the standard error from samples given by different test beams is overestimated.

Accordingly, we found the existing methodology is not appropriately to be adopted to guarantee 95% confidence interval.

Proposal 3: If the existing non-random test beam options (such as 9 beams in diamond constellation and 17 beams in ellipsoid constellation) are adopted, the standard error-based method to guarantee 95% confidence interval should not be used.

# 3. Conclusion

In this contribution, we provided our analysis on the expected EIRP mask for upper 6GHz (i.e., the OTA spatial emission requirement), and discuss the related BS conformance testing based on the WRC-23 outcome. Specifically, the following observations and proposals are provided:

Proposal 1: EEIRP mask conformance testing is performed under the declared minimum mechanical down-tilt angle (and accordingly the declared beam steering range associated to this mechanical down-tilt angle).

Observation 1: The declared parameters min, max, min, max shall come from the corresponding beam directions pair(s) given in D.10 (OTA peak directions set maximum steering direction(s)).

Observation 2: The declared min, max, min, max is different form the beam directions pair(s) given in D.10 (OTA peak directions set maximum steering direction(s)) at least in terms of

(1) declared min, max, min, max is the declarations associated with a certain mechanical tilt, rather than the D.10 declared for every beam;

(2) declared min, max, min, max is the parameters for one beam direction rather than direction pairs.

Proposal 2: During EEIRP mask conformance testing, CAR is provided as below:

| Declaration identifier | Declaration | Description | Applicability (Note 1) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | BS type 1-H (Note 2) | BS type 1-O | BS type 2-O |
| D.x | OTA coverage angular range (CAR) | The max and min in Φ direction, and max and min in θ direction: 1) The beam peak direction max corresponding to the maximum steering from the reference beam centre direction in the positive Φ direction, while the θ value being the closest possible to the reference beam centre direction. 2) The beam peak direction min corresponding to the maximum steering from the reference beam centre direction in the negative Φ direction, while the θ value being the closest possible to the reference beam centre direction. 3) The beam peak direction max corresponding to the maximum steering from the reference beam centre direction in the positive θ direction, while the Φ value being the closest possible to the reference beam centre direction. 4) The beam peak direction min corresponding to the maximum steering from the reference beam centre direction in the negative θ direction, while the Φ value being the closest possible to the reference beam centre direction. | x | x | x |

Observation 3: In Resolution 220 (WRC-23), the confidence interval is corresponding to the averaging processes in both steps 1 and 2, to guarantee the accurate averaging.

Observation 4: In Resolution 220 (WRC-23), the confidence interval of 95% for accurate averaging is just an exemplary value, rather than a regulated value.

Observation 5: Only by assuming K “randomly selected” test beams which provides K sampled EEIRPθLθH, test beam i  and accordingly the averaged value  avg(EEIRPθLθH, test beam i ), the actual EEIRP is smaller than the spatial emission mask (marked as TθLθH ) with 95% confidence only if:

avgEEIRPθLθH, test beam i +1.86∙sK< TθLθH

Observation 6: The assumption of “K randomly selected test beams” can’t be held for the existing test beam options, such as 9 beams in diamond constellation and 17 beams in ellipsoid constellation.

Observation 7: By adopting the existing test beam options, such as 9 beams in diamond constellation and 17 beams in ellipsoid constellation, the standard error from samples given by different test beams is overestimated.

Proposal 3: If the existing non-random test beam options (such as 9 beams in diamond constellation and 17 beams in ellipsoid constellation) are adopted, the standard error-based method to guarantee 95% confidence interval should not be used.

# 4. Reference

[1] RP-240829, “NR base station (BS) RF requirement evolution for FR1/FR2 and testing”, Huawei, RAN#103, March 2024.

[2] Final Acts, World Radiocommunication Conference 2023 (WRC-23), 20th Nov.-15th Dec. 2023.

[3] R4-2405656, “Discussion on expected EIRP mask for upper 6GHz”, Samsung, RAN4#110bis, April 2024.

[4] R4-2406088, “Way Forward for [110bis][311] NR_BS_RF”, ZTE, RAN4#110bis, April 2024.

[5] R4-2409766, “Discussion on expected EIRP mask for upper 6GHz”, Samsung, RAN4#111, May 2024.

[6] R4-2409955, “Way Forward for [111][311] NR_BS_RF”, ZTE, RAN4#111, May 2024.

[7] R4-2411643, “Discussion on OTA spatial emission conformance testing”, Samsung, RAN4#112, Aug. 2024.

[8] R4-2413509, “Way Forward for [112][305] NR_BS_RF”, ZTE, RAN4#112, Aug. 2024.

[9] R4-2416502, “Discussion on OTA spatial emission conformance testing”, Samsung, RAN4#112bis, Oct. 2024.

[10] R4-2416572, “Way Forward for [112bis][302] NR_BS_RF_Part1_E_EIRP”, ZTE, RAN4#112bis, Oct. 2024.