3GPP TSG-RAN WG4 Meeting #113		R4-2418111

Orlando, US, 18th – 22nd November, 2024

Agenda item:		7.24.2.1

Source:		Samsung

Title:	Discussion on RRM requirements of event triggered L1 measurement reporting for LTM

Document for:		Discussion

- Introduction

In previous RAN4 meetings, the RRM impacts of event triggered L1 measurement reporting of LTM has been discussed. The agreements in RAN4#112bis are captured in WF [1]. In this contribution, we continue to discuss the RRM requirements of event triggered L1 measurement reporting of LTM.

- Discussion

Event triggered L1 measurement reporting

#### Issue 1-3: Requirements for reporting criteria per measurement category - Candidate solutions: - Option 1: (MTK) - Define a separate capability for supported number of events triggered L1 measurement reporting. - Focus on SSB based L1-RSRP measurement for event triggered reporting at first. - For SSB based event triggered L1-RSRP measurement reporting, regard the events related to different candidate cells on different frequency layers as different events. Discuss how to differentiate intra-frequency and inter-frequency events. Further discuss the following options: - Option 1: if the LTM-CSI-ReportConfig is linked to multiple cells on different frequency layer, then the number of events related to this LTM-CSI-ReportConfig should be counted no less than the number of frequency layers. - Option 2: inform RAN2 that the events related to different candidate cells on different frequency layers are regarded as different events and ask RAN2 to take this into consideration when design the signaling. - Option 2: discuss and confirm in RAN4#112bis that whether requirements for reporting criteria per measurement category needs to be updated to cover L1-RSRP measurement on neighbor cell. If needed, it is proposed to use the following table as baseline. (Apple) Measurement category; Ecat; Note. Intra-frequency Note 1,2,3,4,5; 9; Events for any one or a combination of intra-frequency SS-RSRP, SSB based L1-RSRP, CSI-RS based L1 RSRP, SS-RSRQ, SS-SINR, CSI-RSRP, CSI-RSRQ, and CSI-SINR for NG-RAN intra-frequency cells. Inter-frequency Note 2,3,4,5; 10; Events for any one or a combination of inter-frequency SS-RSRP, SSB based L1-RSRP, CSI-RS based L1 RSRP, SS-RSRQ, SS-SINR, CSI-RSRP, CSI-RSRQ, and CSI-SINR for NG-RAN inter-frequency cells. Inter-RAT (E-UTRA FDD, E-UTRA TDD) Note 2,4,5; 10; Only applicable for UE with this (inter-RAT) capability. These reporting criteria apply for any E-UTRA carrier frequencies other than the carrier frequency of the E-UTRA PSCell or E-UTRA SCell.. Inter-RAT (E-UTRA FDD, E-UTRA TDD) RSTD Note 2,4,5; 1; Inter-RAT RSTD measurement reporting for UE supporting OTDOA; 1 report capable of minimum 16 inter-RAT cell measurements. Only applicable for UE with this (inter-RAT RSTD via LPP [22]) capability. These reporting criteria apply for any E-UTRA carrier frequencies other than the carrier frequency of the E-UTRA PSCell or E-UTRA SCell.. Inter-RAT (E-UTRA FDD, E-UTRA TDD) RSRP and RSRQ measurements for E-CID Note 2,4,5; 1; Inter-RAT RSRP and RSRQ measurements for E-CID reported to E-SMLC via LPP [22]. One report capable of at least in total 10 inter-RAT RSRP and RSRQ measurements. Applicable to UE capable of reporting inter-RAT RSRP and RSRQ to E-SMLC via LPP. These reporting criteria apply for any E-UTRA carrier frequencies other than the carrier frequency of the E-UTRA PSCell or E-UTRA SCell.. Intra-frequency RSSI and channel occupancy measurements with CCA Note 1,2,3; 1; One report capable of one RSSI and one channel occupancy measurements over a channel [TS 37.213] with CCA. Applicable for UE capable of performing and reporting RSSI and channel occupancy on carrier frequencies under CCA.. Inter-frequency RSSI and channel occupancy measurements with CCA Note 2,3; 1; One report capable of one RSSI and one channel occupancy measurements over a channel [TS 37.213] with CCA. Applicable for UE capable of performing and reporting RSSI and channel occupancy on carrier frequencies under CCA.. Intra-frequency SSB-based measurements for NR E-CID Note 1,2,3,4,5; 1; Intra-frequency SS-RSRP and SS-RSRQ measurements for NR E-CID reported to LMF via LPP [34]. One report capable of at least in total 9 intra-frequency SS-RSRP and SS-RSRQ measurements. Applicable to UE capable of reporting at least one of SS-RSRP and SS-RSRQ to LMF via LPP.. Intra-frequency CSI-RS based measurements for NR E-CID Note 1,2,3,4,5; 1; Intra-frequency CSI-RSRP and CSI-RSRQ measurements for NR E-CID reported to LMF via LPP [22]. One report capable of at least in total 9 intra-frequency CSI-RSRP and/or CSI-RSRQ measurements. Applicable to UE capable of reporting any of CSI-RSRP and CSI-RSRQ to LMF via LPP, as indicated in nr-ECID-MeasSupported-r16.. Inter-frequency SSB-based measurements for NR E-CID Note 2,3,4,5; 1; Inter-frequency SS-RSRP and SS-RSRQ measurements for NR E-CID reported to LMF via LPP [34]. One report capable of at least in total 10 inter-frequency SS-RSRP and SS-RSRQ measurements. Applicable to UE capable of reporting at least one of SS-RSRP and SS-RSRQ to LMF via LPP.. Inter-frequency CSI-RS based measurements for NR E-CID Note 2,3,4,5; 1; Inter-frequency CSI-RSRP and CSI-RSRQ measurements for NR E-CID reported to LMF via LPP [22]. One report capable of at least in total 10 inter-frequency CSI-RSRP and CSI-RSRQ measurements. Applicable to UE capable of reporting any of CSI-RSRP and CSI-RSRQ to LMF via LPP, as indicated in nr-ECID-MeasSupported-r16.. DL RSTD Note 2,4,5; 1; DL RSTD measurement reporting; 1 report capable of multiple (within the UE PRS measurement capability, nr-DL-TDOA-MeasCapability, indicated via LPP [34]) DL RSTD measurements and if supported also multiple corresponding DL PRS-RSRP measurements configured for DL-TDOA. Only applicable for UE capable of reporting measurements for DL-TDOA to LMF via LPP [34].. UE Rx-Tx Note 2,4,5; 1; UE Rx-Tx measurement reporting; 1 report capable of multiple (within the UE PRS measurement capability, nr-DL-PRS-MeasCapability, indicated via LPP [34] for multi-RTT) UE Rx-Tx measurements and if supported also multiple corresponding DL PRS-RSRP measurements configured for multi-RTT. Only applicable for UE capable of reporting measurements for multi-RTT to LMF via LPP [34].. DL PRS-RSRP Note 2,4,5; 1; DL PRS-RSRP measurement reporting; 1 report capable of multiple (within the UE PRS measurement capability, nr-DL-PRS-MeasCapability, indicated via LPP [34] for AoD) DL PRS-RSRP measurements configured for DL-AoD. Only applicable for UE capable of reporting measurements for DL-AoD to LMF via LPP [34].. DL PRS-RSRPP Note 2,4,5; 1; DL PRS-RSRPP measurement reporting; 1 report capable of multiple (within the UE PRS measurement capability, nr-DL-PRS-MeasCapability, indicated via LPP [34] for AoD) DL PRS-RSRPP measurements configured for DL-AoD. Only applicable for UE capable of reporting measurements for DL-AoD to LMF via LPP [34].. SRS-RSRP Note 2,3,4,5; 1; SRS-RSRP measurement reporting for CLI; 1 report capable of up to 32 SRS resources measurements. Only applicable for UE supporting cli-SRS-RSRP-Meas-r16.. CLI-RSSI Note 2,3,4,5; 1; CLI-RSSI measurement reporting for CLI; 1 report capable of up to 64 CLi-RSSI resources measurements. Only applicable for UE supporting cli-RSSI-Meas-r16.. NOTE 1: When the UE is configured with PSCell and SCell carrier frequencies, Ecat for Intra-frequency is applied per corresponding NR serving frequency. NOTE 2: Applicable for UE configured with SA NR operation mode. NOTE 3: Applicable for UE configured with EN-DC operation mode. NOTE 4: Applicable for UE configured with NE-DC operation mode. NOTE 5: Applicable for UE configured with NR-DC operation mode.. - Option 3: RAN4 to introduce requirements of “Capabilities for Support of Event Triggering and Reporting Criteria” (OPPO)

In legacy event triggered L3 measurement, the UE shall be able to support in parallel per category of the capability of reporting criteria according to Table 9.1.4.2-1.

In L3 measurement, different of events for any or combination can be measured. And this capability depends on the UE soft processing and reporting capability.

In L1 measurement, a similar capability can be defined for event-triggered L1 measurement. However, no need for UE to be able to support different events of L3 and L1 in parallel per category.

For each number of the reporting criteria, most the values are leverage from LTE. In addition, for L1 measurement, there is no such more events to parallel reporting like L3 measurement.

Proposal 1: RAN4 to define the capability for support the L1 event trigging and reporting criteria but separate from L3 capabilities.

#### Issue 1-4: How to coordinate with R19 MIMO - Candidate solutions: - Option 1: (MTK) - Events configured for mobility and MIMO should be considered as different events even when they are configured for the same beams and the same cell with the same threshold. - Define the capability for L1 events shared by mobility L1 events and MIMO L1 events.

Event triggered L1 measurement reporting will be introduced in Rel-19 MIMO and Rel-19 mobility separately. However, the motivation and procedures are different. For example, one is for FR2 different beams, while the other is for FR1 and FR2 beams for candidate cells. The Events are also different. In addition, there might be different UE capabilities to support either of them or both of them.

Proposal 2: Define separate capabilities for MIMO and mobility.

#### Issue 1-5: filtering assumption in event evaluation - Candidate solutions: - Option 1: Wait for more RAN1 progress to discuss how to handle L1 filtering in RAN4 for event triggered L1 report (MTK, Apple, HW, Nokia, E///, Qualcomm, CATT). - Option 1a: For whether to consider the RRM impact due to L1 filtering for event triggered L1 report, RAN4 needs to wait for more RAN1 input. (CATT) - If L1 filtering is introduced in RAN1, RAN4 needs to define additional SSB based L1 measurement delay requirements with filtering for event triggered L1 report. - Otherwise, there is no impact on RAN4. - Option 2: For filtering, we think it is better to wait the conclusion of RAN1. If it is up to UE implementation, it is unnecessary to define the related requirements. (CTC) - Event triggered reporting delay is up to whether the filtering is needed, when filtering is used an additional delay can be expected. - In case without filtering, we think current LTM L1-RSRP measurement delay can be as baseline. - Option 3: no need to define additional SSB based L1 measurement delay requirements with filtering, since the agreement about filtering in RAN2 is about filtering with filter coefficient as defined in Clause 5.5.3.2, TS38.331 or about Time-to-Trigger (TTT), neither of them have impact on L1 measurement delay requirements. (CMCC)

In last RAN1#118bis meeting, the agreements are captured as below:

Agreement The agreement “Rel-18 LTM CSI reporting framework is the baseline for CSI-RS based L1-measurement report by gNB scheduled measurement reporting” made in RAN#118 is further clarified for L1-RSRP as follows: - UCI format defined in Table 6.3.1.1.2-8C of TS38.212 can be used by replacing SSBRI with CRI. - Whether the L1-RSRP(s) of serving cell is always included is configurable (in line with Rel-18) - The quantization method defined in clause 5.2.1.4.3 of TS38.214 and bit width defined in Table 6.3.1.1.2-6 of TS38.212 can be used - No L1 specified filtering for time and spatial domain is introduced - No enhancement on how to report L cells x M beams - Periodic reporting on PUCCH is supported - FFS: semi-persistent reporting on PUCCH/PUSCH, and aperiodic reporting on PUSCH

There is no L1 filter introduced in RAN1.

Proposal 3: In event triggered L1 measurement in mobility, do not consider L1 filter for requirements.

#### Issue 1-7: beam level or cell level measurement result for new LTM event evaluation - Candidate solutions: - Option 1: For LTM event triggered L1 reporting, beam level measurement result will be used for evaluation. (Xiaomi, CMCC, Apple, Samsung)

In RAN2 127 meeting, RAN2 agreement shows:

- Event LTM1 is not defined.

- Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell.

- Any beam in candidate RS configuration can be used for LTM event evaluation.

- Beam level measurement result, not cell level measurement result, is used LTM event evaluation. FFS on the conditional LTM case.

- R19 LTM event triggered measurement configuration can be taken as baseline:

- LTM measurement resource configuration is provided in LTM-config.

- Event triggered report config is provided in serving cell config.

6. MAC layer handles the event evaluation and measurement report triggering.

According to the agreement in RAN2#127 meeting listed above, “Beam level measurement result, not cell level measurement result, is used LTM event evaluation.”, for Issue 2-2-2, use beam level measurement result for event evaluation, not cell level

Proposal 4: For LTM event triggered L1 reporting, beam level measurement result will be used for evaluation.

#### Issue 1-9: L1 event triggered reporting delay - Candidate solutions: - Option 1: The existing framework in L3 event triggered reporting RRM requirements can be as a starting point. (CTC, Samsung, ZTE) - Option 2: RAN4 to discuss how to consider the total measurement time if multiple cells are measured with different measurement period. (Xiaomi) - It’s possible that the periodicity of RS is different from each other and then the measurement period is different. How to define total measurement time for multiple cells is FFS. - Option 3: (CMCC) - for SSB based event triggered L1 measurement reporting, it is proposed to define L1 measurement reporting delay requirements. - The delay is less than TL1-RSRP_Measurement_Period or TL1-RSRP_Measurement_Perioa + TSSB_time_index, which are the L1-RSRP measurement period defined for Rel-18 LTM. - for CSI-RS based event triggered L1 measurement reporting, it is proposed to define L1 measurement reporting delay requirements, and L1 measurement reporting accuracy requirements - Option 4: Reporting delay is defined as one L1-RSRP measurement period on target cell. (Apple) - Option 4a: The event triggered measurement reporting delay, shall be less than LTM L1-RSRP measurement period based on SSB or CSI-RS. (Samsung) - Option 5: SSB-based LTM L1 event triggered reporting delay is TL1-RSRP_Measurement_Period_SSB_intra, TL1-RSRP_Measurement_Period_SSB_inter or TL1-RSRP_Measurement_Period_SSB_inter_withoutGap (Nokia) - Option 6: RAN4 to discuss and define the event evaluation time (i.e., the minimum time required to complete the event evaluation and be ready to send on the first reporting occasion from the measurement occasion). (Ericsson)

L1 event triggered reporting delay shall include the measurement time. From the procedure and definition:

- What is the type of RS for serving cell and candidate cell?

- How to perform the evaluation?

In RAN1 latest agreements, it is defined for RS to evaluation.

RAN1#118bis The serving cell RS for event evaluation is at least derived from QCL RS or SSB QCLed with the QCL RS of the indicated joint/DL TCI state for the serving cell - QCL RS above is the RS w.r.t. QCL-TypeD when the indicated joint/DL TCI state is configured with two QCL RSs. - FFS: Details on determination of QCL RS or SSB QCLed with QCL RS. Note: This does not imply the support of mTRP scenarios. RAN1#118 Agreement Explicit configuration of CSI-RS resource(s) for candidate cell(s) for L1-measurement is supported.

In RAN2 agreements:

- Event LTM2: Beam of serving cell becomes worse than absolute threshold; - Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell; - Event LTM4: Beam of candidate cell becomes better than absolute threshold; - Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

It only agreed that the Event condition is to compare with beam quality to threshold/other beams/combination of other beams and threshold.

What is the relationship between serving cell and candidate cells? Whether to support the RS types can be different from cells?

It’s early to conclude in RAN4 for L1 event triggered reporting delay.

Proposal 5: For the L1 event triggered reporting delay, evaluation time shall be considered at least. It is not so clear in RAN1/RAN2 for procedure. Wait for further RAN1 progress.

- Conclusion

In this contribution, we provide our proposals for RRM requirements of event triggered L1 measurement reporting of LTM:

Proposal 1: RAN4 to define the capability for support the L1 event trigging and reporting criteria but separate from L3 capabilities.

Proposal 2: Define separate capabilities for MIMO and mobility.

Proposal 3: In event triggered L1 measurement in mobility, do not consider L1 filter for requirements.

Proposal 4: For LTM event triggered L1 reporting, beam level measurement result will be used for evaluation.

Proposal 5: For the L1 event triggered reporting delay, evaluation time shall be considered at least. It is not so clear in RAN1/RAN2 for procedure. Wait for further RAN1 progress.

- Reference

[1] R4-2416862, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple