3GPP TSG-RAN WG4 Meeting #113		R4-2418113

Orlando, US, 18th – 22nd November, 2024

Agenda item:		7.24.2.3

Source:		Samsung

Title:	Discussion on RRM requirements of conditional Intra-CU LTM

Document for:		Discussion

- Introduction

In previous RAN4 meetings, the potential RRM impacts of conditional Intra-CU LTM has been discussed. The agreements in RAN4#112bis are captured in WF [1]. In this contribution, we continue to discuss the RRM requirements of conditional Intra-CU LTM.

- Discussion

Conditional Intra-CU LTM

In RAN4#112bis meeting, it is agreed RAN4 to define cell switch delay for conditional LTM. But how to define such delay is still open as:

Issue 1-1-1: RAN4 scope of Conditional LTM < Agreement> - RAN4 to define cell switch delay requirements for conditional LTM - RAN4 to define subsequent conditional LTM requirement unless there is any technical issue identified. - Wait for other WGs input to further discuss detailed RRM requirements.

Issue 1-2-1: General aspects on Conditional LTM delay < Way Forward > FFS the following options: - Option 1: CHO requirements framework can be used as baseline. - Option 1a: Rel-18 LTM requirements framework can be used as references. - Option 2: Revisit conditional LTM delay when other WGs achieve further conclusions. - Option 3: Wait for RAN2 progress for defining cell switch delay requirements for conditional LTM. - Option 4: RAN4 to define cell switch delay requirements for conditional PCell switch in intra-CU scenario. - Option 5: Rel-18 UE capability for early ASN.1 decoding or fast RRC processing shall be supported for Rel-19 Conditional LTM. Issue 1-2-2: The starting point of conditional LTM delay < Way Forward > FFS the following options: - Option 1: The starting point of conditional LTM delay is the end of the last TTI containing the RRC command for conditional LTM. - Option 2: The starting point of conditional LTM delay is the time when UE transmits an RRCReconfigurationComplete message. - Option 3: The starting point of CLTM cell switch delay depends on when the condition evaluation begins, which is up to RAN1/RAN2. Issue 1-2-3: The ending point of conditional LTM delay < Way Forward > FFS the following options: - Option 1: - The ending point of conditional LTM delay is the time when the UE transmits the first UL message on the target cell. - FFS the support of RACH-based and/or RACH-less, which is up to RAN1/2 agreement. - It can be revisited if any technical issue will be identified considering the progress in other WGs. - Other option is not precluded.

For the delay definition, it is related to conditional LTM procedure which defined in RAN2. In last meeting, it is the first meeting in RAN2 as well.

In RAN2#127bis meeting, the agreement for conditional Intra-CU LTM is agreed as below:

1. Source cell sends the conditional LTM configuration via RRCReconfiguration to UE, which includes the LTM candidate configurations, and the corresponding execution conditions. 2. Event LTM3-like and LTM5-like are used as the conditional LTM execution condition. FFS on reuse of CHO conditions. 3. Source cell and each candidate cell provides its own execution condition for conditional LTM. 4. It is DU to generate the L1 execution condition. FFS on a case that L3 measurement is used. 5. RACH-less Conditional intra-CU LTM is supported. 6. RACH based conditional intra-CU LTM is supported. 7. UE based TA measurement mechanism is supported for conditional intra-CU LTM. 8. PDCCH ordered early TA acquisition is supported for conditional LTM. 9. Rel-18 Early candidate TCI State activation/deactivation is supported for conditional intra-CU LTM. 10. For RACH-less conditional LTM, CG-based first UL transmission on target cell is supported. FFS on DG-based approach. 11. The LTM completion defined for Rel-18 intra-CU LTM is reused for conditional LTM.

From the definition of LTM completion in RAN2, after the conditional LTM execution, the UE send the first UL message to the target cell/beam as the completion of procedure. Therefore, the end point of conditional cell switch delay should be the same as Rel-18 LTM Cell Switch delay which is that UE transmits the first UL message on the target cell.

Proposal 1: For ending point of conditional LTM delay, reuse the same as that in Rel-18 LTM cell switch delay: UE transmits the first UL message on the target cell.

According to RAN2 agreement, both RACH-based and RACH-less conditional LTM are supported.

Proposal 2: Both RACH-based and RACH-less conditional LTM are supported.

Then the question is for the start point of conditional LTM delay:

In legacy NW based LTM in Rel-18, there is the MAC-CE command for cell switch command. Therefore, the start point is the end of the last TTI containing the MAC-CE command.

In legacy L3 conditional handover, the start point is the time of UE receives a RRC message implying conditional handover.

However, in conditional LTM delay, there is no explicit MAC-CE command to indicate UE to perform conditional LTM. According to RAN2 agreement, there is a RRC message includes the LTM candidate configurations and execution conditions.

Observation 1: For start point of conditional LTM delay, there is no explicit MAC-CE command like Rel-18 NW based LTM. There is a RRC message includes the LTM candidate configurations and execution conditions.

There are two approaches for the start point of conditional LTM delay. The first one is the start point is from conditional LTM configuration in RRC message. The second one is similar as event-triggered reporting which is the condEvent is met over the air.

For first one, it is similar as that in L3 CHO. The delay will be from RRC message and contain TRRC and TEvent_DU. However, TRRC and TEvent_DU are quite long and uncertainty compared to L3 CHO delay. And in the repetition tests, the real delay is only reflected to the measurement time/execution time/interruption time.

For second one, it is similar as that the discussion on event-triggered L1 report. The delay is related to the measurement time and so on.

From current RAN1/RAN2 agreements, it is not so clear whether any further definition of the procedure of condEvent and how to perform the measurement.

Proposal 4: RAN4 to discuss on start point of conditional LTM delay:

- Same as L3 CHO. RRC message includes the LTM candidate configurations and execution conditions.

- Similar as event-triggered L1 reporting. The CondEvent is met over the air.

Wait for further RAN1/RAN2 progress.

For the component of conditional LTM delay, it is open in [1] as below:

Issue 1-2-4: The components of conditional LTM delay < Way Forward > FFS the following options: - Option 1: DCLTM = TRRC + TEvent_DU + TTCI-state activate + Tmeasure + TCLTM_execution + Tinterrupt. - Option 2: DLTM_Conditional = TEvent_DU + Tmeasure + TLTM_execution + TLTM-interrupt. - Option 3: DCLTM = TRRC + TEvent_DU + Tmeasure + TLTM-interrupt + TCLTM_execution. - Option 4: DCLTM = TLTM_event_DU + TLTM_measure + TLTM-processing + TLTM-IU. - need of TLTM-RRC-processing, Tfirst-RS, TRS-proc and any additional components in the cell switch delay depends on when DL sync and ASN.1 decoding are to happen, which may depend on RAN1/2 CLTM design. - Option 5: At least the delay uncertainty (TEvent_DU), the measurements time (Tmeasure) and the conditional execution preparation time (TCLTM_execution) also need to be considered for Rel-19 conditional LTM Cell Switch delay. Issue 1-2-5: TRRC < Way Forward > FFS the following options: - Option 1: TRRC is the RRC procedure delay defined in clause 12 in TS 38.331. - Other option is not precluded. Issue 1-2-6: TEvent_DU < Way Forward > FFS the following options: - Option 1: TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional LTM command until a condition exists at the measurement reference point which will trigger the conditional LTM. - Option 2: TEvent_DU is the delay uncertainty which is the time from when UE transmits RRCReconfigurationComplete message confirming the LTM configuration or the RRCReconfigurationcomplete message for the previous cell switch until a condition exists at the measurement reference point which will trigger the conditional LTM. - Option 3: Wait for more progress in RAN2. Issue 1-2-7: Tmeasure < Way Forward > FFS the following options: - Option 1: Tmeasure is measurement time. - Option 1a: Tmeasure is defined from the end of TTCI-state activate until UE executes LTM to a target cell and interruption time starts. - Option 1b: Tmeasure is the measurements time. It is defined from the end of TEvent_DU until UE executes LTM cell switch to a target cell and interruption time starts. - If SSB based L1-RSRP measurement is used in the event, Tmeasure is same as one measurement period of SSB based L1-RSRP measurement as introduced in R18. - If CSI-RS based L1-RSRP measurement is used in the event, Tmeasure is same as one measurement period of CSI-RS based L1-RSRP measurement which will be introduced in R19. - If L3 measurement is used in the event, Tmeasure is same as one measurement period of legacy L3 measurement. - Option 2: Wait for more progress in RAN2. Issue 1-2-8: TCLTM_execution / TLTM_execution < Way Forward > FFS the following options: - Option 1: TCLTM_execution / TCLTM_execution is the UE execution preparation time for conditional LTM and starts after UE realizes the condition of LTM is met and identity of the target cell is determined. - Option 1a: TCLTM_execution can be up to [10ms]. - Option 2: TCHO_execution may need update in conditional LTM. Issue 1-2-9: Tinterrupt < Way Forward > FFS the following options: - Option 1: Tinterrupt is the time between when the UE starts to execute the conditional LTM to the target cell and the time the UE starts to transmit the first UL message on the target cell. - Option 2: TLTM-interrupt is the interruption time as specified in Rel-18 LTM cell switch. - Option 3: Tinterrupt may induce ambiguity when define conditional LTM delay, as interruptions may also happen in early PRACH procedure (if supported in conditional LTM). - Option 4: The interruption time defined in Rel-18 LTM Cell Switch delay can be a baseline. - Option 5: RAN4 to discuss the starting point of the CLTM interruption, and the components included in TCLTM_interrupt. The goal shall be that CLTM interruption is shorter than CHO interruption. Issue 1-2-10: TTCI-state activate < Way Forward > FFS the following options: - Option 1: TTCI-state activate is the time for active the TCI state of candidate cell(s). - Other option is not precluded. Issue 1-2-11: The components of Tinterrupt < Way Forward > FFS the following options: - Option 1: Tinterrupt = TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU. - TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change. - Tfirst-RS is the time for fine time tracking and acquiring full timing information of the target cell. - TRS-proc is the time for SSB processing. - TLTM-IU is the interruption uncertainty during LTM cell switch.

Since we mentioned above, there is no explicit MAC-CE command like Rel-18 NW based LTM. Whether TRRC and TEvent_DU depend the procedure. At least, the measurement time and conditional execution preparation time and interruption time (Tinterrupt) should be included.

Proposal 5: The delay shall include the measurements time (Tmeasure) and the conditional execution preparation time (TCLTM_execution) and interruption time (Tinterrupt) at least. Others are waiting for further progress in other groups.

- Conclusion

In this contribution, we provide our proposals for RRM requirements of conditional Intra-CU LTM:

Proposal 1: For ending point of conditional LTM delay, reuse the same as that in Rel-18 LTM cell switch delay: UE transmits the first UL message on the target cell.

Proposal 2: Both RACH-based and RACH-less conditional LTM are supported.

Observation 1: For start point of conditional LTM delay, there is no explicit MAC-CE command like Rel-18 NW based LTM. There is a RRC message includes the LTM candidate configurations and execution conditions.

Proposal 4: RAN4 to discuss on start point of conditional LTM delay:

- Same as L3 CHO. RRC message includes the LTM candidate configurations and execution conditions.

- Similar as event-triggered L1 reporting. The CondEvent is met over the air.

Wait for further RAN1/RAN2 progress.

Proposal 5: The delay shall include the measurements time (Tmeasure) and the conditional execution preparation time (TCLTM_execution) and interruption time (Tinterrupt) at least. Others are waiting for further progress in other groups.

- Reference

[1] R4-2416863, WF on RRM requirements for NR_Mob_Ph4_Part2, China Telecom