3GPP TSG-RAN4 Meeting #113	R4-2418169

Orlando, US, Nov 18 – 22, 2024

Agenda item:			7.6.3

Source:	vivo

Title:	Impacts on UE RF requirements and DL performance

Document for:	Discussion

- Introduction

In RAN4#112bis meeting, the WF[1] has been agreed as follows, and this contribution will discuss about the remaining open issues.

# Main session online agreements #### Sub-topic 1-2: Study scope clarification Agreement: - Frequency span ≤ 100 MHz containing two non-contiguous CCs means the frequency separation between lower edge of lowest CC and upper edge of highest CC in downlink of a frequency band is less than or equal to 100MHz as illustrated below: - Any update of scope needs be decided by RAN #### Issue 3-1: Discussion on assumptions on test configurations for evaluation Agreement: (a) RAN4 agree on the general assumptions that are same for legacy NR RF performance evaluation: o UE self-band uplink Tx re-use same assumptions for NR PC3 PA i.e., ACLR=30dB, with MPR=1dB, Full RB allocation. Tx LO leakage and image rejection ratio are 28dB o Reuse the Rx chain image rejection, which is 25dB, as the starting point. Agreement: - The configuration defined in Table 7.3A.2.2-1 and Table 7.3A.2.2-2 of TS 38.101-1 could be re-used for performance evaluation, for ΔRIBNC, REFSENS, both out-of-gap and in-gap ACS, IBB and NBB requirements for selected example combos Agreement: - Priority to one uplink carrier Agreement: - Evaluation may consider impairment factors such as ACLR, phase noise or IMDx of aggressor ability, duplexer isolation…etc - The example bands should be in the scope of existing intra-band non-contiguous CA operating bands - No relaxation on any Tx RF requirements should be assumed in this study item - Both PC3 and PC2 should be considered - Prioritize PC3 - The minimum Wgap for ACS, narrow band blocking and in-band blocking verification should meet: - Wgap ≥ 2∙¦FInterferer (offset),j¦ – BWChannel(j) - It is the condition to apply the requirements - Both the symmetric CC scenario and the asymmetric CC scenario should be considered in the FC SI, and companies are encouraged, but not limited, to study the CC scenarios shared by the operators - [Adjusted requirements for fragmented carriers, if any, shall be separated from the existing requirements] - RAN4 shall not re-discuss the power imbalance assumption of the wanted carriers further in relation to the SI on fragmented carriers #### Issue 3-2: Example band consideration for evaluation Agreement: - RAN4 doesn’t consider 3MHz channel bandwidth for fragmented carriers - The example band(s) should already have NC DLCA specified - Prioritize the frequency bands of ≤2700MHz - Remove n39 and include n26 in the example bands: Consider n2/n25, n3, n7, n66, n41, n39, n26 as the example band # Ad hoc meeting agreements #### Merge issue 2-0/2-1 discussion: Agreement: The terminology of “One Rx RF Chain” is from single antenna to the end of ADC (before digital BB filter) with a single down-converter - To reduce the number of Rx RF chains, a fully shared RF chain (one Rx RF chain) is the reference architecture for fragmented carriers - A partially shared RF chain architecture is the reference architecture for existing DL NCCA requirements (refer to TR 36.823) #### Issue 2-2: Study on power spectral density difference between carriers of co-located adjacent channel operators Agreement: To evaluate acceptable [PSD] difference from UE side between wanted signal and un-wanted signal based on the deployment where BSs from different operators are collocated but their radio units (RUs) are not shared. - The [PSD] difference means the difference between unwanted signal and the wanted carrier with the lower PSD - Start evaluation with wanted signal level currently existing ACS requirements as baseline and to see the performance difference

- Discussion

OPEN ISSUES #### Issue 2-3: Signaling related #### Issue 2-4: Discussion on semi-statically switch hardware resources #### Issue 3-3: Observations of impacts on DL Rx requirements RAN4 discuss if following general aspects can be agreeable: - The largest interferer power in-gap shouldn’t larger than -25dBm for fragmented carriers using a single Rx RF chain - It is proposed to consider only same SCS for both CCs to ease receiver design for the common RF chain - ΔRIBNC for SCC may need to be updated to reflect the filter rejection reduction of common RF chain compared to separate RF chains for each intra-band non-contiguous CA band combination. Furthermore, ΔRIBNC needs to apply to PCC too, although the value of ΔRIBNC can be different for PCC and SCC - RAN4 shall not consider requirements for fragmented carriers which will result in performance degradation as compared to legacy NC IB CA or – only in case of a strong in-gap interferer – single CC operation

The following table is the reference sensitivity for PC3 intra-band NC CA with one UL in TS38.101-1. It can be seen that almost all the DL configuration is the carrier centre frequency of SCC is closer to the UL operating band, except for n66. As the current ΔRIBNC is the REFSENS relaxation value for SCC, so for n66 the evaluation of ΔRIBNC should be different from the other bands as the impact of sharing filter to SCC is smaller than PCC. But at the same time, the frequency separation of the UL and DL band for n66 is as wide as 330MHz, so ΔRIBNC is always equal to 0 for all the aggregated channel bandwidth, Wgap and UL PCC allocation. Therefore, even for fully shared RX chain it may not be deteriorated too much, and the evaluation of n66 could also be deprioritized. The configuration comparation of n66 with n2 is also illustrated in fig1. Besides, among the example bands in the agreement, n41 is the only TDD band that no longer needs to be evaluated for ΔRIBNC.

Table 7.3A.2.2-1: Power class 3 intra-band non-contiguous CA reference sensitivity with one uplink.

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation (LCRB) | SCC ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n1(2A) | 15/15 | 5MHz + 5MHz | 0.0 < Wgap ≤ 50.0 | 25 | 0.5 | FDD |
| CA_n2(2A) | 15/15 | 5MHz + 5MHz | Wgap = 50.0 | 105 | 5.0 | FDD |
|  |  |  | Wgap = 30.0 | 25 | 0.0 |  |
| CA_n3(2A) | 15/15 | 5MHz + 5MHz | Wgap = 65.0 | 125 | 4.7 | FDD |
|  |  |  | Wgap = 45.0 | 255 | 0.0 |  |
| CA_n5(2A) | 15/15 | 15MHz + 5MHz | Wgap = 5.0 | 55 | 6.3 | FDD |
| CA_n7(2A) | 15/15 | 10MHz + 5MHz | Wgap = 55 | 325 | 0.0 | FDD |
|  |  |  | Wgap = 30 | 505 | 0.0 |  |
| CA_n12(2A) | 15/15 | 5MHz + 5MHz | 0.0 < Wgap ≤ 7.0 | 5 (RBstart=12) | 3 | FDD |
| CA_n25(2A) 9 | 15/15 | 5MHz + 5MHz | Wgap = 55.0 | 105 | 5.0 | FDD |
|  |  |  | Wgap = 30.0 | 25 | 0.0 |  |
| CA_n25(2A) 10 CA_n25(3A) | 15/15 | 40MHz + 5MHz | Wgap = 20.0 | 40 (RBstart = 176) | [24.6] 8 | FDD |
| CA_n26(2A) | 15/15 | 15MHz + 10MHz | Wgap = 10.0 | 5 (RBstart = 74) | 25.2 | FDD |
| CA_n66(2A) CA_n66(3A) | N/A | NOTE 1 | NOTE 2 | NOTE 3, NOTE 4 | 0.0 | FDD |
| CA_n71(2A) | 15/15 |  |  |  |  |  |
|  |  | 15MHz + 10MHz | Wgap = 10.0 | 5 (RBstart = 2) | 22.2 |  |
|  |  | 25MHz + 5MHz11 | Wgap = 5.0 | 20 (RBstart = 19) | 25.1 |  |
| NOTE 1: All combinations of channel bandwidths defined in Table 5.5A.2-1. NOTE 2: All applicable sub-block gap sizes. NOTE 3: The PCC allocation is same as Transmission bandwidth configuration NRB as defined in Table 5.3.2-1. NOTE 4: The carrier center frequency of PCC in the DL operating band is configured closer to the UL operating band. NOTE 5: Refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission. NOTE 6: Wgap is the sub-block gap between the two sub-blocks. NOTE 7: The carrier centre frequency of SCC in the DL operating band is configured closer to the UL operating band. NOTE 8: For operation with three or more non-contiguous component carriers, ΔRIBNC applies to all secondary component carriers. NOTE 9: Bandwidth Combination Set 0. NOTE 10: Bandwidth Combination Set 1 NOTE 11: Applicable only to Bandwidth Combination Set 4 and 5 and for UEs supporting the symmetrical UL/DL channel bandwidths. |  |  |  |  |  |  |

Fig1. Configuration comparation of CA_n66(2A) with CA_n2(2A)

Observation1：For CA_n66(2A), the configuration of reference sensitivity testing is the center frequency of PCC in the DL is closer to the UL, which is different from the other bands.

Observation2：For CA_n66(2A), the frequency separation of the UL and DL band for n66 is as wide as 330MHz, so ΔRIBNC may not be deteriorated too much with RX chain sharing.

Proposal 1：Prioritize the re-evaluation of ΔRIBNC in n2, n3, n7, n25, n26.

- n66 needs separate evaluation and could be deprioritized as its wide frequency separation of UL and DL.

When the in-gap interferer falls in the frequency range further than IBB2, then there is no corresponding requirement to evaluate the interference, and as it is still in the UE receive band the definition of OOB blocking could not apply either. Considering the same frequency separation between the interferer and the target CC, for the separate chain the impact of the interference is rather low as the independent narrow filter could provide enough attenuation especially for the range away from the passband. However, for the shared filter the impact could not be ignored anymore. Whether or not to just limit the absolute power of the in-gap interferer is enough needs to be discussed.

Observation 3: There is no corresponding RX requirement to evaluate the power level of the in-gap interferer falling in the frequency range further than IBB2.

Proposal 2: RAN4 should discuss whether to specify a new RX requirement for the in-gap interferer falling in the frequency range further than IBB2.

- Conclusion

Observation1：For CA_n66(2A), the configuration of reference sensitivity testing is the center frequency of PCC in the DL is closer to the UL, which is different from the other bands.

Observation2：For CA_n66(2A), the frequency separation of the UL and DL band for n66 is as wide as 330MHz, so ΔRIBNC may not be deteriorated too much with RX chain sharing.

Proposal 1：Prioritize the re-evaluation of ΔRIBNC in n2, n3, n7, n25, n26.

- n66 needs separate evaluation and could be deprioritized as its wide frequency separation of UL and DL.

Observation 3: There is no corresponding RX requirement to evaluate the power level of the in-gap interferer falling in the frequency range further than IBB2.

Proposal 2: RAN4 should discuss whether to specify a new RX requirement for the in-gap interferer falling in the frequency range further than IBB2.

- References

- R4-2417201, WF on NR FR1 DL fragmented carriers study, MediaTek Inc. RAN4#112bis.