3GPP TSG-RAN4 Meeting #113	R4-2418173

# Orlando, US, Nov 18 – 22, 2024

Agenda item:			7.1.1.2.3

Source:	vivo

Title:	Discussion on MPR applicability for FR2

Document for:	Discussion

- Introduction

In RAN#112bis meeting, the WF[1] has been agreed, and the remaining issue is as follows:

# Topic #3: MPR applicability for FR2 #### Issue 3-1-2: Applicable MPR for FR2 UL CA with DL intra band CA Agreement in main session: - In general, with UE indication of new capability [FR2 MPR enhancements-R19] for MPR improvement, MPR based on UL BWchannel_CA applies instead that based on cumulative aggregated channel BW (CABW). - If only 1 UL CC is activated, the MPR requirements of single carrier could be reused. - A note should be added to the MPR table, e.g. - For FR2-1, for 200MHz BW granularity, RAN4 to change the MPR calculation for DFT-s-BPSK or DFT-s-QPSK from “MPRC_CA = MAX(MPR1, MPR2)” to “MPRC_CA = MPR2”, for UE with UE indication of new capability for MPR improvement. #### Issue 3-2-1: Optional UE capability Agreement in main session: - An optional per UE capability for FR2 is introduced - The capability is implementation agnostic - Supporting FR2 MPR enhancement is applicable only from Rel-19 #### Issue 3-2-2: CC activation-based MPR improvement - WF - Further discuss activated based MPR improvement. - FFS using common UE capability between configuration based and activation based, or another UE capability for activated-based improvements. - FFS MPR can be determined based on activated UL CCs BW instead of Cumulative aggregated channel bandwidth (CABW). (i.e., activation based also applies to UL CA case) - FFS limitation the enhancement to cases where the activated UL CCs form a contiguous block.

- Discussion

As has been recorded in the WF of #112bis meeting, with UE indication of new capability [fr2-MPR-ImprovementUplinkDependent-r19] for MPR improvement, MPR based on UL BWchannel_CA applies instead that based on cumulative aggregated channel BW (CABW). However, there is still controversy when discussing activated-based improvements.

And in the WID there is one objective as follows: MPR requirement is not applicable until the SCell is activated. Therefore, MPR will not be applicable when the CCs are only configured. When only 1CC is configured and also activated then the MPR of single carrier applies. However, when there are several CCs are all configured but only one of them is activated, with a common and unclear UE capability, whether to choose the MPR based on configured aggregated CBW or single activated CC is tricky.

Observation 1: If using common UE capability between configuration based and activation based, the BW selection for MPR would be tricky.

For the sake of distinguishing the referred bandwidth of configured CCs and activated CCs, it is necessary to set a new UE capability to indicate the activated-based MPR improvements, and the mechanism is as follows:

When [fr2-MPR-ImprovementUplinkDependent-r19] is supported, and the intra-band contiguous CA is configured with single UL CC, MPRWT_C_CA shall be determined from Tables 6.2.2.1-1 and 6.2.2.1-2, i.e., the MPR of single carrier is used.

When [fr2-MPR-ImprovementUplinkDependent-ActivationBased-r19] is supported, and the intra-band contiguous CA is activated with single UL CC, MPRWT_C_CA shall be determined from Tables 6.2.2.1-1 and 6.2.2.1-2, i.e., the MPR of single carrier is used.

If [fr2-MPR-ImprovementUplinkDependent-r19] is supported, configured UL BWchannel_CA is used as bandwidth basis, otherwise Cumulative aggregated channel bandwidth (CABW) is used as bandwidth basis.

If [fr2-MPR-ImprovementUplinkDependent-ActivationBased-r19] is supported, activated UL BWchannel_CA is used as bandwidth basis, otherwise Cumulative aggregated channel bandwidth (CABW) is used as bandwidth basis.

Proposal 1: A new UE capability is needed to support the activated-based MPR improvements.

Proposal 2: MPR can be determined based on activated UL CCs BW instead of Cumulative aggregated channel bandwidth (CABW) with UE indicating the corresponding capability, and NOTE4 in the following example table should be added to all the FR2-1 & FR2-2 intra-band C CA MPR tables with all power classes:

Table 6.2A.2.2-1: Maximum power reduction (MPRWT_C_CA) for UE power class 1 in FR2-1

| Waveform Type |  | bandwidth basis3,4 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | < 400 MHz | ≥ 400 MHz and < 800 MHz | ≥ 800 MHz and ≤ 1400 MHz | > 1400 MHz and ≤ 2400 MHz |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 5.51 | ≤ 7.7 | ≤ 8.2 | ≤ 8.7 |
|  | QPSK | ≤ 6.51 | ≤ 8.7 | ≤ 9.7 | ≤ 9.7 |
|  | 16 QAM | ≤ 6.5 | ≤ 8.7 | ≤ 9.2 | ≤ 9.7 |
|  | 64 QAM | ≤ 9.0 | ≤ 10.7 | ≤ 11.2 | ≤ 11.7 |
|  | 256 QAM2 | ≤ 12.5 | ≤ 14.2 | ≤ 14.7 | ≤ 15.7 |
| CP-OFDM | QPSK | ≤ 6.5 | ≤ 8.7 | ≤ 8.7 | ≤ 9.7 |
|  | 16 QAM | ≤ 6.5 | ≤ 8.7 | ≤ 8.7 | ≤ 9.7 |
|  | 64 QAM | ≤ 9.0 | ≤ 10.7 | ≤ 11.2 | ≤ 11.7 |
|  | 256 QAM2 | ≤ 12.5 | ≤ 14.2 | ≤ 14.7 | ≤ 15.7 |
| NOTE 1: (Void) NOTE 2: Refer to clause 6.1 for 256 QAM applicability. NOTE 3: If [fr2-MPR-ImprovementUplinkDependent-r19] is supported, configured UL BWchannel_CA is used as bandwidth basis, otherwise Cumulative aggregated channel bandwidth (CABW) is used as bandwidth basis. NOTE 4: If [fr2-MPR-ImprovementUplinkDependent- ActivationBased -r19] is supported, activated UL BWchannel_CA is used as bandwidth basis, otherwise Cumulative aggregated channel bandwidth (CABW) is used as bandwidth basis. |  |  |  |  |  |

- Conclusion

Observation 1: If using common UE capability between configuration based and activation based, the BW selection for MPR would be tricky.

Proposal 1: A new UE capability is needed to support the activated-based MPR improvements.

Proposal 2: MPR can be determined based on activated UL CCs BW instead of Cumulative aggregated channel bandwidth (CABW) with UE indicating the corresponding capability, and NOTE4 in the following example table should be added to all the FR2-1 & FR2-2 intra-band C CA MPR tables with all power classes:

Table 6.2A.2.2-1: Maximum power reduction (MPRWT_C_CA) for UE power class 1 in FR2-1

| Waveform Type |  | bandwidth basis3,4 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | < 400 MHz | ≥ 400 MHz and < 800 MHz | ≥ 800 MHz and ≤ 1400 MHz | > 1400 MHz and ≤ 2400 MHz |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 5.51 | ≤ 7.7 | ≤ 8.2 | ≤ 8.7 |
|  | QPSK | ≤ 6.51 | ≤ 8.7 | ≤ 9.7 | ≤ 9.7 |
|  | 16 QAM | ≤ 6.5 | ≤ 8.7 | ≤ 9.2 | ≤ 9.7 |
|  | 64 QAM | ≤ 9.0 | ≤ 10.7 | ≤ 11.2 | ≤ 11.7 |
|  | 256 QAM2 | ≤ 12.5 | ≤ 14.2 | ≤ 14.7 | ≤ 15.7 |
| CP-OFDM | QPSK | ≤ 6.5 | ≤ 8.7 | ≤ 8.7 | ≤ 9.7 |
|  | 16 QAM | ≤ 6.5 | ≤ 8.7 | ≤ 8.7 | ≤ 9.7 |
|  | 64 QAM | ≤ 9.0 | ≤ 10.7 | ≤ 11.2 | ≤ 11.7 |
|  | 256 QAM2 | ≤ 12.5 | ≤ 14.2 | ≤ 14.7 | ≤ 15.7 |
| NOTE 1: (Void) NOTE 2: Refer to clause 6.1 for 256 QAM applicability. NOTE 3: If [fr2-MPR-ImprovementUplinkDependent-r19] is supported, configured UL BWchannel_CA is used as bandwidth basis, otherwise Cumulative aggregated channel bandwidth (CABW) is used as bandwidth basis. NOTE 4: If [fr2-MPR-ImprovementUplinkDependent- ActivationBased -r19] is supported, activated UL BWchannel_CA is used as bandwidth basis, otherwise Cumulative aggregated channel bandwidth (CABW) is used as bandwidth basis. |  |  |  |  |  |

- References

- R4-2417204, WF on power domain enhancements, Huawei, HiSilicon, RAN4 #112bis.