3GPP TSG-RAN WG4 Meeting # 113      	R4-2418174

# Orlando, US, Nov 18 – 22, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR |  | rev |  | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR to TS 38.101-2 on MPR applicability for FR2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_RF_Ph4-Core |  |  |  |  |  | Date: |  |  | 2024-11-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | If MPR improvement based on activation CBW is approved, the corresponding UE capability should also be added in the spec as the configured ULBW is different from the activated ULBW. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new UE capability and BW changes to relevant MPR tables and context descriptions. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The referred bandwidth of configured CCs and activated CCs could not be dinstinguished if the UE capability is not divided. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.2.2, 6.2A.2.3, 6.2A.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS38.521 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Unchanged sections are omitted>

#### 6.2A.2.2	Maximum output power reduction for power class 1

##### 6.2A.2.2.1	Maximum output power reduction for power class 1 intra-band contiguous UL CA

For power class 1, MPR for intra-band contiguous UL CA with contiguous allocations within the cumulative aggregated bandwidth is defined as:

MPRC_CA = max(MPRWT_C_CA+∆MPR, MPRnarrow)

Where,

MPRnarrow = 14.4 dB, when BWalloc,RB is less than or equal to 1.44 MHz, MPRnarrow = 10 dB, when 1.44 MHz < BWalloc,RB ≤ 10.8 MHz, where BWalloc,RB is the bandwidth of the RB allocation size.

MPRWT_C_CA is the maximum power reduction due to modulation orders, transmit bandwidth configurations, and waveform types. MPRWT_C_CA is defined in Tables 6.2A.2.2-1 and 6.2A.2.2-2. If [fr2-MPR-ImprovementUplinkDependent-r19] is supported and the intra-band contiguous CA is configured with single UL CC, MPRWT_C_CA shall be determined from Tables 6.2.2.1-1 and 6.2.2.1-2. If [fr2-MPR-ImprovementUplinkDependent-ActivationBased-r19] is supported and the intra-band contiguous CA is activated with single UL CC, MPRWT_C_CA shall be determined from Tables 6.2.2.1-1 and 6.2.2.1-2.

∆MPR for FR2-1 256 QAM as specified in Table 6.2.2.1-5 applies.

Table 6.2A.2.2-1: Maximum power reduction (MPRWT_C_CA) for UE power class 1 in FR2-1

| Waveform Type |  | bandwidth basis3,4 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | < 400 MHz | ≥ 400 MHz and < 800 MHz | ≥ 800 MHz and ≤ 1400 MHz | > 1400 MHz and ≤ 2400 MHz |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 5.51 | ≤ 7.7 | ≤ 8.2 | ≤ 8.7 |
|  | QPSK | ≤ 6.51 | ≤ 8.7 | ≤ 9.7 | ≤ 9.7 |
|  | 16 QAM | ≤ 6.5 | ≤ 8.7 | ≤ 9.2 | ≤ 9.7 |
|  | 64 QAM | ≤ 9.0 | ≤ 10.7 | ≤ 11.2 | ≤ 11.7 |
|  | 256 QAM2 | ≤ 12.5 | ≤ 14.2 | ≤ 14.7 | ≤ 15.7 |
| CP-OFDM | QPSK | ≤ 6.5 | ≤ 8.7 | ≤ 8.7 | ≤ 9.7 |
|  | 16 QAM | ≤ 6.5 | ≤ 8.7 | ≤ 8.7 | ≤ 9.7 |
|  | 64 QAM | ≤ 9.0 | ≤ 10.7 | ≤ 11.2 | ≤ 11.7 |
|  | 256 QAM2 | ≤ 12.5 | ≤ 14.2 | ≤ 14.7 | ≤ 15.7 |
| NOTE 1: (Void) NOTE 2: Refer to clause 6.1 for 256 QAM applicability. NOTE 3: If [fr2-MPR-ImprovementUplinkDependent-r19] is supported, configured UL BWchannel_CA is used as bandwidth basis, otherwise Cumulative aggregated channel bandwidth (CABW) is used as bandwidth basis. NOTE 4: If [fr2-MPR-ImprovementUplinkDependent- ActivationBased -r19] is supported, activated UL BWchannel_CA is used as bandwidth basis, otherwise Cumulative aggregated channel bandwidth (CABW) is used as bandwidth basis. |  |  |  |  |  |

Table 6.2A.2.2-2: Maximum power reduction (MPRWT_C_CA) for UE power class 1 in FR2-2

| Waveform Type | Cumulative aggregated channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | < 400 MHz | ≥ 400 MHz and < 800 MHz | ≥ 800 MHz and ≤ 1400 MHz | > 1400 MHz and ≤ 2000 MHz |
| Pi/2 BPSK | ≤ 7.0 | ≤ 5.0 | ≤ 2.0 | ≤ 2.0 |
| QPSK | ≤ 8.0 | ≤ 6.0 | ≤ 3.0 | ≤ 3.0 |
| 16 QAM | ≤ 8.0 | ≤ 6.0 | ≤ 4.0 | ≤ 4.0 |
| 64 QAM | ≤ 10.0 | ≤ 10.0 | ≤ 10.0 | ≤ 10.0 |

In case of a contiguous RB, DFT-s-BPSK or DFT-s-QPSK UL allocation in a single CC of a CA configuration with contiguous CCs, and whose cumulative aggregated BW  400 MHz, MPRWT_C_CA shall instead be MPR2 if [fr2-MPR-ImprovementUplinkDependent-r19] or [fr2-MPR-ImprovementUplinkDependent- ActivationBased -r19] is supported and derived as MAX(MPR1, MPR2) otherwise, where:

MPR1 shall be determined from Table 6.2.2.1-1 if CABW  200 MHz, from Table 6.2.2.1-2 if CABW > 200 MHz.

MPR2 shall be determined from Table 6.2.2.1-1 if UL BWchannel_CA  200 MHz, from Table 6.2.2.1-2 if UL BWchannel_CA > 200 MHz.

When [fr2-MPR-ImprovementUplinkDependent-r19] is supported, MPR2 shall be determined from Table 6.2.2.1-1 if configured UL BWchannel_CA  200 MHz, from Table 6.2.2.1-2 if configured UL BWchannel_CA > 200 MHz.

When [fr2-MPR-ImprovementUplinkDependent- ActivationBased -r19] is supported, MPR2 shall be determined from Table 6.2.2.1-1 if activated UL BWchannel_CA  200 MHz, from Table 6.2.2.1-2 if activated UL BWchannel_CA > 200 MHz.

and assume all UL CCs use the same SCS for the purpose of determination of inner and outer RB allocations in Table 6.2.2.1-1 and Table 6.2.2.1-2:

NRB shall be chosen as the sum of NRB of all constituent UL CCs in the CA configuration.

LCRB shall be chosen as BWalloc,RB

RBstart shall be derived as: RBstart_allocatedCC+NRB_unallocatedCC_low

RBstart_allocatedCC is the index of the first allocated RB in the CC with allocation

NRB_unallocatedCC_low is the sum of NRB in all UL CCs lower in frequency compared to the CC with allocation

When different waveform types exist across CCs, the requirement is set by the waveform type used in the configuration with the largest MPRC_CA.

For intra-band contiguous UL CA with non-contiguous RB allocations, the following rule for MPR applies:

MPR = max(MPRC_CA, -10*A +  14.4)

Where:

A = NRB_alloc / NRB_agg_C.

NRB_alloc is the total number of allocated UL RBs

NRB_agg_C is the number of the aggregated RBs within the fully allocated cumulative aggregated channel bandwidth assuming lowest SCS among all configured CCs; If [fr2-MPR-ImprovementUplinkDependent-r19] is supported, NRB_agg_C is the number of the configured aggregated RBs within the fully allocated UL aggregated BW assuming lowest SCS among all configured CCs; If [fr2-MPR-ImprovementUplinkDependent- ActivationBased -r19] is supported, NRB_agg_C is the number of the activated aggregated RBs within the fully allocated UL aggregated BW assuming lowest SCS among all configured CCs.

<Unchanged sections are omitted>

#### 6.2A.2.3	Maximum output power reduction for power class 2

For power class 2, MPR (except 256 QAM) specified in sub-clause 6.2A.2.4.1 applies for intra-band contiguous UL CA and sub-clause 6.2A.2.4.2 applies for intra-band non-contiguous UL CA.

Table 6.2A.2.3-1: (Void)

For FR2-1 256 QAM, for intra-band contiguous UL CA with contiguous allocations within the cumulative aggregated bandwidth, the following rule for MPR applies:

MPR = MPRC_CA+∆MPR

MPRC_CA is defined in Table 6.2A.2.3-2. △MPR as specified in Table 6.2.2.1-5 applies. If [fr2-MPR-ImprovementUplinkDependent-r19] is supported and the intra-band contiguous CA is configured with single UL CC, MPRC_CA shall be determined from Tables 6.2.2.2-2 and 6.2.2.2-3. If [fr2-MPR-ImprovementUplinkDependent-ActivationBased-r19] is supported and the intra-band contiguous CA is activated with single UL CC, MPRWT_C_CA shall be determined from Tables 6.2.2.2-2 and 6.2.2.2-3.

Table 6.2A.2.3-2: Maximum power reduction (MPRC_CA) for UE power class 2 in FR2-1

|  |  | bandwidth basis2,3 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | ≤ 400 MHz | > 400 MHz and < 800 MHz | ≥ 800 MHz and ≤ 1400 MHz | > 1400 MHz and ≤ 2400 MHz |
| DFT-s-OFDM | 256 QAM1 | ≤ 12.5 | ≤ 14.2 | ≤ 14.7 | ≤ 15.7 |
| CP-OFDM | 256 QAM1 | ≤ 12.5 | ≤ 14.2 | ≤ 14.7 | ≤ 15.7 |
| NOTE 1: Refer to clause 6.1 for 256QAM applicability. NOTE 2: If [fr2-MPR-ImprovementUplinkDependent-r19] is supported, configured UL BWchannel_CA is used as bandwidth basis, otherwise Cumulative aggregated channel bandwidth (CABW) is used. NOTE 3: If [fr2-MPR-ImprovementUplinkDependent- ActivationBased -r19] is supported, activated UL BWchannel_CA is used as bandwidth basis, otherwise Cumulative aggregated channel bandwidth (CABW) is used as bandwidth basis. |  |  |  |  |  |

For FR2-1 256 QAM, for intra-band contiguous UL CA with non-contiguous RB allocations, the following rule for MPR applies:

MPR = max(MPRC_CA+∆MPR, -10*A +7.0)

Where:

A = NRB_alloc / NRB_agg_C.

NRB_alloc is the total number of allocated UL RBs

NRB_agg_C is the number of the aggregated RBs within the fully allocated cumulative aggregated channel bandwidth assuming lowest SCS among all configured CCs; If [fr2-MPR-ImprovementUplinkDependent-r19] is supported, NRB_agg_C is the number of the configured aggregated RBs within the fully allocated UL aggregated BW assuming lowest SCS among all configured CCs; If [fr2-MPR-ImprovementUplinkDependent- ActivationBased -r19] is supported, NRB_agg_C is the number of the activated aggregated RBs within the fully allocated UL aggregated BW assuming lowest SCS among all configured CCs.

<Unchanged sections are omitted>

#### 6.2A.2.4	Maximum output power reduction for power class 3

##### 6.2A.2.4.1	Maximum output power reduction for power class 3 intra-band contiguous CA

For power class 3, MPR for intra-band contiguous UL CA with contiguous allocations within the cumulative aggregated bandwidth is denoted as MPRC_CA and is defined in Tables 6.2A.2.4-1 and 6.2A.2.4-2. If [fr2-MPR-ImprovementUplinkDependent-r19] is supported and the intra-band contiguous CA is configured with single UL CC, MPRC_CA shall be determined from Tables 6.2.2.3-1 and 6.2.2.3-2. If [fr2-MPR-ImprovementUplinkDependent-ActivationBased-r19] is supported and the intra-band contiguous CA is activated with single UL CC, MPRWT_C_CA shall be determined from Tables 6.2.2.3-1 and 6.2.2.3-2.

Table 6.2A.2.4-1: Maximum power reduction (MPRC_CA) for UE power class 3 in FR2-1

|  |  | bandwidth basis2,3 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | ≤ 400 MHz | > 400 MHz and < 800 MHz | ≥ 800 MHz and ≤ 1400 MHz | > 1400 MHz and ≤ 2400 MHz |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 5.01 | ≤ 7.7 | ≤ 8.2 | ≤ 8.7 |
|  | QPSK | ≤ 5.01 | ≤ 7.7 | ≤ 8.2 | ≤ 9.7 |
|  | 16 QAM | ≤ 6.5 | ≤ 8.7 | ≤ 9.3 | ≤ 9.7 |
|  | 64 QAM | ≤ 9.0 | ≤ 10.7 | ≤ 11.2 | ≤ 11.7 |
| CP-OFDM | QPSK | ≤ 5.0 | ≤ 7.5 | ≤ 8.0 | ≤ 9.7 |
|  | 16 QAM | ≤ 6.5 | ≤ 8.7 | ≤ 9.2 | ≤ 9.7 |
|  | 64 QAM | ≤ 9.0 | ≤ 10.7 | ≤ 11.2 | ≤ 11.7 |
| NOTE 1: (Void). NOTE 2: If [fr2-MPR-ImprovementUplinkDependent-r19] is supported, configured UL BWchannel_CA is used as bandwidth basis, otherwise Cumulative aggregated channel bandwidth (CABW) is used. NOTE 3: If [fr2-MPR-ImprovementUplinkDependent- ActivationBased -r19] is supported, activated UL BWchannel_CA is used as bandwidth basis, otherwise Cumulative aggregated channel bandwidth (CABW) is used as bandwidth basis. |  |  |  |  |  |

Table 6.2A.2.4-2: Maximum power reduction (MPRWT_C_CA) for UE power class 3 in FR2-2

| Waveform Type | Cumulative aggregated channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | < 400 MHz | ≥ 400 MHz and < 800 MHz | ≥ 800 MHz and ≤ 1400 MHz | > 1400 MHz and ≤ 2000 MHz |
| Pi/2 BPSK | ≤ 1.0 | ≤ 1.0 | ≤ 1.0 | ≤ 1.0 |
| QPSK | ≤ 2.0 | ≤ 2.0 | ≤ 2.0 | ≤ 2.0 |
| 16 QAM | ≤ 4.0 | ≤ 4.0 | ≤ 4.0 | ≤ 4.0 |
| 64 QAM | ≤ 10.0 | ≤ 10.0 | ≤ 10.0 | ≤ 10.0 |

In case of a contiguous RB, DFT-s-BPSK or DFT-s-QPSK UL allocation in a single CC of a CA configuration with contiguous CCs, and whose cumulative aggregated BW  400 MHz, MPRC_CA shall instead be MPR2 if [fr2-MPR-ImprovementUplinkDependent-r19] or [fr2-MPR-ImprovementUplinkDependent- ActivationBased -r19] is supported, and derived instead as MAX(MPR1, MPR2) otherwise, where:

MPR1 shall be determined from Table 6.2.2.3-1 if CABW  200 MHz, from Table 6.2.2.3-2 if CABW > 200 MHz.

MPR2 shall be determined from Table 6.2.2.3-1 if UL BWchannel_CA  200 MHz, from Table 6.2.2.3-2 if UL BWchannel_CA > 200 MHz.

When [fr2-MPR-ImprovementUplinkDependent-r19] is supported, MPR2 shall be determined from Table 6.2.2.3-1 if configured UL BWchannel_CA  200 MHz, from Table 6.2.2.3-2 if configured UL BWchannel_CA > 200 MHz.

When [fr2-MPR-ImprovementUplinkDependent- ActivationBased -r19] is supported, MPR2 shall be determined from Table 6.2.2.3-1 if activated UL BWchannel_CA  200 MHz, from Table 6.2.2.3-2 if activated UL BWchannel_CA > 200 MHz.

and assume all UL CCs use the same SCS for the purpose of determination of inner and outer RB allocations in Table 6.2.2.3-1 and Table 6.2.2.3-2:

NRB shall be chosen as the sum of NRB of all constituent UL CCs in the CA configuration.

LCRB shall be chosen as BWalloc,RB

RBstart shall be derived as: RBstart_allocatedCC+NRB_unallocatedCC_low

RBstart_allocatedCC is the index of the first allocated RB in the CC with allocation

NRB_unallocatedCC_low is the sum of NRB in all UL CCs lower in frequency compared to the CC with allocation

When different waveform types exist across CCs, the requirement is set by the waveform type used in the configuration with the highest contiguous MPR.

For intra-band contiguous UL CA with non-contiguous RB allocations, the following rule for MPR applies:

MPR = max(MPRC_CA, -10*A +7.0)

Where:

A = NRB_alloc / NRB_agg_C.

NRB_alloc is the total number of allocated UL RBs

NRB_agg_C is the number of the aggregated RBs within the fully allocated cumulative aggregated channel bandwidth assuming lowest SCS among all configured CCs; If [fr2-MPR-ImprovementUplinkDependent-r19] is supported, NRB_agg_C is the number of the configured aggregated RBs within the fully allocated UL aggregated BW assuming lowest SCS among all configured CCs; If [fr2-MPR-ImprovementUplinkDependent- ActivationBased -r19] is supported, NRB_agg_C is the number of the activated aggregated RBs within the fully allocated UL aggregated BW assuming lowest SCS among all configured CCs.

<Unchanged sections are omitted>