3GPP TSG-RAN WG4 Meeting # 103 	R4-2418179

Orlando, US, 18th – 22nd November, 2024

Source: 	vivo

Title: 	Discussion on remaining issue for FR2 OTA

Agenda Item:	7.13.2

Document for:	Approval

- Introduction

In this contribution, we provide our views on the remaining issue for sTxMP test method.

- Discussion

  - Alternative test direction for min peak EIRP

Agreements:

  - The test direction other than the beam peak direction of single CC operation without STxMP can be used for the first AoA if the test results based on the default AoA separation fail the limits in the spec. The following options can be considered:

    - Option 1: A local scan in xz plane is conducted. As long as test results do not pass the spec yet during scan, the local scan does not stop until AoA1 EIRP fail the legacy beam peak EIRP limit in both directions. FFS on the measurement grid of local scan.

    - Option 2: Alternative test directions on top of the default AoA separation can be declared by UE. FFS on the allowed offset of alternative test directions to the default test directions.

  - Option 1 and Option 2 only apply for EIRP PUMAX,f,c,k testing and will not have the impact on the EIRPmax and TRPmax test directions selection.

  - RAN4 to make down selection from Option 1 and Option 2 in next meeting

The following agreements were achieved in the last meeting:

In our understanding, there is no big difference between option 1 and option 2, and the intention is to allow UE can be verified in more directions. It is understandable since when both antenna array are activated, the beam pattern may change and the beam peak may offset a little. The option 2 seems simpler to let UE declare the offset on top of the default direction, but the main problem here is that for different UE the offset may not be exactly same, which may bring more test effort for UE vendor. The option 1 is a general procedure for all UE which is more stable.

Observation 1: The option 2 give UE flexibility to declare the alternative direction but may bring more test effort for UE vendor.

Proposal 1: Take the option 1 as the method for finding alternative test direction for min peak EIRP in sTxMP and the test grid is same as beam peak search.

  - Link polarization combinations

The polarization combination during the test is also not decided yet. In multi-Rx, (θ, θ) and (φ,φ) is used which is considered as the worst case for the interference between beams. However, such consideration may not be needed for sTxMP which is only measure the radiated power. Since the TE needs to differentiate the beams during the test, it may be beneficial to use different polarization from this perspective.

Proposal 2: It is suggested to only test the polarization combination (θ,φ) and (φ,θ) during the test.

- Conclusion

In this contribution, we provide our views on the remaining issue of sTxMP test method.

Observation 1: The option 2 give UE flexibility to declare the alternative direction but may bring more test effort for UE vendor.

Proposal 1: Take the option 1 as the method for finding alternative test direction for min peak EIRP in sTxMP and the test grid is same as beam peak search.

Proposal 2: It is suggested to only test the polarization combination (θ,φ) and (φ,θ) during the test.

- References

- R4-2416580	Way Forward for [112bis][325] NR_FR2_OTA, Qualcomm, RAN4#112bis