3GPP TSG-RAN WG4 Meeting # 103 	R4-2418183

Orlando, US, 18th – 22nd November, 2024

Source: 	vivo

Title: 	Discussion on the RF requirement for AIoT BS

Agenda Item:	7.21.3.1

Document for:	Approval

- Introduction

In the previous meeting, the RF requirements for AIoT BS were discussed, but there are still several requirements are marked as TBD. In this contribution, we provide our views on these pending requirements.

- Discussion

  - Tx

The following table was agreed in the last meeting [1]:

| RF Requirement for A-IoT BS- TX requirements |  | Potentially needed or not |
| --- | --- | --- |
| Base station output power |  | YES, as RAN4#112 agreed |
| Output power dynamic | RE power control dynamic range | TBD |
|  | Total power dynamic range | TBD |
|  | Power boosting | NO |
| Some requirements like Transmit ON/OFF power | Transmitter OFF power | TBD |
|  | Transmitter transient period | TBD |
| Transmit signal quality | Frequency error | YES, as RAN4#112 agreed |
|  | Some requirements like EVM (Modulation quality) | YES, as RAN4#112 agreed |
|  | TAE | NA, as RAN4#112 agreed |
| Unwanted emissions | Occupied bandwidth | YES, as RAN4#112 agreed |
|  | ACLR | YES, as RAN4#112 agreed |
|  | IBE | NO |
|  | Operating band unwanted emissions | YES, as RAN4#112 agreed |
|  | Transmitter spurious emissions | YES, as RAN4#112 agreed |
| Transmitter intermodulation |  | TBD |

- Output power dynamic (RE power control dynamic range, Total power dynamic range)

The power dynamic range is a requirement to reflect the performance of power amplifier and coverage, which should be a basic requirement for BS. For NB-IoT, this requirement is defined and is related to power boosting. So this requirement should be defined but may need to studied based on the OOK waveform.

- Transmit ON/OFF power/Transmitter transient period

These two requirements is only defined for TDD band in current BS spec, so for the AIoT which is work in the FDD band, these two requirements are not needed.

- Transmitter intermodulation

This requirement mainly defined for co-located BS scenario which doesn’t happen based on current AIoT assumption, so this requirement is also not needed.

Proposal 1: For AIoT BS Tx requirements, take the following table as baseline:

| RF Requirement for A-IoT BS- TX requirements |  | Potentially needed or not |
| --- | --- | --- |
| Base station output power |  | YES |
| Output power dynamic | RE power control dynamic range | YES |
|  | Total power dynamic range | YES |
|  | Power boosting | NO |
| Some requirements like Transmit ON/OFF power | Transmitter OFF power | NO |
|  | Transmitter transient period | NO |
| Transmit signal quality | Frequency error | YES |
|  | Some requirements like EVM (Modulation quality) | YES |
|  | TAE | NO |
| Unwanted emissions | Occupied bandwidth | YES |
|  | ACLR | YES |
|  | IBE | NO |
|  | Operating band unwanted emissions | YES |
|  | Transmitter spurious emissions | YES |
| Transmitter intermodulation |  | NO |

  - Rx

The following table was agreed in the last meeting [1]:

| A-IoT BS- RX RF Requirement |  | Potentially needed or not |
| --- | --- | --- |
| Reference sensitivity level |  | YES, as RAN4#112 agreed |
| Dynamic range |  | Discussed in the WI phase |
| Adjacent Channel Selectivity |  | YES, as RAN4#112 agreed |
| In-band blocking | general blocking | YES |
|  | narrowband blocking | YES |
| Out-of-band blocking |  | YES |
| Receiver spurious emission |  | YES, as RAN4#112 agreed |
| Receiver intermodulation | General intermodulation | Wait for RAN1 conclusion. |
|  | Narrowband intermodulation | YES |
| In-channel selectivity |  | TBD |

- In-channel selectivity

This requirement may be related to whether the FDMA is used which is still under discussion in RAN1, so we can mark it as discussed in WI phase

Proposal 2: For AIoT BS Rx requirements, take the following table as baseline:

| A-IoT BS- RX RF Requirement |  | Potentially needed or not |
| --- | --- | --- |
| Reference sensitivity level |  | YES |
| Dynamic range |  | Discussed in the WI phase |
| Adjacent Channel Selectivity |  | YES |
| In-band blocking | general blocking | YES |
|  | narrowband blocking | YES |
| Out-of-band blocking |  | YES |
| Receiver spurious emission |  | YES |
| Receiver intermodulation | General intermodulation | Discussed in the WI phase |
|  | Narrowband intermodulation | YES |
| In-channel selectivity |  | Discussed in the WI phase |

- Conclusion

In this contribution, we provide our views on TBD requirements.

Proposal 1: For AIoT BS Tx requirements, take the following table as baseline:

| RF Requirement for A-IoT BS- TX requirements |  | Potentially needed or not |
| --- | --- | --- |
| Base station output power |  | YES |
| Output power dynamic | RE power control dynamic range | YES |
|  | Total power dynamic range | YES |
|  | Power boosting | NO |
| Some requirements like Transmit ON/OFF power | Transmitter OFF power | NO |
|  | Transmitter transient period | NO |
| Transmit signal quality | Frequency error | YES |
|  | Some requirements like EVM (Modulation quality) | YES |
|  | TAE | NO |
| Unwanted emissions | Occupied bandwidth | YES |
|  | ACLR | YES |
|  | IBE | NO |
|  | Operating band unwanted emissions | YES |
|  | Transmitter spurious emissions | YES |
| Transmitter intermodulation |  | NO |

Proposal 2: For AIoT BS Rx requirements, take the following table as baseline:

| A-IoT BS- RX RF Requirement |  | Potentially needed or not |
| --- | --- | --- |
| Reference sensitivity level |  | YES |
| Dynamic range |  | Discussed in the WI phase |
| Adjacent Channel Selectivity |  | YES |
| In-band blocking | general blocking | YES |
|  | narrowband blocking | YES |
| Out-of-band blocking |  | YES |
| Receiver spurious emission |  | YES |
| Receiver intermodulation | General intermodulation | Discussed in the WI phase |
|  | Narrowband intermodulation | YES |
| In-channel selectivity |  | Discussed in the WI phase |

- References

- R4-2417088	WF on impacts of A-IoT on RF requirements	Huawei