3GPP TSG-RAN WG4 Meeting # 103 	R4-2418186

Orlando, US, 18th – 22nd November, 2024

Source: 	vivo

Title: 	Discussion on the RF requirement for intermediate UE

Agenda Item:	7.21.3.3

Document for:	Approval

- Introduction

In the previous meeting, the RF requirements for intermediate UE were discussed, but there are still several requirements are marked as TBD. In this contribution, we provide our views on these pending requirements.

- Discussion

  - Tx

The following table was agreed in the last meeting [1]:

| RF Requirement for A-IoT intermediate UE- TX part |  |  | Potentially needed or not |
| --- | --- | --- | --- |
| Maximum output power |  |  | YES, as RAN4#112 agreed |
| Output power dynamics | Minimum output power |  | NO |
|  | Transmit OFF power |  | TBD |
|  | Transmit ON/OFF time mask |  | TBD |
|  | Power control |  | NO |
| Transmitted signal quality | Frequency error |  | YES, as RAN4#112 agreed |
|  | Transmit modulation quality | EVM | YES |
|  |  | Carrier leakage | TBD |
|  |  | In-band emissions | TBD |
| Output RF spectrum emissions | Occupied bandwidth |  | YES, as RAN4#112 agreed |
|  | ACLR |  | Depends on the co-existence, as RAN4#112 agreed |
|  | Spurious emissions |  | YES, as RAN4#112 agreed |
|  | Transmit intermodulation |  | TBD |

- Transmit OFF power

Transmit OFF power is defined to ensure the UE will not affect the reception of other UE or BS when the transmitter is inactive. Since the AIoT and NR share same hardware at Tx side is a typical assumption, it is enough to verify this requirement in NR side and not need to be defined for AIoT.

- Transmit ON/OFF time mask

Since the AIoT operation and waveform is different from NR, this requirement is potentially needed which may be also related to RAN1 design.

- Carrier leakage

The carrier leakage requirement intends to reduce the DC offset which will distort the constellation for QAM signal. For OOK signal, the DC offset will increase the amplitude of waveform and the carrier leakage should also be low enough to ensure the received signal still within the dynamic range. Considering OOK/BPSK is less sensitive than the higher order modulation in NR when carrier leakage exists, the NR requirement is enough to limit the impairment and no need to defined same requirement for AIoT.

- In-band emissions

The in-band emission in NR can limit the interference between UEs, but when intermediate UE perform as AIoT reader, this requirement is unnecessary.

- Transmit intermodulation

The transmit intermodulation in NR is the capability of the transmitter to inhibit the generation of signals in its non-linear elements caused by presence of the wanted signal and an interfering signal reaching the transmitter via the antenna. Since the AIoT and NR share same RF hardware in Tx side, the non-linear performance of component can be verified in NR side and it is unnecessary to define it for AIoT intermediate UE.

Proposal 1: For intermediate UE Tx requirements, take the following table as baseline:

| RF Requirement for A-IoT intermediate UE- TX part |  |  | Potentially needed or not |
| --- | --- | --- | --- |
| Maximum output power |  |  | YES, |
| Output power dynamics | Minimum output power |  | NO |
|  | Transmit OFF power |  | NO |
|  | Transmit ON/OFF time mask |  | YES |
|  | Power control |  | NO |
| Transmitted signal quality | Frequency error |  | YES |
|  | Transmit modulation quality | EVM | YES |
|  |  | Carrier leakage | NO |
|  |  | In-band emissions | NO |
| Output RF spectrum emissions | Occupied bandwidth |  | YES |
|  | ACLR |  | Depends on the co-existence |
|  | Spurious emissions |  | YES |
|  | Transmit intermodulation |  | NO |

  - Rx

The following table was agreed in the last meeting [1]:

| A-IoT intermediate UE- RX RF Requirement | Potentially needed or not |
| --- | --- |
| Reference sensitivity | YES, as RAN4#112 agreed |
| Maximum input power | YES, as RAN4#112 agreed |
| ACS | YES, Depends co-existence study |
| In-band blocking | YES, Depends co-existence study |
| Out-of-band blocking | YES |
| Narrow band blocking | TBD |
| Spurious response | TBD |
| Receiver intermodulation | YES, as RAN4#112 agreed |
| RX spurious emissions | YES |

- Narrow band blocking/ Spurious response

In our understanding, these two requirements intend to guarantee the receiver performance when CW interference exist. Considering the intermediate UE may need implement new receiver in UL spectrum which depends on the D2R spectrum usage, these requirements still needed

Proposal 2: For intermediate UE Rx requirements, take the following table as baseline:

| A-IoT intermediate UE- RX RF Requirement | Potentially needed or not |
| --- | --- |
| Reference sensitivity | YES |
| Maximum input power | YES |
| ACS | YES |
| In-band blocking | YES |
| Out-of-band blocking | YES |
| Narrow band blocking | YES |
| Spurious response | YES |
| Receiver intermodulation | YES |
| RX spurious emissions | YES |

- Conclusion

In this contribution, we provide our views on TBD requirements.

Proposal 1: For intermediate UE Tx requirements, take the following table as baseline:

| RF Requirement for A-IoT intermediate UE- TX part |  |  | Potentially needed or not |
| --- | --- | --- | --- |
| Maximum output power |  |  | YES, |
| Output power dynamics | Minimum output power |  | NO |
|  | Transmit OFF power |  | NO |
|  | Transmit ON/OFF time mask |  | YES |
|  | Power control |  | NO |
| Transmitted signal quality | Frequency error |  | YES |
|  | Transmit modulation quality | EVM | YES |
|  |  | Carrier leakage | NO |
|  |  | In-band emissions | NO |
| Output RF spectrum emissions | Occupied bandwidth |  | YES |
|  | ACLR |  | Depends on the co-existence |
|  | Spurious emissions |  | YES |
|  | Transmit intermodulation |  | NO |

Proposal 2: For intermediate UE Rx requirements, take the following table as baseline:

| A-IoT intermediate UE- RX RF Requirement | Potentially needed or not |
| --- | --- |
| Reference sensitivity | YES |
| Maximum input power | YES |
| ACS | YES |
| In-band blocking | YES |
| Out-of-band blocking | YES |
| Narrow band blocking | YES |
| Spurious response | YES |
| Receiver intermodulation | YES |
| RX spurious emissions | YES |

- References

- R4-2417088	WF on impacts of A-IoT on RF requirements	Huawei