# 3GPP TSG-RAN WG4 Meeting #113	R4-2418188

Orlando, US, 18th – 22nd November, 2024

Source: 	vivo

Title: 	Updated simulation results of MIMO layer evaluation for 6Rx UE

Agenda Item:	7.1.1.3.2

Document for:	Approval

Introduction

In the last four meetings, we discussed the feasibility of 6MIMO layer however there was no consensus [1][2][3][4]. For 6Rx, we tried to align simulation methodology and parameters for 4&6 MIMO layer performance comparison, however some parameters were still open. The tentative agreed table for simulation assumptions are shown in the Annex. In this contribution, we give updated simulation results for the comparisons between 4 and 6 MIMO layers, based on that, our views on 6 MIMO layer support are provided.

Discussion

For MIMO layer support for 6Rx, we had following WF [4] in the last meeting:

Issue 3-1-1: 6-layer Support Way Forward: RAN4 to further discuss the following options (options are not necessarily mutually exclusive). - Option 1: 6-layer support is feasible for handheld and FWA based on some company evaluations considering realistic antenna correlation assumptions and deployment scenarios. - Option 2: 6-layer support is feasible for FWA devices. - Option 3: Companies to further discuss and align on the feasibility criteria and simulation assumptions by RAN4#113 to evaluate 6-Layer feasibility for handhelds using the following as a starting point. - Feasibility criteria - Consider 4-layer vs 6-layer throughput performance. - Simulation assumptions - Only consider link-level simulation for performance evaluations. - Updated simulation assumptions are provided in Annex A for discussions on alignment at RAN4#113 and can be used by companies to provide simulation results at RAN4#113 for discussion. Companies to clarify and justify (e.g., where the number comes from) the settings used in the simulation assumptions for BS correlation matrix and Tx EVM. - Option 4: 6-layer support is not feasible for handheld UEs. - Option 5: Companies to further discuss the following aspect of 6-layer support. - If the handheld feasibility is confirmed, then the handheld and FWA devices will share the same 6-layer requirements. - If the handheld feasibility is not confirmed, 6-layer requirements will be specified for FWA device only. Issue 3-1-2: 6-layer Support as optional feature Way Forward: RAN4 to further discuss if 6-Layer support should be considered an optional feature per the following. - Introduce 6 MIMO layers support as an optional feature, if 6-layer performance requirements are introduced for 6Rx. Issue 3-1-3: Tx EVM for feasibility study Way Forward: FFS on Tx EVM values to be used for feasibility evaluation. RAN4 to further discuss the following: - Option 1: Use Tx EVM = 1% - Option 2: Use Tx EVM = 3% for all of modulation order in MCS Table 2 for performance evaluation - Option 3: Consider TxEVM of 6% for 64QAM and TxEVM 3% for 256QAM - Option 4: use the existing BS Tx EVM core requirement - Option 5: The representative BS Tx EVM values provided by the BS vendor. This agreement was from chairman note. Agreement: - FFS on whether and how to set the Rx EVM assumption.

For 6 MIMO layer evaluation, we took link level simulation as baseline, using TDL channel models with antenna correlation matrix. We conducted performance simulation for 4&6 MIMO layer and the detailed simulation assumptions were listed in the Annex.

Simulation assumptions

If we check the simulation parameters in the Annex, there are many items with more than one option. In this case, there are many combinations of simulation cases if there is no down selection. In our simulation, we choose two typical sets of simulation assumptions:

Priority 1：TDLA30-10+DMRS 1+1 for 4layer and 2+2 for 6layer + UE Correlation matrix: Option 3

Priority 2：TDLA30-10+DMRS 1+1 for 4layer and 2+0 for 6layer+ UE Correlation matrix: Option 1

UE antenna correlation

Antenna correlation is an important aspect for designing MIMO antenna systems. With lower antenna correlation, it indicates better MIMO performance that can be achieved with the underlying antenna system. The antenna correlation coefficient is mostly given as the range of 0.1~0.5 between any two antennas, which is reasonable considering the actual antenna performances. The UE antenna parameter is given in Table 1 in the annex. In the last meeting, Option 3 and Option 4 were added based on real measurements and they are realistic UE antenna correlation matrix. In our simulation, we picked Option 3 as Priority 1 case. For comparison, we also used Option 1 as Priority 2 case.

BS Tx EVM and correlation matrix

For BS antenna number, we assumed 8Tx as a baseline in our simulation.

BS Tx EVM and correlation matrix are two controversial simulation assumptions for 4&6 MIMO layer simulation. For Tx EVM, RAN4 agreed not to tighten this requirement, which confirmed the rationality of minimum requirement. Unless BS venders challenge this requirement, we suggest to use this requirement as a starting point. In the field, in order to past the test for Tx EVM, we agreed the BS performance can be better than the minimum requirement and Option 3 is reasonable to consider TxEVM of 6% for 64QAM and TxEVM 3% for 256QAM, which is commonly used in demod session. Option 1 is too Optimistic and far away from the minimum requirement. We doubt than whether 1% Tx EVM can be achieved for 256QAM. We suggest to take Option 3 as baseline.

For BS correlation matrix, we think ideal non-correlation matrix is too optimistic. We suggest BS vendors can come up realistic correlation matrix. In our simulation, we considered both cases in the simulation table, that is, 3GPP MIMO correlation matrix with Alpha = 0.1/0 to compare the performance.

PDSCH DMRS configuration

For PDSCH DMRS configuration, we use DMRS Type 1 in Rel-15, and DMRS 1+1 for 4layer and 2+2 for 6layer as baseline in our simulation.

Simulation results

### Simulation results for Priority 1

Priority 1：TDLA30-10+DMRS 1+1 for 4layer and 2+2 for 6layer + UE Correlation matrix: Option 3

| a) UE correlation: option 3, BS correlation: option 1 | b) UE correlation: option 3, BS correlation: option 2 |
| --- | --- |

Figure 1. Performance comparison of 4&6 layer for Priority 1 Case

Figure 1 shows the SNR-Throughput curves of 4&6 layer performance for Priority 1 Case. Figure 1a uses the assumption of 3GPP MIMO correlation matrix with Alpha = 0.1. Figure 1b uses ideal BS correlation matrix with Alpha = 0. With the introduction of BS correlation Option 1, large throughput degradation can be observed compared with ideal BS correlation. For example, 4 rank with SNR equals 40dB, the maximum throughput is 753.61Mbps with ideal BS correlation and 327.87 Mbps with BS correlation with Alpha=0.1. The throughput degradation is 56% in this case with the introduction with realistic BS correlation matrix. Large throughput degradation can be observed for 6 rank under the same SNR condition with the introduction of BS correlation: option 2.

Observation 1: Under fixed rank and same SNR condition, large throughput degradation can be observed with BS correlation (Alpha = 0.1), compared with ideal BS correlation (Alpha = 0).

From our simulation, the BS correlation matrix will have a huge impact on the performance for 4 rank and 6 rank. Ideal BS correlation matrix is too optimistic. Low BS correlation with Alpha=0.1 seems more reasonable. It should be considered in the feasibility study.

For the comparison of 4&6 layer performance, we see no gain under the SNR up to 40dB for 6 layer. With or without introduction of BS correlation matrix with Alpha=0.1, 4 layer performance always out performs 6 layer. Under ideal BS correlation matrix, the performance of 6 rank will be close with 4 rank under SNR=40. With SNR higher than 40, 6 rank performance may out perform 4 layer, however it is not realistic to achieve such high SNR condition. Under BS correlation matrix with Alpha = 0.1, the performance of 6 layer is far behind 4 layer.

Observation 2: With or without introduction of BS correlation matrix with Alpha=0.1, 4 layer performance always out performs 6 layer under SNR less than 40dB.

Priority 2：TDLA30-10+DMRS 1+1 for 4layer and 2+2 for 6layer+ UE Correlation matrix: Option 1

| a) UE correlation: option 1, BS correlation: option 1 | b) UE correlation: option 1, BS correlation: option 2 |
| --- | --- |

Figure 2. Performance comparison of 4&6 layer for Priority 2 Case

For Priority Case 2, the only difference is UE correlation matrix compared with Priority Case 1. UE correlation Option 1 reflects another typical UE antenna design for 6Rx, which is also considered in our simulation. Figure 2 shows the SNR-Throughput curves of 4&6 layer performance for Priority 2 Case. Figure 2a uses the assumption of 3GPP MIMO correlation matrix with Alpha = 0.1. Figure 2b uses ideal BS correlation matrix with Alpha = 0. From Figure 2, it can be observed that it shares similar trend for the SNR-Throughput cures for 4 and 6 MIMO layers with different UE Rx correlation. With or without introduction of BS correlation matrix with Alpha=0.1, 4 layer performance always out performs 6 layer.

Observation 3: For different Rx correlation, it shares similar trend for the SNR-Throughput cures for 4 and 6 MIMO layers.

Observed from our simulation results Figure 1~2, it is not feasible for 6 layer since it doesn’t have gain compared with 4 layer. We suggest not to introduce this feature since it has no gain.

Proposal 1: At least for handheld UE, do not support 6 MIMO layer for 6Rx.

Conclusion

This contribution discusses 6 MIMO-layer support for 6Rx. The following observations and proposal are made:

Observation 1: Under fixed rank and same SNR condition, large throughput degradation can be observed with BS correlation (Alpha = 0.1), compared with ideal BS correlation (Alpha = 0).

Observation 2: With or without introduction of BS correlation matrix with Alpha=0.1, 4 layer performance always out performs 6 layer under SNR less than 40dB.

Observation 3: For different Rx correlation, it shares similar trend for the SNR-Throughput cures for 4 and 6 MIMO layers.

Proposal 1: At least for handheld UE, do not support 6 MIMO layer for 6Rx.

- References

[1] R4-2406585, WF on requirements for 6Rx, AT&T, RAN4#110-bis.

[2] R4-2410751, WF on requirements for 6Rx, AT&T, RAN4#111.

[3] R4-2414285, WF on 6Rx UE requirements, AT&T, RAN4#112.

[4] R4-2417209, WF on 6Rx UE requirements, AT&T, RAN4#112bis.

# Annex

Table 1 Feasibility Study Link-Level Simulation Assumptions

| Parameters |  | Values |
| --- | --- | --- |
| Duplex mode |  | TDD |
| Bandwidth |  | 40MHZ |
| SCS |  | 30 |
| Antenna configuration |  | {8Tx, 6Rx} as baseline Interested companies can submit results with {32Tx, 6Rx} and specify the precoding type |
| Propagation channel |  | TDLA30-10 |
| Rank |  | Fixed rank {4,6}, interested companies can bring {5} |
| MCS |  | Adaptive MCS (target BLER 10%) MCS Table 2 Interested companies can bring MCS Table 1 |
| PDSCH configuration | Mapping type | Type A |
|  | Starting symbol | 2 |
|  | Length | 12 |
|  | PRB bundling size | 2 |
|  | VRB-to-PRB mapping type | Non-interleaved |
|  | Precoding | Random PMI (Rel-15 Type 1 codebook) for 8Tx SVD based precoding for 32Tx; companies are encouraged to provide details on the SVD update periodicity |
| PDSCH DMRS configuration | DMRS Type | Type 1 [Rel-15] and eType-1 [Rel-18] |
|  | DMRS Configurations | Option 1: Rel-18 1+1(6L), Rel-15 1+1(4L) Option 2: Rel-15 1+1(4L), Rel-15 2+0(6L) Option 3: Rel-15 1+1(4L), Rel-15 2+2(6L) Option 4: Rel-15 2+2(4L), Rel-15 2+2(6L) [To be further discussed online] |
|  | Precoding | Random PMI (Rel-15 Type 1 codebook) for 8Tx SVD based precoding for 32Tx, companies are encouraged to provide details on the SVD update periodicity |
| UE Correlation matrix |  | Option 1: Option 2: 1.000.100.101.000.300.10 0.300.150.100.141.000.12 0.130.120.300.200.200.120.150.140.130.300.120.20 0.121.000.200.110.120.30 0.110.301.000.100.101.00 Option 3: 1.000.220.221.000.300.580.600.150.150.100.050.150.300.600.580.151.000.150.151.000.150.150.150.100.150.050.100.150.150.150.150.101.000.300.301.00 Option 4: 1.000.100.101.000.250.350.500.100.100.050.050.100.250.500.350.101.000.100.101.000.100.150.100.050.100.050.050.100.100.100.150.051.000.150.151.00 Option 5: 3GPP MIMO correlation matrix with beta=0.00718 (Option 5 is parametric and to be justified since Options 1-4 are based on measurements) Note: For the feasibility analysis, antenna efficiency can be considered. Companies are encouraged to provide the antenna efficiency numbers. |
| BS correlation matrix |  | Option 1: 3GPP MIMO correlation matrix with Alpha = 0.1 Option 2: 3GPP MIMO correlation matrix with Alpha = 0.0 Other options are not precluded. Matrices requested from interested BS vendors for RAN4#113 |
| BS Tx EVM |  | Tx EVM: 3% for 256QAM 6% for 64QAM |