# 3GPP TSG-RAN WG4 Meeting #113	R4-2418189

Orlando, US, 18th – 22nd November, 2024

Source: 	vivo

Title: 	Further discussion on SRS antenna switching requirements

Agenda Item:	7.1.1.3.3

Document for:	Approval

Introduction

SRS antenna switching requirements are one of important aspects for 6Rx requirements. In the last meeting, we agreed to introduce additional breakpoint for band n104 and decided ΔTRxSRS values for t1r6, t2r6, t1r6-t2r6, however, there is more work to be done with the addition of t3r6. The WFs are captured in [1]. In this contribution, we will further discuss the SRS antenna switching requirements with the addition of t3r6.

Discussion

The WF [1] for SRS antenna switching and ΔTRxSRS of last meeting was captured as follows:

- Topic 2: SRS antenna switching and ΔTRxSRS - Sub-topic 2-1: General considerations for SRS antenna switching and ΔTRxSRS Issue 2-1-1: Whether to consider an additional breakpoint for bands whose FUL_high is higher than the FUL_low of n104 Agreement: RAN4 agrees to consider additional breakpoint for bands whose FUL_high is higher than the FUL_low of n104. - Sub-topic 2-2: ΔTRxSRS values Issue 2-2-1: Proposed ∆TRxSRS values for t1r6, t2r6, and t1r6-t2r6 Agreement: RAN4 agrees to adopt the values for Bands n79 and n104 as specified below. Table: ΔTRxSRS values Operating Bands; ΔTRxSRS t1r6 (dB); ΔTRxSRS t2r6 (dB); ΔTRxSRS t1r6-t2r6 (dB). Band n79; 5.5; 5.0; 6.0. Band n104; 6.5; 5.5; 7.0. Issue 2-2-2: Proposed ∆TRxSRS values for t3r6, t1r6-t3r6, t2r6-t3r6, and t1r6-t2r6-t3r6 Agreement: As this is the first RAN4 meeting since the confirmation that RAN1 will consider t3r6, companies are encouraged to provide ∆TRxSRS values for c at RAN4#113 for further discussion with the Apple and Qualcomm proposals in R4-2415324 and R4-2416264, respectively.

For SRS antenna switching configurations, there was one update for t3r6 SRS antenna switching for R19 MIMO WID in RAN#105:

RAN#105 Conclusion

- Item A: SRS port grouping for TDD low-complexity 6/8RX receiver

- Item B: 2TA without CoresetPoolIdx association for asymmetric DL sTRP/UL mTRP

- Item C: 3T6R SRS antenna switching

- Item D: UE capability for 3T3R SRS antenna switching

- Item G: UE capability for non-codebook-based UL transmission

Update the Rel-19 MIMO WID accordingly.

It can be observed that RAN1 will specify 3T6R SRS antenna switching. In RAN4, we can start the work on ΔTRxSRS requirements with the addition of 3tr6.

For SRS antenna switching requirements, we need to study the ΔTRxSRS requirements based on possible RF architectures for t3r6, t1r6-t3r6, t2r6-t3r6, and t1r6-t2r6-t3r6. This value is the additional insertion loss when UE transmits SRS on Rx antenna port and it is highly related to the RF architecture for SRS antenna switching. Figure 1~4 shows the example RF architectures for supporting t3r6, t1r6-t3r6, t2r6-t3r6, and t1r6-t2r6-t3r6.

Figure 1. Example RFFE for t3r6

Figure 2. Example RFFE for t1r6-t3r6

Figure 3. Example RFFE for t2r6-t3r6

Figure 4. Example RFFE for t1r6-t2r6-t3r6

In all, the value of ΔTRxSRS is highly related to the RF architecture for SRS antenna switching, including the insertion loss of RF switch (e.g. SP4T, SP2T) and additional SRS trace. For the insertion loss, we give the numbers for each item:

- SP2T: 0.5/0.6dB

- SP3T: 0.8/1dB

- SP4T: 1/1.3dB

- DP3T: 1.1/1.4dB

- The IL of additional SRS Trace: 1.8/2.4 dB

The number before the dash is the insertion loss for band n41, n77, n78, and the number after the dash is for band n79.

Take t3r6 as an example, the insertion loss for Ant 2 is assumed as the worst, which is 2.8 dB (0.5+0.5+1.8=2.8dB) for band n41, n77, n78. The insertion loss for Ant 1 is TRx path with minimum insertion loss, which is 0.5 dB. Then the ΔTRxSRS value is 2.3dB for t1r6 for band n41, n77, n78.

Based on our calculation, we derive the ΔTRxSRS values for t1r6, t2r6 and t1r6-t2r6.

| Operating bands | ΔTRxSRS requirement for t3r6 | ΔTRxSRS requirement for t1r6-t3r6 | ΔTRxSRS requirement for t2r6-t3r6 | ΔTRxSRS requirement for t1r6-t2r6-t3r6 |
| --- | --- | --- | --- | --- |
| Band n41, n77, n78 | 0.5+0.5+1.8-0.5=2.3dB | 1+0.5+0.5+1.8-1=2.8dB | 0.8+0.8+1.8-0.8=2.4dB | 1+1.1+0.8+1.8-1=3.7dB |
| Band n79 | 0.6+0.6+2.4-0.6=3dB | 1.3+0.6+0.6+2.4-1.3=3.6dB | 1+1+2.4-1=3.4dB | 1.3+1.4+1+2.4-1.3=4.8dB |

The actual RFFE is more complex than the above example RF architectures considering supporting multiple bands and other factors. It is reasonable to consider some implementation margins for ΔTRxSRS requirements for 6Rx.

In the last meeting we agreed to introduce additional breakpoint for band n104. The IL loss difference is 0.5dB for t1r6, 0.5dB for t2r6 and 1dB for t1r6-t2r6 due to the frequency difference between band n79 and n104. We suggest to adopt the same margin, 0.5dB for t3r6, 1dB for t1r6-t3r6 and t2r6-t3r6, 1.5dB for t1r6-t2r6-t3r6 for band n104.

Thus we propose to take ΔTRxSRS values of t3r6, t1r6-t3r6, t2r6-t3r6, t1r6-t2r6-t3r6 for 6Rx as follows:

| Operating bands | ΔTRxSRS requirement for t3r6 | ΔTRxSRS requirement for t1r6-t3r6 | ΔTRxSRS requirement for t2r6-t3r6 | ΔTRxSRS requirement for t1r6-t2r6-t3r6 |
| --- | --- | --- | --- | --- |
| Band n41, n77, n78 | 3.5dB | 5dB | 4dB | 5.5dB |
| Band n79 | 4dB | 6 dB | 5.5dB | 6.5dB |
| Band n104 | 4.5dB | 7dB | 6.5dB | 8 dB |

Conclusion

This contribution discusses ΔTRxSRS requirements for 6Rx considering SRS antenna switching configurations t3r6, t1r6-t3r6, t2r6-t3r6 and t1r6-t2r6-t3r6. The following proposal is made:

Proposal 1: It is proposed to define ΔTRxSRS values for t3r6, t1r6-t3r6, t2r6-t3r6, t1r6-t2r6-t3r6 with 6Rx as follows:

| Operating bands | ΔTRxSRS requirement for t3r6 | ΔTRxSRS requirement for t1r6-t3r6 | ΔTRxSRS requirement for t2r6-t3r6 | ΔTRxSRS requirement for t1r6-t2r6-t3r6 |
| --- | --- | --- | --- | --- |
| Band n41, n77, n78 | 3.5dB | 5dB | 4dB | 5.5dB |
| Band n79 | 4dB | 6 dB | 5.5dB | 6.5dB |
| Band n104 | 4.5dB | 7dB | 6.5dB | 8 dB |

- References

[1] R4-2417209, WF on 6Rx UE requirements, AT&T, RAN4#112bis.