# 3GPP TSG-RAN WG4 Meeting #113	R4-2418198

Orlando, US, 18th – 22nd November, 2024

Source: 	vivo

Title: 	Further discussion on HPUE for intra-band contiguous and non-contiguous CA

Agenda Item:	7.1.1.1.2

Document for:	Approval

Introduction

In the last meeting, RAN4 further discussed MPR evaluation methodology and Pcmax,c/Pcmax issue on PC1.5 for intra-band contiguous and non-contiguous and the WF was captured in [1]. In this contribution, we will continue on the discussion on PC1.5 for intra-band contiguous and non-contiguous CA.

Discussion

MPR evaluation is one of the remaining issues for PC1.5 intra-band contiguous and non-contiguous CA. The WF for MPR evaluation was captured as follows:

## 1.1 MPR assumption Agreement: - Keep the following last meeting’s agreement for intra-band non-contiguous ULCA - Define MPR/A-MPR requirements based on equal PSD assumption for intra-band contiguous ULCA - It does not preclude companies to bring evaluation for unequal PSD - MPRc = MPR, which means no change of spec

In the last meeting, there were MPR value proposals based on simulation and analysis. MPR simulation didn’t consider the reverse IMD introduce by multiple Tx, and some additional MPR margin was added to derive the final MPR results. MPR analysis was conducted based on existing PC 1.5 single CC MPR and PC2 single CC and contiguous UL CA MPR. The concept of derivation of PC1.5 contiguous UL CA MPR was well explained in [2] and we thought it was reasonable since we think the RF architecture extensions are similar for PC2 and PC1.5 single carrier and UL contiguous CA. PC1.5 2Tx CA MPR can be derived using PC2 2Tx CA MPR with addition of MPR offset between PC1.5 2Tx single carrier MPR and PC2 2Tx single carrier MPR, which can be shown in the following equation. This equation is quite aligned with the concept in [2].

PC1.5 2Tx CA MPR ~= PC2 2Tx CA MPR + [PC1.5 2Tx SC MPR - PC2 2Tx SC MPR]

The benefit of this analysis is that it saves a lot of simulation or measurement efforts and the results are of some degree reasonable. For now, there were no MPR measurements for PC 1.5 UL CA yet. For MPR measurements, it requires a lot of PA set up, RF parameters set up, and baseband set up before the measurements. In our understanding, only few companies can do the measurements and derive MPR results. Also, different companies may provide results with difference because of different measurement setups, which are hard to align. Therefore, we propose MPR analysis in [2] as a starting point using exiting PC1.5 single carrier and PC2 MPR data. Of course, simulation and measurement MPR results can further modify the MPR results calculated using exiting PC1 single carrier and PC2 MPR data.

We didn’t give MPR proposals based on pure measurements. However, for MPR evaluation, we did some measurements to verify the MPR difference between 1Tx and 2Tx due to reverse IMD by 2Tx. The PA setup for PC1.5 2Tx single carrier is illustrated as Figure 1. This setup referred to the one used in earlier reverse IMD evaluation [3].

Figure 1. PA setup for PC 1.5 with 2Tx

The test parameters are given as follows:

- Test Frequency：2593MHz

- Channel Bandwidth: 100MHz, single carrier;

- SCS: 30KHz

- Modulation: 256QAM and QPSK

- Allocation: Outer_Full and inner_Full

- MPR = 0dB reference point: 29dBm

- CP-OFDM and DFTS-OFDM

In the MPR measurements, only ACLR and EVM are tested and this should be sufficient for outer_full and inner_full since they are the gating factors for these RB allocations. SEM is not considered for these scenarios. Figure 1 shows the MPR measurements for PC2 1Tx and PC1.5 2Tx single carrier.

| WF | TX mode | Pmax | condition | 1TX VS MPR0 | 2TX VS MPR0 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 10dB ISO | 20dB ISO |
| CP-OFDM | 100MHz_30kHz_256QAM_Outer_Full | 29dBm | ACLR≥-36；EVM≤3.5% | -4 | -5 | -4 |
|  | 100MHz_30kHz_256QAM_inner_Full | 29dBm |  | -4 | -4 | -4 |
|  | 100MHz_30kHz_QPSK_Outer_Full | 29dBm | ACLR≥-36；EVM≤17.5% | -4 | -4 | -4 |
|  | 100MHz_30kHz_QPSK_inner_Full | 29dBm |  | -2 | -2 | -2 |
| DFT-OFDM | 100MHz_30kHz_256QAM_Outer_Full | 29dBm | ACLR≥-36；EVM≤3.5% | -3 | -3 | -3 |
|  | 100MHz_30kHz_256QAM_inner_Full | 29dBm |  | 0 | -1 | -1 |
|  | 100MHz_30kHz_QPSK_Outer_Full | 29dBm | ACLR≥-36；EVM≤17.5% | -2 | -3 | -2 |
|  | 100MHz_30kHz_QPSK_inner_Full | 29dBm |  | 0 | 0 | 0 |

Table 1. MPR measurements for PC 2 1Tx and PC 1.5 2Tx single carrier

It can be seen from Table 1 that four RB configurations are considered in the measurements, 256QAM_Outer_Full, 256QAM_inner_Full, QPSK_Outer_Full, QPSK_inner_Full. The MPR results for 1Tx and 2Tx with 10/20 antenna isolation are given. The MPR difference between 1Tx and 2Tx considering full RB allocation is ranging from 0~1dB.

Observation 1: The MPR difference between 1Tx and 2Tx considering full RB allocation is ranging from 0~1dB.

It is difficult for to derive MPR proposals for PC 1.5 UL CA based on pure measurements. However, it may be possible to verify some test points for MPR. Simulation results can also be considered considering some margin considering the impact of reverse IMD. We suggest to use MPR analysis as a starting point, measurement/simulation results can assist in deriving MPR requirements.

Proposal 1：MPR analysis for PC 1.5 UL CA can be a starting point using exiting PC1.5 single carrier and PC2 single carrier and UL CA MPR data (as shown in below equation), measurement/simulation results can assist in deriving MPR requirements.

PC1.5 2Tx CA MPR ~= PC2 2Tx CA MPR + [PC1.5 2Tx SC MPR - PC2 2Tx SC MPR]

Conclusion

This contribution discusses MPR evaluation framework for PC1.5 intra-band contiguous and non-contiguous CA. The following proposals are made:

Observation 1: The MPR difference between 1Tx and 2Tx considering full RB allocation is ranging from 0~1dB.

Proposal 1：MPR analysis for PC 1.5 UL CA can be a starting point using exiting PC1.5 single carrier and PC2 single carrier and UL CA MPR data (as shown in below equation), measurement/simulation results can assist in deriving MPR requirements.

PC1.5 2Tx CA MPR ~= PC2 2Tx CA MPR + [PC1.5 2Tx SC MPR - PC2 2Tx SC MPR]

- References

[1] R4-2417098, WF on HPUE RF requirements, Samsung, RAN4#112bis.

[2] R4-2411168, MPR for PC1.5 intra-band contiguous UL CA, Apple, RAN4#112.

[3] R4-2111009, Evaluation of Reverse IMD versus antenna isolation and its impact to MPR, Skyworks, RAN4#99e.