3GPP TSG-RAN WG4 Meeting # 113	R4-2418201

Orlando, US, 18th – 22nd November, 2024

Source: 	vivo

Title: 	NR NTN HPUE Rx Requirements

Agenda Item:	7.8.2.2.2

Document for:	Approval

Introduction

In RAN4#112, the HPUE for NTN [1] was started. A summary can be referenced in [2] and a WF [3] has been agreed. In it, the key agreements:

Issue 4-1: General considerations

- Agreement

  - Rx RF requirements including Max input level, ACS, blocking characteristics, spurious response, intermodulation characteristics, Rx spurious emissions in existing spec can be applied to other power class.

Issue 4-2: RSD for NR-NTN

- Agreement

  - Reference sensitivity degradation (RSD) requirements is needed for NTN HPUE.

    - Further discuss the following aspects:

      - Use similar way in TN spec by introducing a new table of RSD compared to the PC3

      - Whether default Tx-Rx frequency separation is used for test point

A paper [4] was submitted for including an initial study on NTN HPUE RF Rx Requirements for both NR-based and Iot-based NTN HPUE Tx requirements, in which some topics were not really discussed.

In RAN4#112bis, a paper to discuss NR NTN HPUE RX requirements were submitted in [12]. However, not too much progress is made. A summary is in [13] and approved WF in [14]. The key agreements are as following:

Issue 3-1: RSD form in spec

Agreement:

- Use similar way in TN spec to derive RSD, if needed.

Issue 3-2: RSD requirements for PC2

Agreement:

- For n256: [0dB] RSD for PC2 NR-NTN for 1Tx and 2Tx

  - FFS on the duplexer isolation and PA interference

Issue 3-4: REFSENS/RSD testing

Agreement:

- For PC2 NR NTN UE, the REFSENs test can adopt the same UL configuration with PC3 NR NTN UE as starting point.

In this paper, a discussion of the impact of HPUE to RSD requirements for NR NTN are provided. Among them, some of the proposals and analysis in [12] were re-submitted with certain update.

HPUE Impact for NR-NTN RSD

General

It should be noted that the scenarios needed to be defined for RSD is related to what Tx requirements would be defined. Here a table from [16]:

Table 1: Current Status for requirements consideration and feasibility (Uncertain part in [])

|  | Antenna Numbers | Handheld and/or non-handheld |
| --- | --- | --- |
| PC1 | 1Tx | Non-handheld |
|  | [2Tx] | Non-handheld |
|  | [3Tx] | [TBD] |
| PC1.5 | [1Tx] | Non-handheld |
|  | [2Tx] | [Both] |
| PC2 | 1Tx | Both |
|  | 2Tx | Both |

It can be seen that there are still a lot of uncertain scenarios. For non-handheld cases, generally better condition would be considered for duplexer, and multi-Tx would also have an impact for RSD analysis. For handheld and non-handheld, if there are different assumptions on components, e.g. there might be some differences.

Observation 1: RSD definition would depend on Tx scenarios including power class, antenna numbers and UE type (handheld and non-handheld).

In this situation, confirmed scenarios can be discussed with hither priority, while others can also be discussed but may given less priority.

Proposal 1: The confirmed scenarios (PC2 and PC1 1Tx) can be discussed with higher priority, while scenarios under discussion (PC1.5 and PC1 2/3 Tx) may be postponed.

PC2

As discussed in [12], it is proposed that for n256 and n254, the RSD can be 0 considering the comparison of frequency separation to n1. N255 may need more study.

Although most companies choose to decide RSD in this way, still some company propose to use more MSD analysis based on some tentative duplexer isolation and PA noise configuration, such as proposed in [15].

We believe that as long as appropriate comparison can be found such as n256, more MSD analysis may not be necessary. It is sufficient for reuse current requirements.

For other cases such as n255, real duplexer data can be used as reference to obtain the isolation, such as those referenced in [5]. Although strictly speaking PC2 components may still not exist for certain bands, current components data with some margin may still be starting point considering the power can almost fulfilled.

Proposal 2a: For NR-NTN power class 2,

- For n256 and n254, RSD = 0dB, for both 2Tx and 1Tx cases.

- For n255, RSD is FFS

As already been suggested in [5], power class 2 is confirmed to be feasible for both handheld and non-handheld UE. Currently there is no differentiation of handheld / non-handheld UE for the RSD requirements, and the understanding is these requirements are based on common components and architectures.

Proposal 2b: For power class 2, there is no differentiation of handheld and non-handheld for RSD.

PC1.5

As discussed, the PC1.5 requirements definition are still under discussion. However, some preliminary thoughts can also be discussed. The following observation and proposals are re-submitted with update:

Since there is no FDD PC1.5 defined in TN yet, there is no clear precedence for PC1.5 NR NTN requirements, and there is no conclusion yet on the PC1.5 of NR-NTN yet. However, according to the discussion on [5], PC1.5 is feasible for PC1.5 2Tx NR NTN, and there is proposal to keep PC1.5 for 2Tx at least.

If this could be agreed, then it should not be too difficult for PC1.5 2Tx RSD derivation, since its similarity to PC2 2Tx in the derivation process.

Observation 2: Derivation of PC1.5 2Tx RSD can be similar to PC2 2Tx.

Furthermore, it should be noted the feasibility study of PC1.5 2Tx for NR NTN is based on handheld UE, thus the requirements should also be based on handheld UE. According to tradition and typical implementation, even the non-handheld type can share the same requirements with handheld ones as long as handheld is feasible.

Proposal 3: For PC1.5, 2Tx RSD are the same for handheld and non-handheld, while 1Tx RSD is likely to consider non-handheld only.

PC1

It is noted that there are already a number of PC1 requirements defined in NR and LTE, and all of them are 1Tx architecture and for non-handheld type. This is easy to be understand since the duplexer that could be working in the PC1 is beyond what a handheld UE could use.

It should be noted that currently PC1 often typically reuse Rx requirements from PC3, basically means that similar self-dense is expected. A number of TRs such as [9][10][11] have documented some study history and derivation of requirements.

In addition, only 1Tx were considered currently, but we are having other Tx options under discussion that may need further discussion.

Observation 3: Currently PC1 is only for 1Tx and non-handheld case in LTE and NR TN, and typically share PC3 Rx requirements without any RSD.

According to the feasibility analysis in [5], PC1 1Tx for NR-NTN is also proposed to be non-handheld only.

According to clause 6.3 “UE self-dense” of [9], the RSD of several new bands were based on the comparison of bands 14, 20 and 28 which at that time is already in the spec. The comparison is based on a “difficulty rank” which is associated with the spectrum arrangement including the channel bandwidth and UL-DL separation. Although the details can still be discussed, it is highly likely that the current band under discussion n254/n255/n256 would be categorized as “easy bands” since the duplex gap is fairly large.

Observation 4: For PC1 1Tx, according to de-sense analysis in 37.880, it appears that the current NTN band n254/n255/n256 are easier compared to legacy PC1 bands.

Based on the analysis and observations, the following proposal is provided:

Proposal 4a: For PC1 1Tx for NR-NTN, which is only considered feasible for non-handheld, the RSD is proposed to be 0dB.

Proposal 4b: FFS for PC1 with 2 or 3 Tx cases.

Conclusion

In this paper, the RSD requirements have been discussed for NR NTN HPUE.

General

Observation 1: RSD definition would depend on Tx scenarios including power class, antenna numbers and UE type (handheld and non-handheld).

Table 1: Current Status for requirements consideration and feasibility (Uncertain part in [])

|  | Antenna Numbers | Handheld and/or non-handheld |
| --- | --- | --- |
| PC1 | 1Tx | Non-handheld |
|  | [2Tx] | Non-handheld |
|  | [3Tx] | [TBD] |
| PC1.5 | [1Tx] | Non-handheld |
|  | [2Tx] | [Both] |
| PC2 | 1Tx | Both |
|  | 2Tx | Both |

Proposal 1: The confirmed scenarios (PC2 and PC1 1Tx) can be discussed with higher priority, while scenarios under discussion (PC1.5 and PC1 2/3 Tx) may be postponed.

PC2

Proposal 2a: For NR-NTN power class 2,

- For n256 and n254, RSD = 0dB, for both 2Tx and 1Tx cases.

- For n255, RSD is FFS

Proposal 2b: For power class 2, there is no differentiation of handheld and non-handheld UE.

PC1.5

Observation 2: Derivation of PC1.5 2Tx RSD can be similar to PC2 2Tx.

Proposal 3: For PC1.5, 2Tx RSD are the same for handheld and non-handheld, while 1Tx RSD is likely to consider non-handheld only.

PC1

Observation 3: Currently PC1 is only for non-handheld case in LTE and NR TN, and typically share PC3 Rx requirements without any RSD.

Observation 4: For PC1, according to de-sense analysis in 37.880, it appears that the current NTN band n254/n255/n256 are easier compared to legacy PC1 bands.

Proposal 4a: For PC1 1Tx for NR-NTN, which is only considered feasible for non-handheld, the RSD is proposed to be 0dB.

Proposal 4b: FFS for PC1 with 2 or 3 Tx cases.

- References

[1] RP-240857, New WID: Enhanced requirements and test methodology for NR and IoT NTN, RAN#103

[2] R4-2412829, Topic summary for [112][127] NR_IoT_NTN_HPUE , Samsung, RAN4#112

[3] R4-2414452, WF on UE RF requirements for NTN HPUE, Samsung, RAN4#112

[4] R4-2412100, Initial analysis for NTN HPUE RF Rx Requirements, vivo, RAN4#112

[5] R4-2415788, Feasibility study and analysis for NR NTN HPUE Tx Requirements, vivo, RAN4#112bis

[6] TR 38.861, Study on high power UE (power class 2) for one NR FDD band

[7] TR 38.896, High power UE (power class 2) for NR FR1 FDD single band

[8] R4-2307239, n8 PC2 RSD for 25MHz DL CBW, Skyworks, RAN4#109

[9] TR 37.880, High-power UE operation for fixed-wireless/vehicle-mounted use cases in Band 12, Band 5, and Band n71

[10] TR 37.828, High-power UE operation for fixed-wireless/vehicle-mounted use cases in LTE bands and NR bands

[11] TR 37.829, High-power UE operation for fixed-wireless/vehicle-mounted use cases in LTE bands and NR bands for Rel-18

[12] R4-2415789, Analysis for NR NTN HPUE Rx Requirements, vivo, RAN4#112bis

[13] R4-2415228, Topic summary for [112bis][124] NR_IoT_NTN_HPUE_part1, Samsung, RAN4#112bis

[14] R4-2417122, WF on co-existence study and HPUE for NR-NTN, Samsung, RAN4#112bis

[15] R4-2416058, Discussion on Rx requirements for NR NTN HPUE, Huawei, HiSilicon, RAN4#112bis

[16] R4-2418200, NR NTN HPUE Tx Requirements and feasibility analysis, vivo, RAN4#113