3GPP TSG-RAN WG4 Meeting # 113	R4-2418202

Orlando, US, 18th – 22nd November, 2024

Source: 	vivo

Title: 	Iot NTN HPUE Tx Requirements and feasibility analysis

Agenda Item:	7.8.2.3.1

Document for:	Approval

Introduction

In RAN4#112, the HPUE for NTN [1] was started. A summary can be referenced in [2] and a WF [3] has been agreed. A paper [4] was submitted including an initial feasibility study and analysis for both NR-based and Iot-based NTN HPUE Tx requirements.

In RAN4#112bis, a comprehensive discussion paper including measurement results for 2 PAs are provided for Iot-NTN in [5]. A summary is in [6] and WF in [7] including the duty cycle related discussion.

Up till now, the RF requirements has not been fully discussed. In this paper, further discussion was made for Iot NTN HPUE Tx requirements, related duty cycle solution, and the feasibility and requirements applicability.

HPUE Impact for Tx Requirements

General

In last meeting, a list of agreements on “General & Feasibility” has been agreed in [7]. In order to make it more clear and provide a baseline for next stage discussion, the following table is provided to reflect the current status of what scenario and UE type has been considered in the requirements, and this will serve as a basic reference and starting point for further requirements discussion.

Observation 1:

Table 1: Current status for requirements consideration and feasibility for Iot NTN

|  | Power Class | Antenna Numbers | Handheld and/or non-handheld |
| --- | --- | --- | --- |
| NB-Iot based | PC1 | 1Tx | [Both] |
|  |  | [2Tx] | FFS |
|  | PC2 | 1Tx | Both |
| eMTC based with half duplex | PC1 | 1Tx | FFS |
|  |  | [2Tx] | FFS |
|  | PC2 | 1Tx | Both |
| eMTC based with full duplex | PC1 | 1Tx | [Non-handheld] |
|  |  | [2Tx] | FFS |
|  | PC2 | 1Tx | Both |

It was found that the requirements of NB-Iot Based NTN has encounter more problems compared to eMTC-based when considering higher power. The details are discussed as following sections.

NB-Iot Based

ACLR

Currently for NB-Iot based Iot-NTN, the ACLR is consists of two requirements: GSM_ACLR and UTRA_ACLR as following:

Actually In the on-going co-existence study, it is the NR signal behaves as a victim. The current study can be easily extended to UTRA_ACLR requirements which is currently cover UTRA and E-UTRA.

However, co-existence with GSM is not considered, and GSM_ACLR was introduced for PC3 with the spirit of reusing requirements without study. In the early days for NB-Iot in LTE, the study for co-existence with GSM is likely to be based on some GSM specific metric other than throughput, and is quite different from what we are doing right now.

Observation 1: Co-existence with GSM was not considered in co-existence study.

In addition, this GSM_ACLR may not be more strict than the occupied channel band requirements, in which 99% energy is required inside the channel. In the worst condition that all the leaked power is on the adjacent channel, the 0.5% energy is comparable a 23dB ACLR to a certain extent. This is almost comparable to 20dB GSM_ACLR.

Furthermore, a 100KHz guard band would exist between the NB1/2 channel edge and the measured victim channel edge for GSM_ACLR, since there is 300KHz channel distance between two GSM carriers. This is reflected by the GSM_ACLR in the adjacent channel center frequency offset from the NB1/2 channel edge is 200KHz. This would means that the emission adjacent to the NB1/2 channel edge might be even worse than the GSM_ACLR measured, thus make the GSM_ACLR even more relaxed.

Observation 2: Current GSM_ACLR seeems no more stringent compared to occupied channel bandwidth.

So, some discussion is needed on whether GSM_ACLR is needed. If it is needed, whether we should keep the requirements for HPUE.

Based on the observations, currently remove the requirement seems one way.  If it is kept, from the previous test verification in [5], it seems that this would also be fulfilled with margin even for GSM PA. However, any changing of this value would likely to needed dedicated further study thus not preferred.

Proposal 1: Whether and how to define GSM_ACLR for NB-Iot based NTN can be further discussed.

Option 1: Remove this requirement;

Option 2: Keep this requirement and reusing the PC3 requirement for HPUE;

Option 3: Others. (Dedicated study may be needed)

SEM

Background

For NB-Iot based Iot-NTN, SEM is reused from NB-Iot which is as following:

This mask was actually from GSM 45.005, which is also copied below:

This SEM is quite different for GSM, compared to what we are more familiar in LTE/NR. It has some main differences:

- The in-band part is also included;

- Defined for some key points, while the emission is required to below the interpolation of these points;

- Defined as relative value with first value as 0dB;

As another reference, the NB-Iot based NTN SEM defined in the testing spec is a interpolated version as following:

Analysis and extention

The NB-Iot has used the same offset value and key points power offset of GSM mask, but changed the power to absolute values as in the table. It should be noted the 26dBm value currently is based on P=33dBm/180kHz, as in the [8].

Observation 3: Currently SEM definition for NB-Iot based Iot NTN include the in-band part, while the value 26dBm for PC3 (centered in 0kHz foob) is based on 33dBm MOP on 180KHz.

Currently, the measurement bandwidth is 30KHz, this means that for edge single tone with 3.75kHz SCS, a measurement centered on 0Hz foob would cover the whole edge tone, thus cover the whole transmission power!

Based on this scenario the following observation is provided:

Observation 4: Currently SEM definition for NB-Iot based Iot NTN,

- the value 26dBm for PC3 (centered in 0kHz foob) is based on 33dBm MOP on 180KHz.

- the value centered in 0kHz foob may cover the entire MOP of NB-Iot based Iot NTN, e.g. for edge single tone 3.75KHz configuration.

For PC3, this should not be a problem, since even with +2dB tolerance, 23+2 = 25dBm is still within 26dBm limit. However, if the power class extends to PC2 and PC1, it is apparent the current value 26dBm can not be kept.

Observation 5: Current SEM for PC3 should be revised for PC2 and PC1, since the in-band part is currently incorporated into SEM and MOP has to be taken into account.

The most straightforward way is sing nominal maximum output power + upper MOP tolerance as the substitute of 26dBm, that is 28dBm for PC2 and 33dBm for PC1.

In addition, for the other key points, if legacy way was followed, all the values should be relative to the first value, that means all the sections need corresponding relaxation by 2 and 7dB for PC2 and PC1 respectively. There may be some doubt on whether this would be over relaxed or not, thus this still need some discussion. If only the in-band part is changed, some more study on feasibility may be needed, since the

Proposal 2: Considering to extend the SEM for NB-Iot based NTN for PC2 and PC1 as following, with the understanding that in-band MOP is also accounted.

Option 1: Changing all key points values:

| ΔfOOB (kHz) | PC2 Emission limit (dBm) | PC1 Emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 | 28 (26+2) | 33 (26+7) | 30 kHz |
| 100 | -3 (-5+2) | 2 (-5+7) | 30 kHz |
| 150 | -6 (-8+2) | -1 (-8+7) | 30 kHz |
| 300 | -27 (-29+2) | -22 (-29+7) | 30 kHz |
| 500-1700 | -33 (-35+2) | -28 (-35+7) | 30 kHz |

Option 2: Changing only the in-band values:

| ΔfOOB (kHz) | PC2 Emission limit (dBm) | PC1 Emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 | 28 (26+2) | 33 (26+7) | 30 kHz |
| 100 | -5 | -5 | 30 kHz |
| 150 | -8 | -8 | 30 kHz |
| 300 | -29 | -29 | 30 kHz |
| 500-1700 | -35 | -35 | 30 kHz |

Option 3: Others

There might be also other ways to define SEM, but the changes would be fairly radical, e.g. removing the in-band part, thus would need more discussion.

Verification

Interestingly, we did some measurements on SEM for PC1 based on the same GSM PA module used in last meeting [5].

Main test configurations and descriptions:

- Carrier Frequency: One per band from 255/256

- SCS: 15kHz,

- Modulation: QPSK

- Full tone & Edge single tone,

- Power: Various power from 31/30dBm for full tone and single power option for single tone

- Measured results description: Only the value closest to the SEM mask is recorded;

- In-band part not tested by the test equipment

The detailed analysis can be considered in later, e.g. in MPR etc.

Those results might be used in later stage.

Observation 6: Some SEM results were collected for a GSM PA module in 31/30dBm.

MPR

MPR is generally should be studied after ACLR and SEM confirmed. The method of simulation and testing can be considered. However, there can are still early discussion on some of the evaluation baseline, e.g. what is the MPR=0 reference point. For example, for NB-Iot, all the single Tone transmission are regarding as MPR=0；

Observation 7: While waiting for ACLR/SEM conclusion, other evaluation assumption can also be discussed.

After checking single Tone SEM for HPUE, we found that the Edge single Tone configuration would have significant difference in the two sides. It is likely that one side would face some difficulty in fulfil SEM. In this sense, it might be needed to consider another MPR = 0 reference point.

Proposal 3: For the MPR study of for NB-Iot based NTN, considering center single tone allocation as MPR=0 reference point, and introduce MPR for edge single tone allocation.

P-MPR

In RAN4#112, the following agreement is reached in the WF [3]:

Issue 3-3-4: SAR for handheld

Agreement:

- Using P-MPR as the starting point

- The other solutions are not precluded

  - E.g., the solution similar to TN duty cycle based on UE capability and network scheduling

In RAN4#112bis, the following agreement in WF [7]:

Agreement:

- Iot NTN can be differentiated by handheld and non-handheld

In the spirit Iot-NTN equipment would also need SAR solution for handheld devices, P-MPR as a basic SAR solution is agreed to be introduced.

For eMTC based Iot-NTN, the configured transmitted power directly reference LTE, since there are is no dedicated configured transmitted power definition for eMTC in 36.101:

However, for NB-Iot, a dedicated and simplified configured transmitted power was defined for 36.101:

Correspondingly, the NB-Iot based Iot-NTN referenced this section:

Based on these conditions, the following observation is confirmed:

Observation 8: Currently NB-Iot and NB-Iot based Iot-NTN do not support P-MPR.

In the current status, P-MPR can not be utilized for NB-Iot based Iot NTN.

Since P-MPR is introduced in NTN, revising the legacy NB-Iot is not reasonable. So it is preferred to add P-MPR into 36.102 for NB-Iot based Iot NTN only. The details can be FFS.

Proposal 4：Revise the configured transmitted Power seciton for NB-Iot Based Iot-NTN in 36.102 to incorporate P-MPR. Details FFS.

eMTC-Based

For eMTC based Iot-NTN, the requirements are basically reused from eMTC, which is quite aligned with LTE in many basic requirements. This would means the HPUE study for LTE would be easily to work.

So as a general guideline, legacy eMTC/LTE requirements can be reused for HPUE as starting point. The details can FFS.

Proposal 5: As a general guideline, for eMTC based Iot-NTN, the HPUE requirements can using legacy LTE requirements as starting point.

Duty cycle Related

In last meeting, there were some preliminary discussion on duty cycle based approach. In [9], some general discussion is provided.

However, currently there is not much discussion on whether some difference exists between Iot NTN and NR NTN, particularly between NB-Iot based NTN and NR-NTN.

Proposal 6: Study if there are differences between Iot-NTN and NR-NTN when try to introducing tentative duty cycle scheme.

Feasibility and Requirements applicability

In last meeting, some PA testing and analysis, plus duplexer analysis were made in [5]. Some agreements were reached in the WF [7] as follows:

Issue 1-1-2: How many Tx number should be considered for Iot-NTN (in the specification).

Agreement:

- Focus on 1Tx for PC2 [and PC1], further study [2Tx for PC1]

  - FFS on the applicability for handheld for PC1.

  - Continue work on 1Tx PC1 requirements.

Issue 1-1-3: For NB-Iot based Iot-NTN, whether PC1 is feasible for handheld device.

- Proposals

  - Option 1: Yes, (vivo, Qualcomm)

    - Based on measurements for two different PAs. (vivo)

    - Handheld satellite phones which have very high output power exist.(Qualcomm)

  - Option 2: Others

- WF

  - Continue discussion in next meeting.

Issue 1-1-4: For half duplex eMTC based Iot NTN, whether PC1 is feasible for handheld device.

- WF

  - FFS.

Issue 1-1-5: For full duplex eMTC based Iot NTN, whether PC1 is feasible for handheld device.

- WF

  - FFS.

Issue 1-1-6: For Iot-NTN, whether PC1 is feasible for non-handheld device.

Agreement:

- For Iot-NTN, PC1 is feasible for non-handheld device.

To make it simple enough, the current status of what scenario and UE type has been summarized in observation 1 which also copied below:

Table 1: Current status for requirements consideration and feasibility for Iot NTN

|  | Power Class | Antenna Numbers | Handheld and/or non-handheld |
| --- | --- | --- | --- |
| NB-Iot based | PC1 | 1Tx | [Both] |
|  |  | [2Tx] | FFS |
|  | PC2 | 1Tx | Both |
| eMTC based with half duplex | PC1 | 1Tx | FFS |
|  |  | [2Tx] | FFS |
|  | PC2 | 1Tx | Both |
| eMTC based with full duplex | PC1 | 1Tx | [Non-handheld] |
|  |  | [2Tx] | FFS |
|  | PC2 | 1Tx | Both |

Currently the main unsettled part is for PC1 and 2Tx.

In last meeting , [5] has provided a lot of testing results to verify the feasibility of current available components for handheld device to reach power class 1 or even higher power. Although some proprietary PA are not used in TN cellular before, they are already being used in many commercial smartphones to have a satellite communication capability.

If cost and availability is still a concern, the testing of current GSM PA module also show promising results, can can even satisfy the tentative requirements at least in 1.6GHz band though margin may not sufficient. With some enhancements, there are even more potentials after requirements can be introduced.

Proposal 7: Considering the current widespread proprietary implementation satellite PA, and also the potential to reuse GSM PA, PC1 for handheld requirements should be defined for NB-Iot based Iot-NTN.

In addition, the following observation summary is provided based on last meeting’s material and analysis.

Observation 9:

Table 2. Current PC1 feasibility summary for handheld UE for Iot NTN

|  | PA | Duplexer | Conclusion based on PA&Duplexer |
| --- | --- | --- | --- |
| NB-Iot Based | Sufficient margin for specific designed for satellite communication. Margin may not sufficient for GSM PA depending on bands etc. | N/A | Feasible |
| half duplex eMTC based | May achieve power class | N/A | FFS |
| full duplex eMTC based | May not be achieve power class. (Comparable to NR NTN) | Not feasible (Needed as NR NTN) | Not Feasible |

In addition, as discussed in [5], introducing 2Tx for Iot NTN may bring a lot of extra work for 2Tx TxD related definition, and this is also not that meaningful from implementation point of view.

Based on previous observation and proposal, the following proposal for requirements to be defined for Iot NTN is proposed. The item “(discuss)” means this item needs to be discussed.

Proposal 8:

Table 3: Proposal for requirements consideration and feasibility for Iot NTN

|  | Power Class | Antenna Numbers | Handheld and/or non-handheld |
| --- | --- | --- | --- |
| NB-Iot based | PC1 | 1Tx | [Both] (discuss) |
|  |  | [2Tx](discuss) | FFS |
|  | PC2 | 1Tx | Both |
| eMTC based with half duplex | PC1 | 1Tx | [Both] (discuss) |
|  |  | [2Tx] (discuss) | FFS |
|  | PC2 | 1Tx | Both |
| eMTC based with full duplex | PC1 | 1Tx | [Non-handheld] (discuss) |
|  |  | [2Tx] (discuss) | FFS |
|  | PC2 | 1Tx | Both |

Conclusion

Tx General

Observation 1:

Table 1: Current status for requirements consideration and feasibility for Iot NTN

|  | Power Class | Antenna Numbers | Handheld and/or non-handheld |
| --- | --- | --- | --- |
| NB-Iot based | PC1 | 1Tx | [Both] |
|  |  | [2Tx] | FFS |
|  | PC2 | 1Tx | Both |
| eMTC based with half duplex | PC1 | 1Tx | FFS |
|  |  | [2Tx] | FFS |
|  | PC2 | 1Tx | Both |
| eMTC based with full duplex | PC1 | 1Tx | [Non-handheld] |
|  |  | [2Tx] | FFS |
|  | PC2 | 1Tx | Both |

Tx NB-Iot Based

ACLR:

Observation 1: Co-existence with GSM was not considered in co-existence study.

Observation 2: Current GSM_ACLR seeems no more stringent compared to occupied channel bandwidth.

Proposal 1: Whether and how to define GSM_ACLR for NB-Iot based NTN can be further discussed.

Option 1: Remove this requirement

Option 2: Keep this requirement and reusing the PC3 requirement for HPUE

Option 3: Others. (Dedicated study may be needed)

SEM:

Observation 3: Currently SEM definition for NB-Iot based Iot NTN include the in-band part, while the value 26dBm for PC3 (centered in 0kHz foob) is based on 33dBm MOP on 180KHz.

Observation 4: Currently SEM definition for NB-Iot based Iot NTN,

- the value 26dBm for PC3 (centered in 0kHz foob) is based on 33dBm MOP on 180KHz.

- the value centered in 0kHz foob may cover the entire MOP of NB-Iot based Iot NTN, e.g. for edge single tone 3.75KHz configuration.

Observation 5: Current SEM for PC3 should be revised for PC2 and PC1, since the in-band part is currently incorporated into SEM and MOP has to be taken into account.

Proposal 2: Considering to extend the SEM for NB-Iot based NTN for PC2 and PC1 as following, with the understanding that in-band MOP is also accounted.

Option 1: Changing all key points values:

| ΔfOOB (kHz) | PC2 Emission limit (dBm) | PC1 Emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 | 28 (26+2) | 33 (26+7) | 30 kHz |
| 100 | -3 (-5+2) | 2 (-5+7) | 30 kHz |
| 150 | -6 (-8+2) | -1 (-8+7) | 30 kHz |
| 300 | -27 (-29+2) | -22 (-29+7) | 30 kHz |
| 500-1700 | -33 (-35+2) | -28 (-35+7) | 30 kHz |

Option 2: Changing only the in-band values:

| ΔfOOB (kHz) | PC2 Emission limit (dBm) | PC1 Emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 | 28 (26+2) | 33 (26+7) | 30 kHz |
| 100 | -5 | -5 | 30 kHz |
| 150 | -8 | -8 | 30 kHz |
| 300 | -29 | -29 | 30 kHz |
| 500-1700 | -35 | -35 | 30 kHz |

Option 3: Others

Observation 6: Some SEM results were collected for a GSM PA module in 31/30dBm.

MPR:

Observation 7: While waiting for ACLR/SEM conclusion, other evaluation assumption can also be discussed.

Proposal 3: For the MPR study of for NB-Iot based NTN, considering center single tone allocation as MPR=0 reference point, and introduce MPR for edge single tone allocation.

P-MPR:

Observation 8: Currently NB-Iot and NB-Iot based Iot-NTN do not support P-MPR.

Proposal 4：Revise the configured transmitted Power seciton for NB-Iot Based Iot-NTN in 36.102 to incorporate P-MPR. Details FFS.

Tx eMTC Based

Proposal 5: As a general guideline, for eMTC based Iot-NTN, the HPUE requirements can using legacy LTE requirements as starting point.

Duty cycle Related

Proposal 6: Study if there are differences between Iot-NTN and NR-NTN when try to introducing tentative duty cycle scheme.

Feasibility and Requirements applicability

Proposal 7: Considering the current widespread proprietary implementation satellite PA, and also the potential to reuse GSM PA, PC1 for handheld requirements should be defined for NB-Iot based Iot-NTN.

Observation 9:

Table 2. Current PC1 feasibility summary for handheld UE for Iot NTN

|  | PA | Duplexer | Conclusion based on PA&Duplexer |
| --- | --- | --- | --- |
| NB-Iot Based | Sufficient margin for specific designed for satellite communication. Margin may not sufficient for GSM PA depending on bands etc. | N/A | Feasible |
| half duplex eMTC based | May achieve power class | N/A | FFS |
| full duplex eMTC based | May not be achieve power class. (Comparable to NR NTN) | Not feasible (Needed as NR NTN) | Not Feasible |

Proposal 8:

Table 3: Proposal for requirements consideration and feasibility for Iot NTN

|  | Power Class | Antenna Numbers | Handheld and/or non-handheld |
| --- | --- | --- | --- |
| NB-Iot based | PC1 | 1Tx | [Both] (discuss) |
|  |  | [2Tx](discuss) | FFS |
|  | PC2 | 1Tx | Both |
| eMTC based with half duplex | PC1 | 1Tx | [Both] (discuss) |
|  |  | [2Tx] (discuss) | FFS |
|  | PC2 | 1Tx | Both |
| eMTC based with full duplex | PC1 | 1Tx | [Non-handheld] (discuss) |
|  |  | [2Tx] (discuss) | FFS |
|  | PC2 | 1Tx | Both |

- References

[1] RP-240857, New WID: Enhanced requirements and test methodology for NR and IoT NTN, RAN#103

[2] R4-2412829, Topic summary for [112][127] NR_IoT_NTN_HPUE , Samsung, RAN4#112

[3] R4-2414452, WF on UE RF requirements for NTN HPUE, Samsung, RAN4#112

[4] R4-2412099, Feasibility study and Initial analysis for NTN HPUE RF Tx Requirements, vivo, RAN4#112

[5] R4-2415790, Feasibility study and analysis for Iot NTN HPUE Tx Requirements, vivo, RAN4#112bis

[6] R4-2415229, Topic summary for [112bis][125] NR_IoT_NTN_HPUE_part2,vivo, RAN4#112bis

[7] R4-2417205, WF on HPUE for IoT NTN, vivo, RAN4#112bis

[8] R4-77AH-IoT-0019, On UE emission requirement for NB-IoT, Huawei, RAN4#77 NB-IOT AH

[9] R4-2418200, NR NTN HPUE Tx Requirements and feasibility analysis, vivo, RAN4#113