3GPP TSG-RAN WG4 Meeting #113	R4-2418258

Orlando, USA, 18th – 22nd November 2024

Source:	Nokia

Title:	IMT parameters: General system and UE aspects

Agenda item:	7.2.1

Document for:	Approval

Introduction

ITU-R WP 5D has requested that external organizations provide IMT parameters for 4400-4800 MHz, 7125-8400 MHz and 14800-15350 MHz for the sharing studies towards WRC-27 [1]. RAN has agreed SI FS_4400_15350MHz_NR and tasked RAN4 to work on these parameters [2].

RAN4 has already agreed LS replies for 4400-4800 MHz and 7125-8400 MHz in [3][4]. In this contribution, we discuss remaining open issues regarding the system and UE parameters for 14800-15350 MHz.

Discussion

Channel bandwidth

Issue 4-5: Typical channel bandwidth

Agreement:

- [200MHz] as typical bandwidth, document in TR other bandwidths not precluded

  - The note should capture the association of channel bandwidth with maximum transmission power /PSD

  - If the practical issues are identified or additional work required for replying LS for 200MHz, RAN4 will consider 100MHz as typical bandwidth

The agreement about the typical channel bandwidth in RAN4#112-bis [5] is in the following for 14800-15350 MHz. 200 MHz is assumed unless there is any practical issue.

Although the practical issues are not clarified in the WF, we understand they are related to the spectrum mask, spurious emissions, blocking and spurious response for 200 MHz channel bandwidth. For 100 MHz channel bandwidth, these parameters can be simply reused from NR FR1 specifications. These is no requirement for 200 MHz channel bandwidth in NR FR1 specifications, e.g., TS 38.101-1. However, we can reuse the requirement for the intra-band contiguous carrier aggregation. NR CA bandwidth classes are defined up to 400 MHz aggregated channel bandwidth.

Spurious emission

          - FFS if RAN4 to adopt the UE general spurious emissions defined in TS 38.101-1 clause 6.5.3.1

In the WF in RAN4#112-bis [6], the following agreement was made for spurious emissions.

For the channel bandwidth up to 100 MHz, TS 38.101-1 clause 6.5.3.1 can be referred. For the channel bandwidth wider than 100 MHz, the emissions defined for the aggregated channel bandwidth in NR CA can be reused. Spurious emissions can be derived by assuming BWchannel_CA =200 MHz in Table 6.5A.3.1-1 of TS 38.101-1 so that the spurious region boundary is FOOB=205 MHz.

| Aggregated Channel bandwidth | OOB boundary FOOB (MHz) |
| --- | --- |
| BWChannel_CA | BWChannel_CA + 5 |

Spurious emissions up to 100 MHz channel bandwidth is defined in TS 38.101-1 clause 6.5.3.1. For wider channel bandwidth such as 200 MHz, TS 38.101-1 clause 6.5A.3 is referred.

In the WF in RAN4#112-bis [6], the following agreement was made for the emission mask.

Emissions mask

- Option 1: UE SEM given in Section 7.1.2, TR 38.921.

- Option 2: Other options.

For the channel bandwidth up to 100 MHz, option 1 works or alternatively TS 38.101-1 clause 6.5.2.2 can be referred as they are the same. For the channel bandwidth 200 MHz, the mask defined for the aggregated channel bandwidth in NR CA can be reused. Therefore, spectral mask can be derived by assuming BWchannel_CA =200 MHz in Table 6.5A.2.2.1-1 of TS 38.101-1.

| ΔfOOB (MHz) | Spectrum emission limit(dBm) | MBW(MHz) |
| --- | --- | --- |
| ± 0 - 1 | -13 | 0.2 |
| ± 1 - 5 | -10 | 1 |
| ± 5 – 200 | -13 | 1 |
| ± 200 – 205 | -25 | 1 |

Emission mask up to 100 MHz channel bandwidth is defined in TS 38.101-1 clause 6.5.2.2. For wider channel bandwidth, TS 38.101-1 clause 6.5A.2.2 is referred.

Blocking and spurious response

          - FFS if RAN4 to consider the blocking characteristic specified in clause 7.6 of TS 38.101-1 for frequency larger than 3300 MHz could be applied.

In the WF in RAN4#112-bis [6], the following agreement was made for the blocking and spurious response.

The blocking and spurious response characteristics, i.e., in-band blocking, out-of-band blocking and spurious response for high NR band CA (n77, n78 and n79) are specified in TS 38.101-1 clause 7.6A.2.1, 7.6A3.1 and 7.7A.1. The tables 7.6A.2.1-1, 7.6A.2.1-2, 7.6A3-1, 7.6A.3-2, 7.7A-1, and 7.7A-2 specify the parameters and requirements. They can be the baseline for the requirement of 200 MHz channel bandwidth in the 15 GHz band, assuming NR CA bandwidth class C with BWchannel CA=200 MHz.

Blocking and spurious response up to 100 MHz channel bandwidth is specified in clause 7.6 of TS 38.101-1. For wider channel bandwidth, the aggregated channel bandwidth of NR intra-band contiguous CA with bandwidth class C in clause 7.6A.2.1, 7.6A3.1 and 7.7A.1 is referred.

For the base station side, the current assumption to reuse FR1 requirement is also applicable to multicarrier operation wider than 100 MHz channel bandwidth. No additional clarification to assume aggregated channel bandwidth is needed for 200 MHz channel bandwidth.

No additional clarification is needed for channel bandwidth wider than 100 MHz for BS.

In conclusion, we have no practical issue to assume 200 MHz typical channel bandwidth. Therefore, we propose 200 MHz is concluded as the typical maximum channel bandwidth for 14800 – 15350 MHz in the LS reply to WP5D.

200 MHz is communicated to WP5D as the typical maximum channel bandwidth for 14800 – 15350 MHz in the LS reply.

UE Noise Figure

Noise figure

- For LS to ITU, noise figure is [13] dB

- The common understanding is that the current co-existence simulation work (based on 11 dB UE NF) is not changed due to above noise figure value.

In the WF in RAN4#112-bis [6], the following agreement was made.

The value in square bracket 13 dB is worse than the typical NF value agreed earlier in TR 38.921 (10 dB) and the one agreed for the coexistence simulation work (11 dB). It is confusing that different values are used in different contexts, so the TR should capture how NF=13 dB in the LS reply is different from NF=11 dB in the coexistence simulations. We understand NF=11 dB is the typical value but NF=13 dB is the worst value (such as for the extreme condition). It is proposed that the NF=11 dB is mentioned in the TR as the typical value and 13 dB is the worst value including margins such as temperature or product variation, etc.

It is proposed that the NF=11 dB is mentioned in the TR as the typical value and 13 dB is the worst value including margins such as temperature or product variation, etc.

ACLR and ACS

ACLR

- Decide based on co-existence study

ACS

- Decide based on co-existence study

In the WF in RAN4#112-bis [6], the following agreement was made.

The simulation results in [7] and [8] have shown that for both downlink and uplink, the average and 5%-tile throughput losses of the victim UE in all simulated cases are limited to around 5% with downlink ACIR offset of -9 dB compared to the current ACLR and ACS specified for FR1. This is also aligned with the trend that ACLR and ACS can be more relaxed in higher frequency range from coexistence viewpoint. Therefore, it is proposed the ACLR and ACS for 14800 to 15350 MHz should be relaxed for 9 dB compared to the current ones specified in FR1, i.e., 21 dB ACLR and 24 dB ACS.

The ACLR and ACS for 14800 to 15350 MHz should be relaxed for 9 dB compared to the current ones specified in FR1, i.e., 21 dB ACLR and 24 dB ACS.

Conclusion

The following Observations and Proposals were made:

Proposal 1: Spurious emissions up to 100 MHz channel bandwidth is defined in TS 38.101-1 clause 6.5.3.1. For wider channel bandwidth such as 200 MHz, TS 38.101-1 clause 6.5A.3 is referred.

Proposal 2: Emission mask up to 100 MHz channel bandwidth is defined in TS 38.101-1 clause 6.5.2.2. For wider channel bandwidth, TS 38.101-1 clause 6.5A.2.2 is referred.

Proposal 3: Blocking and spurious response up to 100 MHz channel bandwidth is specified in clause 7.6 of TS 38.101-1. For wider channel bandwidth, the aggregated channel bandwidth of NR intra-band contiguous CA with bandwidth class C in clause 7.6A.2.1, 7.6A3.1 and 7.7A.1 is referred.

Observation 1: No additional clarification is needed for channel bandwidth wider than 100 MHz for BS.

Proposal 4: 200 MHz is communicated to WP5D as the typical maximum channel bandwidth for 14800 – 15350 MHz in the LS reply.

Proposal 5: It is proposed that the NF=11 dB is mentioned in the TR as the typical value and 13 dB is the worst value including margins such as temperature or product variation, etc.

Proposal 6: The ACLR and ACS for 14800 to 15350 MHz should be relaxed for 9 dB compared to the current ones specified in FR1, i.e., 21 dB ACLR and 24 dB ACS.

- References

- R4-2400333 Parameters of terrestrial component of IMT for sharing and compatibility studies in the frequency bands 4 400-4 800 MHz, 7 125-8 400 MHz and 14.8-15.35 GHz, ITU-R Working Party 5D

- RP-240787 New SI proposal: Study on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz

- R4-2410576, Draft LS on Parameters for 4400 to 4800 MHz of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27, RAN4#111

- R4-2414449	LS Reply on Parameters for 7125 to 8400 MHz of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27, RAN4#112

- DRAFT Meeting Report for TSG RAN WG4 meeting: 112-bis

- R4-2417108 WF on 15GHz UE parameters, RAN4#112-bis

- R4-2417536, “Urban macro coexistence simulation results for 14800 to 15350 MHz frequency range with 1 UE per slot”, Nokia.

- R4-2417537, “Urban macro coexistence simulation results for 14800 to 15350 MHz frequency range with 3 UE per slot”, Nokia.