3GPP TSG-RAN WG4 Meeting # 113					       			       			R4-2418283

Orlando, US, November 18-22, 2024

Agenda item:			7.22.3

Source:	Moderator (Huawei)

Title:	Topic summary for [113][225] Netw_Energy_NR_enh_Part2

Document for:	Information

# Introduction

This topic summary includes RRM core requirements for Rel-19 network energy saving (7.22.2.2).

Recommended Topics to be treated online (in order of decreasing priority):

Issue 1-1-1: Scenario clarification based on RAN1 progress

Issue 1-2-1: L1 requirements at transition

Issue 1-2-2: L1 requirements subsequent to  “transition period”

Issue 1-3-1: L3 requirements for SSB adaptation – general

Issue 1-4-1: SCell activation requirement for SSB adaptation

# Topic #1: Adaptation of common signal/channel transmission

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2417514 | Qualcomm Technologies Ireland | Proposal 1: RAN4 does not define requirements for SSB adaptation in idle mode. Proposal 2: RAN4 should not discuss requirements that depend on the SMTC window as long as no answer from RAN2 has been received. Observation 1: Transition requirements are already defined for e.g., beam failure detection and L3 measurements when the configuration changes. The underlying principles should also be applied when SSB periodicity is adapted. Observation 2: During a transition in SSB periodicity TSSB, the shorter measurement period may not contain enough samples to achieve the sufficient measurement accuracy. Proposal 3: For measurements that are defined based on a measurement period, requirements during the transition phase in case of SSB adaptation should be based on the longer measurement period, i.e., based on the longer of both SSB periodicities, i.e., max(TSSB1, TSSB2). Observation 3: In case of beam failure detection, requirements during the transition phase are based on the more stringent evaluation period, i.e., the shorter one of both evaluation periods. Proposal 4: In case of beam failure detection, requirements for the transition phase should be based on min(TSSB1, TSSB2). Proposal 5: RAN4 to investigate whether for CBD requirements in the transition phase can be defined based on min(TSSB1, TSSB2). |
| R4-2417626 | MediaTek inc. | Proposal 1: When the UE transitions when SSB periodicity changes, the L1/L3 measurement requirements apply based on the longer delay before or after the transition. |
| R4-2417727 | CATT | Proposal 1: RAN4 to discuss the L3 measurement requirements impact due to SSB adaptation after receiving RAN2 reply on the SMTC configuration. Proposal 2: RAN4 to discuss minimum requirement at transitions for L3 measurement due to SSB adaptation after receiving RAN2 reply on the SMTC configuration. Proposal 3: For SSB periodicity adaptation in time-domain, the existing L1 measurement requirements can still apply. Proposal 4: For SSB periodicity adaptation in time-domain, define the minimum requirement at transitions for L1 measurement: - When SSB periodicity changes upon SSB periodicity adaptation, the UE shall use an evaluation period that is the maximum of the evaluation periods corresponding to the SSB periodicity before and after the adaptation. Subsequent to this duration, the UE shall use an evaluation period corresponding to the SSB periodicity after adaptation. Proposal 5: No impact is expected for SCell activation and TCI state switching due to SSB adaptation. Proposal 6: The defined requirements due to SSB periodicity adaptation apply to SCell. And RAN4 wait for RAN1 progress on whether the requirements apply to PCell. |
| R4-2417959 | Xiaomi | Observation 1: the adaptation SSB configuration happened within a successful measurement report period may result in the measurement requirements updates. Proposal 1: When defining the requirement with adaptation SSB, RAN4 needs only consider the case in which there is no any SSB adaptation period changed within one reporting interval. Proposal 2: The requirements for transition period can be decoupled with L1/L3 measurement. FFS on how to defined them separately. Proposal 3: The current SSB based measurement requirements in 9.2, 9.3 and 9.4 [2,TS38.133] shall update the applicability for the adaptation SSB for PCell. |
| R4-2418163 | LG Electronics Inc. | Proposal 1: RAN4 to focus on L1 measurement requirement, and defer L3 measurement requirement Proposal 2: Apply indicated period of SSB adaptation for L1 measurement requirement Proposal 3: Reusing legacy transition requirements for SSB adapation |
| R4-2418431 | CMCC | Proposal 1: No Cell DTX impact on SSB adaptation related requirements. Proposal 2: For both L1 and L3 requirements during the transition period, requirements apply based on the longer delay before or after the transition. Subsequent to the transition period, requirements apply based on the second SSB configuration/periodicity. |
| R4-2418472 | China Telecom | Proposal 1: For L1 measurement requirements, RAN4 to define RRM requirements for transition period of SSB adaptation, it’s proposed to reuse exiting transition requirements for L1 measurement. Proposal 2: After transition period for L1 measurement, UE shall perform measurement based on new SSB configurations, e.g. new SSB periodicity. Proposal 3: After SSB adaptation, for L1 measurement during SCell activation, UE shall perform SCell activation based on new SSB configurations, e.g. new SSB periodicity. Proposal 4: After SSB adaptation, for L3 measurement during SCell activation, wait for the conclusions on SMTC impact from other WGs. Proposal 5: For L3 requirements for transition period and after transition period, wait for the conclusions on SMTC impact from other WGs. |
| R4-2418586 | Apple | Proposal 1: minimum requirements for L1 operations upon SSB periodicity adaptation are needed: Proposal 2: after transition period, existing requirements still apply with clarification that TSSB is the updated SSB periodicity after adaptation. Observation 1: impact on L3 measurement due to SSB adaptation depends on whether there is impact on SMTC configuration upon SSB adaptation. Proposal 3: more RAN1/2 inputs are expected before RAN4 can conclude on impact from SSB adaptation on L3 measurement. |
| R4-2418654 | Huawei, HiSilicon | Proposal 1: For SSB adaptation in L3 measurement, RAN4 assumes that UE is still expected to perform measurement based on SMTC. RAN4 to wait for RAN2 LS reply to decide whether there is RAN4 requirements impacts (e.g. whether to define requirements at transition). Proposal 2: For L1 measurement (L1-RSRP/L1-SINR/BFD/CBD), RAN4 to define requirements at transmission at follows: - UE shall use an evaluation/measurement period that is no less than the minimum of evaluation/measurement period corresponding to the SSB before and after the adaptation, and no more than the maximum of evaluation/measurement period corresponding to the SSB before and after the adaptation. Subsequent to this duration, the UE shall use the evaluation/measurement period corresponding to the SSB after the adaptation. Proposal 3: Based on proposal 2, after the transition period, L1 requirements apply based on the SSB after the adaptation. Proposal 4: UE shall use SSB after adaption for SCell activation regardless of the SMTC configuration when SSB adaption happens before or at the same time as SCell activation. Deprioritize the requirements when SSB adaptation happens during SCell activation. - Detailed triggering on SSB adaptation depends on further RAN1 discussion. Observation 1: In realistic scenario, it is less likely that gNB trigger the TCI state switching simultaneously with SSB adaption. |
| R4-2419009 | Ericsson | Proposal 1: RAN4 to define the NCD-SSB adaptation requirement for SCell first and wait RAN1’s conclusion on whether SSB adaptation can be applied to PCell. Proposal 2: RAN4 to focus on NW controlled SSB adaptation firstly. Proposal 3: RAN4 not to define SSB adaptation requirement for IDLE mode. Proposal 4: RAN4 to wait RAN2’s LS about SMTC adaptation in L3 measurement. Proposal 5: RAN4 not to define SSB adaptation requirement during SCell activation, TCI state switching. Proposal 6: During L1 measurement, UE shall combine the SSB measurements before and after SSB adaptation with different periodicities. Proposal 7: During L1 measurement, subsequent to the transition measurement period, the measurement period requirements will follow the SSB periodicity after SSB adaptation. |
| R4-2419105 | vivo | Proposal 1 RAN4 to follow legacy rule that no RRM requirement is defined if the adaptation of SSB periodicity happens during the time UE performs CBD. Proposal 2 For SSB periodicity adaptation, RAN4 further check whether existing transition period requirements for L3 measurement can be re-used for BFD, L1-RSRP measurement of serving cell and L1-SINR measurement of serving cell. Proposal 3 The support of SSB adaptation under inter-cell scenario needs to be discussed and agreed in RAN1/2 first. RAN4 to postpone corresponding discussion until RAN1/2 make conclusions. Proposal 4 RAN4 to follow legacy rule that no RRM requirement is defined if the adaptation of SSB periodicity happens during the time UE performs TCI switch or SCell activation. Proposal 5 For impacts to L3 measurements, RAN4 to wait for RAN2 conclusion regarding whether there is impact to SMTC due to SSB adaptation. |
| R4-2419191 | Samsung | Observation 1: Based on RAN1 agreements on SSB adaptation: - Rel-19 NES-capable UE in idle mode is not supported - For Cell DTX extension to SSBs not on sync-raster for connected mode UEs, no RAN1 spec. impact Observation 2: Based on the simulation results provided in TR 38.864, larger periodicity can bring BS energy saving gains Observation 3: Option 2 can enable broader SSB-related parameters adaptation and more scheduling flexibility, but is more complicated, and it is one of the solutions to realize option 1 for some functionalities Observation 4: Based on RAN1 discussions, there are two applicable cases for adaptation of SSB in time domain. 1) Serving cell with only Rel-19 NES-capable UE; 2) Serving cell with both legacy and Rel-19 NES-capable UE Observation 5: Due to the mis-match of adapted SSB and SMTC window, if the adapted SSB triggered by MAC-CE/DCI does not fall into the configured SMTC window, the UE cannot perform L3 measurement based on the adapted SSBs Proposal 1: There is no need to discuss the RRM requirements for the scenarios of Rel-19 NES-capable UEs in idle mode - Whether to discuss the RRM requirements for the scenarios of Rel-19 NES-capable UEs in inactive mode should wait for RAN1 progress Proposal 2: There is no need to discuss the RRM requirements related to Cell DTX for connected mode Rel-19 NES-capable UEs Proposal 3: - Suggest RAN4 to consider legacy SSB burst periodicity as starting point to define the requirements - For discussion on RRM requirements impacts based on longer SSB adaptation periodicity can wait for RAN1 progress on whether longer SSB burst adaptation periodicity can be supported Proposal 4: - RAN4 to focus on NW controlled SSB adaptation firstly - Suggest to wait for RAN1 progresses on whether MAC-CE and/or DCI can be supported to indicate SSB adaptation transmission on top of the RRC reconfiguration Proposal 5: Suggest to wait for RAN1 agreements on the applicable scenarios and functionalities of option 1 and option 2 (SSB adaptation based on two SSB configurations) for SSB adaptation Proposal 6: For the SSB adaptation deployment assumption, RAN4 to confirm whether the following applicable case should be considered to define the new requirements (if any) - Serving cell with only Rel-19 NES-capable UE - Serving cell with both legacy and Rel-19 NES-capable UE Proposal 7: For adaptation of common signal/channel transmissions, RAN4 to discuss: - Which deployment assumption should be considered to define the new requirements (if any) - Intra-cell and/or inter-cell deployment Proposal 8: - RAN4 to discuss the transition period requirements by SSB periodicity adaptation - FFS: Requirements on interrupts at state transitions Proposal 9: RAN4 to wait RAN2 reply LS on how to cope with the mis-match of adapted SSB transmission and SMTC window by using which SMTC mechanism (s). Proposal 10: For L3 requirements for transition period, the possible UE measurement behavior can be: - If the first candidate additional SSB resource is activated after receiving the indication signaling and after Time Gap 1 - Both SSB resources, the NES-capble UEs can to measure. - Whether to/How to define the new measurement rules and the combination/sample average rules of measurement based on two SSBs with different periodicities shall be further discussed in RAN4 - FFS: Time Gap 1 - If the candidate additional SSB resources are activated beyond the first whole configured periodicity unit after receiving the indication signaling and after Time Gap 2 - No need to combine the measurements - How to define the NES related measurement period - Whether to/How to define NES related TFirstSSB - FFS: Time Gap 2 Proposal 11: If adaptation based on two SSB configurations where two configurations are active, for L3 requirements after transition period, the possible UE measurement behavior can be: - NES-capable UEs to measure these additional NES-based SSB resources with the highest priority - Both SSB resources (legacy SSBs and adapted SSBs) the NES-capable UEs can to measure. FFS how to/whether to combine the measurements, FFS how to define the NES based measurement period - Based on UE implementation. Follow the measurement rules concluded in on-demand SSB. Proposal 12: No need to define RRM requirements for the case: UE receive the adaptation indication during SCell activation procedure but after MAC-CE activation command reception Proposal 13: For Rel-19 NES-capable UE’s SCell - RAN4 to focus on MAC CE based SCell activation case as starting point for SSB adaptation SCell operation - RAN4 to study the SCell activation and deactivation delay impact. - Combine the RAN1 conclusions of on-demand SSB and SSB adaptation to define the corresponding SCell requirements Proposal 14: To define the L1 requirements for “transition period”/after “transition period”, M=3 assumption should be considered. |
| R4-2419266 | ZTE Corporation, Sanechips | Proposal 1: In time domain, the adaptation of NCD-SSB not on sync raster for PCell could be supported. Proposal 2: The impact caused by the SSB adaptation in time domain is to identify the relevant L1/L3 measurement requirements based on the adapted SSB periodicity. When the transition happens, UE identifies the relevant L1/L3 measurement requirements based on the adapted SSB periodicity. |
| R4-2419324 | Nokia, Nokia Shanghai Bell | Proposal 1: RAN4 to start investigation of RRM impacts for SSB adaptation in time domain for a single active SSB pattern, due to usage of different SSB burst periodicities. Further scenarios can be considered based on further RAN1 agreements. Proposal 2: RAN4 to maintain the specified range of SSB burst periodicity (5ms to 160ms) for SSB periodicity adaptation. Proposal 3: RAN4 to consider the L3 measurement impact from SSB periodicity adaptation to NR intra-/inter-frequency cell detection delay and time index delay as well as to NR intra-/inter-frequency measurement period, i.e. the impact to configured SMTC period in case of active SCell. Proposal 4: RAN4 to consider the L1 measurement impact from SSB periodicity adaptation both to L1-RSRP measurement delay for active SCells for BFD and CBD, and to TCI state switching delay. Proposal 5: RAN4 to investigate whether a transition period with no or relaxed RRM requirements can serve as starting point when changing SSB burst periodicity for defining the impact to L3 measurements. Proposal 6: RAN4 to consider the impact of processing time for decoding the updated SSB burst periodicity to be ready to receive the SSBs with the adjusted SSB burst periodicity, identified as the SSB adaptation delay. |
| R4-2418771 | OPPO | Observation 1: The SMTC impact should be considered for OD-SSB and SSB adaptation together. Proposal 1: RAN4 not to define L3 measurement based on SMTC periodicity before and during SCell activation if OD-SSB is triggered before the complete of SCell activation. Proposal 2: SMTC adaptation is not expected from UE’s perspective due to dynamic SSB adaptation or OD-SSB activation/deactivation. Proposal 3: Prefer to wait for RAN2’s LS reply on SMTC impact, but it should not impact the discussion of OD-SSB based SCell activation. |

## Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 1-1 SSB adaptation – Scenario

Issue 1-1-1: Scenario clarification based on RAN1 progress

Background:

RAN4 agreements in last meeting R4-2416860 Issue 1-1-1: Scenario clarification based on RAN1 progress Agreement: - Per RAN1 agreements, RAN4 to take following as starting point for SSB adaption discussion: - CONNECTED mode, and - SSB adaptation for SCell, and - SSB adaptation for NCD-SSB, and - SSB periodicity adaptation in time-domain Note: Above could be further updated based on further RAN1 progress (e.g. PCell, other adaptation mechanism) RAN1#118 bis Conclusion There is no consensus on the support of adaptation of SSB for idle mode UEs in Rel-19 Agreement For Cell DTX extension to SSBs not on sync-raster for connected mode UEs, select Option 3, i.e. Cell DTX does not impact UE assumption on SSB transmissions (i.e. legacy behavior). - No spec impact

- Proposals about IDLE mode

  - Option 1: RAN4 does not define requirements for SSB adaptation in idle mode (QC, Ericsson, Samsung).

    - Option 1a: Whether to discuss the RRM requirements for the scenarios of Rel-19 NES-capable UEs in inactive mode should wait for RAN1 progress (Samsung)

- Proposals about PCell

  - Option 1: The defined requirements due to SSB periodicity adaptation apply to SCell. And RAN4 wait for RAN1 progress on whether the requirements apply to PCell. (CATT)

    - Option 1a: RAN4 to define the NCD-SSB adaptation requirement for SCell first and wait RAN1’s conclusion on whether SSB adaptation can be applied to PCell. (Ericsson)

  - Option 2: The current SSB based measurement requirements in 9.2, 9.3 and 9.4 [2,TS38.133] shall update the applicability for the adaptation SSB for PCell. (Xiaomi)

  - Option 3: In time domain, the adaptation of NCD-SSB not on sync raster for PCell could be supported. (ZTE)

- Proposals about Cell DTX

  - Option 1: No Cell DTX impact on SSB adaptation related requirements. (CMCC, Samsung)

- Moderator:

Based on moderator’s understanding, agreements in last RAN4 meeting still hold. In addition, following could be agreed per RAN1 latest agreements.

- Recommended WF:

  - Per RAN1 agreements:

    - No RRM impacts for SSB adaptation requirement for IDLE mode:

    - No RRM impacts for Cell DTX.

Issue 1-1-2: Scenario clarification- SSB periodicity

- Proposals

  - Option 1: RAN4 to consider legacy SSB burst periodicity as starting point to define the requirements (Samsung, Nokia)

    - Option 1a: RAN4 to maintain the specified range of SSB burst periodicity (5ms to 160ms) for SSB periodicity adaptation. (Nokia)

    - Option 1b:  wait for RAN1 progress on whether longer SSB burst adaptation periodicity can be supported. (Samsung)

- Recommended WF:

  - Postpone the discussion and wait for RAN1 progress.

Issue 1-1-3: Scenario clarification-  SSB adaptation triggering

- Proposals

  - Option 1: RAN4 to focus on NW controlled SSB adaptation firstly (Samsung, Ericsson)

    - Option 1a: wait for RAN1 progresses on whether MAC-CE and/or DCI can be supported to indicate SSB adaptation transmission on top of the RRC reconfiguration. (Samsung)

- Recommended WF:

  - RAN4 to focus on NW controlled SSB adaptation firstly.

Issue 1-1-4: Scenario clarification-  SSB adaptation mechanism

- Proposals

  - Option 1: RAN4 to start investigation of RRM impacts for SSB adaptation in time domain for a single active SSB pattern, due to usage of different SSB burst periodicities. Further scenarios can be considered based on further RAN1 agreements (Nokia)

  - Option 2: Suggest to wait for RAN1 agreements on the applicable scenarios and functionalities of option 1 and option 2 (SSB adaptation based on two SSB configurations) for SSB adaptation. (Samsung)

- Recommended WF:

  - Postpone the discussion and wait for RAN1 progress.

Issue 1-1-5: Scenario clarification-  Others

- Proposals

  - Option 1: For the SSB adaptation deployment assumption, RAN4 to confirm whether the following applicable case should be considered to define the new requirements (if any). (Samsung)

    - Serving cell with only Rel-19 NES-capable UE

    - Serving cell with both legacy and Rel-19 NES-capable UE

  - Option 2: For adaptation of common signal/channel transmissions, RAN4 to discuss: (Samsung)

    - Which deployment assumption should be considered to define the new requirements (if any)

    - Intra-cell and/or inter-cell deployment

  - The support of SSB adaptation under inter-cell scenario needs to be discussed and agreed in RAN1/2 first. RAN4 to postpone corresponding discussion until RAN1/2 make conclusions. (Vivo)

- Recommended WF: Discuss above proposals

### Sub-topic 1-2 SSB adaptation – L1 Requirements impacts

Issue 1-2-1: L1 requirements at transition

- Proposals

  - Option 1: Requirements during the transition phase in case of SSB adaptation should be based on the longer measurement period: i.e., based on the longer of both SSB periodicities, i.e., max(TSSB1, TSSB2).  (QC)

    - Option 1a: the L1/L3 measurement requirements apply based on the longer delay before or after the transition. (Apple, MTK)

    - Option 1b: When SSB periodicity changes upon SSB periodicity adaptation, the UE shall use an evaluation period that is the maximum of the evaluation periods corresponding to the SSB periodicity before and after the adaptation. Subsequent to this duration, the UE shall use an evaluation period corresponding to the SSB periodicity after adaptation. (CATT)

    - Option 1c: For both L1 and L3 requirements during the transition period, requirements apply based on the longer delay before or after the transition. Subsequent to the transition period, requirements apply based on the second SSB configuration/periodicity. (CMCC)

    - Option 1d: 	UE shall use an evaluation/measurement period that is no less than the minimum of evaluation/measurement period corresponding to the SSB before and after the adaptation, and no more than the maximum of evaluation/measurement period corresponding to the SSB before and after the adaptation. Subsequent to this duration, the UE shall use the evaluation/measurement period corresponding to the SSB after the adaptation. (Huawei)

    - Option 1e: For SSB periodicity adaptation, RAN4 further check whether existing transition period requirements for L3 measurement can be re-used for BFD, L1-RSRP measurement of serving cell and L1-SINR measurement of serving cell. (Vivo)

  - Option 2: BFD/CBD, requirements for the transition phase should be based on min (TSSB1, TSSB2). (QC).

  - Option 3: For L1 measurement requirements, RAN4 to define RRM requirements for transition period of SSB adaptation, it’s proposed to reuse exiting transition requirements for L1 measurement. (CTC, LGE)

    - Reusing legacy transition requirements for SSB adaptation. (LGE)

  - Option 4: RAN4 to follow legacy rule that no RRM requirement is defined if the adaptation of SSB periodicity happens during the time UE performs CBD. (Vivo)

  - Option 5: During L1 measurement, UE shall combine the SSB measurements before and after SSB adaptation with different periodicities. (Ericsson)

  - Option 6: The requirements for transition period can be decoupled with L1/L3 measurement. FFS on how to defined them separately. (Xiaomi)

  - Option 7: To define the L1 requirements for “transition period”/after “transition period”, M=3 assumption should be considered. (Samsung)

  - Option 8: UE identifies the relevant L1/L3 measurement requirements based on the adapted SSB periodicity. (ZTE)

  - Option 9: RAN4 to consider the L1 measurement impact from SSB periodicity adaptation both to L1-RSRP measurement delay for active SCells for BFD and CBD, and to TCI state switching delay. (Nokia)

- Moderator:

Most companies supported to define requirements at transition at least for some L1 measurement requirements. Proposal could be divided in to following directions. Some of the proposals are for upper bound of the transition period, while others are for the lower bound.

  - Alt 1: evaluation/measurement period is the maximum of the period corresponding to the SSB before and after adaptation.

  - Alt 2: evaluation/measurement period is the minimum of the period corresponding to the SSB before and after adaptation.

  - Alt 3: evaluation/measurement period is no less than the minimum of the period corresponding to the SSB before and after adaptation.

  - Alt 4: evaluation/measurement period is the period corresponding to the SSB after adaptation.

  - Alt 5: evaluation/measurement period is defined assuming UE combines SSB before and after SSB adaptation with different periodicities

- Recommended WF:

  - RAN4 to define requirements at transition for L1 requirements.

  - Discuss Alt 1/2/3/4/5 in this meeting and discuss the applicable L1 requirements.

Issue 1-2-2: L1 requirements subsequent to  “transition period”

- Proposals

  - Option 1: Subsequent to the transition period, requirements apply based on the second SSB configuration/periodicity. (CMCC, CATT, LGE, CTC, Apple, Huawei, Ericsson)

    - Option 1a: For SSB periodicity adaptation in time-domain, the existing L1 measurement requirements can still apply. (CATT)

    - Option 1b: Apply indicated period of SSB adaptation for L1 measurement requirement. (LGE)

    - Option 1c: Subsequent to the transition period, requirements apply based on the second SSB configuration/periodicity. (CMCC)

    - Option 1d: After transition period for L1 measurement, UE shall perform measurement based on new SSB configurations, e.g. new SSB periodicity. (CTC)

    - Option 1e: after transition period, existing requirements still apply with clarification that TSSB is the updated SSB periodicity after adaptation. (Apple)

    - Option 1f: Subsequent to this duration, the UE shall use the evaluation/measurement period corresponding to the SSB after the adaptation. (Huawei)

    - Option 1g: subsequent to the transition measurement period, the measurement period requirements will follow the SSB periodicity after SSB adaptation. (Ericsson)

- Moderator:

All companies agree that subsequent the transition period, requirements apply based on the SSB after transition.

- Recommended WF:

  - Subsequent the transition period, requirements apply based on the SSB after transition.

  - Note: The transition period is discussed in other issues.

### Sub-topic 1-3 SSB adaptation – L3 Requirements impacts

Issue 1-3-1: L3 requirements for SSB adaptation – general

- Proposals

  - Option 1: Wait for RAN2 inputs on whether there is impact on SMTC. (QC, CATT, CTC, Apple, Huawei, Ericsson, Vivo, Samsung, LGE, OPPO)

    - Option 1: RAN4 should not discuss requirements that depend on the SMTC window as long as no answer from RAN2 has been received. (QC)

    - Option 1a: RAN4 to discuss the L3 measurement requirements impact due to SSB adaptation after receiving RAN2 reply on the SMTC configuration. (CATT)

    - Option 1b: For L3 requirements for transition period and after transition period, wait for the conclusions on SMTC impact from other WGs. (CTC)

    - Option 1c: more RAN1/2 inputs are expected before RAN4 can conclude on impact from SSB adaptation on L3 measurement. (Apple)

    - Option 1d: For SSB adaptation in L3 measurement, RAN4 assumes that UE is still expected to perform measurement based on SMTC. RAN4 to wait for RAN2 LS reply to decide whether there is RAN4 requirements impacts (e.g. whether to define requirements at transition). (Huawei)

    - Option 1e: RAN4 to wait RAN2’s LS about SMTC adaptation in L3 measurement. (Ericsson)

    - Option 1f: For impacts to L3 measurements, RAN4 to wait for RAN2 conclusion regarding whether there is impact to SMTC due to SSB adaptation. (Vivo)

    - Option 1g: RAN4 to wait RAN2 reply LS on how to cope with the mis-match of adapted SSB transmission and SMTC window by using which SMTC mechanism (s). (Samsung)

    - Option 1h: RAN4 to focus on L1 measurement requirement, and defer L3 measurement requirement. (LGE)

    - Option 1i: Prefer to wait for RAN2’s LS reply on SMTC impact, but it should not impact the discussion of OD-SSB based SCell activation. (Oppo)

  - Option 2: Define requirements at transition for L3 measurement. (MTK, CATT, Xiaomi, CMCC, ZTE, Xiaomi)

    - Option 2a: When the UE transitions when SSB periodicity changes, the L1/L3 measurement requirements apply based on the longer delay before or after the transition. (MTK)

    - Option 2b: RAN4 to discuss minimum requirement at transitions for L3 measurement due to SSB adaptation after receiving RAN2 reply on the SMTC configuration. (CATT)

    - Option 2c: The requirements for transition period can be decoupled with L1/L3 measurement. FFS on how to defined them separately. (Xiaomi)

    - Option 2d: For both L1 and L3 requirements during the transition period, requirements apply based on the longer delay before or after the transition. Subsequent to the transition period, requirements apply based on the second SSB configuration/periodicity. (CMCC)

    - Option 2e: The impact caused by the SSB adaptation in time domain is to identify the relevant L1/L3 measurement requirements based on the adapted SSB periodicity. When the transition happens, UE identifies the relevant L1/L3 measurement requirements based on the adapted SSB periodicity. (ZTE)

    - Option 2f: RAN4 to investigate whether a transition period with no or relaxed RRM requirements can serve as starting point when changing SSB burst periodicity for defining the impact to L3 measurements. (Nokia)

  - Option 3: When defining the requirement with adaptation SSB, RAN4 needs only consider the case in which there is no any SSB adaptation period changed within one reporting interval. (Xiaomi)

  - Option 4: RAN4 to consider the L3 measurement impact from SSB periodicity adaptation to NR intra-/inter-frequency cell detection delay and time index delay as well as to NR intra-/inter-frequency measurement period, i.e. the impact to configured SMTC period in case of active SCell. (Nokia)

  - Option 5: SMTC adaptation is not expected from UE’s perspective due to dynamic SSB adaptation or OD-SSB activation/deactivation. (Oppo)

- Moderator:

  - 9 companies supported to wait for RAN2 LS reply on whether there is impact on SMTC. 6 companies supported to define requirements at transition.

- Recommended WF:

  - Wait for LS reply on whether there is impact on SMTC.

  - Discuss whether to define requirements at transition for L3 measurement base on LS reply.

Issue 1-3-2: L3 requirements for SSB adaptation – measurement behaviour

- Proposals

  - Option 1: Samsung

For L3 requirements for transition period, the possible UE measurement behavior can be:

- If the first candidate additional SSB resource is activated after receiving the indication signaling and after Time Gap 1

  - Both SSB resources, the NES-capble UEs can to measure.

  - Whether to/How to define the new measurement rules and the combination/sample average rules of measurement based on two SSBs with different periodicities shall be further discussed in RAN4

  - FFS: Time Gap 1

- If the candidate additional SSB resources are activated beyond the first whole configured periodicity unit after receiving the indication signaling and after Time Gap 2

  - No need to combine the measurements

  - How to define the NES related measurement period

  - Whether to/How to define NES related TFirstSSB

  - FFS: Time Gap 2

If adaptation based on two SSB configurations where two configurations are active, for L3 requirements after transition period, the possible UE measurement behavior can be:

- NES-capable UEs to measure these additional NES-based SSB resources with the highest priority

- Both SSB resources (legacy SSBs and adapted SSBs) the NES-capable UEs can to measure. FFS how to/whether to combine the measurements, FFS how to define the NES based measurement period

- Based on UE implementation. Follow the measurement rules concluded in on-demand SSB.

- Recommended WF:

  - Discuss above proposal.

### Sub-topic 1-4 SSB adaptation – Other requirements impacts

Issue 1-4-1: SCell activation requirement for SSB adaptation

- Proposals

  - Option 1: RAN4 not to define SSB adaptation requirement during SCell activation, TCI state switching (CATT, Ericsson, Vivo, Samsung, OPPO)

    - Option 1: No impact is expected for SCell activation and TCI state switching due to SSB adaptation. (CATT)

    - Option 1a: RAN4 not to define SSB adaptation requirement during SCell activation, TCI state switching. (Ericsson)

    - Option 1b: RAN4 to follow legacy rule that no RRM requirement is defined if the adaptation of SSB periodicity happens during the time UE performs TCI switch or SCell activation. (Vivo)

    - Option 1c: No need to define RRM requirements for the case: UE receive the adaptation indication during SCell activation procedure but after MAC-CE activation command reception. (Samsung)

    - Option 1d: RAN4 not to define L3 measurement based on SMTC periodicity before and during SCell activation if OD-SSB is triggered before the complete of SCell activation. (Oppo)

  - Option 2a: (CTC)

    - After SSB adaptation, for L1 measurement during SCell activation, UE shall perform SCell activation based on new SSB configurations, e.g. new SSB periodicity (CTC)

    - After SSB adaptation, for L3 measurement during SCell activation, wait for the conclusions on SMTC impact from other WGs. (CTC)

  - Option 2b: (Huawei)

    - UE shall use SSB after adaption for SCell activation regardless of the SMTC configuration when SSB adaption happens before or at the same time as SCell activation. Deprioritize the requirements when SSB adaptation happens during SCell activation.

    - Detailed triggering on SSB adaptation depends on further RAN1 discussion.

  - Option 2c: RAN4 to study the SCell activation and deactivation delay impact. (Samsung)

    - RAN4 to focus on MAC CE based SCell activation case as starting point for SSB adaptation SCell operation

    - Combine the RAN1 conclusions of on-demand SSB and SSB adaptation to define the corresponding SCell requirements

- Recommended WF:

  - No requirements when SSB adaptation happens during SCell activation.

  - FFS the requirements impact when SSB adaptation happen before or at the same time as SCell activation considering.

Issue 1-4-2: TCI state switching requirements for SSB adaptation

- Proposals

  - Option 1: No impact is expected for SCell activation and TCI state switching due to SSB adaptation. (CATT)

    - Option 1a: RAN4 not to define SSB adaptation requirement during SCell activation, TCI state switching. (Ericsson)

    - Option 1b: RAN4 to follow legacy rule that no RRM requirement is defined if the adaptation of SSB periodicity happens during the time UE performs TCI switch or SCell activation. (Vivo)

  - Option 2: RAN4 to consider the L1 measurement impact from SSB periodicity adaptation both to L1-RSRP measurement delay for active SCells for BFD and CBD, and to TCI state switching delay. (Nokia)

- Recommended WF:

  - No requirements when SSB adaptation happens during TCI state switching.

Issue 1-4-3: Other requirements impacts by SSB adaptation

- Proposals

  - Option 1: RAN4 to consider the impact of processing time for decoding the updated SSB burst periodicity. (Nokia)

  - Option 2: RAN4 to discuss the transition period requirements by SSB periodicity adaptation. (Samsung)

    - FFS: Requirements on interrupts at state transitions

- Recommended WF:

  - Discuss above proposal