3GPP TSG-RAN WG4 Meeting#113		R4-2418319

Orlando, United States, Nov 18th -22nd, 2024

Agenda item:	7.21.3.2

Source: 	Spreadtrum Communications

Title: 	Discussion on RF requirements impact for ambient IoT device

Document for:	Discussion

Introduction

In RAN4#112bis meeting, a WF [1] was approved, in which there were remaining open issues as listed. This contribution continues to be discussed as follow.

Discussion

TX requirements

Agreements: - Whether requirements potentially needed or not is summarized in the following table: A-IoT device TX (D2R) RF Requirement; Potentially needed or not. Device 1; Device 2a; Device 2b. Transmit output power; Maximum output power /output power; NO for maximum output power YES for backscattering power or power backscattering loss; NO for maximum output power YES for backscattering power or power backscattering loss; YES. Output power dynamic; Transmit OFF power; No; NO; YES, as RAN4#112 agreed. Transmit ON/OFF time mask; NO, as RAN4#112 agreed; NO, as RAN4#112 agreed; NO, as RAN4#112 agreed. Minimum output power; N/A; N/A; YES. Power control; NO, as RAN4#112 agreed; NO, as RAN4#112 agreed; YES, as RAN4#112 agreed. Transmit signal quality; Frequency error; NO, as RAN4#112 agreed; TBD; YES, as RAN4#112 agreed. Modulation quality requirements; TBD; TBD; TBD. In band emissions; NO, as RAN4#112 agreed; TBD; TBD.. Carrier leakage; NO, as RAN4#112 agreed; TBD; YES, as RAN4#112 agreed. Output RF spectrum emissions; Occupied bandwidth; YES; YES; YES. SEM; TBD; TBD; TBD. ACLR; TBD; TBD; TBD. Spurious emissions; TBD; TBD; TBD. Unwanted emissions; TBD; TBD; TBD. Transmit intermodulation; NO; NO; TBD.

According to the agreements in RAN4#112bis meeting, there are some requirements are TBD which need to be discussed in this meeting. They are Modulation quality requirements, SEM, ACLR and Spurious emissions respectively.

Modulation quality requirement

According to RAN1#118 meeting agreement, OOK and Binary PSK for baseband modulation are feasible for D2R for all devices. In current spec, pi/2 BPSK EVM is 30%. On the whole, it is needed to define EVM for devices. However, device1 has ultra-low complexity and cost. The number of kinds of RF requirements should be less. In our understanding, EVM is not needed to consider for device1 in view of testing cost and testing time. In addition, as for device 2a and device2b, these two devices have more power consumption and cost. Maybe with higher positioning accuracy and higher data carrying capacity in the future release is needed. Perhaps EVM is not applicable for OOK signal. Maybe we can use modulation quality requirements to define for device 2a and 2b to evaluation performance.

Proposal 1: Modulation quality requirement is not needed to define for device1. Modulation quality requirement is needed to define for device2a and device2b.

Spectrum emission mask(SEM)

What SEM measures is the absolute value of noise in a small bandwidth. Generally speaking, SEM needs to rely on baseband filtering and processing. Obviously, for device1 and device 2a just has a LPF to suppress SEM. Hence, in our understanding, there is no need to define SEM for device1 and device2a. For device2b, defining SEM is necessary.

Proposal 2: SEM is not needed to define for device1 and deviec2a. SEM is needed to define for device2b.

Adjacent channel leakage ratio

Adjacent Channel Leakage power Ratio (ACLR) is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency. ACLR is the average power considering the leakage into adjacent channels, and it is a relative power value. In our understanding, it is needed to define for all devices. The value of ACLR is related to co-existence study.

Proposal 3: ACLR is needed to define for all devices. The value of ACLR is related to co-existence study.

Spurious emissions

Other spurious except near-end spurious such as ACLR/SEM Belong to spurious emissions. As an RFID reference, device needs to meet the regulation requirements and it is related to co-existence study. Hence, spurious emissions is needed to be defined for all devices.

Proposal 4: Spurious emissions is needed to define for all devices. The value of spurious is related to co-existence study and regulation requirements.

RX requirements

Agreement: - Whether requirements potentially needed or not is summarized in the following table: A-IoT device- RX Requirement; Potentially needed or not. Device 1; Device 2a; Device 2b. Reference sensitivity; YES, as RAN4#112 agreed; YES, as RAN4#112 agreed; YES, as RAN4#112 agreed. Maximum input power; TBD; TBD; TBD. ACS; NO; NO; Depend on co-existence study. ACSC; NO; NO; NO. In-band blocking; NO; NO; TBD. Out-of-band blocking; NO; NO; TBD. Receiver intermodulation; NO; NO; TBD. Rx spurious emission; NO; NO; TBD. Spurious response; NO, as RAN4#112 agreed; NO, as RAN4#112 agreed; NO, as RAN4#112 agreed. Interference rejection; TBD; TBD; TBD.

Maximum input power

Maximum input power is defined as the maximum mean power received at the UE antenna port. The purpose of the definition is to prevent the whole link from being saturated, thus causing the deterioration of SNR and performance. According to the investigation for envelop detector, the dynamic range of envelop detector is about 45dB, and the reference sensitivity of device 1 and device2a is -36dBm and -46dBm. The transmission power of A-IoT BS is 33dBm, assumed for co-existence study. Which means that it may exceed the dynamic range of envelop detector. Hence, it is needed to define maximum input power for device 1and device2a. And device2b has higher power consumption compared to device 1and device2a. Device2b also needs to define maximum input power.

Proposal 5: Maximum input power is needed to define for all devices.

Conclusions

In this paper, we provide our views on RF requirements for ambient IoT device, we have made the following proposals.

Proposal 1: Modulation quality requirement is not needed to define for device1. Modulation quality requirement is needed to define for device2a and device2b.

Proposal 2: SEM is not needed to define for device1 and deviec2a. SEM is needed to define for device2b.

Proposal 3: ACLR is needed to define for all devices. The value of ACLR is related to co-existence study.

Proposal 4: Spurious emissions is needed to define for all devices. The value of spurious is related to co-existence study and regulation requirements.

Proposal 5: Maximum input power is needed to define for all devices.

Reference

- R4-2417088, WF  on impacts of A-IoT on RF requirements, Huawei, TSG RAN WG4 meeting #112bis