# 3GPP TSG-RAN WG4 Meeting #113	R4-2418354

# Orlando, US, 18th – 22nd November, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.108 | CR | - | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR to TS38.108 Introduction of NR-NTN S band |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_Sband-Core |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduction of NR-NTN S band. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Relevant sections for NR-NTN S band are updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements for NR-NTN S band are not defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2, 5.3.5, 5.4.2.3, 5.4.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# << Start of change >>

5.2	Operating bands

Satellite is designed to operate in the operating bands defined in table 5.2-1 and 5.2-2.

Table 5.2-1: Satellite operating bands in FR1-NTN

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n256 | 1980 MHz – 2010 MHz | 2170 MHz – 2200 MHz | FDD |
| n255 | 1626.5 MHz – 1660.5 MHz | 1525 MHz – 1559 MHz | FDD |
| n254 | 1610 MHz – 1626.5 MHz | 2483.5 MHz – 2500 MHz | FDD |
| n252 | 2000 MHz - 2020 MHz | 2180 MHz - 2200 MHz | FDD |
| NOTE: Satellite bands are numbered in descending order from n256. |  |  |  |

Table 5.2-2: Satellite operating bands in FR2-NTN

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n512 (NOTE 1) | 27500 - 30000 MHz | 17300 - 20200 MHz | FDD |
| n511 (NOTE 2) | 28350 - 30000 MHz | 17300 - 20200 MHz | FDD |
| n510 (NOTE 3) | 27500 - 28350 MHz | 17300 - 20200 MHz | FDD |
| NOTE 1: This band is applicable in the countries subject to CEPT ECC Decision(05)01 [14] and CEPT ECC Decision (13)01 [15]. NOTE 2: This band is applicable in the USA subject to FCC 47 CFR part 25 [16]. NOTE 3: This band is applicable for Earth Station operations in the USA subject to FCC 47 CFR part 25 [16]. FCC rules currently do not include ESIM operations in this band (47 CFR 25.202 [17]). |  |  |  |

# << Next change >>

5.3.5	SAN channel bandwidth per operating band

The requirements in this specification apply to the combination of SAN channel bandwidths, SCS and operating bands shown in table 5.3.5-1 for FR1-NTN and table 5.3.5-2 for FR2-NTN. The transmission bandwidth configuration in table 5.3.2-1 and table 5.3.2-2 shall be supported for each of the SAN channel bandwidths within the SAN capability. The SAN channel bandwidths are specified for both the Tx and Rx path.

Table 5.3.5-1: SAN channel bandwidths and SCS per operating band in FR1-NTN

| SAN Operating Band | SCS (kHz) | SAN channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 30 (NOTE) |
|  | 15 | 5 | 10 | 15 | 20 |  |
| n256 | 30 |  | 10 | 15 | 20 |  |
|  | 60 |  | 10 | 15 | 20 |  |
|  | 15 | 5 | 10 | 15 | 20 |  |
| n255 | 30 |  | 10 | 15 | 20 |  |
|  | 60 |  | 10 | 15 | 20 |  |
|  | 15 | 5 | 10 | 15 |  |  |
| n254 | 30 |  | 10 | 15 |  |  |
|  | 60 |  | 10 | 15 |  |  |
|  | 15 | 5 | 10 | 15 | 20 |  |
| n252 | 30 |  | 10 | 15 | 20 |  |
|  | 60 |  | 10 | 15 | 20 |  |
| NOTE: Deployment of 30 MHz channel bandwidth for NTN SAN needs to be preceded by introduction of all applicable Tx RF, Rx RF, and demodulation requirements. |  |  |  |  |  |  |

Table 5.3.5-2: SAN channel bandwidths and SCS per operating band in FR2-NTN

| SAN Operating Band | SCS (kHz) | SAN channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 | 100 | 200 | 400 |
| n512 | 60 | 50 | 100 | 200 |  |
|  | 120 | 50 | 100 | 200 | 400 |
| n511 | 60 | 50 | 100 | 200 |  |
|  | 120 | 50 | 100 | 200 | 400 |
| n510 | 60 | 50 | 100 | 200 |  |
|  | 120 | 50 | 100 | 200 | 400 |

# << Next change >>

5.4.2.3	Channel raster entries for each operating band

The RF channel positions on the channel raster in each SAN operating band are given through the applicable NR-ARFCN in table 5.4.2.3-1 for FR1-NTN and table 5.4.2.3-3 for FR2-NTN, using the channel raster to resource element mapping in clause 5.4.2.2.

For SAN operating bands with 100 kHz channel raster, ΔFRaster = 20 × ΔFGlobal. In this case, every 20th NR-ARFCN within the operating band are applicable for the channel raster within the operating band and the step size for the channel raster in table 5.4.2.3-1 is given as <20>.

For SAN operating bands with 60 kHz channel raster above 3 GHz, ΔFRaster = I ×ΔFGlobal, where I ϵ {4, 8}. In this case, every Ith NR-ARFCN within the operating band are applicable for the channel raster within the operating band and the step size for the channel raster in table 5.4.2.3-3 is given as <I>.

Table 5.4.2.3-1: Applicable NR-ARFCN per operating band in FR1-NTN

| SAN operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n256 | 100 | 396000 – <20> – 402000 | 434000 – <20> – 440000 |
| n255 | 100 | 325300 – <20> – 332100 | 305000 – <20> – 311800 |
| n254 | 100 | 322000 – <20> – 325300 | 496700 – <20> – 500000 |
| n252 | 100 | 400000 – <20> – 404000 | 436000 – <20> – 440000 |

For SAN operating bands with 100 kHz channel raster, enhanced channel raster is defined with ΔFRaster = 2 × ΔFGlobal. In this case every 2nd NR-ARFCN within the operating band are applicable for the channel raster within the operating band and the step size for the channel raster in table 5.4.2.3-1 is <2>.

Table 5.4.2.3-2: Applicable NR-ARFCN per operating band for enhanced channel raster

| SAN operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n256 | 10 | 396000 – <2> – 402000 | 434000 – <2> – 440000 |
| n255 | 10 | 325300 – <2> – 332100 | 305000 – <2> – 311800 |
| n254 | 10 | 322000 – <2> – 325300 | 496700 – <2> – 500000 |
| n252 | 10 | 400000 – <2> – 404000 | 436000 – <2> – 440000 |
| NOTE 1: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. These channel numbers shall also be such that the minimum guard band for each channel bandwidth and SCS specified in Table 5.3.3-1 are met for carriers located at the upper or lower edge of an operating band. |  |  |  |

Table 5.4.2.3-3: Applicable NR-ARFCN per operating band in FR2-NTN

| SAN operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n512 | 60 | 2070833 – <1> – 2112499 | 1553336 – <4> – 1746664 |
|  | 120 | 2070833 – <2> – 2112499 | 1553336 – <8> – 1746664 |
| n511 | 60 | 2084999 – <1> –2112499 | 1553336 – <4> – 1746664 |
|  | 120 | 2084999 – <2> –2112499 | 1553336 – <8> – 1746664 |
| n510 | 60 | 2070833 – <1> – 2084999 | 1553336 – <4> – 1746664 |
|  | 120 | 2070833 – <2> – 2084999 | 1553336 – <8> – 1746664 |

# << Next change >>

5.4.3.3	Synchronization raster entries for each operating band

The synchronization raster for each band is given in table 5.4.3.3-1 and table 5.4.3.3-2. The distance between applicable GSCN entries is given by the <Step size> indicated in table 5.4.3.3-1 for FR1-NTN and table 5.4.3.3-2 for FR2-NTN.

Table 5.4.3.3-1: Applicable SS raster entries per operating band (FR1-NTN)

| SAN operating band | SS Block SCS | SS Block pattern (NOTE) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n256 | 15 kHz | Case A | 5429 – <1> – 5494 |
| n255 | 15 kHz | Case A | 3818 – <1> – 3892 |
|  | 30 kHz | Case B | 3824 – <1> – 3886 |
| n254 | 15 kHz | Case A | 6215 – <1> – 6244 |
|  | 30 kHz | Case C | 6218 – <1> – 6241 |
| n252 | 15 kHz | Case A | 5456 – <1> – 5494 |
| NOTE: SS Block pattern is defined in clause 4.1 in TS 38.213 [7]. |  |  |  |

Table 5.4.3.3-2: Applicable SS raster entries per operating band (FR2-NTN)

| SAN operating band | SS Block SCS | SS Block pattern (NOTE) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n512 | 120 kHz | Case D | 17448 – <12> – 19428 |
|  | 240 kHz | Case E | 17472 – <24> – 19416 |
| n511 | 120 kHz | Case D | 17448 – <12> – 19428 |
|  | 240 kHz | Case E | 17472 – <24> – 19416 |
| n510 | 120 kHz | Case D | 17448 – <12> – 19428 |
|  | 240 kHz | Case E | 17472 – <24> – 19416 |
| NOTE: SS Block pattern is defined in section 4.1 in TS 38.213 [7]. |  |  |  |

# << End change >>