3GPP TSG-RAN WG4 Meeting #113	R4-2418369

Orlando, US, 18-22 November 2024

Source:	Nokia

Title:	(NR_newRAT-Core) On Rel-15 Active TCI state list update

Agenda item:	4.5.1

Document for:	Discussion

Introduction

In this contribution we continue the discussion about Rel-15 active TCI state list update requirement clarification in section 8.10.6 that has been ongoing under the maintenance agenda for some meetings.

Discussion

The discussion about potential need for clarification has been about the highlighted text part:

### 8.10.6 Active TCI state list update delay If the target TCI state is known, upon receiving PDSCH carrying MAC-CE for activation/deactivation of UE-specific PDSCH TCI state as defined in clause 6.1.3.14 of TS 38.321 [7] at slot n, UE shall be able to receive PDCCH to schedule PDSCH with the new target TCI state at the first slot that is after n+ THARQ +3Nslotsubframe,µ +TOk*(Tfirst-SSB + TSSB-proc) / NR slot length. Where THARQ, Tfirst-SSB, TSSB-proc and TOk are defined in clause 8.10.3.

The reason for the discussion has been that the requirement may be falsely understood to mean that the network schedules PDCCH with the target TCI state, while the target TCI state is for PDSCH, and the delay is the time until the UE can be scheduled with the target PDSCH TCI state.

The requirement is written as the time from UE receiving TCI state activation MAC-CE activating new PDSCH TCI state in slot n until the UE shall be able to receive a PDCCH scheduling the UE with the target PDSCH TCI state after slot n+ THARQ +3Nslotsubframe,µ +TOk*(Tfirst-SSB + TSSB-proc) / NR slot length. Hence, even though the requirement is for PDSCH TCI state activation delay, the delay is actually defined to end at the PDCCH that schedules the PDSCH.

Active TCI state list update delay is to define time after which the UE can be scheduled using PDCCH on a newly activated target PDSCH TCI state. The delay is defined from the time the UE receives the TCI state activation MAC-CE until the UE receives PDCCH scheduling the PDSCH with the target PDSCH TCI state.

Figure 1 illustrates the active TCI state list update delay in section 8.10.6 with blue color.

If the UE is scheduled on the new PDSCH TCI state after the defined delay, depending on whether the tci-PresentInDCI is set as 'enabled' or ‘disabled’, the first reception with the new PDSCH TCI state may happen timeDurationForQCL after this. The delay due to timeDurationForQCL is defined in section 8.10.4 as DCI based TCI state switch delay. This is also shown in Figure 1.

The delay from the time the UE receives PDCCH scheduling the PDSCH with the target PDSCH TCI state is taken into account in DCI based TCI state switch delay requirement.

There have been multiple proposals to update the yellow highlighted part in section 8.10.6. However, companies have not been able to conclude which wording would be the most suitable one.

The requirement is for Rel-15 and hence there can be no technical changes in the requirement which could impact actual implementations. The technical understanding of the requirement shall be kept the same. This means the that the delay will continue to be from the time the UE receives the TCI state activation MAC-CE until the UE at latest shall be able to receive a PDCCH scheduling the PDSCH with the new PDSCH TCI state.

The technical meaning of the current Rel-15 requirement shall not be changed as this impact actual implementations.

There have been proposals about changing the requirement to e.g. UE not being expected to receive a PDCCH scheduling PDSCH with the new TCI state before slot n+ THARQ +3Nslotsubframe,µ +TOk*(Tfirst-SSB + TSSB-proc) / NR slot length. In our view such addition is not helpful. Even though the delay is up to the point the UE receives the PDCCH, the requirement is anyways about when the UE can be scheduled with the new PDSCH TCI state. Hence, such addition is not serving the purpose of the requirement and shall not be included.

Initially we may not see a great need to update the current requirement. However, if RAN4 group see that a clarification to the requirement would be helpful, we propose the following wording, which we have also included in our CR:

If the target TCI state is known, upon receiving PDSCH carrying MAC-CE for activation/deactivation of UE-specific PDSCH TCI state as defined in clause 6.1.3.14 of TS 38.321 [7] at slot n, UE shall be able to receive PDCCH to schedule scheduling the PDSCH with the new target TCI state at the first slot that is after n+ THARQ +3Nslotsubframe,µ +TOk*(Tfirst-SSB + TSSB-proc) / NR slot length. Where THARQ, Tfirst-SSB, TSSB-proc and TOk are defined in clause 8.10.3.

Conclusion

The following observations and proposals were made in this contribution:

- Active TCI state list update delay is to define time after which the UE can be scheduled using PDCCH on a newly activated target PDSCH TCI state. The delay is defined from the time the UE receives the TCI state activation MAC-CE until the UE receives PDCCH scheduling the PDSCH with the target PDSCH TCI state.

- The delay from the time the UE receives PDCCH scheduling the PDSCH with the target PDSCH TCI state is taken into account in DCI based TCI state switch delay requirement.

- The technical meaning of the current Rel-15 requirement shall not be changed as this impact actual implementations.

- If the target TCI state is known, upon receiving PDSCH carrying MAC-CE for activation/deactivation of UE-specific PDSCH TCI state as defined in clause 6.1.3.14 of TS 38.321 [7] at slot n, UE shall be able to receive PDCCH to schedule scheduling the PDSCH with the new target TCI state at the first slot that is after n+ THARQ +3Nslotsubframe,µ +TOk*(Tfirst-SSB + TSSB-proc) / NR slot length. Where THARQ, Tfirst-SSB, TSSB-proc and TOk are defined in clause 8.10.3.