3GPP TSG-RAN WG4 Meeting #113	R4-2418389

Orlando, USA, Nov 18 – Nov 22, 2024

Source:	Nokia

Title:	Specification improvements in R19 timeframe

Agenda item:	9.1.2.2

Document for:	Discussion

Introduction

In the last couple of meetings, RAN4 has been discussing possible specification improvements. The outcome of the RRM part of the discussions is captured in the agreed WF’s [1, 2 and 3].

In RAN4#111 a number of concrete actions were agreed to implemented. These were further discussed in RAN4#112 meeting where following was agreed and captured in [3]:

- Interested companies are encouraged to collaborate on CR review and drafting before RAN4#112bis

  - CR will be based on TS38.133 v18.6.0

  - The updated CR will be submitted in RAN4#112bis

  - The proposed revision should be strictly limited to the following three issues

    - Issue1: [], “TBD” and “FFS” clean up

    - Issue2: Terminology/style inconsistency, incorrect notation/symbols/abbreviation, undefined abbreviations, redundant information/notes clean up

    - Issue3: Empty test cases clean up

A CR drafting and review plan was made. Interested companies has spent a great amount of time and effort on reviewing the CRs in the time between RAN#112 and RAN4#112bis.

Besides above agreement also following agreements were reached in RAN4#112 [3]:

- Modal verbs

  - Agreement: deprioritized

- Duplicated requirements

  - Agreement: deprioritized

- Unused test configurations

  - Agreement: deprioritized

- Suffix alignment

  - Agreement: deprioritized

- References or mapping tables in the core part requirements that point to the relevant test cases from Rel-19

  - Agreement: Deprioritized. Meanwhile, interested companies can bring examples on how to implement.

- Abbreviations

  - “PRB” or “RB”:

    - Current status: both are used, choose one of them? None of them is in Abbreviation of TS 38.133.

    - Agreement: “PRB” should be consistently used in all sections instead of “RB” and the abbreviation is to be included in section 3.

  - “BW”:

    - Current status: used, but not in the Abbreviation section in TS 383.133.

    - Agreement: Include the abbreviation in the Abbreviations section 3 of TS 38.133.

  - “TRS”: is used but not defined in Abbreviations section in TS 38.133.

    - Agreement: Include the abbreviation in the Abbreviations section 3 of TS 38.133.

In RAN4#112bis following was agreed:

- Agreement

  - Generic editing aspects to be addressed

    - Edit Help (i.e. no need to fix in the CR)

      - “table”, “annex” and “clause” should be in lower case, except in the caption of the table or title of the clause.

      - DRx -> DRX

      - RB->PRB

      - EUTRA(N) -> E-UTRA(N)

      - n.a.-> N/A (edit help)

      - “5ms”->”5 ms”

        - ms, dB, dBm, s, us, kHz, MHz, GHz, Hz,

      - Cell1-> Cell 1

        - TRP, BWP, Note

      - SSB GSCN -> SSB ARFCN in test configurations with clauses start with A.

    - To be addressed in the CR

      - italic on UE capability or signalling IE

      - Update abbreviation section to include the following terms

        - PRB, BW, TRS

      - Missing references should be included.

        - Example: TS 38.101-1 [18]

      - The caption of the table in general should not be changed for sake of simplification, unless error is identified.

  - The following aspects may or may not be pure editorial and should be reviewed and decided case by case.

    - Remove [], TBD, FFS

    - Remove/update notes

- Hierarchy of indent

  - Agreement:

    - Target release:

      - Step 1: Take 8.3.2 in TS38.133 v18.7.0 as the example to start working with

        - This exercise should not be part of CR drafting and approval in Nov.

      - Step 2: Rel-19 specification for R19 features with independent clause

    - Approach

      - Option 1: RAN2 pseudo-code

      - Option 2: RAN1/4 existing approach

      - Interested companies are encouraged to choose their preferred approaches in next meetings and the decision is not expected to be made before next Feb meeting.

In section 2, we give our view on how RAN4 could potentially improve the use of hierarchy of indent by use of ‘pseudo-code’-style.

In this paper we discuss a few aspects identified during the process of drafting the example in the Appendix, which is a continuation of our earlier proposals in [4 and 6]. We have left the comments in the TP to make it easier to locate each identified issue. The TP in section is the same as provided in [6].

Discussion

Following is one TP example how to apply the pseudo-code in section 8.3.2 which is a school example of complex RAN4 specification writing.

<Start of Change #1>

### 8.3.2	SCell Activation Delay Requirement for Deactivated SCell

The requirements in this clause shall apply for the UE configured with at least one downlink SCell in EN-DC, or in standalone NR carrier aggregation or in NE-DC or in NR-DC and when one SCell is being activated.

The delay within which the UE shall be able to activate the deactivated SCell depends upon the specified conditions.

Upon receiving SCell activation command in slot n, the UE shall be capable to transmit valid CSI report and apply actions related to the activation command for the SCell being activated no later than in slot:

Tactivation_delay = n+THARQ+Tactivation_time+TCSI_ReportingNR slot length

where:

- THARQ (in ms) is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3]

- Tactivation_time is the SCell activation delay in millisecond.

- TCSI_reporting is the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [2].

1> If the SCell is known and belongs to FR1:

2> if the measurement period of the SCell being activated is equal to or smaller than 2400ms:

3> Tactivation_time = TFirstSSB+ 5ms.

2> if the measurement period of the SCell being activated is larger than 2400ms:

3> Tactivation_time = TFirstSSB_MAX + Trs + 5ms.

1> If the SCell is unknown and belongs to FR1:

2> if ‘ssb-PositionInBurst’ indicates only one SSB is being actually transmitted, or

2> if ‘ssb-PositionInBurst’ indicates multiple SSBs and TCI indication is provided in same MAC PDU with SCell activation,

3> if the side condition Ês/Iot ≥ -2dB is fulfilled:

4> if UE supports shortMeasInterval-r18 and:

5> if the SCell is contiguous to an active serving cell in the same band, and

5> if the ssb-PositionInBurst is same as the one of contiguous FR1 active serving cell, and

5> if the SMTC offset of the sCell is same as the one of contiguous FR1 active serving cell, and

5> if the RTD with contiguous FR1 active serving cell is smaller than or equal to 260ns with respect to the to-be-activated SCell’s SSB numerology, and its reception power difference with contiguous FR1 active serving cell is smaller than or equal to 6dB.

6> Tactivation_time = TFirstSSB_MAX, enhanced + TSMTC_MAX, enhanced + Trs, enhanced + 5ms.

5> otherwise,

6> Tactivation_time = TFirstSSB_MAX, enhanced + TSMTC_MAX, enhanced + 2*Trs, enhanced + 5ms.

4> Otherwise:

5> if the SCell is contiguous to an active serving cell in the same band, and

5> if the ssb-PositionInBurst is same as the one of contiguous FR1 active serving cell, and

5> if the SMTC offset is same as the one of contiguous FR1 active serving cell, and

5> if the RTD with contiguous FR1 active serving cell is smaller than or equal to 260ns, and its reception power difference with contiguous FR1 active serving cell is smaller than or equal to 6dB

6> Tactivation_time = TFirstSSB_MAX + TSMTC_MAX + Trs + 5ms.

5>  otherwise:

6> Tactivation_time = TFirstSSB_MAX + TSMTC_MAX + 2*Trs + 5ms.

2> otherwise:

3> if the side condition Ês/Iot ≥ -2dB is fulfilled:

4> If UE supports shortMeasInterval-r18:

5> if semi-persistent CSI-RS is used for CSI reporting:

6> Tactivation_time = 6ms + TFirstSSB_MAX, enhanced + TSMTC_MAX, enhanced + Trs, enhanced + TL1-RSRP, enhanced_measure + TL1-RSRP, report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP).

5> if periodic CSI-RS is used for CSI reporting:

6> Tactivation_time = 3ms + TFirstSSB_MAX, enhanced + TSMTC_MAX, enhanced + Trs, enhanced + TL1-RSRP, enhanced_measure + TL1-RSRP ,report + max(THARQ + Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay).

4> Otherwise:

5> if semi-persistent CSI-RS is used for CSI reporting:

6> Tactivation_time = 6ms + TFirstSSB_MAX + TSMTC_MAX + Trs + TL1-RSRP, measure + TL1-RSRP,report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP).

5> if periodic CSI-RS is used for CSI reporting:

6> Tactivation_time = 3ms + TFirstSSB_MAX + TSMTC_MAX + Trs + TL1-RSRP, measure + TL1-RSRP,report + max(THARQ + Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay).

-	However, when the following conditions are fulfilled, no activation requirement will be applied for this unknown SCell:

-	the SCell is contiguous to an active serving cell in the same band, and

-	A single SSB is used in the unknown SCell; or multiple SSBs are used in the SCell and TCI state indication for PDCCH is provided by the same MAC PDU used for SCell activation; and

-	its ssb-PositionInBurst is same as the one of contiguous FR1 active serving cell, and

-	its SMTC offset is same as the one of contiguous FR1 active serving cell

-	its RTD with contiguous FR1 active serving cell is larger than 260ns, or its reception power difference with contiguous FR1 active serving cell is larger than 6dB;

1> If the SCell being activated belongs to FR1:

2> if the UE is supporting scellWithoutSSB-InterBandCA-r18:

The reference serving cell can be indicated by higherlayer parameter referenceCell-r18. If UE is not indicated with referenceCell-r18, the reference serving cell is assumed to be the QCL-typeC source cell if there is only one active QCL-typeC source cell configured.

Editor notes: FFS whether and how to capture the wording “after the compensation for AGC”.

2> if the UE is not provided with SSB configuration (absoluteFrequencySSB) in the target SCell (FrequencyInfoDL); and

2> if the UE is not provided with an SMTC configuration for the target SCell, and

2> if there is one collocated active reference serving cell on different FR1 band, and:

3> if the RTD between the target SCell and the collocated reference serving cell is within CP where CP is corresponding to the SCS of SSB-less SCell, and

3> if the [EPRE] difference at the UE is smaller than or equal to [12] dB, where, [EPRE] difference is the power difference between TRS/A-TRS symbol on the SSB-less SCell and SSB symbol on the reference serving cell [after the compensation for AGC], and

3> if the RS(s) of the SSB-less SCell being activated is (are) QCL-TypeA with TRS(s) of the SSB-less SCell being activated, and the TRS(s) of the SSB-less SCell being activated is (are) further QCL-TypeC with SSB(s) of an inter-band active serving cell, and the inter-band active serving cell shall be same as the reference serving cell.

4> if aperiodic CSI-RS resources are not configured for SCell activation, or

4> if the UE do not support aperiodicCSI-RS-FastScellActivation-r17:

5> Tactivation_time = Tfirst_TRS + TTRS +5 ms.

4> if aperiodic CSI-RS resources are configured for Scell activation, and

4> if the UE supporting aperiodicCSI-RS-FastScellActivation-r17

5> Tactivation_time = Tfirst_ATRS + Tgap + TATRS + 5 ms.

1> else, if the SCell being activated belongs to FR1:

2> if the UE supporting scellWithoutSSB, and:

2> if there is at least one active serving cell contiguous to the SCell on that FR1 band, and

2> if the UE is not provided with SSB configuration (absoluteFrequencySSB), and

2> if the UE is not provided with SMTC configuration for the target SCell, and:

3> if the RTD between the target SCell and the contiguous active serving cell is within within ±260ns, and

3> if he difference of the reception power with the contiguous active serving cell is <= 6dB, and

3> if the The RS(s) of SCell being activated is (are) QCL-TypeA with TRS(s) of the SCell being activated, and the TRS(s) of the SCell being activated is (are) further QCL-TypeC with SSB(s) of any active serving cell that is contiguous to the SCell being activated on that FR1 band:

4> Tactivation_time = 3 ms.

1> If the SCell belongs to FR2:

2> if there is at least one active serving cell on that FR2 band:

3> if the UE is provided with SMTC for the target SCell, and

3> if the SSBs in the serving cell(s) and

3> if the SSBs in the SCell fulfil the condition defined in clause 3.6.3, and

3> if the parameter ssb-PositionsInBurst is same for the serving cell(s) and the Scell, and

3> if the SSB is in the same half-frame on the SCell and the contiguous FR2 active serving cell.

4> Tactivation_time is TFirstSSB+ 5ms.

3> if the UE supporting scellWithoutSSB:

4> if UE is not provided with any SMTC for the target SCell:

5> if the RS(s) of SCell being activated is (are) QCL-TypeD with RS(s) of one active serving cell on that FR2 band:

6> Tactivation_time = 3 ms.

4> if the UE is a FR2 power class 6:

5> if the UE supports [Enhanced FR2 HST RRM requirements for intra-band CA and inter-frequency measurements in connected mode], and

5> if the RS(s) of SCell being activated is (are) QCL-TypeD with RS(s) of one active serving cell on that FR2 band:

6> Tactivation_time = 3 ms.

2> if there is no active serving cell on that FR2 band:

3> if the target SCell is known to UE:

4> if semi-persistent CSI-RS is used for CSI reporting:

5> if UE receives the SCell activation command, semi-persistent CSI-RS activation command and TCI state activation command at the same time:

6> Tactivation_time = 3ms + TFineTiming + 2ms.

5> otherwise:

6> Tactivation_time = 3ms + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP)

4> if periodic CSI-RS is used for CSI reporting:

5> if UE receives the SCell activation command and TCI state activation commands at the same time:

6> Tactivation_time = max(5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay-THARQ).

5> otherwise:

6> Tactivation_time = max(Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay-THARQ).

3> if the target SCell is unknown to UE:

4> if the PCell/PSCell and the target SCell are configured as FR1-FR2-1 CA, or

4> if the PCell/PSCell and the target SCell are in a FR2-1 band pair with independent beam management:

5> if semi-persistent CSI-RS is used for CSI reporting, and

5> if the side condition Ês/Iot ≥ -2dB:

6> if UE supports reduceForCellDetection, and/or

6> if UE supports reduceForSSB-L1-RSRP-Meas, and/or

6> if UE support shortMeasInterval-r18, and

7> if SCell activation triggered L3 report is not configured, or

7> if SCell activation triggered L3 report is configured but not reported:

8> Tactivation_time = 6ms + TFirstSSB_MAX,enhanced + 15*TSMTC_MAX, enhanced + X1*Trs, enhanced  + TL1-RSRP, enhanced_measure + TL1-RSRP, report  + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP).

6> otherwise:

7> Tactivation_time = 6ms + TFirstSSB_MAX + 15*TSMTC_MAX + 8*Trs  + TL1-RSRP, measure + TL1-RSRP, report  + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP).

5> if periodic CSI-RS is used for CSI reporting, and

5> if the side condition Ês/Iot ≥ -2dB:

6> if UE supports reduceForCellDetection, and/or

6> if UE supports reduceForSSB-L1-RSRP-Meas, and/or

6> if UE support shortMeasInterval-r18, and

7> if SCell activation triggered L3 report is not configured, or

7> if SCell activation triggered L3 report is configured but not reported:

8> Tactivation_time = 3ms + TFirstSSB_MAX, enhanced + 15*TSMTC_MAX, enhanced + X1*Trs, enhanced + TL1-RSRP, enhanced_measure + TL1-RSRP, report + max {(THARQ + Tuncertainty_MAC + 5ms + TFineTiming), (Tuncertainty_RRC + TRRC_delay)}.

6> otherwise:

7> Tactivation_time = 3ms + TFirstSSB_MAX + 15*TSMTC_MAX + 8*Trs + TL1-RSRP, measure + TL1-RSRP, report + max {(THARQ + Tuncertainty_MAC + 5ms + TFineTiming), (Tuncertainty_RRC + TRRC_delay)}.

4> else, if the PCell/PSCell and the target SCell are configured as FR1-FR2-2 CA, or

4> if the PCell/PSCell and the target SCell are in a FR2-2 band pair with independent beam management:

5> if semi-persistent CSI-RS is used for CSI reporting, and

5> if the side condition Ês/Iot ≥ -2dB:

6> Tactivation_time = 6ms + TFirstSSB_MAX + 23*TSMTC_MAX + 12*Trs  + TL1-RSRP, measure + TL1-RSRP, report  + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP).

5> if periodic CSI-RS is used for CSI reporting, and

5> if the side condition Ês/Iot ≥ -2dB:

6> 3ms + TFirstSSB_MAX + 23*TSMTC_MAX + 12*Trs + TL1-RSRP, measure + TL1-RSRP, report + max {(THARQ + Tuncertainty_MAC + 5ms + TFineTiming), (Tuncertainty_RRC + TRRC_delay)}.

Where:

- TSMTC_MAX:

-	In FR1, in case of intra-band contiguous SCell activation or in case of intra-band non-contiguous SCell activation for UE not capable of intraBandNR-CA-non-collocated-r18 or UE is capable of intraBandNR-CA-non-collocated-r18 and nonCollocatedTypeNR-CA-r18 is provided, TSMTC_MAX is the longer SMTC periodicity between active serving cells and SCell being activated provided the cell specific reference signals from the active serving cells and the SCells being activated or released are available in the same slot; in case of intra-band non-contiguous SCell activation for UE capable of intraBandNR-CA-non-collocated-r18 and nonCollocatedTypeNR-CA-r18 is not provided or in case of inter-band SCell activation, TSMTC_MAX is the SMTC periodicity of SCell being activated. TSMTC_MAX is the SMTC periodicity of SCell being activated.

-	In FR2, in case of intra-band SCell activation, TSMTC_MAX is the longer SMTC periodicity between active serving cells and SCell being activated provided that in Rel-15 only support FR2 intra-band CA; in case of FR2 inter-band SCell activation, TSMTC_MAX is the SMTC periodicity of SCell being activated.

-	TSMTC_MAX is bounded to a minimum value of 10ms.

- TSMTC_MAX, enhanced:

-	In FR1 or FR2-1, a UE supporting shortMeasInterval-r18 if the SMTC for SCell being activated is only configured in measObjectNR, TSMTC_MAX, enhanced is the SSB periodicity of SCell being activated. Otherwise, TSMTC_MAX, enhanced = TSMTC_MAX.

- Trs:

  - is the SMTC periodicity of the SCell being activated if the UE has been provided with an SMTC configuration for the SCell in SCell addition message, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the measObjectNRs having the same SSB frequency and subcarrier spacing configured by MN and SN have different SMTC, Trs is the periodicity of one of the SMTC which is up to UE implementation. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement which involves Trs is applied with Trs = 5ms assuming the SSB transmission periodicity is 5ms. There are no requirements if the SSB transmission periodicity is not 5ms.

- Trs, enhanced:

  - is the SSB periodicity of the SCell being activated for a UE supporting shortMeasInterval-r18 in FR1 or FR2-1, if the SMTC for SCell being activated is only configured in the measObjectNR. Otherwise, Trs, enhanced = Trs

- TFirstSSB:

  - is the time to the end of the first complete SSB burst indicated by the SMTC, or within 5ms if SMTC is not configured, after slot n + THARQ+3msNR slot length.

- TFirstSSB_MAX:

  - Is the time to the end of the first complete SSB burst indicated by the SMTC, or within 5ms if SMTC is not configured, after slot n + THARQ+3msNR slot length, further fulfilling:

-	In FR1, in case of intra-band contiguous SCell activation or in case of intra-band non-contiguous SCell activation for UE not capable of intraBandNR-CA-non-collocated-r18, the occasion when all active serving cells and SCells being activated or released are transmitting SSB bursts in the same slot; in case of intra-band non-contiguous SCell activation for UE capable of intraBandNR-CA-non-collocated-r18 or in case of inter-band SCell activation, the first occasion when the SCell being activated is transmitting SSB burst.

-	In FR2, the occasion when all active serving cells and SCells being activated or released are transmitting SSB bursts in the same slot.

- TFirstSSB_MAX, enhanced:

- For a UE supporting shortMeasInterval-r18 in FR1 or FR2-1, if the SMTC for SCell being activated is only configured in the measObjectNR, TFirstSSB_MAX, enhanced is the time to the end of the first complete SSB burst indicated by the SSB periodicity of the SCell being activated, after slot n + THARQ+3msNR slot length. Otherwise, TFirstSSB_MAX, enhanced = TFirstSSB_MAX

- TFineTiming

  - is the time period between UE finish processing the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and the timing of first complete available SSB corresponding to the TCI state.

- X1:

  - is equal to the reported value in reduceForCellDetection in FR2-1. Otherwise, if reduceForCellDetection is absent, X1 is 8.

- TL1-RSRP, measure:

  - is L1-RSRP measurement delay TL1-RSRP_Measurement_Period_SSB ms or TL1-RSRP_Measurement_Period_CSI-RS based on applicability as defined in clause 9.5 assuming M=1 and TReport=0.

- TL1-RSRP, enhanced_measure is:

-	SSB based L1-RSRP measurement delay TL1-RSRP_Measurement_Period_SSB ms based on applicability as defined in clause 9.5 assuming M=1 and TReport=0; N is equal to the value reported by the UE in  reduceForSSB-L1-RSRP-Meas. Otherwise, if  reduceForSSB-L1-RSRP-Meas is absent, N= 8. Or,

-	CSI-RS based L1-RSRP measurement delay TL1-RSRP_Measurement_Period_CSI-RS ms based on applicability as defined in clause 9.5 assuming M=1 and TReport=0.

-	If UE supports shortMeasInterval-r18 capability, L1-RSRP measurement for TL1-RSRP, enhanced_measure can be performed based on non-DRX mode even if DRX is configured.

- TL1-RSRP, report:

  - is delay of acquiring CSI reporting resources.

- Tuncertainty_MAC:

- is the time period between reception of the last activation command for PDCCH TCI, PDSCH TCI (when applicable) relative to

-	SCell activation command for known case;

-	First valid L1-RSRP reporting for unknown case.

- Tuncertainty_RRC:

- is the time period between reception of the RRC configuration message for TCI of periodic CSI-RS for CQI reporting (when applicable) relative to

-	SCell activation command for known case;

-	First valid L1-RSRP reporting for unknown case.

- Tuncertainty_SP:

- is the time period between reception of the activation command for semi-persistent CSI-RS resource set for CQI reporting relative to:

-	SCell activation command for known case;

-	First valid L1-RSRP reporting for unknown case.

- TRRC_delay:

- is the RRC procedure delay as specified in TS38.331 [2].

  - Longer delays for RRM measurement requirements, and in case of FR2 also SSB based RLM/BFD/CBD/L1-RSRP measurement requirements, can be expected during the cell detection time for unknown SCell activation.

  - When absoluteFrequencySSB is not configured in DownlinkConfigCommon for target SCell but SMTC for target SCell is configured, no requirement would be applied.

- TFirstTRS:

- is the time to the end of the first complete periodic CSI-RS burst for SCell activation after slot n + THARQ+3msNR slot length.

- TTRS:

- is the periodicity of periodic CSI-RS burst for SCell activation.

- TFirstATRS:

- is the time to the end of the first complete CSI-RS burst for SCell activation after slot n + THARQ+3msNR slot length, where the CSI-RS burst is defined as four CSI-RS resources in two consecutive slots.

- TATRS:

- is the CSI-RS burst for SCell activation where the CSI-RS burst is defined as four CSI-RS resources in two consecutive slots.

- Tgap:

- is a gap length between two aperiodic CSI-RS bursts,

-	at least 2 slots for 15kHz and 30kHz

-	at least 3 slots for 60kHz

SCell in FR1 is known if it has been meeting the following conditions:

1> if during the period equal to max(5*measCycleSCell,  5*DRX cycles) for FR1 before the reception of the SCell activation command:

2> the UE has sent a valid measurement report for the SCell being activated and

2> the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3.

1> if the SSB measured during the period equal to max(5*measCycleSCell, 5*DRX cycles) also remains detectable during the SCell activation delay according to the cell identification conditions specified in clause 9.2 and 9.3.

Otherwise SCell in FR1 is unknown.

SCell in FR2 is known if it has been meeting the following conditions:

1> If the SCell is the first SCell to be activated in an FR2 bands, and following conditions:

2> if, during the period equal to 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4 before UE receives the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and semi-persistent CSI-RS for CQI reporting (when applicable):

3> if the UE has sent a valid L3-RSRP measurement report with SSB index, and

3> the SCell activation command is received after L3-RSRP reporting and no later than the time when UE receives MAC-CE command for TCI activation

2> if during the period from L3-RSRP reporting to the valid CQI reporting, the reported SSBs with indexes remain detectable according to the cell identification conditions specified in clauses 9.2 and 9.3, and the TCI state is selected based on one of the latest reported SSB indexes.

Otherwise, the first SCell in FR2 band is unknown.

The requirement for unknown SCell applies provided that the activation commands for PDCCH TCI, PDSCH TCI (when applicable), semi-persistent CSI-RS for CQI reporting (when applicable), and configuration message for TCI of periodic CSI-RS for CQI reporting (when applicable) are based on the latest valid L1-RSRP reporting.

If the UE has been provided with higher layer in TS 38.331 [2] signaling of smtc2 prior to the activation command, TSMTC_Scell follows smtc1 or smtc2 according to the physical cell ID of the target cell being activated. TSMTC_MAX follows smtc1 or smtc2 according to the physical cell IDs of the target cells being activated and the active serving cells.

In addition to CSI reporting defined above, UE shall also apply other actions related to the activation command specified in TS 38.331 [2] for a SCell at the first opportunities for the corresponding actions once the SCell is activated.

The starting point of an interruption window on spCell or any activated SCell, as specified in clause 8.2, shall not occur before slot n+1+THARQNR slot length  and not occur after slot slot n+1+THARQ+3ms+TXNR slot length, where NR slot length is with respect to the numerology used in the SCell being activated, and TX is:

-	0, if Tactivation_time is 3ms;

-	TFirstSSB, for any scenario where Tactivation_time  includes TFirstSSB;

-	TFirstSSB_MAX, for any scenario where Tactivation_time  includes TFirstSSB_MAX;

-	Tuncertainty_MAC +TFineTiming, for any scenario where Tactivation_time  includes only TFineTiming and no TFirstSSB_MAX;

-	Tfirst_TRS, for FR1 inter-band SSB-less SCell activation scenario where Tactivation_time   includes Tfirst_TRS;

-	Tfirst_ATRS, for FR1 inter-band SSB-less SCell activation scenario where Tactivation_time   includes Tfirst_ATRS.

The length of the interruption window may be different for different victim cells, and depends on the applicable scenario and on the frequency band relation between the aggressor cell and the victim cell.

The requirements in this clause and requriements on interruption due to SCell activation in clause 8.2 apply provided that the SSB of the to-be-activated SCell is within the first active DL BWP of the Scell.

Starting from the slot specified in clause 4.3 of TS 38.213 [3] (timing for secondary Cell activation/deactivation) and until the UE has completed the SCell activation, the UE shall report out of range if the UE has available uplink resources to report CQI for the SCell.

Starting from the slot specified in clause 4.3 of TS 38.213 [3] (timing for secondary Cell activation/deactivation) and until the UE has completed a first L1-RSRP measurement, the UE shall report lowest valid L1 SS-RSRP range if the UE has available uplink resources to report L1-RSRP for the SCell.

<End of Changes>

Conclusion

Discuss the proposed specification layout as part of the baseline candidates.

- References

- R4-2406710, WF on [110bis][142] RRM_Spec_Improvement, Apple.

- R4-2410715, WF on [111][140] RRM_Spec_Improvement, Apple

- R4-2414103, WF on [112[229] RRM_Spec_Improvement, Apple

- R4-2404701, Discussion on RRM specification quality improvement, Nokia, Nokia Shanghai Bell

- R4-2411785, On improvements of Rel-19 RRM specifications, Qualcomm

- R4-2416339, Specification improvements in R19 timeframe, Nokia

- R4-2416917, WF for [112bis][225] RRM_Spec_Improvement, Apple