3GPP TSG-RAN WG4 Meeting # 113	R4-2418399

# Orlando, US, 18th – 22nd November, 2024

Source:	LG Electronics

Title:	Discussion on Rel-16 Relaxed measurement for RAN2 LS

Agenda item:	4.5.1

Document for:	Discussion

Introduction

Based on the LS [1], RAN2 revised the TS38.304 to align RAN2 and RAN4 specification for Rel-16 measurement relaxation in IDLE/INACTIVE state. In this contritubtion, we provide our views on the LS whether RAN4 specification needs to be updated.

Discussion

For Rel-16 measurement relaxation, there was inconsistency measurement relaxation requirements for inter-frequency/inter-RAT between RAN2 and RAN4 specification for Case (B) when Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ below (e.g., inter-frequency measurement).

4.2.2.10.2 Measurements for UE fulfilling low mobility criterion This clause contains requirements for measurements on inter-frequency NR cells provided that: - UE is only configured with lowMobilityEvaluation [2] criterion and UE has fulfilled the lowMobilityEvaluation [2] criterion, or - UE is configured with both lowMobilityEvaluation [2] and cellEdgeEvaluation [2] criterion and combineRelaxedMeasCondition [2] is not configured, and - UE has fulfilled only the lowMobilityEvaluation [2] criterion. … When Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ then the requirements are defined as follows: - The UE shall be able to evaluate whether a newly detectable inter-frequency NR cell meets the reselection criteria defined in TS38.304 [1] within Ncarrier_Relax * Tdetect,NR_Inter_Relax + Ncarrier_Non_relax * Tdetect,NR_Inter. Cells which have been detected shall be measured at least every Ncarrier_Relax * Tmeasure,NR_Inter_Relax + Ncarrier_Non_relax * Tmeasure,NR_Inter. The UE shall be able to evaluate that an already identified inter-frequency NR cell has met reselection criterion defined in TS 38.304 [1] within Ncarrier_Relax *Tevaluate,NR_Inter_Relax + Ncarrier_Non_relax * Tevaluate,NR_Inter. … When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ and the UE is configured with highPriorityMeasRelax [2] then the UE shall search for inter-frequency layers of higher priority at least every K2*Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7 and, K2 = 60. Otherwise, if the UE is not configured with highPriorityMeasRelax [2] then the UE shall search for inter-frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7.

- Case (A):

  - UE is only configured with lowMobilityEvaluation [2] criterion and UE has fulfilled the lowMobilityEvaluation [2] criterion, or

- Case (B):

  - UE is configured with both lowMobilityEvaluation [2] and cellEdgeEvaluation [2] criterion and combineRelaxedMeasCondition [2] is not configured, and

  - UE has fulfilled only the lowMobilityEvaluation [2] criterion.

For Case (B),

- RAN2 has defined measurement realaxation for higher priority frequency layer only if the serving cell fulfils Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ. [TS38.304 clause 5.2.4.9]

- RAN4 has defined measurement relaxation for higher priority frequency layer both if the serving cell fulfils Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ and if the serving cell fulfils Srxlev > SnonIntraSearchP or Squal > SnonIntraSearchQ. [TS38.133 clause 4.2.2.10.2 and 4.2.2.11.2]

In the last RAN2 meeting, RAN2 agreed to revise the measurement relaxation rules for NR intra-frequency, inter-frequency, or inter-RAT frequency cells by reffering to the RAN4 specification for the case (B) and RAN2 agreed the CR [2] as below.

5.2.4.9 Relaxed measurement 5.2.4.9.0 Relaxed measurement rules When the UE is required to perform measurements of intra-frequency cells or NR inter-frequency cells or inter-RAT frequency cells according to the measurement rules in clause 5.2.4.2: … - if both lowMobilityEvaluation and cellEdgeEvaluation are configured: - if the UE has performed normal intra-frequency, NR inter-frequency, or inter-RAT frequency measurements for at least TSearchDeltaP after (re-)selecting a new cell; and - if the relaxed measurement criterion in clause 5.2.4.9.1 is fulfilled for a period of TSearchDeltaP; and - if the relaxed measurement criterion in clause 5.2.4.9.2 is fulfilled: - … - else: - if the UE has performed normal intra-frequency, NR inter-frequency, or inter-RAT frequency measurements for at least TSearchDeltaP after (re-)selecting a new cell, and the relaxed measurement criterion in clause 5.2.4.9.1 is fulfilled for a period of TSearchDeltaP; or, - if the relaxed measurement criterion in clause 5.2.4.9.2 is fulfilled: - if combineRelaxedMeasCondition is not configured: - the UE may choose to perform relaxed measurements for NR intra-frequency cells, inter-frequency cells or inter-RAT frequency cells according to relaxation methods in clauses 4.2.2.9, 4.2.2.10, and 4.2.2.11 in TS 38.133 [8];

Since RAN2 agreed to follow RAN4 requirements, RAN4 specification should be clarified for Case (A) and Case (B). In the Case (A), if the serving cell fulfils Srxlev > SnonIntraSearchP or Squal > SnonIntraSearchQ, measurement relaxation for higher priority frequency layer is allowed if highPriorityMeasRelax is configured based on the approved WF [3] in the RAN4#95e as below.

- When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ and only criteria of low mobility is configured, if the low mobility criteria is fulfilled - UE is not required to do any lower and equal priority inter-frequency/inter-RAT measurement - When NW indicates that higher priority carrier measurements can be relaxed (highPriorityMeasRelax), UE measures higher priority inter-frequency/inter-RAT layers at least every 1 hour - When NW does not indicate that higher priority carrier measurements can be relaxed, UE measures higher priority inter-frequency/inter-RAT layers at least every Thigher_priority_search (60 sec).

In the RAN4#96e, for highPriorityMeasRelax, RAN4 agreed applicable scenario and sent LS [4] to RAN2 below.

RAN4 thanks the LS from RAN2 on RRM relaxation in power saving. RAN4 find there are different understandings between RAN2 and RAN4 about the applicability of IE highPriorityMeasRelax. RAN4 had sufficient discussion and drew the following conclusions: - highPriorityMeasRelax is only applied to the scenario where Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ and only criteria of low mobility is configured, if the low mobility criteria is fulfilled, - When NW indicates that higher priority carrier measurements can be relaxed (highPriorityMeasRelax), UE measures higher priority inter-frequency/inter-RAT layers at least every 1 hour - When NW does not indicate that higher priority carrier measurements can be relaxed, UE measures higher priority inter-frequency/inter-RAT layers at least every Thigher_priority_search (60 sec).

Based on the above agreements, the highPriorityMeasRelax IE is only applicable for the Case (A). Therefore, Case (B) is not applicable for highPriorityMeasRelax configuration. And in LS [1], based on RAN2 clarification for s-SearchThresholdP and s-SearchThresholdQ, those parameters are less than or equal to s-IntraSearchP and s-NonIntraSearchP. That means, if Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, UE always has fulfilled the cellEdgeEvaluation criteria. Therefore, in case of the Case (B), when Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, UE has fulfilled both lowMobilityEvaluation and cellEdgeEvaluation criteria. Therefore, in this case, the UE shall search for higher priority frequency layer at least every K2*Thigher_priority_search, so the requirements in clause 4.2.2.10.4 and 4.2.2.11.4 can be applied. The revised CR [5] has been submitted.

- Proposa 1: RAN4 to revise the measurement relaxation requriements for higher priority frequency layer for inter-frequency NR Cells and inter-RAT E-UTRAN cells as below

4.2.2.10.2 Measurements for UE fulfilling low mobility criterion … If UE is only configured with lowMobilityEvaluation [2] criterion and UE has fulfilled the lowMobilityEvaluation [2] criterion, when Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ and the UE is configured with highPriorityMeasRelax [2] then the UE shall search for inter-frequency layers of higher priority at least every K2*Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7 and, K2 = 60. Otherwise, if the UE is not configured with highPriorityMeasRelax [2] then the UE shall search for inter-frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7. 4.2.2.11.2 Measurements for UE fulfilling low mobility criterion … If UE is only configured with lowMobilityEvaluation [2] criterion and UE has fulfilled the lowMobilityEvaluation [2] criterion, when Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ and the UE is configured with highPriorityMeasRelax [2] then the UE shall search for E-UTRA inter-RAT frequency layers of higher priority at least every K2*Thigher_priority_search seconds where Thigher_priority_search is described in clause 4.2.2.7 and, K2 = 60. Otherwise, if the UE is not configured with highPriorityMeasRelax [2] then the UE shall search for E-UTRA inter-RAT frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7.

- Proposal 2: Do not need to send reply LS to RAN2.

Conclusion

In this contribution, we provide our views on the LS whether RAN4 specification needs to be updated, and we propose

- Proposal 1: RAN4 to revise the measurement relaxation requriements for higher priority frequency layer for inter-frequency NR Cells and inter-RAT E-UTRAN cells as below

4.2.2.10.2 Measurements for UE fulfilling low mobility criterion … If UE is only configured with lowMobilityEvaluation [2] criterion and UE has fulfilled the lowMobilityEvaluation [2] criterion, when Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ and the UE is configured with highPriorityMeasRelax [2] then the UE shall search for inter-frequency layers of higher priority at least every K2*Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7 and, K2 = 60. Otherwise, if the UE is not configured with highPriorityMeasRelax [2] then the UE shall search for inter-frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7. 4.2.2.11.2 Measurements for UE fulfilling low mobility criterion … If UE is only configured with lowMobilityEvaluation [2] criterion and UE has fulfilled the lowMobilityEvaluation [2] criterion, when Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ and the UE is configured with highPriorityMeasRelax [2] then the UE shall search for E-UTRA inter-RAT frequency layers of higher priority at least every K2*Thigher_priority_search seconds where Thigher_priority_search is described in clause 4.2.2.7 and, K2 = 60. Otherwise, if the UE is not configured with highPriorityMeasRelax [2] then the UE shall search for E-UTRA inter-RAT frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7.

- Proposal 2: Do not need to send reply LS to RAN2.

Reference

[1] R2-2409308, “LS on Relaxed measurement,” RAN2, LG Electronics

[2] R2-2409282, “Correction to relaxed measurement,” LG Electronics, Nokia, Samsung, Ericsson

[3] R4-2009265, “WF on RRM measurement relaxation for Power Saving,” CATT

[4] R4-2012122, “ Reply LS on RRM relaxation in power saving,” RAN4, Huawei

[5] R4-2418401, “(NR_UE_pow_sav-Core) CR on relaxed measurement in IDLE/INACTIVe mode,” LG Electronics,vivo