3GPP TSG-RAN WG4 Meeting #113	R4-2418410

# Orlando, US, 18th – 22nd November, 2024

Source:	LG Electronics

Title:	Discussion on RRM requirements for ATG CA operation

Agenda item:	7.10.4

Document for:	Discussion

Introduction

In the last RAN4 meeting, general scenario for ATG DL CA was discussed and there were some agreements [1]. In this contribution, we provide our views on open issues for ATG with CA operation.

Discussion

General

UE antenna type

In the last meeting, RAN4 discussed and had revised agreement for UE antenna types for intra-/inter-band DL CA.

Agreement: - Refine the agreement from last meeting as: - For intra-band contiguous CA, same antenna type should be applied on a band. - For inter-band CA, either different or same antenna type can be applied on different bands. - For inter-band CA, consider the following cases: - 1. ATG UE with omnidirectional antennas on the bands of PCC and SCC. - 2. ATG UE with an omnidirectional antenna on the band of PCC and an antenna array on the band of SCC. - 3. ATG UE with an antenna array on the band of PCC and an omnidirectional antenna on the band of SCC. - [4. ATG UE with an antenna array on the bands of PCC and SCC, and one common Rx beam assumed.] - 5. ATG UE with an antenna array on the bands of PCC and SCC, and two simultaneous separate Rx beams assumed. Further check which understanding is correct: - Understanding #1: Case 4 is excluded in RF session. - Understanding #2: Case 4 is not explicitly discussed in RF session, since no case 4 specific RF requirement. Case 4 UE implementation is possible.

For Case 4 antenna type, common Rx beam with array antenna is available since co-located based inter-band DL CA in Rel-19 ATG scenario is only considered. However, generally, RAN4 assumes separate Rx beam for inter-band CA when array antenna is considered like Case 4 antenna type. And as discussed in the last meeting stated above (Understanding#2), Case 4 antenna type is not dicussed in RF session and we do not know whether there have any impact depending on RF architecture. In our understanding, to support Case 4 antenn type, RF architecture is that one LNA and phase shifter would be shared for two bands, and separate local oscillator can be considered. In this case, MRTD, reception power difference for two bands, and DL frequency separation should be restricted to avoid critical receving performance degradataion.

- Observation 1: Separate local oscillator with shared single LNA and phase shifter for two bands can be assumed for the simple RF architecture for Case 4 antenna type.

- Proposal 1: To support Case 4 antenna type, MRTD, reception power difference for two bands, and DL frequency separation should be restricted to avoid critical receving performance degradataion, e.g., conditions for intra-band NCCA operation.

Timing

MRTD

To define MRTD, RAN4 should follow how to derive the MRTD value. MRTD is based on TAE/cell phase synchronication accuracy and propagation delay difference. Existing 33us MRTD requirement for inter-band CA has been defined under assumption that 30us propagation delay difference and 3 us TAE/cell phase synchronization accuracy since non co-located CA deploymenet is considered. However, for ATG inter-band DL CA, since only co-located deployment is considered, MRTD does not need to account for propagation delay differences. Therefore, existing MRTD for intra-band non-contiguous CA as 3us can be reused for ATG intra-band DL CA. If array antenna type with common Rx beam is considered, MRTD should be 3us.

- Proposal 2: For ATG inter-band DL CA, 3us MRTD should be defined.

Signalling characteristics

Interruption requirements

For ATG interruption requirements, exsiting FR1 interruption requirements could be reused for following cases.

- Interruptions at SCell addition/release

- Interruptions at SCell activation/deactivation

- Interruptions at direct SCell activation

- Interruptions at fast SCell activation

- Interruptions during measurements on deactivated SCC

- Interruptions due to SCell dormancy

- Interruptions due to Active BWP switching Requirement

- Interruptions due to UE-specific CBW change

- Interruptions when identifying CGI of an NR cell with autonomous gaps

For interrtupion requirement due to NR SRS antenna port switching, RAN4 needs to discuss whether this feature is useful for ATG scenario first.

- Proposal 3: RAN4 to define the interruption requirements for following cases

  - Interruptions at SCell addition/release

  - Interruptions at SCell activation/deactivation

  - Interruptions at direct SCell activation

  - Interruptions at fast SCell activation

  - Interruptions during measurements on deactivated SCC

  - Interruptions due to SCell dormancy

  - Interruptions due to Active BWP switching Requirement

  - Interruptions due to UE-specific CBW change

  - Interruptions when identifying CGI of an NR cell with autonomous gaps

- Proposal 4: RAN4 to further discuss whether to support SRS antenna port switching

Measurement procedure

SCell activation triggered measurement reporting requirement

In the last meeting, RAN4 agreed to introduce requirement for SCell Activation Delay Requirement for Deactivated SCell with the L3 reporting during activation. So, SCell activation triggered measurement reporting should be defined, and exsiting requirement can be reused.

- Proposal 5: RAN4 to introduce SCell activation triggered measurement reporting for ATG by reusing existing requirement.

Deactivated SCell measurement

RAN4 agreed to define intra-frequency measurement requirements for deactivated SCell by reusing the principle from the existing cell identification and measurement requirements with measCycleScell as baseline. And FFS whether to consider beam sweeping factor N1 and L3/L1 measurement sharing factor Klayer1_measurement. Since ATG DL CA is always co-located and only one SCell is considered, beam sweeping factor N1 and L3/L1 measurement sharing factor Klayer1_measurement can be set to 1, i.e., no need to consider beam sweeping factor N1 and L3/L1 measurement sharing factor Klayer1_measurement.

- Proposal 6: RAN4 not to consider beam sweeping factor N1 and L3/L1 measurement sharing factor Klayer1_measurement for deactivated SCell measurement

Scheduling Availability/Restrictions

For the scheduling availability in ATG CA scenario, in general, exsiting scheduling availability can be applied. So, for UE with omni-directional antenna, existing scheduling availability for FR1 intra-band CA / inter-band CA can be resued, and for UE with antenna array, exsiting scheduling availability for FR2 intra-band CA / inter-band CA can be reused. For UE with different antenna type for inter-band CA, exsiting scheduling availability for FR2 inter-band CA can be reused.

The scheduling availability for case 4 antenna type with common Rx beam can be discussed after final conclusion whether the antenna type is introduced.

- Proposal 7: Existing scheduling availabilty for intra-/inter-band CA can be reused as below

  - For UE with omni-directional antenna, existing scheduling availability for FR1 intra-band CA / inter-band CA can be resued

  - for UE with antenna array, exsiting scheduling availability for FR2 intra-band CA / inter-band CA can be reused

  - For UE with different antenna type for inter-band CA, exsiting scheduling availability for FR2 inter-band CA can be reused

  - FFS for Case 4 antenna type with common Rx beam after final conclusion whether the antenna type is introduced.

Conclusion

In this contribution, we provide views on RRM requireiments for ATG with CA operation, and we propose

- Observation 1: Separate local oscillator with shared single LNA and phase shifter for two bands can be assumed for the simple RF architecture for Case 4 antenna type.

- Proposal 1: To support Case 4 antenna type, MRTD, reception power difference for two bands, and DL frequency separation should be restricted to avoid critical receving performance degradataion, e.g., conditions for intra-band NCCA operation.

- Proposal 2: For ATG inter-band DL CA, 3us MRTD should be defined.

- Proposal 3: RAN4 to define the interruption requirements for following cases

  - Interruptions at SCell addition/release

  - Interruptions at SCell activation/deactivation

  - Interruptions at direct SCell activation

  - Interruptions at fast SCell activation

  - Interruptions during measurements on deactivated SCC

  - Interruptions due to SCell dormancy

  - Interruptions due to Active BWP switching Requirement

  - Interruptions due to UE-specific CBW change

  - Interruptions when identifying CGI of an NR cell with autonomous gaps

- Proposal 4: RAN4 to further discuss whether to support SRS antenna port switching

- Proposal 5: RAN4 to introduce Scell activation triggered measurement reporting for ATG by reusing existing requirement.

- Proposal 6: RAN4 not to consider beam sweeping factor N1 and L3/L1 measurement sharing factor Klayer1_measurement for deactivated SCell measurement

- Proposal 7: Existing scheduling availabilty for intra-/inter-band CA can be resued as below

  - For UE with omni-directional antenna, existing scheduling availability for FR1 intra-band CA / inter-band CA can be resued

  - for UE with antenna array, exsiting scheduling availability for FR2 intra-band CA / inter-band CA can be reused

  - For UE with different antenna type for inter-band CA, exsiting scheduling availability for FR2 inter-band CA can be reused

  - FFS for Case 4 antenna type with common Rx beam after final conclusion whether the antenna type is introduced.

Reference

[1] R4-2416852, “WF on RRM requirements for [112bis][208] NR_ATG_enh,” CMCC