3GPP TSG-RAN WG4 Meeting # 113	R4-2418411

# Orlando, US, 18th – 22nd November, 2024

Source:	LG Electronics

Title:	Discussion on RRM requirements for FR2-1 L3 measurement delay reduction

Agenda item:	7.15.2.1

Document for:	Discussion

Introduction

In the last RAN4 meeting, RAN4 discussed and agreed on high level approaches for triggering condition for multi-Rx L3 measurement [1]. In this contritubtion, we provide our views on activating condition for multi-Rx simultaneous reception and how to define core requirements for L3 measurement delay reduction.

Discussion

Conditions to trigger UE to activate L3 measurement delay reduction by optimizing Rx BSF

RAN4 agreed the high level approaches for condition to trigger UE to activate L3 measurement by optimizing Rx BSF.

For scenario 1/2/3/4: - RAN4 to consider conditions for triggering FBS for L3 measurement, including active condition and de-active conditions. - (De-)Activating multi-Rx for L3 measurements (intra/inter-frequency) is depending on the criteria/threshold(s) and UE current conditions. - The detailed criteria/condition(s) is FFS. - Meanwhile, RAN4 agree that the criteria/threshold(s) are configured by RRC signaling. FFS the further details. For Scenario 5/7: - For UE supporting Rel-19 BSF reduction capability, faster beam sweeping always applies during handover (Tsearch). - For UE supporting Rel-19 BSF reduction capability, faster beam sweeping always applies during RRC re-establishment.

For scenario 1/2/3/4, activating / deactivating multi-Rx for L3 measurement would be based on current UE conditions with some criteria/thresholds. The FR2-1 L3 measurement delay reduction would be beneficial for UE mobility when UE is experiencing lower signal quality to serving cell (i.e., cell edge region). If UE supports multi-Rx reception simultaneously for L3 measurement and signal quality for serving cell is low, L3 measurement delay reduction by optimizing Rx BSF could be applied. Therefore, at least the condition for serving cell signal quality to activate multi-Rx for L3 measurement should be introduced as triggering condition.

- Proposal 1: RAN4 to introduce signal quality (e.g., cell edge) based triggering condition to activate multi-Rx for L3 measurement

Conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF”

RAN4 needs to define the condition to trigger UE to quit multi-Rx simultaneous reception for L3 measurement since UE cannot always turn on the multi-Rx chains due to power consumption issue. Therefore, if UE needs power saving during L3 measurement with multi-Rx simultaneous reception, Rel-18 UAI ‘multiRx-PreferenceFR2’ for power saving can be considered.

- Proposal 2: RNA4 to introduce condition to trigger UE to quit multi-Rx reception for L3 measurement based on Rel-18 UAI ‘multiRx-PreferenceFR2’ for UE power saving.

RRM measurement with two panels activated, two searchers are occupied by this single carrier

[Options in RAN4#112]: - Option A/1a: Two searchers are occupied by this single carrier to the requirements of enhanced BSF for single carrier. Discuss CA case later. - Option B: Consider one searcher for single carrier.

Based on the agreement of applicability requirement and the WID, the main target is L3 measurement delay reduction by Rx BSF for single carrier in FR2-1 as below.

Applicability requirement: Baseline: L3 delay enhancements in Rel-19 by optimizing Rx BSF for UE supporting multi-rx simultaneous reception are applicable provided that: - the target carrier(s) to be measured: only one carrier in the single FR2-1 band is configured for L3 SSB measurement and - UE serving carrier(s): UE is configured with single carrier on FR2-1 band, i.e. FR2-1 PCell without CA/DC. Note: Target and serving carrier frequency can be the same or different. Note: The ‘other UE CA/DC modes (e.g., 1 or 2 FR2-1 bands CA, or FR1+FR2 CA/DC, or EN-DC)’ and/or the ‘other number of target to-be-measured carrier(s) on FR2-1 band’ can be FFS after concluding the baseline above. These extra FFS parts will NOT delay the WI completion. WID FR2-1 SSB based L3 measurement delay reduction for connected mode - For UE supporting multiple-Rx simultaneous reception on single carrier: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - Rx beam sweeping factor

And to reduce L3 measurement using multi-Rx simultaneous reception, two searchers should be occupied by the measured single carrier.

- Proposal 3: Two searchers are occupied by this single carrier to the requirements of enhanced BSF for single carrier to reduce L3 measurement delay in FR2-1.

L3 measurement delay reduction by optimizing Rx BSF with/without DRX

If UE supports the multi-Rx reception for L3 measurement, both non-DRX and DRX should be supported. If UE needs to power saving for DRX cycle, UE can quit multi-Rx operation for L3 measurement depdning on conclusion of other issue.

- Proposal 4: RAN4 to defin L3 requirements delay reduction with optimizing Rx BSF for both non-DRX and DRX

Solutions to apply/specify L3 measurement delay reduction by optimizing Rx BSF

The SSB based intra-frequency measurement requirements without gaps for FR2-1 in Table 1 has been defined considering UE Rx beam sweeping.

Table 1 Measurement period for intra-frequency measurements without gaps (FR2)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max(400ms, ceil(Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(400ms, ceil(1.5x Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil(Mmeas_period_w/o_gaps xKp x Klayer1_measurement ) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

M values depending on power class are below.

Mmeas_period_w/o_gaps: For a UE supporting FR2-1 power class 1 or 5, Mmeas_period_w/o_gaps =40. For a UE supporting FR2-1 power class 2, Mmeas_period_w/o_gaps =24. For a UE supporting FR2-1 power class 3, Mmeas_period_w/o_gaps =24. For a UE supporting power class 4, Mmeas_period_w/o_gaps =24. For a UE supporting FR2-2 power class 1, Mmeas_period_w/o_gaps = 60. For a UE supporting FR2-2 power class 2, Mmeas_period_w/o_gaps = 36. For a UE supporting FR2-2 power class 3, Mmeas_period_w/o_gaps = 36.

For UE supporting FR2-1 power class 3, M=24, and it means that 4 measurement samples with 6 Rx beam sweeping or 3 measurement samples with 8 Rx beam sweeping. For example, for UE supporting multi-Rx simultaneous reception, the less Rx BSF can be considered and it could be UE capability, so 3 or 4 Rx BSF value or additional scaling factor (<1) could be considered to reduce M value. RAN4 needs to discuss how to reduce M values for SSB based intra- / inter-frequency measurements for UE supporting multi-Rx simultaneous reception.

- Proposal 5: RAN4 to discuss how to reduce M values for SSB based intra- / inter-frequency measurements for UE supporting multi-Rx simultaneous reception. For example, consider 3 or 4 Rx BSF value or additional scaling factor < 1 to reduce M value.

Conclusion

In this contribution, we provide views on activating condition for multi-Rx simultaneous reception and how to define core requirements for L3 measurement delay reduction, and we propose

- Proposal 1: RAN4 to introduce signal quality (e.g., cell edge) based triggering condition to activate multi-Rx for L3 measurement

- Proposal 2: RNA4 to introduce condition to trigger UE to quit multi-Rx reception for L3 measurement based on Rel-18 UAI ‘multiRx-PreferenceFR2’ for UE power saving.

- Proposal 3: Two searchers are occupied by this single carrier to the requirements of enhanced BSF for single carrier to reduce L3 measurement delay in FR2-1.

- Proposal 4: RAN4 to defin L3 requirements delay reduction with optimizing Rx BSF for both non-DRX and DRX

- Proposal 5: RAN4 to discuss how to reduce M values for SSB based intra- / inter-frequency measurements for UE supporting multi-Rx simultaneous reception. For example, consider 3 or 4 Rx BSF value or additional scaling factor < 1 to reduce M value.

Reference

[1] R4-2416853, “WF on RRM requirements for NR_RRM_Ph5_Part1,” Apple.