3GPP TSG-RAN WG4 Meeting # 113		R4-2418432

# Orlando, US, 18th – 22nd November, 2024

Agenda Item:	7.23.4.1

Source: 	CMCC

Title: 			  Simulation assumption and results for LP-WUR

Document for:	Discussion

- Introduction

In last meeting, RAN4 provide the updated RRM simulation assumption. The WF has been approved in [1]. In this contribution, we give our simulation results.

- Discussion

Simulation result

Our initial simulation results are provided below by using the simulation assumption in Annex.1.

Table 1. AWGN, Es/IoT = -3, SNR of cell 2 is 6 dB lower compared with cell 2, 4 OFDM symbol

|  | OOK-1 RSRP error | OOK-1 RSRQ error | OOK-4 M=2 RSRP error | OOK-4 M=2 RSRQ error | OOK-4 M=4 RSRP error | OOK-4 M=4 RSRQ error |
| --- | --- | --- | --- | --- | --- | --- |
| 3 sample | 0.32 | 0.22 | 0.29 | 0.2 | 0.53 | 0.42 |
| 4 sample | 0.27 | 0.19 | 0.24 | 0.16 | 0.41 | 0.32 |
| 5 sample | 0.24 | 0.17 | 0.21 | 0.14 | 0.3 | 0.23 |

Table 2. TDL-C 300ns, Es/IoT = -3, SNR of cell 2 is 6 dB lower compared with cell 2, 4 OFDM symbol

|  | OOK-1 RSRP error | OOK-1 RSRQ error | OOK-4 M=2 RSRP error | OOK-4 M=2 RSRQ error | OOK-4 M=4 RSRP error | OOK-4 M=4 RSRQ error |
| --- | --- | --- | --- | --- | --- | --- |
| 3 sample | 2.76 | 0.93 | 2.97 | 1.13 | 3.02 | 1.16 |
| 4 sample | 2.43 | 0.81 | 2.51 | 0.91 | 2.74 | 1.15 |
| 5 sample | 2.15 | 0.72 | 2.39 | 0.81 | 2.61 | 0.81 |

Table 3. AWGN, Es/IoT = -3, SNR of cell 2 is 6 dB lower compared with cell 2, 8 OFDM symbol

|  | OOK-1 RSRP error | OOK-1 RSRQ error | OOK-4 M=2 RSRP error | OOK-4 M=2 RSRQ error | OOK-4 M=4 RSRP error | OOK-4 M=4 RSRQ error |
| --- | --- | --- | --- | --- | --- | --- |
| 3 sample | 0.25 | 0.17 | 0.21 | 0.15 | 0.18 | 0.11 |
| 4 sample | 0.2 | 0.14 | 0.19 | 0.13 | 0.17 | 0.12 |
| 5 sample | 0.18 | 0.13 | 0.15 | 0.1 | 0.13 | 0.08 |

Table 4. TDL-C 300ns, Es/IoT = -3, SNR of cell 2 is 6 dB lower compared with cell 2, 8 OFDM symbol

|  | OOK-1 RSRP error | OOK-1 RSRQ error | OOK-4 M=2 RSRP error | OOK-4 M=2 RSRQ error | OOK-4 M=4 RSRP error | OOK-4 M=4 RSRQ error |
| --- | --- | --- | --- | --- | --- | --- |
| 3 sample | 2.72 | 0.92 | 2.86 | 1.09 | 3.19 | 1.45 |
| 4 sample | 2.37 | 0.81 | 2.43 | 0.87 | 2.55 | 1.22 |
| 5 sample | 2.15 | 0.73 | 2.17 | 0.75 | 2.41 | 1.04 |

Table 5. AWGN, Es/IoT = -3, SNR of cell 2 is 9 dB lower compared with cell 2, 4 OFDM symbol

|  | OOK-1 RSRP error | OOK-1 RSRQ error | OOK-4 M=2 RSRP error | OOK-4 M=2 RSRQ error | OOK-4 M=4 RSRP error | OOK-4 M=4 RSRQ error |
| --- | --- | --- | --- | --- | --- | --- |
| 3 sample | 0.25 | 0.15 | 0.31 | 0.21 | 0.29 | 0.18 |
| 4 sample | 0.23 | 0.14 | 0.28 | 0.2 | 0.22 | 0.13 |
| 5 sample | 0.21 | 0.12 | 0.21 | 0.13 | 0.19 | 0.11 |

Table 6. TDL-C 300ns, Es/IoT = -3, SNR of cell 2 is 9 dB lower compared with cell 2, 4 OFDM symbol

|  | OOK-1 RSRP error | OOK-1 RSRQ error | OOK-4 M=2 RSRP error | OOK-4 M=2 RSRQ error | OOK-4 M=4 RSRP error | OOK-4 M=4 RSRQ error |
| --- | --- | --- | --- | --- | --- | --- |
| 3 sample | 2.76 | 0.98 | 3.02 | 1.22 | 3.34 | 1.73 |
| 4 sample | 2.37 | 0.83 | 2.72 | 1.12 | 2.93 | 1.43 |
| 5 sample | 2.18 | 0.76 | 2.21 | 0.78 | 2.51 | 1.12 |

Table 7. AWGN, Es/IoT = -3, SNR of cell 2 is 9 dB lower compared with cell 2, 8 OFDM symbol

|  | OOK-1 RSRP error | OOK-1 RSRQ error | OOK-4 M=2 RSRP error | OOK-4 M=2 RSRQ error | OOK-4 M=4 RSRP error | OOK-4 M=4 RSRQ error |
| --- | --- | --- | --- | --- | --- | --- |
| 3 sample | 0.18 | 0.1 | 0.22 | 0.15 | 0.17 | 0.1 |
| 4 sample | 0.15 | 0.09 | 0.18 | 0.12 | 0.14 | 0.08 |
| 5 sample | 0.14 | 0.08 | 0.14 | 0.08 | 0.14 | 0.08 |

Table 8. TDL-C 300ns, Es/IoT = -3, SNR of cell 2 is 9 dB lower compared with cell 2, 8 OFDM symbol

|  | OOK-1 RSRP error | OOK-1 RSRQ error | OOK-4 M=2 RSRP error | OOK-4 M=2 RSRQ error | OOK-4 M=4 RSRP error | OOK-4 M=4 RSRQ error |
| --- | --- | --- | --- | --- | --- | --- |
| 3 sample | 2.78 | 0.98 | 3.06 | 2.03 | 2.99 | 1.26 |
| 4 sample | 2.4 | 0.86 | 3 | 1.55 | 2.49 | 0.93 |
| 5 sample | 2.18 | 0.78 | 2.43 | 1.09 | 2.4 | 1.03 |

Observation 1: For OOK based LP-WUR, in order to fulfilled the accuracy requirement 3.5dB for LP-RSRP and 3.5dB for LP-RSRQ, 3 samples are needed.

- Observation 2: Limited or no RRM accuracy gain is achieved by extending the LP-SS overhead from 4 OFDM symbol to 8 OFDM symbol.

- Conclusion

Based on the discussion above, the following observations are concluded.

Observation 1: For OOK based LP-WUR, in order to fulfilled the accuracy requirement 3.5dB for LP-RSRP and 3.5dB for LP-RSRQ, 3 samples are needed.

- Observation 2: Limited or no RRM accuracy gain is achieved by extending the LP-SS overhead from 4 OFDM symbol to 8 OFDM symbol.

- Reference

- R4-2416861, WF on RRM requirements for NR_LPWUS, Moderator (vivo)

- Annex.A

Simulation assumption

Note: Highlight part is updated base on the WF of last meeting

Table 1: General parameters

| Simulation parameters | Comments/values |
| --- | --- |
| Carrier frequency for Cell 1 and Cell 2 | 2.6 GHz |
| Prior knowledge of Cell 1 / Cell 2 by the UE | Interfering cell (Cell 2) is not known to UE |
| DRX | No applicable for LP-WUR |
| BS transmit antennas for LP-SS blocks | 1 Tx |
| UE receive antennas | 1 Rx |
| Data and control channel subcarrier spacing | Data, SSB and LP-SS have the same SCS |
| Subcarrier spacing | 30kHz |
| Measurement period (in number of measurement samples) | [3, 4, 5] |
| LP-SS/SSB measurement interval | LP-SS: 320 ms SSB: 320 ms |
| LP-SS BW | 132 subcarriers for SCS=30kHz for LP-SS initially |
| SSS | 30KHz for SSS |
| Actual LP-SS transmissions | always transmitted |
| Guard band | 1 RB on each side of LP-SS/LP-WUS signal |

Table 2: Cell-specific parameters

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| RF Channel number | - | Channel 1 | Channel 1 |
| NR-PSS, NR-SSS (OFDM based LP-WUR) | - | To be indicated by companies | To be indicated by companies |
| LP-SS |  | OOK-1; or OOK-4 with M = [2,4] | when Cell 1 uses OOK-1; OOK-1 or NR signal is used for Cell 2 when Cell 1 uses OOK-4, OOK-4 with same M value as cell 1 or NR signal is used for Cell 2 |
| LP-SS pattern |  | [ZC sequence] Note: Company can simulate one or all of them | [ZC sequence] Note: Company can simulate one or all of them |
| PBCH and DMRS power offset with respect to NR-PSS, NR-SSS and LP-SS | dB | 0 | 0 |
| Data and control PSD relative to NR-PSS,NR-SSS and LP-SS | dB | 0 | 0 |
| RB Utilization | % | 100 | 100 |
| Data Modulation | - | QPSK | QPSK |
| Slot length | - | 14 symbols | 14 symbols |
| CP Length | - | Normal | Normal |
| Frequency offset relative to UE frequency reference | Hz | OFDM based receiver [5] ppm OOK based receiver [10] ppm | N/A |
| Timing error |  | Residual timing error + timing drift (frequency offset* 320ms (reference signal periodicity) Residual timing error: T=5us for OOK-1 T=2us for OOK-4 with M=2 T=1us for OOK-4 with M=4 |  |
| 1)Relative Delay of 1st Path (synchronous) | µs | 0 | CP/2 |
| 2) Relative Delay of 1st Path (asynchronous): Fixed delay | ms | 0 | 3 ms |
| SNR | dB | SNR setting for serving and interference cell are derived based on agreement of Issue 2-1-1-1 |  |
|  |  | When Ês/Iot = -3 dB - When SNR of cell 2 is 9 dB lower compared with cell 2; SNR = [-2.7] - When SNR of cell 2 is 6 dB lower compared with cell 2 SNR = [-2.4] | When Ês/Iot = -3 dB - When SNR of cell 2 is 9 dB lower compared with cell 2; SNR = [-11.7] - When SNR of cell 2 is 6 dB lower compared with cell 2; SNR = [-8.4] |
|  |  | When Ês/Iot = -0.5 dB Note: Determine the SNR based on based on agreement of Issue 2-1-1-1 | When Ês/Iot = -0.5 dB Note: Determine the SNR based on based on agreement of Issue 2-1-1-1 |
|  |  | When Ês/Iot = 2 dB Note: Determine the SNR based on based on agreement of Issue 2-1-1-1 | When Ês/Iot = 2 dB Note: Determine the SNR based on based on agreement of Issue 2-1-1-1 |
| Ês/Iot | dB | -3; -0.5dB; 2dB | N/A |
| Propagation conditions | - | FR1: AWGN TDL-C 300ns |  |
| UE speed |  | 3 km/h |  |

Table 3: UE-specific parameters

| [Receiver Filter] | [3th/5th Order Butterworth with 3.96MHz bandwidth] |
| --- | --- |
| [Receiver ADC bit width] | [4 or 8-bitADC] |
| [Receiver Sampling Rate for LP-SS only] | [7.68MHz] |