3GPP TSG-RAN WG4 Meeting #113			R4-2418435

# Orlando, US, 18th – 22nd November, 2024

Agenda Item:	7.26.4.2

Source: 	CMCC

Title: 			   Discussion on the RRM requirement for NTN phase3

Document for:	Discussion

- Introduction

In last meeting, RAN4 achieved some general agreements for objectives of DL coverage enhancement and support of regenerative payload. The WF has been approved in [1]. In this contribution, we further discuss the RRM impact and requirements on these two objectives.

- Downlink coverage enhancements

Issue 1-1: SSB periodicity enhancement

Agreement:

- RAN4 to wait for RAN1’s definitive decision on the maximum periodicity of SSB and the range of SMTC before discussing whether and how to modify the relevant RRM requirements. FFS on the detailed list of RRM requirements to be affected, e.g.,

  - uplink timing requirements, RRM requirements during transmission between different SSB periodicities, default assumption on the SSB and SMTC periodicities in handover/satellite switch with re-sync/RRC re-establishment/RRC connection release with redirection, cell identification, measurements, etc.

For downlink coverage enhancements, following further agreement from RAN1 has been achieved in last meeting, we list here for information:

Agreement from RAN1 For NR NTN, support extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - The maximum of the additional default value (apart from the existing 20ms value) is at least 160ms. - FFS: whether 320ms can be supported as the maximum of the additional default value instead of or in addition to 160ms.

RAN1 agreed to consider extending the periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access, maximum of the additional default value (apart from the existing 20ms value) is at least 160ms so far. If 320ms will be finally supported by RAN1, RAN2 will extend the SMTC, MG and so on correspondingly.

If RAN1/2 will extend the SSB/SMTC/MG periodicity larger than 320ms (the smallest DRX cycle), then almost all RRM requirements will be impacted including IDLE/Inactive mode and connected mode, such as: cell (re)-selection, L3 measurement, L1 measurement and so on. While if the extended SSB/SMTC/MG periodicity will not larger than 320ms, we think most of current requirements can be reused.

Observation 1: Almost all RRM requirements will be impacted if RAN1/2 will extend the SSB/SMTC/MG periodicity larger than 320ms (the smallest DRX cycle).

Since the extension of the SSB periodicity is still under RAN1’s discussion, RAN4 can wait for more progress from RAN1/2.

Proposal 1: RAN4 to wait for more progress from RAN1/2 on SSB periodicity extension

- Support of regenerative payload

Issue 2-2: Mobility requirements

Agreement:

- For those requirements not applicable for regenerative payload NTN systems, if any, RAN4 to add an explicit statement to TS38.133 to clarify their applicability to regenerative payload NTN systems. FFS on the exact text and clause(s) for such clarifications.

According to the RAN plenary progress, satellite switching with re-sync over Regenerative Architecture will not be pursued in REL-19. Therefore, the requirement of satellite switching with re-sync will not be applied for NTN regenerative mode.

Proposal 2: The requirement of satellite switching with re-sync will not be applied for NTN regenerative mode.

- Conclusion

Based on the discussion above, the following observations and proposals are concluded.

Observation 1: Almost all RRM requirements will be impacted if RAN1/2 will extend the SSB/SMTC/MG periodicity larger than 320ms (the smallest DRX cycle).

Proposal 1: RAN4 to wait for more progress from RAN1/2 on SSB periodicity extension

Proposal 2: The requirement of satellite switching with re-sync will not be applied for NTN regenerative mode.

- Reference

- R4-2416866, WF on RRM requirements for NR_NTN_Ph3_Part2, Moderator (Qualcomm Incorporated)