# 3GPP TSG-RAN WG4 Meeting #113                                 R4-2418445

# Orlando, US, 18th – 22nd November, 2024

Agenda Item:	7.24.2.3

Source: 	CMCC

Title: 			 Discussion on conditional intra-CU LTM

Document for:	Discussion

- Introduction

In RAN #102 meeting, the new WID on NR mobility enhancements Phase 4 was approved [1]. In RAN #105 meeting, the revised WID was approved [2]. The detailed objectives are duplicated as following.

- Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support - Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure. - Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint - Specify RRM requirements related to the above objectives as necessary [RAN4]

In last meetin, there was initial discussion on measurements related enhancements for LTM, and a WF was agreed [3]. This contribution provides discussion on conditional intra-CU LTM.

- Discussion

Issue 1-1-1: RAN4 scope of Conditional LTM < Agreement> - RAN4 to define cell switch delay requirements for conditional LTM - RAN4 to define subsequent conditional LTM requirement unless there is any technical issue identified. - Wait for other WGs input to further discuss detailed RRM requirements.

It was agreed that RAN4 will define cell switch delay requirements for conditional LTM and subsequent conditional LTM. From delay requirements point of view, the main difference between these two cases is that the starting point is different, while other components of delay are same. One high level issue is whether to define delay requirements seperately or a unified requirements to cover both cases.

For legacy Conditional PSCell Addition/Change (CPA/C), Subsequent Conditional PSCell Addition/Change (SCPA/C), the requirements are defined seperately, since CPA/C and SCPA/C is introduced in different release. While for now, conditional LTM and subsequent conditional LTM are considered in the same release. It is possible to define a unified requirements to cover both cases. It is proposed to firstly decide whether cell switch delay requirements for conditional LTM and subsequent conditional LTM are specified seperately or a unified requirements are defined to cover both cases.

Proposal 1: it is proposed to discuss how to define cell switch delay requirements for conditional LTM and subsequent conditional LTM

- Alt1: define seperate cell switch delay requirements for conditional LTM and subsequent conditional LTM

- Alt2: define unified cell switch delay requirements to cover both conditional LTM and subsequent conditional LTM

2.1 Define seperate cell switch delay requirements for conditional LTM and subsequent conditional LTM

Rel-18 LTM cell switch delay can be used as baseline to discuss delay for conditional LTM. Rel-18 LTM cell switch delay DLTM is specified as the delay from the end of the last TTI containing the MAC-CE command for cell switch until the time the UE transmits the first UL message on the target cell.  DLTM = Tcmd + TLTM-interrupt. Tcmd equals to THARQ + 3ms, where THARQ is the timing between cell switch command and acknowledgement. TLTM-interrupt is interruption time, and TLTM-interrupt = TLTM-RRC-processing + TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU ms. In detail, TLTM-RRC-processing is the time for ASN.1 decoding and validity/compliance check for the RRC configuration of the LTM target cell indicated in the LTM cell switch command. TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change. Tfirst-RS is the time for fine time tracking and acquiring full timing information of the target cell. TRS-proc is the time for SSB processing. TLTM-IU is the interruption uncertainty during LTM cell switch which is related with whether RACH-based LTM cell switch or RACH-less LTM cell switch is in use.

If it is agreed to define seperate cell switch delay requirements for conditional LTM and subsequent conditional LTM, the detailed requriements for two cases will be discussed seperately as following.

For conditional LTM, there is no cell switch command, which means that it is not necessary to include Tcmd which equal to THARQ + 3ms, where THARQ is the timing between cell switch command and acknowledgement. Similar like CHO, CPAC, or SCPAC, UE will receive a RRC message implying conditional LTM. The starting point of conditional LTM is the time when UE receives RRC command triggering conditional LTM, similar as CHO, CPAC. The corresponding delay requirements include the component of TRRC_delay, which is the RRC processing delay. The ending point is the time the UE transmits the first UL message on the target cell.

For subsequent conditional LTM, if we follow the approach for subsequent CPAC, the starting point is the time when UE transmits RRCReconfigurationcomplete message for the previous cell switch. The ending point is the time the UE transmits the first UL message on the target cell.

Proposal 2: with the assumption that it is agreed to define seperate cell switch delay requirements for conditional LTM and subsequent conditional LTM:

- For conditional LTM, the starting point is the time when UE receives RRC command triggering conditional LTM. The ending point is the time the UE transmits the first UL message on the target cell.

- For subsequent conditional LTM, the starting point is the time when UE transmits RRCReconfigurationcomplete message for the previous cell switch. The ending point is the time the UE transmits the first UL message on the target cell.

In detail, the cell switch delay for conditionl LTM is proposed as DLTM_Conditional = TRRC_delay + TEvent_DU + Tmeasure + TLTM_execution + TLTM-interrupt. TRRC is the RRC procedure delay. TEvent_DU is the delay uncertainty which is the time from when UE successfully decodes a conditional LTM command until a condition exists at the measurement reference point which will trigger the conditional LTM. Tmeasure is measurements time. TLTM-interrupt is the interruption time as specified in Rel-18. TLTM_execution is the UE execution preparation time for conditional LTM, and starts after UE realizes the condition of conditional LTM is met and identity of the target cell is determined. TLTM_execution can be up to 10ms.

The cell switch delay for subsequent conditionl LTM is proposed as DLTM_Conditional_subsequent = TEvent_DU + Tmeasure + TLTM_execution + TLTM-interrupt. TEvent_DU is the delay uncertainty which is the time from when the UE successfully transmits an RRCReconfigurationcomplete message for the previous cell switch until a condition exists at the measurement reference point which will trigger the conditional LTM. Tmeasure is measurements time. TLTM-interrupt is the interruption time as specified in Rel-18. TLTM_execution is the UE execution preparation time for conditional LTM, and starts after UE realizes the condition of conditional LTM is met and identity of the target cell is determined. TLTM_execution can be up to 10ms.

Proposal 3: with the assumption that it is agreed to define seperate cell switch delay requirements for conditional LTM and subsequent conditional LTM:

- For conditional LTM, cell switch delay is proposed as DLTM_Conditional = TRRC_delay + TEvent_DU + Tmeasure + TLTM_execution + TLTM-interrupt.

- TRRC is the RRC procedure delay.

- TEvent_DU is the delay uncertainty which is the time from when UE successfully decodes a conditional LTM command until a condition exists at the measurement reference point which will trigger the conditional LTM.

- Tmeasure is measurements time.

- TLTM_execution is the UE execution preparation time for conditional LTM, and starts after UE realizes the condition of conditional LTM is met and identity of the target cell is determined. TLTM_execution can be up to 10ms.

- TLTM-interrupt is the interruption time as specified in Rel-18.

- For subsequent conditional LTM, cell switch delay is proposed as DLTM_Conditional_subsequent = TEvent_DU + Tmeasure + TLTM_execution + TLTM-interrupt.

- TEvent_DU is the delay uncertainty which is the time from when the UE successfully transmits an RRCReconfigurationcomplete message for the previous cell switch until a condition exists at the measurement reference point which will trigger the conditional LTM.

- Tmeasure is measurements time.

- TLTM_execution is the UE execution preparation time for conditional LTM, and starts after UE realizes the condition of conditional LTM is met and identity of the target cell is determined. TLTM_execution can be up to 10ms.

- TLTM-interrupt is the interruption time as specified in Rel-18.

2.1 Define unified cell switch delay requirements to cover both conditional LTM and subsequent conditional LTM

As discussed above, the main difference for cell switch delay between conditional LTM and subsequent conditional LTM is that the starting point is different. For conditional LTM, the starting point is the time when UE receives RRC command triggering conditional LTM. For subsequent conditional LTM, the starting point is the time when UE transmits RRCReconfigurationcomplete message for the previous cell switch. Considering that after receriving RRC command implying conditional LTM, UE need to transmit RRCReconfigurationcomplete message confirming the LTM configuration. In order to cover both cases, it is proposed that the starting point can be the time from when the UE successfully transmits RRCReconfigurationComplete message confirming the LTM configuration or the RRCReconfigurationcomplete message for the previous cell switch.

Proposal 4: with the assumption that it is agreed to define unified cell switch delay requirements to cover both conditional LTM and subsequent conditional LTM:

- the starting point to cover both conditional LTM and subsequent conditional LTM is proposed as the time when UE transmits an RRCReconfigurationComplete message.

- for conditional LTM, the transmitted RRCReconfigurationComplete message is for confirmation of the LTM configuration

- for subsequent conditional LTM, the transmitted RRCReconfigurationComplete message is for the previous cell switch.

- The ending point is the time the UE transmits the first UL message on the target cell.

The cell switch delay to cover both conditionl LTM and subsequent conditional LTM is proposed as DLTM_Conditional = DLTM_Conditional_subsequent = TEvent_DU + Tmeasure + TLTM_execution + TLTM-interrupt. TEvent_DU is the delay uncertainty which is the time from when UE transmits RRCReconfigurationComplete message confirming the LTM configuration or the RRCReconfigurationcomplete message for the previous cell switch until a condition exists at the measurement reference point which will trigger the conditional LTM. Tmeasure  is measurements time. TLTM-interrupt is the  interruption time as specified in Rel-18. TLTM_execution is the UE execution preparation time for conditional LTM, and starts after UE realizes the condition of conditional LTM is met and identity of the target cell is determined. TLTM_execution can be up to 10ms.

Proposal 5: with the assumption that it is agreed to define unified cell switch delay requirements to cover both conditional LTM and subsequent conditional LTM:

- The cell switch delay to cover both conditionl LTM and subsequent LTM is proposed as DLTM_Conditional = DLTM_Conditional_subsequent = TEvent_DU + Tmeasure + TLTM_execution + TLTM-interrupt.

- TEvent_DU is the delay uncertainty which is the time from when UE transmits RRCReconfigurationComplete message confirming the LTM configuration or the RRCReconfigurationComplete message for the previous cell switch until a condition exists at the measurement reference point which will trigger the conditional LTM

- Tmeasure is measurements time.

- TLTM-interrupt is the interruption time as specified in Rel-18.

- TLTM_execution is the UE execution preparation time for conditional LTM, and starts after UE realizes the condition of conditional LTM is met and identity of the target cell is determined

Issue 1-3-2: Early UL sync < Way Forward > FFS the following options: - Option 1: For UL-sync, we can consider some situations as follows, - Do not need to acquire the TA, i.e., the NTA of candidate cell(s) is/are same as serving cell or NTA = 0. - RAR-based TA acquisition. - UE based TA measurement. - Option 2: How/whether to support early RACH on candidate cells during conditional LTM. - Option 3: Whether updates are needed to the Rel-18 PDCCH ordered RACH delay requirements. - Option 4: If DL TODA-based UE autonomous TA estimation is found beneficial for the conditional LTM cell switch, RAN4 to define the requirements for DL TODA-based UE autonomous TA estimation. - Option 5: Wait for more progress from other WGs.

According to RAN2 agreements, UE based TA measurement mechanism is supported for conditional intra-CU LTM. Autonomous TA adjustment for LTM is introduced in Rel-18 LTM. However, no dedicated requirements for UE based TA adjustment are specified in R18 due to limited timeline. In order to guarantee the performance, it is necessary to define related RAN4 requirements, e.g.timing requirements, for UE based TA adjustment.

RAN2 Agreements on C-LTM in RAN2#127bis

- Source cell sends the conditional LTM configuration via RRCReconfiguration to UE, which includes the LTM candidate configurations, and the corresponding execution conditions.

- Event LTM3-like and LTM5-like are used as the conditional LTM execution condition. FFS on reuse of CHO conditions.

- Source cell and each candidate cell provides its own execution condition for conditional LTM.

- It is DU to generate the L1 execution condition. FFS on a case that L3 measurement is used.

- RACH-less Conditional intra-CU LTM is supported.

- RACH based conditional intra-CU LTM is supported.

- UE based TA measurement mechanism is supported for conditional intra-CU LTM.

- PDCCH ordered early TA acquisition is supported for conditional LTM.

- Rel-18 Early candidate TCI State activation/deactivation is supported for conditional intra-CU LTM.

- For RACH-less conditional LTM, CG-based first UL transmission on target cell is supported. FFS on DG-based approach.

- The LTM completion defined for Rel-18 intra-CU LTM is reused for conditional LTM.

Proposal 6: since RAN2 is agreed that UE based TA measurement mechanism is supported for conditional intra-CU LTM, it is proposed to define related RAN4 requirements, e.g.timing requirements, for UE based TA adjustment.

Issue 1-4-1: Whether cover both RACH-based and RACH-less conditional LTM < Way Forward > FFS the following options: - Option 1: Cover both RACH-based and RACH-less conditional LTM. - Option 2: FFS whether both RACH-less and RACH-based CLTM are supported by RAN1/2.

Since RAN2 has agreed that RACH-less Conditional intra-CU LTM and RACH based conditional intra-CU LTM are supported, it is necessary to cover both cases.

Proposal 7: it is proposed to cover both RACH-less conditional intra-CU LTM and RACH based conditional intra-CU LTM

- Conclusion

This contribution provides discussion on conditional LTM. The proposals are:

Proposal 1: it is proposed to discuss how to define cell switch delay requirements for conditional LTM and subsequent conditional LTM

- Alt1: define seperate cell switch delay requirements for conditional LTM and subsequent conditional LTM

- Alt2: define unified cell switch delay requirements to cover both conditional LTM and subsequent conditional LTM

Proposal 2: with the assumption that it is agreed to define seperate cell switch delay requirements for conditional LTM and subsequent conditional LTM:

- For conditional LTM, the starting point is the time when UE receives RRC command triggering conditional LTM. The ending point is the time the UE transmits the first UL message on the target cell.

- For subsequent conditional LTM, the starting point is the time when UE transmits RRCReconfigurationcomplete message for the previous cell switch. The ending point is the time the UE transmits the first UL message on the target cell.

Proposal 3: with the assumption that it is agreed to define seperate cell switch delay requirements for conditional LTM and subsequent conditional LTM:

- For conditional LTM, cell switch delay is proposed as DLTM_Conditional = TRRC_delay + TEvent_DU + Tmeasure + TLTM_execution + TLTM-interrupt.

- TRRC is the RRC procedure delay.

- TEvent_DU is the delay uncertainty which is the time from when UE successfully decodes a conditional LTM command until a condition exists at the measurement reference point which will trigger the conditional LTM.

- Tmeasure is measurements time.

- TLTM_execution is the UE execution preparation time for conditional LTM, and starts after UE realizes the condition of conditional LTM is met and identity of the target cell is determined. TLTM_execution can be up to 10ms.

- TLTM-interrupt is the interruption time as specified in Rel-18.

- For subsequent conditional LTM, cell switch delay is proposed as DLTM_Conditional_subsequent = TEvent_DU + Tmeasure + TLTM_execution + TLTM-interrupt.

- TEvent_DU is the delay uncertainty which is the time from when the UE successfully transmits an RRCReconfigurationcomplete message for the previous cell switch until a condition exists at the measurement reference point which will trigger the conditional LTM.

- Tmeasure is measurements time.

- TLTM_execution is the UE execution preparation time for conditional LTM, and starts after UE realizes the condition of conditional LTM is met and identity of the target cell is determined. TLTM_execution can be up to 10ms.

- TLTM-interrupt is the interruption time as specified in Rel-18.

Proposal 4: with the assumption that it is agreed to define unified cell switch delay requirements to cover both conditional LTM and subsequent conditional LTM:

- the starting point to cover both conditional LTM and subsequent conditional LTM is proposed as the time when UE transmits an RRCReconfigurationComplete message.

- for conditional LTM, the transmitted RRCReconfigurationComplete message is for confirmation of the LTM configuration

- for subsequent conditional LTM, the transmitted RRCReconfigurationComplete message is for the previous cell switch.

- The ending point is the time the UE transmits the first UL message on the target cell.

Proposal 5: with the assumption that it is agreed to define unified cell switch delay requirements to cover both conditional LTM and subsequent conditional LTM:

- The cell switch delay to cover both conditionl LTM and subsequent LTM is proposed as DLTM_Conditional = DLTM_Conditional_subsequent = TEvent_DU + Tmeasure + TLTM_execution + TLTM-interrupt.

- TEvent_DU is the delay uncertainty which is the time from when UE transmits RRCReconfigurationComplete message confirming the LTM configuration or the RRCReconfigurationComplete message for the previous cell switch until a condition exists at the measurement reference point which will trigger the conditional LTM

- Tmeasure is measurements time.

- TLTM-interrupt is the interruption time as specified in Rel-18.

- TLTM_execution is the UE execution preparation time for conditional LTM, and starts after UE realizes the condition of conditional LTM is met and identity of the target cell is determined

Proposal 6: since RAN2 is agreed that UE based TA measurement mechanism is supported for conditional intra-CU LTM, it is proposed to define related RAN4 requirements, e.g.timing requirements, for UE based TA adjustment.

Proposal 7: it is proposed to cover both RACH-less conditional intra-CU LTM and RACH based conditional intra-CU LTM

- Reference

- RP-234036, New WID: NR mobility enhancements Phase 4

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4

- R4-2416863, WF on RRM requirements for NR_Mob_Ph4_Part2, China Telecom

- R4-2414060, WF on R19 Further NR mobility enhancement, Apple

- RP-233970, Revised WID on Further NR Mobility Enhancements

- R2-2404102, Report of 3GPP TSG RAN WG2 meeting #125bis

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4