# 3GPP TSG-RAN WG4 Meeting #113                                R4-2418454

# Orlando, US, 18th - 22nd November, 2024

Agenda Item:	7.17.3.1

Source: 	CMCC

Title: 			 Discussion on testability and interoperability issues for CSI prediction

Document for:	Discussion

- Introduction

In RAN #102 meeting, the WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [1]. In RAN #105 meeting, the revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [2]. The objectives for RAN4 are duplicated as following.

- Core requirements for the above two use cases for AI/ML LCM procedures and UE features [RAN4]: - Specify necessary RAN4 core requirements for the above three use cases. This includes the test setup and methodology for the beam management use case. - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring. - Testability and interoperability [RAN4]: - Finalize the testing framework and procedure for one-sided models and further analyse the various testing options for two-sided models, in collaboration with RAN1, and including at least: - Relation to legacy requirements - Performance monitoring and LCM aspects considering use-case specifics - Generalization aspects - Static/non-static scenarios/conditions and propagation conditions for testing (e.g., CDL, field data, etc.) - UE processing capability and limitations - Post-deployment validation due to model change/drift

In last meeting, there is discussion on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, and a WF was approved [3]. This contribution provides discussion on testability and interoperability issues for  CSI prediction.

- Discussion

Issue 4-7: CSI prediction tests - Proposals - Option 1: Use TDL model for testing and static condition (similar to legacy test) - Option 2: Define test requirements relative to performance with random PMI Type codebook - Option 3: RAN4 to consider Spatially Correlated Channel models (SCM) for the requirements/test on AIML CSI prediction. - Option 4: Define test requirements relative to performance with eTypeII Codebook - Option 5: others

According to the revised WID [2], CSI prediction (one-sided model) is one of the objectives for WI, and RAN4 need to specify necessary RAN4 core requirements. In RAN4#110 meeting, it was agreed that thoughput should be the default metric for inference. In last meeting, it was agreed to use relative throughput as requirement metric for CSI prediction. As for what is the baseline that throughput would be compared to, our consideration is to follow the throughput related requirements for legacy PMI reporting. The legacy requirements of PMI reporting are defined based on the precoding gain, expressed as the relative increase in throughput when the transmitter is configured according to the UE reported PMI compared to the case when the transmitter is using random precoding, respectively. Similarly, for CSI feedback, the throughput is the throughput gain achieved with predicted PMI compared to random PMI.

Proposal 1: for CSI prediction accuracy metrics for inference, the relative throughput is the throughput gain achieved with predicted PMI compared to random PMI.

Inference performance is eventully performance, which is impacted not only by the AI/ML model performance but may also be impacted by other factors. While model monitoring targets to evaluate whether a model works well under a centain configuation/scenario. Inference performance is good does not mean monitoring performance is good. From this point of view, intermediate KPI, at least cosine similarity (SGCS), ranging within [0, 1], can be considered as test metrics for LCM.

Proposal 2: for CSI prediction, it is proposed to use intermediate KPI, e.g. SGCS, as requirements/tests metrics for LCM.

- Conclusion

This contribution provides discussion on testability and interoperability issues for CSI prediction. The proposals are:

Proposal 1: for CSI prediction accuracy metrics for inference, the relative throughput is the throughput gain achieved with predicted PMI compared to random PMI.

Proposal 2: for CSI prediction, it is proposed to use intermediate KPI, e.g. SGCS, as requirements/tests metrics for LCM.

- Reference

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- RP-242399, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- R4-2417212, WF on requirements for AI/ML air interface, Qualcomm Incorporated

- R4-2414323, WF on AI/ML, Qualcomm Incorporated

- R4-2410570, WF on AI/ML, Qualcomm Incorporated

- R4-2406617, WF on AI/ML, Qualcomm Incorporated

[5] 3GPP TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface